3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119380

# Electronic Meeting, 1 – 12 November 2021

Agenda item:	9.1.1.1

Source:	Keysight Technologies, Rohde & Schwarz

Title:	Preliminary MU assessment of CFFDNF and CFFNF methodologies

Document for:	Approval

# Introduction

This contribution intends to complete the preliminary MU assessment for methods/elements related to CFFDNF and CFFNF. A revision of this contribution will include the draft text/text proposal to [1].

# MU Descriptions

The MU contributions for CFFNF and CFFDNF have been captured in Annex B of [1], i.e.,

Table B.1.1.1-1: Uncertainty contributions for CFFNF system UID; Description of uncertainty contribution; Details in annex. Measurement stage. 1; DUT antenna location estimation; TBD. 2; Probe antenna pattern; TBD. 3; EIRP measurement error; TBD. 4; Near-field interaction between probe antenna and DUT antenna; TBD. 5; Influence of power measurement uncertainty; TBD. Table B.1.1.1-2: Uncertainty contributions for CFFDNF system UID; Description of uncertainty contribution; Details in annex. Measurement stage. 1; DUT antenna location estimation; TBD. 2; Probe antenna pattern; TBD. 3; EIRP measurement error; TBD. 4; TRP measurement error; TBD. 5; Near-field interaction between probe antenna and DUT antenna; TBD.

It is proposed to use the following MU descriptions for these MU elements.

##### DUT antenna location estimation

Description: This contribution originates from estimating the phase centre of the active antenna array and thus declaring the phase centre offset incorrectly in the vendor declaration for the black&white-box approach. The factors included in this uncertainty are the differences in path losses between the declare and the actual offsets and the difference in compensated probe pattern. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to CFFDNF&CFFNF methodologies with the black&white-box approach.

Additional background: This MU element was analysed in Clause 5.1.4.9.

##### Probe antenna pattern

Description: The probe pattern is assumed to be known so that the measurement in the near field can be corrected when the probe pattern needs to be compensated. There is no direct dependence between the UE pattern and the probe pattern in near field measurements. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF&CFFNF methodologies.

Additional background: This MU element is similar to the ‘A3-14 Probe pattern knowledge’ of the Near Field Test Range in [2] and should be considered 0 since the probe pattern can readily be measured.

##### EIRP measurement error in NF

Description: The cause of this uncertainty contributor is due to the reduction of distance between the measurement antenna and the DUT from Far-Field to Near-Field. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF&CFFNF methodologies. This MU element, if a non-insignificant mean error was identified, shall also be listed under System Uncertainties.

Additional background: This MU element was analysed in Clauses 5.1.4.4, 5.1.4.5, and 5.1.4.6.

##### TRP measurement error in NF

Description: This contribution originates from calculating the Total Radiated Power based on EIRP measurements in the NF. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF methodology.

Additional background: This MU element was analysed in Clause 5.1.4.4.

##### Near-field interaction between probe antenna and DUT antenna

Description: This uncertainty term is related to the coupling between the UE antenna and the measurement antenna in the near field at different distances. This uncertainty can be determined by multiple measurements of the DUT when at different distance from the probe. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFDNF&CFFNF methodologies.

Additional background: This MU element is similar to the ‘A3-15 Multiple Reflections’ MU element of the Near Field Test Range in [2] and can be assumed to be 0 but needs to be evaluated for each system separately.

##### Influence of power measurement uncertainty

Description: This contribution originates from relative power measurement uncertainties at different radii when applied to the asymptotic expansion transformation. This uncertainty is assumed to have a Gaussian distribution. This MU element is applicable to the CFFNF methodology.

Additional background: This MU element was analysed in Clause 5.1.4.7.

Proposal 1: Adopt the above MU descriptions in [1]

# MU Assessment for CFFNF

The following MU assessments are proposed for CFFNF to be captured in [1], i.e.,

Table B.1.1.3-1: Uncertainty contributions for CFFNF system for PC3 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 (Note 1) |  |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.24 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error | 0.09 | 5.1.4.6 (Table 5.1.4.6-4) |
| Note: The range length for CFFNF with black-box approach is reported for radius r3>r2>r1 and r3=r2+1 cm=r1+2 cm=22 cm. Note 1: Black-box does not require the declaration of the antenna offset Note 2: Probe antenna pattern must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

Table B.1.1.3-2: Uncertainty contributions for CFFNF system for PC1 UEs utilizing the black-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | 0 (Note 1) |  |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.53 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error | 0.16 (Note 2) | 5.1.4.6 (Table 5.1.4.6-4) |
| Note: The range length for CFFNF with black-box approach is reported for radius r3>r2>r1 and r3=r2+1 cm=r1+2 cm=32 cm. Note 1: Black-box does not require the declaration of the antenna offset Note 2: Probe antenna pattern must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

Table B.1.1.3-3: Uncertainty contributions for CFFNF system for PC3 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.9 |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.02 (Note 2) | 5.1.4.5 (Table 5.1.4.5-3) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| 5 | Influence of power measurement uncertainty | 0.37 | 5.1.4.7 |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1 and r2=r1+2 cm=22 cm. Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated. |  |  |  |

Table B.1.1.3-4: Uncertainty contributions for CFFNF system for PC1 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.9 |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.04 (Note 2) | 5.1.4.5 (Table 5.1.4.5-4) |
| 4 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| 5 | Influence of power measurement uncertainty | FFS (Note 3) | 5.1.4.7 |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error | 0.12 (Note 2) | 5.1.4.5 (Table 5.1.4.5-4) |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r2>r1 and r2=r1+2 cm=32 cm. Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated. Note 3: MU can be finalized in RAN5 based on the analysis framework in Clause 5.1.4.7. |  |  |  |

# MU Assessment for CFFDNF

The following MU assessments are proposed for CFFDNF to be captured in [1], i.e.,

Table B.1.1.3-5: Uncertainty contributions for CFFDNF system for PC3 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.9 |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.02 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| 4 | TRP measurement error | 0.04 (Note 3) | 5.1.4.4 (Table 5.1.4.4-9) |
| 5 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r=35 cm (EIRP) and r=20 cm (TRP). Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated Note 3: Min. range lengths, need for offset compensation, and measurement grids defined in Clause 5.1.4.4 |  |  |  |

Table B.1.1.3-6: Uncertainty contributions for CFFDNF system for PC1 UEs utilizing the black&white-box approach

| UID | Description of uncertainty contribution | Uncertainty Value | Details in Clause |
| --- | --- | --- | --- |
| Measurement stage |  |  |  |
| 1 | DUT antenna location estimation | FFS (Note 1) | 5.1.4.9 |
| 2 | Probe antenna pattern | 0 |  |
| 3 | EIRP measurement error | 0.05 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| 4 | TRP measurement error | 0.02 (Note 3) | 5.1.4.4 (Table 5.1.4.4-10) |
| 5 | Near-field interaction between probe antenna and DUT antenna | 0 |  |
| Systematic Uncertainties |  |  |  |
| 6 | EIRP measurement error | 0.45 (Note 2) | 5.1.4.4 (Table 5.1.4.4-8) |
| Note: The range length for CFFNF with black&white-box approach is reported for radius r=45 cm (EIRP) and r=20 cm (TRP). Note 1: Pending OEM feedback on typical offset estimation error. Analyses in Clause 5.1.4.9 Note 2: Probe antenna pattern and array offset must be compensated Note 3: Min. range lengths, need for offset compensation, and measurement grids defined in Clause 5.1.4.4 |  |  |  |

Proposal 2: Adopt the above preliminary MUs in [1]

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Adopt the above MU descriptions in [1]

Proposal 2: Adopt the above preliminary MUs in [1]

# References

- TR 38.884, Study on enhanced test methods for FR2 NR UEs, V1.1.0 (2021-08)

- TR 37.941, Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements, V16.3.0 (2021-09)

<<< START OF CHANGES >>>

<<< START OF CHANGE >>>

Annex B:
Measurement uncertainty

# B.1	Measurement uncertainty budget for UE RF testing methodology

Editor’s note: collect the MU elements which are impacted by the enhancements in Clauses 5 and 6 in this clause; if impact on the MU budget of the RRM and/or demodulation setups is identified, the corresponding clauses can be added. Organize the Annex to mirror the TR38.810 structure

## B.1.1	High DL power and low UL power

Editor’s note: the conclusion of MU impacts of the enhanced test methods (i.e. direct Near Field (DNF), Combined Far-Field/Direct Near Field (CFFDNF), and Combined Far-Field/Near Field (CFFNF)) should be captured.

### B.1.1.1	Uncertainty Contributions

This section covers the additional MU elements to DFF for different enhanced test systems in the following tables.

Table B.1.1.1-1: Uncertainty contributions for CFFNF system

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Measurement stage |  |  |
| 1 | DUT antenna location estimation | TBD |
| 2 | Probe antenna pattern | TBD |
| 3 | EIRP measurement error | TBD |
| 4 | Near-field interaction between probe antenna and DUT antenna | TBD |
| 5 | Influence of power measurement uncertainty | TBD |

Table B.1.1.1-2: Uncertainty contributions for CFFDNF system

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Measurement stage |  |  |
| 1 | DUT antenna location estimation | TBD |
| 2 | Probe antenna pattern | TBD |
| 3 | EIRP measurement error | TBD |
| 4 | TRP measurement error | TBD |
| 5 | Near-field interaction between probe antenna and DUT antenna | TBD |

### B.1.1.2	Uncertainty Contributions descriptions

FFS

### B.1.1.3	Uncertainty assessment

FFS

<<< END OF CHANGES >>>