3GPP TSG-RAN WG4 #101-e	R4-2119403

Electronic Meeting, November 01-12, 2021

Agenda item:	8.12.2.1.1

Source:	MediaTek Inc.

Title:	Discussion on the PDSCH requirements for scenarios with inter-cell inter-user interference

Document for:	Discussion

- Introduction

In RAN4#100-e, a way forward for PDSCH demodulation requirements for inter-cell interference MMSE-IRC was agreed [1]. In this contribution, we present simulation results based on the agreed simulation assumptions and provide our views for the remaining issues.

- Discussion

SSB configuration

There might be performance degradation of PBCH under the scenario of inter-cell interference. Also, as we are now focusing on the PDSCH requirement, we suggest to set serving cell SSB and interference cell(s) SSB(s) in the different time/frequency resources. If it is agreed to set SSB(s) of serving cell and interference cell(s)) in the same time/frequency resources, the configuration should be properly chosen to avoid the case that the performance of SSB is the bottleneck.

Proposal 1: Suggest to set serving cell SSB and interference cell(s) SSB(s) in the different time/frequency resources. If it is agreed to set SSB(s) of serving cell and interference cell(s)) in the same time/frequency resources, the configuration should be properly chosen to avoid the case that the performance of SSB is the bottleneck.

Initial simulation results

In Appendix A, we provide initial simulation results for MMSE-IRC and MMSE-MRC receiver considering different INR values, MCS, antenna configuration and propagation channel Based on the simulation assumptions in Appendix B.

Table 1. Summary for the simulation results

| Duplex mode | Channel model | Number of UE Rx antenna | INR1 | INR2 | MCS | SNR for 70% of Max T-put |  | Gain | Corresponding SINR for SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE-MRC |  |  |
| FDD | TDLA | 2 Rx | 5.43 | -1.50 | MCS 4 | 3.10 | 5.01 | 1.91 | -4.06 |
|  |  |  |  |  | MCS 13 | 10.61 | 12.28 | 1.67 | 3.45 |
|  |  |  | 3.10 | N/A | MCS 4 | 1.68 | 2.94 | 1.26 | -3.15 |
|  |  |  |  |  | MCS 13 | 9.15 | 10.23 | 1.08 | 4.32 |
|  |  |  | 7.77 | 2.29 | MCS 4 | 4.37 | 7.04 | 2.67 | -5.01 |
|  |  |  |  |  | MCS 13 | 11.91 | 14.27 | 2.36 | 2.53 |
|  |  |  | 13.91 | 3.34 | MCS 4 | 6.68 | 11.60 | 4.92 | -7.75 |
|  |  |  |  |  | MCS 13 | 14.30 | 18.81 | 4.51 | -0.13 |
|  |  | 4 Rx | 5.43 | -1.50 | MCS 4 | -1.64 | 1.86 | 3.5 | -8.80 |
|  |  |  |  |  | MCS 13 | 5.54 | 8.57 | 3.03 | -1.62 |
|  |  |  | 3.10 | N/A | MCS 4 | -2.30 | -0.22 | 2.08 | -7.13 |
|  |  |  |  |  | MCS 13 | 4.90 | 6.62 | 1.72 | 0.07 |
|  |  |  | 7.77 | 2.29 | MCS 4 | -1.24 | 3.85 | 5.09 | -10.62 |
|  |  |  |  |  | MCS 13 | 5.90 | 10.48 | 4.58 | -3.48 |
|  |  |  | 13.91 | 3.34 | MCS 4 | -0.42 | 8.35 | 8.77 | -14.85 |
|  |  |  |  |  | MCS 13 | 6.75 | 14.84 | 8.09 | -7.68 |
|  | TDLC | 2 Rx | 5.43 | -1.50 | MCS 4 | 3.82 | 5.52 | 1.70 | -3.34 |
|  |  |  |  |  | MCS 13 | 11.79 | 13.30 | 1.51 | 4.63 |
|  |  |  | 3.10 | N/A | MCS 4 | 2.29 | 3.44 | 1.15 | -2.54 |
|  |  |  |  |  | MCS 13 | 10.12 | 11.18 | 1.06 | 5.29 |
|  |  |  | 7.77 | 2.29 | MCS 4 | 5.24 | 7.57 | 2.33 | -4.41 |
|  |  |  |  |  | MCS 13 | 13.31 | 15.34 | 2.04 | 3.93 |
|  |  |  | 13.91 | 3.34 | MCS 4 | 8.00 | 12.18 | 4.18 | -6.43 |
|  |  |  |  |  | MCS 13 | 16.63 | 20.03 | 3.40 | 2.20 |  |
|  |  | 4 Rx | 5.43 | -1.50 | MCS 4 | -0.69 | 2.52 | 3.21 | -7.85 |  |
|  |  |  |  |  | MCS 13 | 7.05 | 9.73 | 2.68 | -0.11 |  |
|  |  |  | 3.10 | N/A | MCS 4 | -1.62 | 0.41 | 2.03 | -6.45 |  |
|  |  |  |  |  | MCS 13 | 5.88 | 7.58 | 1.70 | 1.05 |  |
|  |  |  | 7.77 | 2.29 | MCS 4 | 0.04 | 4.53 | 4.49 | -9.34 |  |
|  |  |  |  |  | MCS 13 | 7.86 | 11.74 | 3.88 | -1.52 |  |
|  |  |  | 13.91 | 3.34 | MCS 4 | 1.66 | 9.12 | 7.46 | -12.77 |  |
|  |  |  |  |  | MCS 13 | 9.49 | 16.38 | 6.89 | -4.94 |  |

According the simulation results, performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-IRC can be observed in all cases.

Observation 1: From the simulation results, the gain of MMSE-IRC over MMSE-IRC is larger than 1dB for all cases.

To determine the configurations used for test cases, the criterion SINR < -6dB can be applied first as there are lots of cases with SINR < -6dB. After filtering out the cases with SINR > -6dB, we can consider the relationship between SNR and INR, e.g., choose the cases for which SNR > INR for further down selection.

Observation 2: From the simulation results, the corresponding SINR for “SNR for 70% of Max T-put” are smaller than -6 dB in many cases.

Proposal 2: Suggest to apply SINR < -6dB as the first criterion for down selection.

- Conclusion

The observations and proposals are summarized as below:

Proposal 1: Suggest to set serving cell SSB and interference cell(s) SSB(s) in the different time/frequency resources. If it is agreed to set SSB(s) of serving cell and interference cell(s)) in the same time/frequency resources, the configuration should be properly chosen to avoid the case that the performance of SSB is the bottleneck.

Observation 1: From the simulation results, the gain of MMSE-IRC over MMSE-IRC is larger than 1dB for all cases.

Observation 2: From the simulation results, the corresponding SINR for “SNR for 70% of Max T-put” are smaller than -6 dB in many cases.

Proposal 2: Suggest to apply SINR < -6dB as the first criterion for down selection.

- Reference

- R4-2115730, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel

Appendix A

| Figure 1. Simulation results for INRs 5.43 dB and -1.50 dB |  |  |
| --- | --- | --- |
| Figure 2. Simulation results for INR 3.1 dB |  |  |

| Figure 3. Simulation results for INRs 7.77 dB and 2.29 dB |  |
| --- | --- |

| Figure 4. Simulation results for INRs 13.91 dB and 3.34 dB |  |
| --- | --- |

Appendix B

Table 1. Simulation assumptions

| Parameters | Serving cell FDD | Interferer cell(s) |
| --- | --- | --- |
| DIP values | - INRs 5.43 and -1.50 dB in case of 2 interference cells - INR 3.1 dB in case of 1 interference cell - INRs 7.77 and 2.29 dB in case of 2 interference cells - INRs 13.91 and 3.34 dB in case of 2 interference cells |  |
| PDSCH allocation in frequency domain | Full PRB |  |
| Physical cell ID | 0 | 1 or 2 (depending on the number of interferer cells) |
| Channel bandwidth | 10MHz | 10MHz |
| SCS | 15kHz | 15kHz |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH |
| SSB configuration | Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources |  |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port FDD: Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 | Same as Serving cell |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ | Type A, Start symbol 2, Duration 12​ |
| PRB bundling size for target PDSCH | 2 | 2 |
| PDSCH DMRS configuration | Type 1, 1+1 | Type 1, 1+1 |
| MCS | MCS 4 (QPSK, CR=0.3) MCS 13 (16QAM, CR=0.5) | 16QAM randomly modulated symbols |
| Rank | 1 | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| PDSCH precoder for both serving cell and interferer cells | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2 |  |
| Antenna configuration | 2T2R, 2T4R |  |
| Antenna Correlation | ULA Low |  |
| Propagation condition | TDLA30-10, TDLC300-100 |  |
| Number of HARQ Processes | 4 | N/A |
| Maximum HARQ transmission | 4 | N/A |