3GPP TSG-RAN WG4 #101-e	R4-2119425

November 1st ‒ 12th, 2021

Electronic Meeting

Agenda item:	10.8.3

Source: 	Qualcomm Incorporated, Puloli

Title: 	UE coexistence and emission requirements for 700 MHz NB-IoT

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new NB-IoT band in the upper 700 MHz frequency block for the US.  One of the objectives is to study the need for any applicable co-location and/or co-existence requirements.  This contribution discusses the UE coexistence requirements for this band.  Additionally, US FCC emission requirements are also studied and a mitigation to introduce a 100 kHz guard band is suggested similar to what has been agreed for other NB-IoT bands operating in the US.

# Discussion

A new E-UTRA band for NB-IoT has been proposed [1] with frequency arrangment illustrated in the following diagram copied from Figure 3 of [1].

Figure 1.  Frequency arrangement of new band.  Copied from Figure 3 of [1].

## UE coexistence

While exceptions exist, the specification in 36.101 conventionally defines a -50 dBm/MHz UE coexistence requirements from the uplink of any 3GPP band into the downlink of another 3GPP band.  If this convention were to be followed, we would expect that a requirement would be imposed on the transmitter in this new band to limit its emissions into another other 3GPP band to -50 dBm/MHz.  Similarly, other 3GPP bands should limit their emissions into this new band’s downlink frequency range to -50 dBm/MHz.

However, it is suggested in this contribution that the conventional approach of -50 dBm/MHz should not be blindly applied to this band in either direction.  Firstly, the new band is applicable only to NB-IoT.  It cannot support an LTE carrier.  Hence, the -50 dBm/MHz derived for LTE which itself was originally derived for WCDMA of -60 dBm/3.84 MHz is not necessarily appropriate to protect NB-IoT.  The assumptions used in the original derivations were based on handset usage which is not the anticipated operation for NB-IoT.  Moreover, the operating bandwidth of NB-IoT is 200 kHz rather than 1 MHz.  Therefore, -50 dBm/MHz should not be blindly applied.

Furthermore, a review of the regulatory requirements from the FCC part 27.53 governing operation in this spectrum does not reveal any such emission requirement of -50 dBm/MHz.  For example, the CFR states that operation in the 746 – 758 MHz band and the 776 – 788 MHz band, the emissions on any frequency outside 746 – 758 MHz and outside of 776 – 788 MHz is attenuated by at least 43 + 10 log (P).  Moreover, within the first 100 kHz a measurement bandwidth of 30 kHz may be used.  This amounts to an emission requirement of -13 dBm with a measurement bandwidth of 30 kHz.  The requirement on frequencies between 763 – 775 MHz and 793 – 805 MHz shall be attenuated by 65 + 10 log (P) for mobile and portable stations.  This equates to a -35 dBm/6.25 kHz requirement.  Thus, there are no regulatory requirements justifying -50 dBm/MHz.

We list the following options for consideration

- Specify UE coexistence emission requirements of -50 dBm/MHz for the UE both to protect the new NB-IoT band as well as to protect other E-UTRA bands.

- Specify -57 dBm/200 kHz requirement (bandwidth scaling from -50 dBm/MHz) from other bands to protect NB-IoT, and -50 dBm/MHz from NB-IoT to protect other bands.

- Study the appropriate protection level for NB-IoT by means of coexistence simulations.  Not preferred and potential impact to existing UE’s

- Do not specify any UE coexistence requirements in 3GPP for this band, but instead focus only on the FCC emission requirements as needed.

UE coexistence requirements are more likely to impact legacy E-UTRA devices than the new NB-IoT devices.  Since NB-IoT operates in a narrow channel of 200 kHz, the emissions are not far reaching.  On the other hand, the imposition of a new -50 dBm/MHz or -57 dBm/MHz requirement on legacy UE’s could be problematic.  Yet, there is no regulatory requirement to justify the new requirement.  A coexistence study to derive new emissions requirements is also not preferred since it will be time consuming potentially causing the band to fall outside of the Rel-17 timeline and yet be of limited value.  The new emission requirement will still impose additional requirements on legacy UE’s.  Given that there are no regulatory requirements necessitating UE coexistence, the potential impact to legacy UE’s if a new 3GPP requirement is imposed, there appears to be limited value to introduce these requirements into the 3GPP specifications.  It is therefore proposed that option 4 is adopted.

Proposal:  Do not specify any UE coexistence requirements in 3GPP for this band, but instead focus only on the FCC emission requirements as needed.

## Additional spurious emissions

To address the additional spurious emission requirement of -13 dBm/30 kHz outside of 788 MHz, the current spectrum emission mask requirement is reviewed.  The spectrum emission mask for an NB-IoT device is shown below

Table 6.6.2F.1-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

It can be seen that emission mask will not conform to the FCC requirement on each side of the operator’s block.  In fact, a similar problem has already been studied for other NB-IoT bands in [2].  The result was an agreement in [3] that has been applied to US NB-IoT bands to introduce NS_04 signaling which points to operating band table for US NB-IoT bands where a 100 kHz guard band is introduced.  For this band, the result is effectively truncating the band from 1 MHz to 800 kHz.

Proposal:  Apply NS_04 for this the new NB-IoT 700 MHz band.

Proposal:  Assign band number 100 for this band.

Table 5.5F-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850.1 MHz | – | 1909.9 MHz | 1930.1 MHz | – | 1989.9 MHz | FDD |
| 4 | 1710.1 MHz | – | 1754.9 MHz | 2110.1 MHz | – | 2154.9 MHz | FDD |
| 5 | 824.1 MHz | – | 848.9 MHz | 869.1 MHz | – | 893.9MHz | FDD |
| 12 | 699 MHz | – | 715.9 MHz | 729 MHz | – | 745.9 MHz | FDD |
| 13 | 777 MHz | – | 786.9 MHz | 746 MHz | – | 755.9 MHz | FDD |
| 17 | 704.1 MHz | – | 715.9 MHz | 734.1 MHz | – | 745.9 MHz | FDD |
| 25 | 1850.1 MHz | – | 1914.9 MHz | 1930.1 MHz | – | 1994.9 MHz | FDD |
| 26 | 814.1 MHz | – | 848.9 MHz | 859.1 MHz | – | 893.9 MHz | FDD |
| 66 | 1710.1 MHz | – | 1779.9 MHz | 2110.1 MHz | – | 2199.0 MHz | FDD |
| [100] |  | 787.1 MHz |  | – |  | 787.9 MHz |  | 757.1 MHz |  | – |  | 757.9 MHz |  | FDD |  |

# Conclusion

The UE coexistence requirements between a new NB-IoT band and existing legacy bands in the 700 MHz frequency range for the US has been discussed.  The conventional approach to defining UE coexistence emission requirements was reviewed with possible applicability to the new band.  Regulatory requirements were also reviewed in search of justification for the conventional 3GPP-defined emission requirements.  No quantitative justification was found.  Moreover, it was observed that a UE coexistence requirement if imposed now might negatively impact existing legacy LTE devices already operating in the 700 MHz bands – i.e., Bands 12, 13, 14, and 17 – yet provide limited value.  It is therefore proposed not to defined 3GPP based UE coexistence requirements due to the introduction of this new NB-IoT band, but rather to focus only on the FCC emission requirements and any necessary accommodations such as truncating the band by 100 kHz on each band edge to allow for a guard band.

Proposal:  Do not specify any UE coexistence requirements in 3GPP for this band, but instead focus only on the FCC emission requirements as needed.

Proposal:  Apply NS_04 for this the new NB-IoT 700 MHz band.

Proposal:  Assign band number 100 for this band.

# Reference

- RP-212618, “New WI: Upper_700_MHz_A_Block new E-UTRA Band in the US,” Puloli, Sequans Communications

- R4-2007564, “Exclusion 100 KHz for NB-IoT to meet FCC band-edge requirements,” Qualcomm Incorporated

- R4-2108012, “CR for 36.101: Introduction of NS Signalling for NB-IoT in the USA,” T-Mobile USA, Qualcomm