3GPP TSG-RAN4 Meeting #101-e	R4-2119429

# Electronic Meeting, November 1st - 12th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0972 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of modifiedMPR-Behavior for PC1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_PC1_5_n77_n78-Core |  |  |  |  |  | Date: |  |  | 2021-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is unclear how the modifiedMPR-Behavior applies to Rel-16 and earlier UE’s compared to Rel-17 and later UE’s supporting PC1.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A clarification is provided in the bit description to indicate the interpretation of the bit for Rel-16 and earlier UE’s compared to Rel-17 and later UE’s. To fully understand the behavior then requires the bit indication as well as awareness of the release of the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on which MPR table is supported by the UE for PC1.5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<<  Start of changes  >>>

# L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2.2-4a of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set, MPR in Table 6.2.2-4 applies. |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n77 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2.2-4a of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2.2-4 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies |
| n78 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2.2-4a of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2.2-4 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies |
| n79 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2.2-4a of 38.101-1 v17.3.0 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, MPR in Table 6.2.2-4 of 38.101-1 v17.3.0 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies |

### <<<  End of changes  >>>