3GPP TSG-RAN WG4 #101-e	R4-2119437

November 1st ‒ 12th, 2021

Electronic Meeting

Agenda item:	7.25.2.2

Source: 	Qualcomm Incorporated

Title: 	MPR and A-MPR for 100 MHz NR-U

Document for:	Approval

# Introduction

When NR-U was first introduced into the RAN4 specifications [1], channel bandwidths of 20, 40, 60, and 80 MHz were included.  The addition of 100 MHz channel bandwidth for NR-U in Bands n46 and n96 is now under consideration [2].  This contribution provides proposal for MPR and A-MPR for NS_53 and NS_54 for 100 MHz channel bandwidth.

# Discussion

## MPR

MPR for NR-U has already been defined in Table 6.2F.2-1 and Table 6.2F.2-2 in 38.101-1, copied below

Table 6.2F.2-1 Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 6.2F.2-2 MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

It can be seen that the MPR is independent of channel bandwidth because the general emission requirements scale with bandwidth.  Simulations for 100 MHz bandwidth were run and it was verified that the existing MPR can apply.  The MPR table is then proposed to be applied for all bandwidths including 100 MHz.  The MPR mapping for wideband operation is modified to add 100 MHz channel; similar to 60 MHz, all sub-band configurations are categorized as A due to the larger separation between allocated sub-band ACLR and IQ image with respect to the center of the wideband channel.

Table 6.2F.2-2 MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| 100 | 00001,00010,00100,01000, 10000,00011,00110,01100, 11000,00111,01110,11100, 01111,11110,11111 | None |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

Proposal:  Existing PC5 MPR for NR-U can be applied for 100 MHz channel bandwidth.  All sub-band configurations for 100 MHz are category A for the purpose of MPR mapping.

## A-MPR

NR-U A-MPR is defined for Band n46 with NS_28, NS_29, NS_30, and NS_31.  For Band n96, A-MPR applies with NS_53 and NS_54.  Because of the additional spurious emission requirements, A-MPR is dependent upon the channel frequency.  In this contribution, Band n96 is the focus due to the wider frequency range and the greater possibility to place 100 MHz channels.  As observed in [3] the limiting requirement for NS_53 is the -1 dBm/MHz maximum in-band PSD for all channel frequencies.  Since the power is spread over a larger bandwidth, it is expected that the 100 MHz channel should be able to meet the PSD requirement with less overall backoff except for higher order modulations where EVM becomes more limiting.  Simulation results confirm this behavior with the proposed A-MPR for NS_53 shown below.  The rightmost column for 100 MHz channel bandwidth has been added.

Table 6.2F.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 2.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 2.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

For NS_54, higher in-band PSD is allowed so it no longer limits the output power.  Instead, the additional spurious emissions requirement of -27 dBm/MHz.  Edge channels are susceptible to A-MPR to meet this emission requirement.  In this case, edge channels are those closest to 5925 MHz as well as the second 100 MHz channel – according to the channelization proposal [4] these are the channels centered at 5995 MHz and 6055 MHz.

Table 6.2F.3.7-1: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz and 6055 MHz].. |  |  |  |  |

Proposal:  A-MPR for NS_53 and NS_54 with 100 MHz channel bandwidths is according to the tables above.

# Conclusion

MPR and A-MPR for NS_53 and NS_54 have been proposed for PC5 NR-U with 100 MHz channel bandwidth.  The extension to 100 MHz channel bandwidth is expected to be straightforward since the general emissions scale according to bandwidth and the NS_54 and NS_54 requirements limit the maximum PSD and the edge channels respectively.  Simulations verified these expectations with proposed table values provided in this contribution.

# Reference

- RP-202117, “Introduction of NR-based access to unlicensed spectrum,” Qualcomm Incorporated, Nokia, AT&T, Charter Communications, Inc., Verizon, Samsung, LGE, Skyworks Solutions, Inc.

- RP-211599, “Revised Basket WID on adding channel bandwidth support to existing NR bands,” Ericsson

- R4-2011344, “Simulation results for NR-U bands n46 and n96,” Qualcomm Incorporated

- R4-2117498, “Channel rasteres for NR-U 100 MHz carrier bandwidth in bands n46 and n96,” Qualcomm Incorporated