3GPP TSG-RAN WG4 Meeting # 101e	R4-2119487

# Electronic Meeting, Nov. 1-12, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	NR-U Contiguous UL-CA Back-Off Measurements

Agenda Item:	7.5.2 NR-U intra-band contiguous UL CA [WI code]

Document for:	Approval

# Introduction

At RAN4 #100e, agreements were reached on UE transmitter (TX) assumptions, requirements and priorities to evaluate MPR and A-MPR for NR-U intra-band contiguous uplink (UL) carrier aggregation (CA) [1]. Based on these agreements, this document presents back-off measurements to evaluate MPR for PC5 using a Wi-Fi 5GHz Power Amplifier (PA). A-MPR has not been evaluated and is proposed for further study (FFS).

# Discussion

## Power Amplifier Calibration and Test Waveforms

The test conditions are set as follows:

- A 5GHz WiFi PA is calibrated to meet 1dB MPR for power class (PC) 5 using the agreed 20MHz 100RB3 QPSK DFT-s-OFDM SCS 15kHz reference waveform;

- Local Oscillator (LO) leakage has been disabled to avoid erroneous NR-U SEM scaling and therefore erroneous back-off gating levels;

- IQ Image rejection: -28dB;

- Gating measurements: only MPR gating is evaluated using:

  - PC 5 ACLR: -27dBc; and

  - Agreed Spectrum Emission Mask (SEM) in WF [1]. The spectrum emission mask for non-transmitted channels is floored at -28dBr.

The list of tested waveforms is captured in Table 1 with the following naming convention:

- All tested waveforms are QPSK modulated.

- UL-CA component carrier (CC) configurations:

  - 20MHz+20MHz and 80MHz+80MHz are evaluated with lowest valid SCS;

  - For 80+80MHz, only contiguous sub-bands across the two CCs are configured. In the waveform naming example of [00011000], the “1” indicate the fourth sub-band of CC1 and the first sub-band of CC2 are transmitted, the “0” indicate that the 1st,2nd,3rd sub-band of CC1 and 2nd,3rd,4th sub-band of CC2 are not transmitted.

  - Full and RB interlaced allocations in the transmitted sub-bands of CC1 and transmitted sub-bands of CC2.

  - Hybrid configurations of full allocations in the transmitted sub-bands of CC1 and interlaced allocations in the transmitted sub-bands of CC2.

- Resource Block (RB) allocations:

  - “Full” refers to fully allocated waveforms for a given channel bandwidth or given sub-band.

  - “CP_1RBI_0”: refers to a CP-OFDM interlaced waveform with 1RB_0 repeated according to the configured SCS wideband configuration and associated nominal intra-cell guard band. The example of waveform “[CP 1RBI_0] + [CP_1RBI_0] [00011000] is shown in Figure 1.

- Single carrier waveform naming “DFT_20M0_100RB3”:

  - “DFT/CP”: refers to either DFT-s-OFDM / CP-OFDM waveform type respectively;

  - “20M” refers to 20MHz channel bandwidth (CBW);

  - “0” refers to numerology “0”, i.e. SCS15kHz; and

  - “100RB3” refers to Lcrb=100RBs with start position RBstart=3.

Figure 1: Sketch of [CP 1RBI_0] + [CP 1RBI_0] [00011000] waveform example: 80+80MHz UL-CA, 10x1RB_0 transmitted in sub-band #4 CC1 and in sub-band #1 of CC2 using lowest SCS of 30kHz. Non-transmitted sub-bands are not shown.

Table 1: List of measured NR-U waveforms

|  | Waveform Name | SCS |
| --- | --- | --- |
| 20+20 MHz UL-CA | [CP Full] + [CP Full] [20+20] | 15+15 |
|  | [CP 1RBI_0] + [CP 1RBI_0] [20+20] | 15+15 |
|  | [CP 1RBI_0] + [CP 1RBI_5] [20+20] | 15+15 |
|  | [DFT Full] + [DFT Full] [20+20] | 15+15 |
|  | [DFT 1RBI_0] + [DFT 1RBI_0] [20+20] | 15+15 |
| 80+80 MHZ UL-CA | [CP Full] + [CP Full] [00011000] | 30+30 |
|  | [CP Full] + [CP Full] [00111000] | 30+30 |
|  | [CP Full] + [CP Full] [01111000] | 30+30 |
|  | [CP Full] + [CP Full] [11111000] | 30+30 |
|  | [CP Full] + [CP Full] [11111100] | 30+30 |
|  | [CP Full] + [CP Full] [11111110] | 30+30 |
|  | [CP Full] + [CP Full] [11111111] | 30+30 |
|  | [CP 1RBI_0] + [CP 1RBI_0] [00011000] | 30+30 |
|  | [CP 1RBI_2] + [CP 1RBI_4] [00011000] | 30+30 |
|  | [CP 1RBI_1] + [CP 1RBI_1] [01111000] | 30+30 |
|  | [CP 1RBI_0] + [CP 1RBI_4] [01111000] | 30+30 |
|  | [CP 1RBI_0] + [CP 1RBI_0] [11111000] | 30+30 |
|  | [CP 1RBI_0] + [CP 1RBI_4] [11111000] | 30+30 |
|  | [DFT Full] + [DFT Full] [00011000] | 30+30 |
|  | [DFT Full] + [DFT Full] [00111000] | 30+30 |
|  | [DFT Full] + [DFT Full] [01111000] | 30+30 |
|  | [DFT Full] + [DFT Full] [11111000] | 30+30 |
|  | [DFT Full] + [DFT Full] [11111100] | 30+30 |
|  | [DFT Full] + [DFT Full] [11111110] | 30+30 |
|  | [DFT Full] + [DFT Full] [11111111] | 30+30 |
|  | [DFT 1RBI_2] + [DFT 1RBI_4] [00011000] | 30+30 |
|  | [DFT 1RBI_2] + [DFT 1RBI_2] [01111000] | 30+30 |
|  | [DFT 1RBI_1] + [DFT 1RBI_4] [01111000] | 30+30 |
|  | [DFT 1RBI_0] + [DFT 1RBI_0] [11111000] | 30+30 |
|  | [DFT 1RBI_0] + [DFT 1RBI_4] [11111000] | 30+30 |
| Hybrid “Full/Interlaced” | [CP Full] + [CP 1RBI_0] [20+20] | 15+15 |
|  | [DFT Full] + [DFT 1RBI_5] [20+20] | 15+15 |
|  | [CP Full] + [CP 1RBI_0] [11111111] | 30+30 |
|  | [CP Full] + [CP 1RBI_0] [11111100] | 30+30 |
|  | [CP Full] + [CP 1RBI_0] [11111000] | 30+30 |
|  | [CP Full] + [CP 1RBI_0] [01111000] | 30+30 |
|  | [DFT Full] + [DFT 1RBI_0] [11111000] | 30+30 |

## Measurement Results

Figure 2 summarizes the measured back-off for each gating factor with waveforms split into 5 groups, and with from left to right:

- DFT-s-OFDM PA calibration waveform;

- Group 1: 20+20MHz and waveforms ordered as in Table 1;

- Group 2: 80+80MHz fully allocated CP OFDM waveforms;

- Group 3: 80+80MHz interlaced CP OFDM waveforms;

- Group 4: 80+80MHz fully allocated DFT-s-OFDM waveforms; and

- Group 5: 80+80MHz interlaced DFT-s-OFDM waveforms.

Figure 2: PC5 back-off measurement results for full/full and interlaced/interlaced allocations.

The measured back-off for hybrid configurations are summarized in Figure 3 and split into three groups: 20+20MHz, 80+80 CP-OFDM, 80+80 DFT-s-OFDM.

Figure 3: PC5 back-off measurement results for “hybrid” full/interlaced allocations.

Observation 1: For full/full and interlaced/interlaced allocations:

- Full/full allocations: for both CP-OFDM and DFT-s-OFDM 80+80 configurations:

  - MPR is gated by SEM for both CP and DFT-s-OFDM.

  - The highest back-off is met for asymmetric WB configurations [11111000] and [01111000] where the -28dB image rejection of the modulated sub-bands fail the SEM.

  - The next highest back-off is met in full WB configurations [11111111].

- Interlaced/interlaced: for 80+80 configurations:

  - MPR is gated by SEM for DFT-s-OFDM waveforms;

  - MPR is gated by ACLR for full WB configurations [11111111], otherwise driven by SEM;

  - The back-off for the tested interlaced waveforms is lower than for the case of fully allocated waveforms;

  - It should be noted however that the selected interlaced waveforms may not be representative of the worst IMD landscape since the tested RB interlaced patterns do not ensure maximum IMD overlap;

  - Further study of power back-off of interlaced/interlaced configurations is required; and

  - There is little difference in measured back-off for CP interlaced/interlaced and DFT interlaced/interlaced for 80+80MHz.

Observation 2: For “hybrid” full/interlaced allocations:

- SEM remains the dominating gating factor for all waveform types on this limited selection of tested waveforms;

- Due to missing interlaced/interlaced waveforms that maximize IMD overlap, it is not possible to conclude if the measured power back-off required to pass SEM for hybrid “Full/interlaced” allocations is covered by the back-off of “interlaced/interlaced” allocations; and

- Further study of power back-off of hybrid configurations is required as these allocations offer a much higher spectrum utilization than the interlaced/interlaced waveforms.

Proposal 1: PC5 contiguous UL-CA MPR:

•	Further study power back-off for interlaced waveforms where IMD overlap is maximized; and

•	Further study MPR table format at next meeting for all allocation types.

# Conclusions

In this contribution, we present our measurement results for NRU intra-band contiguous uplink CA operation. It was not possible to conclude if MPR for interlaced/interlaced waveforms may be as high as full/full allocations. It was also not possible to conclude if MPR for hybrid full/interlaced waveforms may be covered by interlaced/interlaced MPR. Based on observations 1 and 2, we propose the following:

Proposal 1: PC5 contiguous UL-CA MPR:

•	Further study power back-off for interlaced waveforms where IMD overlap is maximized; and

•	Further study MPR table format at next meeting for all allocation types.

# References

[1] 	R4-2114899, WF on NRU ULCA cases and evaluation assumptions, 3GPP TSG-RAN WG4 Meeting #100-e, Skyworks Solutions, Inc.