3GPP TSG-RAN WG4 Meeting # 101e	R4-2119488

# Electronic Meeting, Nov. 1-12, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	Adding 25MHz and 30MHz CBW to Band n71

Agenda Item:	7.25.2.1 Addition of bandwidth and Tx/Rx requirements [NR_bands_R17_BWs -Core]

Document for:	Approval

# Introduction

This document presents transmitter (Tx) noise measurements, REFSENS levels, and the changes required to introduce the agreed-to asymmetric uplink (UL) 20MHz / downlink (DL) 25MHz and 30MHz channel bandwidth (CBW), for band n71 [1].

# Discussion

## Capturing Asymmetric UL / DL 20/25 and 20/30MHz Operation

It is agreed in WF [1] that for band n71, DL 25MHz and DL 30MHz CBW are supported, with UL CBW restricted to 20MHz. We advance proposals 1, 2 and 3 below to align the introduction of these additional CBW for n71 with the agreements captured in bigCR [2].

Proposal 1: For Table 5.3.5-1 (subclause 5.3.5), adopt yellow highlighted changes in Table 1.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n71 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For Table 5.3.6-1 (subclause 5.3.6), adopt yellow highlighted changes in Table 2.

Table 2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n8 | 20 | 35 | 0 |
| n25 | 40 | 45 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 5 | 10 | 2 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 25 |  |
|  | 20 | 30 |  |
|  | 20 | 35 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

Proposal 3: For Table 7.3.2-3 (sub-clause 7.3.2), adopt yellow highlighted changes in Table 3.

Table 3: Proposed changes to Table 7.3.2-3 to capture the 20MHz UL configuration for n71 DL 25 and 30MHz REFSENS

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 15 | 25 | 251 | 201 | 201 | Note 5 | Note 5 | Note 5 |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

## n71 25MHz and 30MHz REFSENS

### Power Amplifier Calibration and REFSENS Assumptions

PA Tx noise in Rx band measurement conditions:

- PA is calibrated to meet -30dBc ACLR at PC3 1dB MPR with the 20 MHz channel bandwidth (CBW), SCS 15 kHz, DFT-S-OFDM QPSK 100RB0 waveform.;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5: -70dBc; and

- UL RB configuration: according to Table 3, i.e. UL 20MHz CBW, 20RB located closest to DL carrier and default n71 Tx-Rx frequency separation.

REFSENS evaluation assumptions:

- 55 dB Tx to RX and Tx to antenna duplexer rejection in Rx band;

- 10 dB Primary antenna to diversity antenna isolation;

- 4 dB Post PA loss;

- 4 dB antenna to LNA front-end losses;

- 3 dB antenna diversity gain;

- -1 dB SNR; and

- Previously agreed-to rules established to compute correlated and uncorrelated MRC contributions.

### Measurement Results and REFSENS

The measured Tx noise level in Rx band is -34.3dBm for 25MHz CBW and -31dBm for 30MHz. The resulting MSD presented in Table 4 ranges from 3.7dB to 5.1dB. Since the measured MSD level for 25MHz CBW is below the agreed legacy MSD level for UL/DL 20MHz/20MHz, we propose n71 REFSENS levels using the delta-MSD approach previously presented for similar cases (eg. band n3 new CBW REFSENS). n71 25MHz and 30MHz REFSENS levels are calculated by adding to the baseline 20/20MHz MSD the difference between the measured 20/20MHz MSD and the measured MSD for 20/25MHz and for 20/30MHz. The delta-MSD relative to the 20/20MHz MSD is +0.8dB for 20/25MHz and +2.2dB for 20/30MHz. The corresponding REFSENS levels are proposed in Proposal 4/Table 5.

Table 4: Measured MSD for n71 asymmetric UL 20MHz/DL 25MHz and UL 20MHz/DL 30MHz

Proposal 4: For Table 7.3.2-1a (sub-clause 7.3.2), adopt yellow highlighted changes in Table 5.

Table 5: n71 25MHz and 30MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 | -84.2 | -82.0 | -80.7 |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -84.3 | -82.1 | -80.8 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This contribution proposes changes required to align the introduction of band n71 asymmetric UL 20MHz DL 25MHz and DL35MHz operation, with the agreements captured in bigCR [2]. REFSENS levels are proposed based on PA Tx noise in Rx band measurement data.

Proposal 1: For Table 5.3.5-1 (subclause 5.3.5), adopt yellow highlighted changes in Table 1.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n71 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 10: These UE channel bandwidths are applicable to sidelink operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For Table 5.3.6-1 (subclause 5.3.6), adopt yellow highlighted changes in Table 2.

Table 2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n8 | 20 | 35 | 0 |
| n25 | 40 | 45 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 5 | 10 | 2 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 25 |  |
|  | 20 | 30 |  |
|  | 20 | 35 |  |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |  |

Proposal 3: For Table 7.3.2-3 (sub-clause 7.3.2), adopt yellow highlighted changes in Table 3.

Table 3: Proposed changes to Table 7.3.2-3 to capture the 20MHz UL configuration for n71 DL 25 and 30MHz REFSENS

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 15 | 25 | 251 | 201 | 201 | Note 5 | Note 5 | Note 5 |  |  |  | FDD |
|  | 30 |  | 121 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: For Table 7.3.2-1a (sub-clause 7.3.2), adopt yellow highlighted changes in Table 5.

Table 5: n71 25MHz and 30MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 | -84.2 | -82.0 | -80.7 |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 | -84.3 | -82.1 | -80.8 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2114914, Way Forward on n71 REFSENS configuration for 25 and 30 MHz, 3GPP TSG-RAN WG4 Meeting #100-e, T-Mobile USA, Skyworks Solutions Inc.

[2]	R4-2115101, bigCR to TS 38.101-1 - Introduction of 35MHz and 45MHz channel bandwidth R4-2112023, 3GPP TSG-RAN WG4 Meeting # 100e, Skyworks Solutions Inc., Apple.