3GPP TGS-RAN WG4 Meeting #101-e	R4-2119510
Electronic Meeting, November 1 – 12, 2021

Agenda item:	8.16.3.2

Source:	Intel Corporation

Title:	Minimum peak EIS for FR2-2

Document for:	Discussion

# 0	Introduction

UE RF discussions for the NR extension to 71 GHz work item are ongoing and most recently we agreed to consider different size arrays in the power class evaluations [1]. In this paper, we focus on the minimum peak EIS derivation of handheld UEs and FWAs for three different antenna array sizes.

# 2	Discussion

## 2.1	Handheld UE

Since PC3 is the most space-limited form factor, we will continue to use the baseline 4-elements in the single-band minimum peak EIS derivations and include an 8-element and 16-element derivation for comparison.

### 2.1.1 Minimum peak EIS

Table 1. Preliminary PC3 minimum peak EIS evaluation for FR2-2

| Parameter | Unit | 4-elements | 8-elements | 16-elements |
| --- | --- | --- | --- | --- |
| Frequency range | GHz | 52.6 - 71 | 52.6 - 71 | 52.6 - 71 |
| Modulation |  | QPSK | QPSK | QPSK |
| Bandwidth | MHz | 100 | 100 | 100 |
| SNR | dB | -1 | -1 | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -94 | -94 | -94 |
| Noise figure | dB | 17 | 17 | 17 |
| Effective array gain - 10log(# ant.) + element gain + roll-off | dB | 7.0 | 10 | 13 |
| Total implementation loss | dB | -12.4 | -12.9 | -13.5 |
| Peak EIS (Minimum) | dBm | -72.6 | -75.1 | -77.5 |

For 100MHz CBW, our 4-element derivation yields a minimum peak EIS of -72.6 dBm, while the 8-element is -75.1 dBm, and the 16-element is -77.5 dBm. As discussed in [2], the 8-element value provides a reasonable balance between improved performance, while still considering potential integration issues.

Observation 1: Our derived PC3 minimum peak EIS (100MHz) value for a 4-element array is -72.6 dBm, -75.1 dBm for an 8-element array, and -77.5 dBm for 16-elements. Given the size limitation of this form factor, an 8-element antenna array is a reasonable size to use.

Proposal 1: Use an 8-element antenna array assumption for handheld UEs in FR2-2 and define the minimum peak EIS as -75.1 dBm (100MHz).

## 2.1	FWA

Table 2 details the parameters for the FWA minimum peak EIS derivation for a 16-element, a 32-element, and a 64-element antenna array.

### 2.1.1 Minimum peak EIS

Table 2. Preliminary PC1 minimum peak EIS evaluation for FR2-2

| Parameter | Unit | 16-elements | 32-elements | 64-elements |
| --- | --- | --- | --- | --- |
| Frequency range | GHz | 52.6 - 71 | 52.6 - 71 | 52.6 - 71 |
| Modulation |  | QPSK | QPSK | QPSK |
| Bandwidth | MHz | 100 | 100 | 100 |
| SNR | dB | -1 | -1 | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -94 | -94 | -94 |
| Noise figure | dB | 17 | 17 | 17 |
| Effective array gain - 10log(# ant.) + element gain + roll-off | dB | 13 | 16 | 19 |
| Total implementation loss | dB | -12.0 | -12.8 | -13.5 |
| Peak EIS (Minimum) | dBm | -79.0 | -81.2 | -83.5 |

Proposal 2: Use a 32-elment array assumption for PC1 in FR2-2 and define the minimum peak EIS single-band requirement as -81.2 dBm (100MHz).

# 3	Conclusions

In this paper we presented our views on the FR2-2 REFSENS requirements of PC1 and PC3. Additionally, we discussed Rx spurious emissions. The following observations and proposals have been made:

Handheld UE

Observation 1: Our derived PC3 minimum peak EIS (100MHz) value for a 4-element array is -72.6 dBm, -75.1 dBm for an 8-element array, and -77.5 dBm for 16-elements. Given the size limitation of this form factor, an 8-element antenna array is a reasonable size to use.

Proposal 1: Use an 8-element antenna array assumption for handheld UEs in FR2-2 and define the minimum peak EIS as -75.1 dBm (100MHz).

FWA

Proposal 2: Use a 32-elment array assumption for PC1 in FR2-2 and define the minimum peak EIS single-band requirement as -81.2 dBm (100MHz).

# 4	References

- R4-2114989, “WF on 60 GHz UE RF requirements,” Qualcomm, RAN4 #100e, August 2021

- R4-2119509, “UE Tx requirements for FR2-2,” Intel, RAN4 #101e, November 2021