3GPP TSG-RAN WG4 Meeting#101-e	R4-2119544

Online, 1st Nov – 12th Nov, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on FR1 HST PDSCH CA Tests

Agenda item:	8.8.3.2

Document for:	Discussion

- Introduction

In RAN4#99-e, simulation assumptions were agreed in [1] for defining HST requirements for CA scenarios. There were also some open issues mentioned in [2]. In this paper, we provide our views on FR1 HST PDSCH CA test cases.

- Applicability Rules and Release Independence

In RAN#90-e, it was agreed to discuss the test applicability rule for CA scenarios between HST-DPS and HST-SFN schemes. Option 2 for applicability rule, proposed in RAN4#100-e, does not seem to have any clear technical reason for splitting the applicability rules in such fashion. On the other hand, HST-SFN channel model is definitely a tougher model to handle from UE implementation perspective compared to DPS. Therefore, to reduce the test burden on UEs, we propose the following.

Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped.

From UE implementation perspective, we don’t see any difficulty in extending the demod requirements for CA. Therefore, we propose the following.

Proposal 2: Use same rule for release independence of demod requirements, as agreed in RRM requirements.

- HST-SFN Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below.

Table 1: HST-SFN Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 10.03 | 12.03 | 7.79 | 9.79 |
| 10 | 10.01 | 12.01 | 8.00 | 10.00 |
| 15 | 10.33 | 12.33 | 8.51 | 10.51 |
| 20 | 10.61 | 12.61 | 8.64 | 10.64 |
| 25 | 10.73 | 12.73 | 8.79 | 10.79 |
| 30 | 10.90 | 12.90 | 9.17 | 11.17 |
| 40 | 10.93 | 12.93 | 9.27 | 11.27 |
| 50 | 11.17 | 13.17 | 9.46 | 11.46 |

Table 2: HST-SFN Simulation results for TDD 30kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 11.22 | 13.22 | 9.50 | 11.50 |
| 10 | 11.66 | 13.66 | 9.92 | 11.92 |
| 15 | 11.45 | 13.45 | 9.83 | 11.83 |
| 20 | 11.30 | 13.30 | 9.57 | 11.57 |
| 25 | 11.24 | 13.24 | 9.59 | 11.59 |
| 30 | 11.39 | 13.39 | 9.71 | 11.71 |
| 40 | 11.60 | 13.60 | 9.94 | 11.94 |
| 50 | 11.81 | 13.81 | 10.09 | 12.09 |
| 60 | 11.90 | 13.90 | 10.17 | 12.17 |
| 80 | 12.00 | 14.00 | 10.19 | 12.19 |
| 90 | 12.19 | 14.19 | 10.41 | 12.41 |
| 100 | 12.04 | 14.04 | 10.50 | 12.50 |

- HST-DPS Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below.

Table 3: HST-DPS Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 10.41 | 12.41 | 7.65 | 9.65 |
| 10 | 10.64 | 12.64 | 7.82 | 9.82 |
| 15 | 10.64 | 12.64 | 7.75 | 9.75 |
| 20 | 10.20 | 12.20 | 7.41 | 9.41 |
| 25 | 10.62 | 12.62 | 7.72 | 9.72 |
| 30 | 10.70 | 12.70 | 7.76 | 9.76 |
| 40 | 10.56 | 12.56 | 7.79 | 9.79 |
| 50 | 10.86 | 12.86 | 7.89 | 9.89 |

Table 4: HST-DPS Simulation results for TDD 30kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 10.37 | 12.37 | 7.59 | 9.59 |
| 10 | 10.63 | 12.63 | 7.80 | 9.80 |
| 15 | 10.51 | 12.51 | 7.76 | 9.76 |
| 20 | 10.43 | 12.43 | 7.71 | 9.71 |
| 25 | 10.51 | 12.51 | 7.75 | 9.75 |
| 30 | 10.38 | 12.38 | 7.65 | 9.65 |
| 40 | 10.32 | 12.32 | 7.62 | 9.62 |
| 50 | 10.68 | 12.68 | 7.80 | 9.80 |
| 60 | 10.64 | 12.64 | 7.77 | 9.77 |
| 80 | 10.53 | 12.53 | 7.75 | 9.75 |
| 90 | 10.78 | 12.78 | 7.67 | 9.67 |
| 100 | 10.60 | 12.60 | 7.80 | 9.80 |

- Conclusions

This paper provides our views on test parameters and simulation results for FR1 HST PDSCH CA requirements. Following has been proposed.

Proposal 1: If UE passes the FR1 HST PDSCH CA tests for HST-SFN scheme, tests with HST-DPS scheme can be skipped.

Proposal 2: Use same rule for release independence of demod requirements, as agreed in RRM requirements.

# References

[1] R4-2108635, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 – May 27, 2021.

[2] R4-2115724, “WF on FR1 HST demodulation”, 3GPP TSG RAN WG4 Meeting #100-e, Online, Aug 16 - 27, 2021.