3GPP TSG-RAN WG4 Meeting RAN4#101-e     				R4-2119555 Online

November 1 – 12, 2021

Agenda Item:	4.1.1.2

Source:	Lenovo, Motorola Mobility

Title:	Issues with FR2 MIMO EVM Measurement Using the Pseudo-Inverse

Document for:  Approval

- Introduction

In [1], it has been proposed that the pseudo-inverse receiver be used to define and measure the EVM for two-layer transmissions. In this contribution, we point out some fundamental issues with this proposal.  As discussed below, the pseudo-inverse in [1] cannot be defined or computed as proposed if the channel does not have full rank. Furthermore, the MIMO layers cannot be separated using the pseudo-inverse when the channel does not have full rank. In this contribution, alternative proposals are presented which address these issues.

- EVM for Two-Layer Transmission

In [1], it has been proposed that the pseudo-inverse be used in the definition and measurement of EVM for two-layer transmission.  Following the notation in [1], the effective channel matrix is given by

H=HP= h0,0h0,1h1,0h1,1

where H is the 2x2 channel matrix and P is the precoding matrix.  It is then proposed that the zero-forcing MIMO equalizer is defined using the pseudo-inverse as

GZF=H+=HHH-1HH.

A few observations can be made on the use of the pseudo-inverse for measuring EVM.

Observation 1:	If a square matrix H has full rank, then H+=H-1, and the pseudo-inverse is not needed.

Observation 2:	If a square matrix H does not have full rank, then HHH does not have full rank and the inverse HHH-1 does not exist.  As a result, if H does not have full rank, then the pseudo-inverse cannot be defined or computed as

H+=HHH-1HH.

So, if the pseudo-inverse H+ were to be used, it would be necessary to define it in some other manner such as in terms of the singular value decomposition of H.

Observation 3:  	If the square matrix H does not have full rank, then it is not possible to separate the MIMO layers using the pseudo-inverse or any other linear receiver and the EVM requirement will be failed.

With respect to Observation 3, we note that if H does not have full rank, then multiplication by the pseudo-inverse does not yield the identity matrix; that is

H+H ≠I,

and as a result, the pseudo-inverse H+ cannot be used to separate the MIMO layers. To demonstrate, let y denote the signal received given by

y=H x+n

where x is the transmitted data vector, n is the transmitter noise vector, and H has dimension 2x2.  Furthermore, let the singular value decomposition of H be given by

H=U Σ VH,

where U and V are unitary matrices and Σ is a diagonal matrix for which the diagonal elements are real and non-negative.  In the Appendix, it is shown that if

GZF=H+,

and H does not have full rank, then the resulting data estimate x is given by

x=H+y=v1 v1Hx+n

=v1* x v1+v1* n v1

where v1 is the right singular vector belonging to V for which the corresponding singular value is non-zero.  Thus, in the case that H does not have full rank, the expected value of the estimate is given by

Ex|x=v1H x v1

which is the projection of the data vector x onto the right singular vector v1.

In the Appendix, it is shown that even if there is no transmitter noise so that n=0, the EVM for the estimate

x=H+y

when H does not have full rank is no less than 71%, so in this case the EVM requirement fails.

Observation 4: 	If H does not have full rank and GZF=H+ is used, the resulting EVM will be no less than 71%.

In summary, if the square matrix H does not have full rank, the pseudo-inverse cannot be defined and computed as

H+=HHH-1HH

because HHH-1 does not exist.  Furthermore, if H does not have full rank, the two layers of data cannot be separated with the pseudo-inverse or any other linear receiver. If the pseudo-inverse is used in this case, the EVM will be no less than 71%.

As the result of the above, we have the following proposal.

Proposal 1:  	If H has full rank, then GZF=H-1.  If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed.

If there is still the desire to use the pseudo-inverse when H does not have full rank, we have the following alternative proposal

Proposal 2: 	The zero-forcing receiver is defined as GZF=H-1 where the singular value decomposition for H is given by

H=U Σ VH,

H-1is given by

H-1=V Σ+UH

and Σ+ is derived from Σ by taking the reciprocal of the non-zero diagonal elements and leaving the zero elements in place.

- Conclusions

# In [1], it has been proposed that the pseudo-inverse be used in the definition and measurement of EVM for two-layer transmission.  However, in this contribution, it has been observed that if the channel matrix H does not have full rank, then HHH does not have full rank and the pseudo-inverse cannot be defined as

# H+=HHH-1HH

# Furthermore, it has been shown, that if H does not have full rank, then the MIMO layers cannot be separated using the pseudo-inverse or any other linear receiver. Based on these observations, we have the following two proposals.

Proposal 1:  	If H has full rank, then GZF=H-1.  If H has does not have full rank, the two layers cannot be separated and the EVM requirement is failed.

If there is still the desire to use the pseudo-inverse when H does not have full rank, we have the following alternative proposal

Proposal 2: 	The zero-forcing receiver is defined as GZF=H-1 where the singular value decomposition for H is given by

H=U Σ VH,

the pseudo-inverse	H-1is given by

H-1=V Σ+UH,

and Σ+ is derived from Σ by taking the inverse of the non-zero diagonal elements and leaving the zero elements in place.

# References

- R4-2107113, ‘Discussion on FR1 UL MIMO EVM measurements’, Rohde and Schwartz, 3GPP TSG-RAN WG4 Meeting #98-e-bis.

Appendix:  EVM for Two-Layer MIMO with a Rank Deficient Channel

In the case that the square matrix H does not have full rank, the pseudo-inverse can be expressed in terms of the singular value decomposition of H which is given by

H=U Σ VH,

where U and V are unitary matrices and Σ is a diagonal matrix for which the diagonal elements are real and non-negative.  If H has dimension 2x2 and rank 1 then H can be expressed as

H=Uλ000 VH

where λ is real and positive. For this example, the pseudo-inverse is given by

H+=V λ-1000 UH

In the absence of receiver noise, the received signal is given by

y=H x+n

where x is the transmitted data and n is the transmitter noise.  Multiplication by the pseudo-inverse yields the data estimate

x=H+y=V λ-1000 U*U λ000 VHx+n

=V 1000 VHx+n,

=v1 v1Hx+n

where v1 is the first column of V.  The resulting estimate of the data vector x can be expressed as

x=v1H x v1+v1H n v1

where v1H x v1 is the projection of the data vector x onto the vector v1.  The expected value of the estimate given by

Ex|x=v1H x v1

which is equal to x if and only if x=v1.  Finally, if the data vector x has mean and covariance given by

Ex=0 and Ex xH= 120012 ,

then Ex2=1, and

Ex-x2=Ev1H x v1-x2= Ev1HxHv1v1H xv1-v1H xH v1x- xHv1Hx v1+xHx

= 12-12-12+1=12 .

As a result, even if there is no transmitter noise so that n=0, the mean-square error of the estimate

x=H+y

is equal to

MSE= Ex-x2= 12

and the EVM is given

EVM=100MSE= 1000.5=71% .