3GPP TSG-RAN WG4 Meeting #101-e  		                                 R4-2119557

Electronic Meeting, 1st-12th of November, 2021

Source:	THALES, Magister Solutions Ltd

Title:	TN-NTN Coexistence Results for Phase 1

Type:	discussion

Document for:	discussion

Agenda Item:	8.13.2.1

Release:	Rel-17

# Introduction

R4-2115749	WF on NTN co-existence study

- Decision:		Approved.

R4-2115750	Simulation assumption for NTN co-existence study

- Decision:		Approved.

During RAN4#100-e, the following documents related to NTN coexistence have been approved:

For further details, please also consider the email discussion summary for [100-e][313] NTN_Solutions_Part2 (R4-2115785, noted).

The simulation calibration phase (Phase 0) almost finished (see for example R4-2115628), and since then companies provided updates on the calibration results for both NTN and TN.

RAN4#100-e Agreements:

- The updated summary of calibration results and assumptions will be captured in the new TR 38.863

- The calibration results indicate the consistency of most companies’ simulations. Therefore, calibration work has mostly been done for NTN coexistence. Companies can continue to contribute on calibration aspect over emails till Sep 30th.

- RAN4 start to discuss the simulation assumption and co-existence results for phase 1 as agreed in previous work plan, RAN4 will check the status in Nov 2021 RAN4 meeting with the target to conclude phase 1 co-existence study by Nov 2021.

Moreover, during RAN4#100-e meeting, the following agreements have been made on the 20th August GTW online meeting:

This contribution therefore presents Phase 1 simulation results for TN-NTN coexistence, i.e., focusing on the scenarios 1 to 6 from the table below.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

# Simulation assumptions

The simulation assumptions are defined in [1].

The simulation methodology states that NTN UEs should be dropped at the edge of TN clusters, which is a vague definition. The definition should include some information with respect to the SINR level received e.g. at those NTN UE when operating as TN UEs (e.g. SINR of 5 dBs or lower).

However, in this contribution the UEs (which are assumed to have both TN and NTN capabilities) are placed to the TN cell coverage area, and UEs are allowed to select the best beam/cell for service. This results that most of the UEs are allocated to TN cells, while the UEs at the edge of the TN cells may select the NTN beam. The simulations are further performed by considering the following use cases/combinations:

Figure 2.1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode (e.g. n1)

Figure 2.2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

where the interference types and varied parameters as respect to each scenarios are represented below:

| Interference Type | Combination | Aggressor | Victim | Notes | Varied & Static parameters |
| --- | --- | --- | --- | --- | --- |
| i1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | NTN UE ACS varied TN gNB ACLR static |
| i2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | NTN satellite ACS varied TN UE ACLR static |
| i3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | NTN satellite ACLR varied TN UE ACS static |
| i4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n1 FDD. | NTN UE ACLR varied TN gNB ACS static |
| i5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | NTN UE ACLR varied TN UE ACS static |
| i6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with TN n34 TDD. | NTN satellite ACS varied TN gNB ACLR static |

The simulation parameters are considered from R4-2115750, with the following updates of Table 2.3-7:

ACLR/ACS for TN (2 GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 46 dB | 46 dB |
| UE | ACLR | 30dB (ACLR1) for UE Power Class 3 43dB (ACLR2) | 37 |
|  | ACS | 33 dB 27dB for 20 MHz channel BW, n1 | 28 |

# Simulation results

# Scenario 1

Figure 3.1: LEO@600 (rural, urban)

Table 3.1.1: Throughput and SINR losses in Rural scenario 1

| LEO 600 km – Rural |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN UE ACS |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 1.4 | 0.5 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 1.1 | 0.4 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 2.0 | 0.5 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 3.1.2: Throughput and SINR losses in Urban scenario 1

| LEO 600 km – Urban |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN UE ACS |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 5.9 | 2.8 | 1.1 | 0.4 | 0.2 | 0.1 | 0.0 | 0.0 |
|  | 5%-tile | 5.2 | 2.4 | 0.9 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 |

Observation 1: In Urban scenario with TN DL aggressor and NTN DL victim, the average SINR of NTN UEs degrades up to 5.9 dB depending on NTN UE ACS. However, this was not carried to throughput loss due to NTN UEs adopting MCS0 into use even with dynamic link adaptation.

# Scenario 2

Observation 2: Link budget from omnidirectional UE to the satellite is very modest, and with TN power control and limitations to the number of scheduled users the TN UEs do not pose a threat to the satellite UL. However, a great number of non-power controlled UEs under a large satellite beam could theoretically cause noticeable interference.

# Scenario 3

Figure 3.3: LEO@600 (rural, urban)

Table 3.3.1: Throughput and SINR losses in Rural scenario 3

| LEO 600 km – Rural |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite ACLR |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 4.4 | 1.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 0.4 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 3.3.2: Throughput and SINR losses in Urban scenario 3

| LEO 600 km – Urban |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite ACLR |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Observation 3: Path loss from satellite to UEs with omnidirectional antennas is high, so leaked interference power to adjacent channel becomes even less. In Rural scenario the TN UE link budget is smaller, so slightly increased interference can be seen in the SINR with low satellite ACLR.

# Scenario 4

Observation 4: The UEs that connect to the satellite have a very large path loss towards TN gNBs. When ACIR is reduced from the NTN UEs’ transmitted power, the NTN UL ACI cannot be heard at the TN gNBs.

# Scenario 5

Figure 3.5: LEO@600 (rural, urban)

Table 3.5.1: Throughput and SINR losses in Rural scenario 5

| LEO 600 km – Rural |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN UE ACLR |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 0.5 | 0.3 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 3.5.2: Throughput and SINR losses in Urban scenario 5

| LEO 600 km – Urban |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN UE ACLR |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Observation 5: Under used simulation assumptions there is a low probability that UEs are close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE.

# Scenario 6

Figure 3.6: LEO@600 (rural, urban)

Table 3.6.1: Throughput and SINR losses in Rural scenario 6

| LEO 600 km – Rural |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite ACS |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 98.8 | 74.7 | 36.2 | 13.7 | 4.9 | 1.9 | 1.0 | 0.7 |
|  | 5%-tile | 100.0 | 83.3 | 33.3 | 16.7 | 8.3 | 0.0 | 0.0 | 0.0 |
| SINR loss [dB] | Mean | 4.9 | 2.2 | 0.9 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 |
|  | 5%-tile | 4.7 | 2.0 | 0.8 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |

Table 3.6.2: Throughput and SINR losses in Urban scenario 6

| LEO 600 km – Urban |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite ACS |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| Throughput loss [%] | Mean | 80.4 | 51.0 | 21.0 | 6.5 | 2.4 | 0.5 | 0.4 | 0.2 |
|  | 5%-tile | 99.9 | 79.5 | 38.6 | 11.5 | 3.0 | 1.1 | 0.6 | 0.3 |
| SINR loss [dB] | Mean | 9.1 | 5.2 | 2.5 | 1.0 | 0.4 | 0.1 | 0.1 | 0.0 |
|  | 5%-tile | 8.1 | 4.3 | 1.9 | 0.7 | 0.3 | 0.1 | 0.1 | 0.0 |

Observation 6: The antenna directivity for TN gNBs is not low enough, and with high transmission power they will interfere NTN UL transmissions with a low link budget even without adjacent channel interference. This situation is very particular and can be considered as worst case.

# Proposals

Following all previous results for Scenario 1 to Scenario 6 and also previous observations, we can reach at the following proposal for NTN UE and Satellite ACLR and ACS values:

Proposal 1. RAN4 shall consider for NTN UE same ACLR and ACS parameter values as for TN UE.

Proposal 2. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACLR.

Proposal 3. RAN4 shall consider for NTN Satellite node a maximum value of 30-35dBs for ACS.

Proposal 4. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACLR when TN FDD.

Proposal 5. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACS when TN FDD.

Proposal 6. RAN4 shall consider for NTN Satellite node a maximum value of 30-35dBs for ACS when TN TDD.

# Conclusions

Observation 1: Scenario 1. In Urban scenario with TN DL aggressor and NTN DL victim, the average SINR of NTN UEs degrades up to 5.9 dB depending on NTN UE ACS. However, this was not carried to throughput loss due to NTN UEs adopting MCS0 into use even with dynamic link adaptation.

Observation 2: Scenario 2. Link budget from omnidirectional UE to the satellite is very modest, and with TN power control and limitations to the number of scheduled users the TN UEs do not pose a threat to the satellite UL. However, a great number of non-power controlled UEs under a large satellite beam could theoretically cause noticeable interference.

Observation 3: Scenario 3. Path loss from satellite to UEs with omnidirectional antennas is high, so leaked interference power to adjacent channel becomes even less. In Rural scenario the TN UE link budget is smaller, so slightly increased interference can be seen in the SINR with low satellite ACLR.

Observation 4: Scenario 4. The UEs that connect to the satellite have a very large path loss towards TN gNBs. When ACIR is reduced from the NTN UEs’ transmitted power, the NTN UL ACI cannot be heard at the TN gNBs.

Observation 5: Scenario 5. Under used simulation assumptions there is a low probability that UEs are close enough to each other cause significant ACI. If UEs are close-by to each other they likely connect to the same cell. However, if the UEs are forced to different networks (NTN and TN) the NTN UE could pose a threat to the neighbour channel TN UE.

Observation 6: Scenario 6. The antenna directivity for TN gNBs is not low enough, and with high transmission power they will interfere NTN UL transmissions with a low link budget even without adjacent channel interference. This situation is very particular and can be considered as worst case.

Following all previous results for Scenario 1 to Scenario 6 and also previous observations, we can reach at the following proposal for NTN UE and Satellite ACLR and ACS values:

Proposal 1. RAN4 shall consider for NTN UE same ACLR and ACS parameter values as for TN UE.

Proposal 2. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACLR.

Proposal 3. RAN4 shall consider for NTN Satellite node a maximum value of 30-35dBs for ACS.

Proposal 4. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACLR when TN FDD.

Proposal 5. RAN4 shall consider for NTN Satellite node a maximum value of 15-20dBs for ACS when TN FDD.

Proposal 6. RAN4 shall consider for NTN Satellite node a maximum value of 30-35dBs for ACS when TN TDD.

# References

- R4-2115750, Simulation assumptions for NTN co-existence study, 3GPP RAN4#100-e.

END