3GPP TSG-RAN WG4 Meeting #101-e	R4-2119696

Electronic Meeting, November 01 – 12, 2021

Agenda item:	8.16.5 – Co-existence simulations

Source: 	Korea Testing Laboratory

Title: 	Coexistence simulation results for NR DL at 60 GHz

Document for:	Discussion

# Introduction

After 3GPP TSG-RAN WG4 meeting #100-bis-e, the system calibration process was conducted among the interesting parties for co-existence simulation. This paper contributes simulation results of DL ACIR after the calibration.

# Discussion

As the agreement of the previous meeting, the simulations are conducted for DL ACIR with the indoor office C scenario at 60 GHz and 70 GHz. Refer to the WF from last meeting for detailed simulation parameters [1].

Figure. 1 DL ACIR simulation results for throughput loss at 60 GHz

Figure. 2 DL ACIR simulation results for throughput loss at 60 GHz

The DL ACIR results are summarized in Table 1.

Table 1 : Summary of DL ACIR simulation results

| Bandwidth | 100 MHz | 400 MHz |
| --- | --- | --- |
| 60 GHz Average | 18.0 | 17.8 |
| 60 GHz 5%-tile | 19.8 | 19.9 |
| 70 GHz Average | 18.5 | 17.6 |
| 70 GHz 5%-tile | 20.8 | 19.3 |

The DL ACIR has slight differences between 100 MHz and 400 MHz bandwidth and 60 GHz and 70 GHz carrier frequencies. Based on these simulation results, both bandwidth and carrier frequency are not a dominant factor to DL ACIR.

# Conclusion

In this contribution we present our simulation results for coexistence study. To keep less than 5% throughput degradation, the DL ACIR shall be stringent as 19.9 dB for 60 GHz and 20.8 dB for 70 GHz.

# Reference

- R4-21114993, “WF on co-existence simulation for NR_ext_to_71GHz”, Qualcomm.