3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2119716

Electronic Meeting, 1st – 12th November, 2021

Agenda item:			7.34

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [101-e][116] NR_Power_Limit_CA_DC

Document for:	Information

# Introduction

This document summarizes the email discussion for agenda item 7.34 on increasing the maximum output power for CA and DC.

# Topic #1: Options for increasing MOP

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117052 | Nokia, Nokia Shanghai Bell | UE RF requirements specific to the sum method |
| R4-2117053 | Nokia, Nokia Shanghai Bell | Feasibility of the sum method for increasing UE power high limit for CA/DC Moderator note: Withdrawn. Revised to R4-2118726 |
| R4-2117751 | Xiaomi | Discussion on increasing UE maximum power high limit |
| R4-2117863 | MediaTek Inc. | Further discussion on capability signaling for HPUE NR DC |
| R4-2117864 | MediaTek Inc. | Draft LS on signalling clarification of NR CA/DC power class in R17 |
| R4-2117988 | Apple | UE maximum output power for inter-band UL CA |
| R4-2118183 | ZTE Corporation | Discussion on increase UE maximum power for NR uplink inter band CA |
| R4-2118292 | vivo | Discussion on the increasing UE power high limit for CA and DC |
| R4-2118547 | Huawei,HiSilicon | Discussion on Increasing UE power high limit for CA and DC |
| R4-2118605 | Mediatek India Technology Pvt. | Discussion on impact of increasing UE high limit for CA and DC |
| R4-2118726 | Nokia, Nokia Shanghai Bell | Feasibility of the sum method for increasing UE power high limit for CA/DC |
| R4-2118888 | OPPO | R17 Discussion on UE power class high limit |
| R4-2119434 | Qualcomm Incorporated | Increasing MOP for CA and DC |
| R4-2119441 | Skyworks Solutions Inc. | Handling of maximum output power limit for PC2 inter-band combinations |

## Open issues summary

### Sub-topic 1-1 General framework

- If increasing the MOP is enabled by specifications and is supported by the UE, should the higher MOP be mandatory (for example, both Pcmax_L and Pcmax_H increased) or as an allowance (for example, only Pcmax_H is increased)?

- Is signaling to the network necessary (either as a capability or by signaling a PC or otherwise)?

- Which PC configurations are in scope?  (PC5+PC5, PC5+PC3, PC3+PC3, PC3+PC2, PC2+PC2, PC2+PC1.5, PC1.5+PC1.5)

### Sub-topic 1-2 Comparison of options

For the following options

- The sum method:  For example, PPowerClass,CA is replaced with 10*log10∑ pPowerClass,c

- A new conventional power class:  For example, define PC1.8 for 27.8 dBm max power

- A conceptual power class (call it power class 0):  For example, remove requirements on composite power, each CC is treated independently.

- CA/DC power class that also indicates the PA configuration for each carrier/cell:  For example, define PC0 as 26 dBm max total output power composed of 23 dBm + 23 dBm PA configuration.

Consider the following aspects

- Are there any regulatory issues?

- Are new MSD requirement needed?

- Are the existing SAR mechanisms sufficient?  If not, what needs to be changed?

- What is the impact to a UE that requires TxD for a single CC power class when it is configured for CA or DC?

- What is the level of work required in RAN4?  (low, medium, high)

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1 General framework

| Company | Comments |
| --- | --- |
| Huawei | Issue #1 Both approaches have pros and cons, however, we tend to support that both Pcmax_L and Pcmax_H should be increased. If Pcmax_L remains the same and only Pcmax_H is increased, the range of Pcmax (for CA) is increased, but the UE is NOT required to deliver extra power. This approach is most flexible for UE implementations but the performance gain is not guaranteed. On the other hand, the max total power P_PowerClass,CA involves in the calculation of both Pcmax_L and Pcmax_H. If P_PowerClass,CA is increased, both Pcmax_L and Pcmax_H should increase. This guarantees that the UE is required to deliver the extra power, which is also beneficial for the network performance. We believe that this is low-hanging fruit for UE implementations, since it’s based on UE’s existing hardware capabilities. And it’s in line with the original intention, i.e., fully utilize the UE power ability in each tx chain. Issue #2 Yes, signaling is needed. Actually it’s the key ingredient of our proposed solution, which is described in sub topic 1-2. Issue #3 We believe the solution should be scalable. Hence all PC configurations are in scope. |
| OPPO | Question 1: If increasing the MOP is enabled by specifications and is supported by the UE, should the higher MOP be mandatory (for example, both Pcmax_L and Pcmax_H increased) or as an allowance (for example, only Pcmax_H is increased)? [OPPO] It might depends on the solution, for example if same power class is kept for UE with and without increased power ability then probably lower limit needs to be kept and only higher limit can be improved. Question 2: Is signaling to the network necessary (either as a capability or by signaling a PC or otherwise)? [OPPO] It depends on how to increase max power higher limit, and how the signalling looks like. Most likely UE needs to indicate the higher power capability to NW. Question 3: Which PC configurations are in scope? (PC5+PC5, PC5+PC3, PC3+PC3, PC3+PC2, PC2+PC2, PC2+PC1.5, PC1.5+PC1.5) [OPPO] The efforts is to find a solution can be applied to all PC combinations in principle, however, not clear the reason why PC5+PC5 UE doesn’t indicate PC3 directly, and PC3+PC3 doesn’t indicate PC2 directly, and PC2+PC2 doesn’t indicate PC1.5 directly, and there is no PC1.5+PC1.5 case. These shouldn’t be included. And the workload is also another aspect need to be considered, enable non typical PA configurations will lead to many different set of requirements no matter which method is used, therefore, sugest to restrict the variation of different PA configurations and only consider the most typical implementations when define 3GPP requirements. |
| MediaTek | Issue 1: We share same view as OPPO. Issue 2: As indicated in R4-2118605, we think indication of new higher MOP capability could be needed irrespective of new UE capability or power class. Further, no matter how to handle MOP of the combo, as we proposed in R4-2117863, A new power class signaling per band per combination is needed for HPUE NR CA/DC Issue 3: At this stage, maybe PC5+PC3, PC3+PC2, PC2+PC1.5 |
| Xiaomi | Issue #1: we slightly prefer only Pcmax_H is increased. Issue #2: Yes, the feature should be necessary as optional per band combination capability to signalling to network Issue #3: some PC combinations have already covered by traditional CA power class approach. In order to harmonize with current CA power class, the soulution may be only applied the PC combination when 10*log10∑ pPowerClass,c>PPowerClass,CA for specific band combination |
| ZTE | Issue #1: we slightly prefer only Pcmax_H is increased. Issue #2: Yes, whichever option is selected, a new signalling is needed since NW should know the higher power of UE. Issue #3: The cases whose total sum of the power of each band are different with the current power class should be discussed, also the Tx chain number restrictions may also need to be considered. So PC5+PC5=PC3, PC3+PC3=PC2, PC1.5+PC1.5 power allocation may not need to be considered at this stage. |
| Qualcomm | Issue #1: We prefer only Pcmax_H is increased. As explained in R4-2119434, this results in significant simplification and reduction in specification work since MPR, A-MPR, MSD would generally not have to be revisited. Issue #2: Signaling would be beneficial to inform the network Issue #3: We agree with the comments from ZTE, Xiaomi, MTK, and OPPO that we do not need to consider the combinations for which there are already defined power classes. |
| Apple | Issue #1: Only per-band Pcmax needs to be considered as the sum of the output power is of no importance since the CA power constraint is removed. Issue #2: Two types of signaling are needed. One is the capability to maximize each band’s power ability to differentiate with the existing CA power class definition. The other is to indicate the per-band power class under CA if it is different from single band power class. This field is not required to be signaled if the per-band power class under CA is the same as single band power class. Issue #3: In principle, all permutations of single-band power classes are applicable. But in Rel-17 we can focus on PC2+PC3, PC2+PC2, PC3+PC5, and PC2+PC5 as these are probable hardware implementation in Rel-17. |
| Vivo | Issue #1, we prefer only Pcmax_H increased. If both Pcmax_L and Pcmax_H increased, it seems no different with defining a new PC1.8, and also if the Pcmax_L is increased, MPR/AMPR, MSD all need to be re-evaluated. Issue #2, an option signaling for the new Pcmax_H is needed, and also the legacy power class for CA is anyway needed to indicate the Pcmax_L. Isssue #3, If only Pcmax_H increased, it may be no harm to include all typical PC combination. In this WI, we prefer to not limit on the power capability or number of Tx chains, try to form a generic solution. |
| Nokia | Pcmax_L and/or Pcmax_H Both limits need to be increased. Signaling to the network The signaling is definitely necessary since there are UEs with power class for inter band CA for PC2. If nothing is signaled, the UE’s power class is considered as PC3. Power class At least PC2+PC3 should be included. |
| Skyworks | Issue#1: Per band PCmax should be sufficient. PCmaxL should not be increased as the default UE does not exceed the inter-band power class today. Issue#2: a per band power capability in inter-band CA mode should be sufficient, and as suggested by Apple if not signaled the per band power class is assumed. But there may still be a need for another signaling which is whether the UE has peak power > inter-band PCmax as the default operation (used for MSD and duty cycle reporting) assume PC2 combined max power. Note that PC3+PC5 or PC3+PC3 with PC2 inter-band can’t exceed the PC2 max power anyhow. So the signaling is only needed for the cases where 26dBm+2dB can be exceeded which is for the cases where at least one of the band is PC2. B1 PC; B2 PC; max. 26; 26; 29.0. 26; 23; 27.8. 26; 20; 27.0. 23; 23; 26.0. 23; 20; 24.8. Issue#3 In our view the adopted PCmax method and signaling should apply to any combinations but for R17 we think it should restrict to max PC2 per band cases. PC3+PC5, PC3+PC3, PC3+PC2, PC2+PC5, PC2+PC2 (with PC2 inter-band power class). |
| Verizon | Issue #1: This work should consider the solution which could be in significant simplification and reduction in specification works from what RAN4 have been done. Issue #2: Signaling to network is ok as long as it would be beneficial to inform the network Issue #3: At least PC5+PC5, PC3+PC3, PC2+PC2, PC1.5+PC1.5 could not be part of this work. |
| Huawei | Would like to clarify the issues around P_CMAX_L and P_CMAX_H. Here’s the formula for inter-band CA: It can be seen that if a 23dBm+26 dBm UE reports CA power of 26 dBm (PC2), the P_CMAX_L won’t increase. But if it reports CA power of 27.8 dBm, the P_CMAX_L will increase, which is the natural result of removing the CA power cap. This does not mean tighter requirements for UE, in other words, as long as the UE can Tx 23 and 26 dBm per band, it will meet the increased P_CMAX_L limit. Some companies seems to think that per band P_CMAX is sufficient. However, there’s the requirement of P_EMAX_CA, which limits the total power per UE for potential regulatory compliance. Hence the above formula should not be abandoned. |

Sub topic 1-2 Comparison of options

| Company | Comments |
| --- | --- |
| Huawei | Option 4 was added upon our request. However, the moderator uses his own wording instead of the one we suggested. Unfortunately, the moderator’s wording does not describe our proposal accurately. As per the moderator’s response, please allow us to clarify our view below. So the real option 4 is: Extend the CA power class signalling: signal the CA power configuration from which the max total power is derived. More explicitly, it’s proposed in our contribution that: A UE shall report both the PA configuration (i.e., PPowerClass, c for serving cell c) and the max total output power (i.e., PPowerClass,CA) for a given band combination. It is signaled via the powerClass parameter of the existing BandCombination IE using a newly defined mapping table. We believe that by such a novel signaling design, both the ambiguity of per-band power class during CA and the increase of CA output power can be solved simultaneously. Also this approach is fundamentally different from conventional power classes. |
| Huawei | We’d like to emphasize the necessity of reporting CA power configuration for UEs employing TxD. Failing this, the UE may not pass the conformance test for configured transmitted power. For example, a UE equipped with 2x PC3 Pas may report PC2 on both band A and band B as well as PC2 for CA_A-B. As per current spec, the Test Equipment (TE) may assume P_PowerClass,a=26 dBm, P_PowerClass,b=26 dBm and P_PowerClass,CA=26 dBm. Ignoring P_EMAX, A-MPR, etc, the lower bound of the CA output power P_CMAX_L may be simplified to: PCMAX_L=min10*log10pPowerClass,amprPC2+pPowerClass,bmprPC2,PPowerClass,CA =min10*log102*pPC2mprPC2,PPowerClass,CA =min29dBm-MPRPC2,26 For most RB allocations, the MPR_PC2 will be < 3 dB. Hence P_CMAX_L becomes 26 dBm, and P_CMAX_H = 26 dBm. As a result, the P_CMAX requirement becomes too stringent for the UE since no MPR is allowed. By contrast, if the UE reports the CA configuration PC3+PC3 to the TE/network, 23 dBm and PC3 MPR will be applied in the above calculation for P_CMAX_L (=min{26-MPR_PC3, 26}). Note that the same problem exists for Ues using 2xPC2 Pas for PC1.5 and TxD is the only way to support PC1.5 up to R17. In summary, our proposal is to report PPowerClass, c for each band/serving cell and the max total output power PPowerClass,CA (could be the sum of PPowerClass, c). But there’s no extra burden for the UE, since the UE still reports a single value via the powerClass parameter of the BandCombination IE. This value is an index to a mapping table which defines the CA power configuration and the max total power. In short, one stone two birds. |
| OPPO | The option 2 conventional power class is always the safe approach that can be fall back to if other more advance approach cannot be agreed. The option 1 generally is ok in our view, but to solve the issue of TxD UE power capability difference in single band and in band combinations, a per band combination UE capability needs to be introduced to make it clear whether max power of this band combination can be summed from each band power class. Besides, to enable the 23+26 PA configurations for PC2 a complete set of requirements should be defined, like the MPR/AMPR, MSD, etc. Just change the Pcmax higher limit might not be enough especially MPR since the MPR is defined according to 23+23 or 26+26 PA configurations. |
| Xiaomi | If no traditional CA/DC power class is introduced, a clarification on how to define MOP requirements should be needed. And If both Pcmax_L and Pcmax_H is increasing, the MSD for inter-band CA shall be evaluated according to our paper 7751. |
| ZTE | “The sum method” is our preference. We think introducing new power classes would cause PC fragments but with litter benefit. Instead, option 1 is more general which can be more flexible to capture all the cases. For ‘A conceptual power class (call it power class 0)’, it may need to exclude some power allocation cases as discussed in issue #3 under Sub-topic 1-1, and it may unclear that the fallback ignalli between PC2 and PC0/PC3 when the duty cycle side conditions are not met. For regulatory issues, in currently specification the Tx requirements such as MPR/A-MPR, SEM, spurious emission etc, are defined per uplink component carrier for inter-band CA. Therefore, per band requirement can also be applied for the UE with a new feature to increase UE maximum power high limit for NR uplink inter band CA. So there might no regulatory issues. For the new MSD, we think it is needed due to the restriction of total power is lifted. Also we give initial evaluation result for the new MSD value for 23+26dBm power allocation in our paper 8183. For the SAR, we think the existing SAR mechanisms P-MPR and duty cycle are sufficient. However, some texts/wordings might need to be corrected which are FFS for now. In addition we think the work level is high considering RAN4 have already spent lots of meeting times to discuss it, and it may not proper to continue to discuss it in rel-18 due to the long break time period. |
| MTK | Regarding the aspect 4, (What is the impact to a UE that requires TxD for a single CC power class when it is configured for CA or DC) as our analysis in the R4-2117863, a new signaling would be needed as we proposed regardless how the UE MOP of the combo is reported. |
| Qualcomm | We are not aware of any regulatory impacts for any of these options. In case of total power constraints, Pemax can be signaled by the network. New MSD would be required in case Pcmax_L is elevated for example with a new power class. If the sum method is used for Pcmax_H only, then new MSD is not needed. SAR mechanism of P-MPR is always available and duty cycle allows the UE to signal among capability among a set of predefined values. The inaccuracy of the duty cycle report does not justify creating new values to accommodate the increases in power proposed here. For a UE that requires TxD in single carrier, if the Pcmax_L is not elevated, then there is no impact to this UE. For example, the UE supports (23+23 TxD) on one carrier and 23 on the other carrier. When aggregated since TxD is not available on the first carrier, then only 26 dBm CA total power is available instead of 27.8 dBm. If the sum method only affects Pcmax_H, then there the TxD UE is still in compliance. If there is a capability defined for increased power, the TxD UE would not signal it. The new power class proposals or the enhanced power class proposal from Huawei would lead to a very large workload since these require new power class, new MPR, new MSD for many possible power class combinations. The specification will become bloated with an excessive number of power classes, each possible PA configuration, and unique MPR, MSD specs for each of these. |
| SoftBank | Regarding the regulatory point of view, the current MOP requirements (defined by sum of powers in the bands) may be referred. If some options impact the current MOP requirements, the framework for prohibiting those new options in the network is needed. |
| Apple | In our view, Option 1 and Option 3 are more similar approach. However, if the intent is to max out each band’s power ability, the sum of the power class for the combined Pcmax limit is just a redundant step. For inter-band UL CA/DC, if there is no constraint on the total power on UE side such that no power sharing is needed, then the network only needs to know Pcmax from each band. The combined Pcmax is essentially of no use to each network cell. From signaling aspect, both Option 1 and Option 3 need a capability signaling to differentiate with the conventional power class definition. Option 1 proposes a new IE which has no relation with power class. So it is not clear whether CA power class still needs to be signaled or not. If power class was not signaled, would the combination still be considered as PC3 (default)? Option 3 proposes a conceptual power class (such as power class 0) which is still within the power class definition. So there would not be confusion with the existing CA power classes. Power class 0 is not tied to a particular combined power value, instead it is used to indicate that all the UL requirements are per band based. Option 2 and Option 4 are basically the same approach as with the conventional inter-band CA power class definition. Option 4 proposes a new Rel-17 IE for CA power class. The numeric expression for each combined power rating may create confusion to the existing power class definition. In addition to the new signaling to indicate the support of maximum output power from each band, all options (including the existing CA power classes) also need a new signaling for NR band power class under CA/DC (EN-DC already has such IE) as indicated in MediaTek’s contribution R4-2117863 and mentioned by several companies. It is to accommodate 2Tx implementation on single band which may fall back to 1Tx when operating under CA/DC. And in our view, this new IE field is only signaled when the NR band power class under CA/DC is different from its single band power class. On the emission aspect, for inter-band UL CA/DC combinations, in most cases the bands are well isolated. Therefore, the interaction between the Pas (such as reversed IMD) have not been identified as a concern and no MPR/A-MPR for inter-band combinations were seen needed as commented by ZTE. On the other hand, the regulatory requirements have been defined as per-band based. That also implies the per-band based UL requirements as proposed in Option 3 bodes well as long as there is no total power constraint on the UE side. Lastly, regarding the MSD due to 2UL transmissions, in our view, the requirements are meant to verify the PA linearity, filter isolation, as well as receiver linearity performance. These performance can already be verified by the PC3 and PC2 UL CA requirements. There is no need to further define separate MSD requirements with different UL CA power compositions other than PC2 and PC3. |
| Vivo | Since most of regulation is per-band, when the MOP of single band compliant with the regulation, increasing the upper limit of CA with the sum should be OK. Considering TxD for a single CC, if the power class of single band may downgrade from PC2 to PC3 after CA configuration because of no full-power PA, an optional signaling for increasing upper limit of MOP can fix it. For example, UE with PC2 in band A (23+23dBm PA implementation) and PC3 in band B, UE cannot reach 27.8dBm with maximum 2 Tx restriction for band A+B inter-band CA. In this case, UE can choose not to report the optional signaling, thus not supporting the sum method for this band combination. |
| Nokia | Regulatory issues The 3rd method has a problem since requirements to ensure the total power is capped by a certain limit are required. In addition, the UE with the 3rd approach cannot utilize p-UE-FR1 and p-NR-FR1. If the 3rd approach was accepted, specifications would not need all the total power related requirements. MSD It depends on proposed CA/DC band configurations. This issue is agnostic to the listed approaches. Whether existing SAR mechanisms sufficient or not We think so. But total duty cycle should be scaled according to the total power. This issue is agnostic to the listed approaches. TxD relation If UE’s power classes for respective bands within a band combination are different between those in signal band operation and those in CA operation mode due to TxD, then, UE should report additional PCs of each band within an UL CA and that is understood to override any legacy (per-band) PC signalling that would limit the power. PA configuration PA configuration is not needed but the power class available in CA mode for each band is necessary. Level of work The amount of work to complete this WI is not that much different among the listed options. But total amount of work in the future to introduce new Power configurations would be different. |
| Skyworks | In our view the best option is based on signaling we each band power capability in the inter-band combination case, then the corresponding maximum power is just a consequence and it not needed to signal explicitly. The main further aspect is how the duty cycle declaration should be understood in this case: i.e is the duty cycle related to the interbank power class declaration or the sum of the declared per band capability For MSD although we are fine not to define an additional MSD for this, we must ensure that the RAN5 up power sequence does not result in a higher power than the inter-band power class so that REFSENS/MSD are not failed by UEs that can reach higher power. |
| CHTTL | We tend to share Softbank’s view that if some options impact the current MOP requirements, the framework for prohibiting those new options in the network is needed. |
| Verizon | This work should be in scope of current regulatory requirements. However, if there is any impact to the current requirements, new options should be considered. |
| Ericsson | Sub-topic 1-2: we propose option 2. No changes of the existing power-class signaling (38.306) nor changes of the configured power. Consider the following aspects - Are there any regulatory issues? No, if there are limitations in a region the cell-specific P-Max and corresponding for a band combination can be used (all UEs should be limited). - Are new MSD requirement needed? This should be governed by the power class indicated per band (if no indication the default applies)? Existence of MSD requirements is not a prerequisite for configuring a UE. Moreover, if the MSD is “large” for a lower power class of a band combination (BC) it is going to be “large” for the higher BC power class - Are the existing SAR mechanisms sufficient? If not, what needs to be changed? The “P-MPR method” can always be used. For duty-cycle reporting the corresponding power class/capability for which this applies can be indicated by the UE. - What is the impact to a UE that requires TxD for a single CC power class when it is configured for CA or DC? The TxD-capable UE should still meet the advertised power class. - What is the level of work required in RAN4? (low, medium, high) Low (in the sense that changes are minimised). For Option 2 possibly limited to MSD for a higher-power band combination. We are supportive of a higher BC power class but within the existing framework. The existing field for BC (note highlight) powerClass, powerClass-v1610 Indicates power class the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. This capability is not applicable to IAB-MT. |
| Huawei | Our proposed solution and the conceptual power class method belong to the same category of the sum method, which can be viewed as variants of the sum method. All of them aim to modify the P_CMAX formula for inter-band CA, and none of them have the burden of conventional power classes, such as MPR, power tolerance, etc. For simplicity, let’s call our proposal the Lookup Table (LUT) method. It’s proposed to define a LUT in the RAN4 spec, and each row of the table contains the configuration of per-band power class, max total power for CA and maybe other info TBD. The exact format could be FFS, but an example table is shown below. Index; Per-band Configuration PPowerClass, c; Total Output Power PPowerClass,CA (dBm). 0; 23dBm + 23dBm; 26. 1; 23dBm + 26dBm; 26. 2; 23dBm + 26dBm; 27.8. …; …; …. The value to be signaled to the network is the index of the table entry. In this way, the network/TE will know both the per-band power class as well as the total CA power which may or may not be increased. Below is a comparison between the LUT-method, the Sum-method and the conceptual power class method (called PC0-method). LUT-method; Sum-method; PC0-method. P_CMAX_L; may be increased, subject to signaling; may be increased, subject to signaling; same as PC2. P_CMAX_H; may be increased, subject to signaling; may be increased, subject to signaling; may be increased, subject to signaling. Report Per-band Power class for CA; Yes; No; No. Signaling Overhead; Reuse existing CA power class IE; conventional CA power class signaling + new UE capability fullPowerUL-CA; conventional PC2/PC3 signaling for P_CMAX_L + new PC0 signaling for P_CMAX_H. MSD impact; can reuse existing PC2 requirements; can reuse existing PC2 requirements; can reuse existing PC2 requirements. |
| DOCOMO | Are there any regulatory issues? We would like to further discuss the necessity of IE indication from NW to UE so that we can prohibit UE enabling this feature of increasing MOP under NW not indicating the said IE, which may be captured in an example of required changes for configured output power described in R4-2117052, while this issue may be addressed by P-max. |

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

### Sub-topic 1-1 General framework

To enable increased maximum output power, it is clear that Pcmax_H should be increased.  However, both views were expressed on whether Pcmax_L should also be increased.  It was generally understood that increasing Pcmax_L may have implication to MPR/A-MPR/MSD.

All companies believe it would be beneficial or necessary to signal to the network.  The details of signaling still need to be resolved.

Most companies believed that the scope could be limited to PC5+PC3, PC3+PC2, and PC2+PC1.5 for Rel-17.

### Sub-topic 1-2 Comparison of options

Except for one company, all other companies did not state a strong preference or preferred not to define a new conventional power class.  Many companies preferred the option 1 ”sum method”, while the proponent of option 3 ”conceptual power class” noted its similarity to option 1 and the proponent of option 4 ”LUT method” also categorized it as a variant of the sum method.

The following views on specific points were expressed

Regulatory:  In general, regulatory requirements are per band so there are not expected to be any issues.  However, in some countries there may be regulatory limits on total power.  So, the Pemax_CA (or equivalent mechanism) needs to be in place to limit total power.

MSD:  Three options are available

1. Pcmax_L is not increased, therefore, existing MSD can still apply

2. Pcmax_L is increased, new MSD needs to be evaluated

3. MSD is not needed for the higher power configuration, since the linearity is already verified at a lower power setpoint

SAR:  Existing mechanisms of P-MPR and duty cycle reporting are sufficient.  Some wording change may be needed.

TxD UE:  Two options are available

1. Since signaling is available, then the TxD UE would not signal the "increased power"

2. Extend signaling (for example, LUT method).

Moderator note: the example problem that Huawei brings up in round 1 discussion is not related to increasing power.  That same problem exists even for PC2 today, so it should be treated generally, not as part of this work item.

Level of work:  If Pcmax_L is not increased, the work load is small.  But if Pcmax_L is increased, then there may be considerable work for MPR/A-MPR/MSD.

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Signaling is needed Scope is limited to PC5+PC3, PC3+PC2, and PC2+PC1.5 for Rel-17 where the inter-band CA/DC power class is nominally PC3, PC2, and PC1.5, respectively. Scalability for future configurations should be considered. The “sum method” or some variant of it shall be pursued. New power classes using the conventional power class is not agreed. Regulatory: In general, regulatory requirements are per band so there are not expected to be any issues. However, in some countries there may be regulatory limits on total power. So, the Pemax_CA (or equivalent mechanism) needs to be in place to limit total power. SAR: Existing mechanisms of P-MPR and duty cycle reporting are sufficient. Some wording change may be needed. Candidate options: Pcmax_L: Three options are available - Pcmax_L is not increased and MPR/A-MPR/MSD can still apply - Pcmax_L is not increased, but need to re-evaluate MPR/A-MPR/MSD - Pcmax_L is increased and need to re-evaluate MPR/A-MPR/MSD MSD: Three options are available 1. Pcmax_L is not increased, therefore, existing MSD can still apply 2. Pcmax_L is increased, new MSD needs to be evaluated 3. MSD is not needed for the higher power configuration, since the linearity is already verified at a lower power setpoint TxD UE: Two options are available 1. Since signaling is available, then the TxD UE would not signal the "increased power" 2. Extend signaling (for example, LUT method). Recommendations for 2nd round: Capture agreements in a WF. Discuss and resolve issues and candidate options listed above. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
| XXX | Pcmax_L: MSD: TxD UE: |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on increasing MOP for CA/DC | Qualcomm Incorporated |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2117052 | UE RF requirements specific to the sum method | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2117053 | Feasibility of the sum method for increasing UE power high limit for CA/DC | Nokia, Nokia Shanghai Bell | Withdrawn. Revised to R4-2118726. |  |
| R4-2117751 | Discussion on increasing UE maximum power high limit | Xiaomi | Noted |  |
| R4-2117863 | Further discussion on capability ignalling for HPUE NR DC | MediaTek Inc. | Noted |  |
| R4-2117864 | Draft LS on signalling clarification of NR CA/DC power class in R17 | MediaTek Inc. | Noted |  |
| R4-2117988 | UE maximum output power for inter-band UL CA | Apple | Noted |  |
| R4-2118183 | Discussion on increase UE maximum power for NR uplink inter band CA | ZTE Corporation | Noted |  |
| R4-2118292 | Discussion on the increasing UE power high limit for CA and DC | vivo | Noted |  |
| R4-2118547 | Discussion on Increasing UE power high limit for CA and DC | Huawei,HiSilicon | Noted |  |
| R4-2118605 | Discussion on impact of increasing UE high limit for CA and DC | Mediatek India Technology Pvt. | Noted |  |
| R4-2118726 | Feasibility of the sum method for increasing UE power high limit for CA/DC | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2118888 | R17 Discussion on UE power class high limit | OPPO | Noted |  |
| R4-2119434 | Increasing MOP for CA and DC | Qualcomm Incorporated | Noted |  |
| R4-2119441 | Handling of maximum output power limit for PC2 inter-band combinations | Skyworks Solutions Inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | James Wang | fucheng_wang@apple.com |
| Vivo | Ziqi Liu | liuziqi@vivo.com |
| Ericsson (‘Ericsson2’) | Christian Bergljung | Christian.Bergljung@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)