3GPP TSG-RAN WG4 Meeting # 101-e 													   R4-2119732

Electronic Meeting, 1st – 12th November 2021

Agenda item:			8.19.1, 8.19.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [101-e][132] NR_feMIMO

Document for:	Information

Introduction

In RAN4#100-e, the discussion on the Rel-17 FeMIMO was continued to see if any impact to RAN4 RF requirements based on the discussion progress of RAN1 on the work item. A WF on the impact to the RF specification was approved during the last meeting of RAN4.

Based on the WF, RAN4 is supposed to resume the discussion about the RF impact to the remaining FFS topics in this meeting again given the status of other WGs. This email thread aims to finalize the common understanding of the RF impact to the RAN4 specification.

Therefore, the discussion summary will try to derive the tentative agreements based on the comments provided in the 1st round discussion. A WF to capture the consensus of the meeting will be discussed during the 2nd round.

Topic #1: Impact to UE RF requirements

Companies’ contributions summary

| T-doc number Sub-topic | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117844 Multi-panel reception, MPE, SRS | Samsung | Proposal 1: No RAN4 requirement assuming simultaneous reception channel/RS with different QCL type D is specified in Rel-17. Proposal 2: No RF impact is identified for the multi-panel UE in Rel-17. Observation 1: It is reasonable for a UE to assume the worst case by reporting the largest P-MPR value only. Observation 2: RAN4 can modify the current note of the P-MPR, or add relevant UE capabilities based on the enhancement in RAN1 without any change of the PCMAX,f,c definition. Observation 3: It would be meaningless without other core requirements for 8 antenna ports Proposal 3: No requirement for 8Rx SRS ports is specified unless a discussion about its full set of requirements is initiated in RAN4. Proposal 4: No RF impact is identified for the SRS enhancement in Rel-17. |
| R4-2118288 Multi-panel reception | vivo | Observation 1: The reference sensitively power level is not likely to be improved by multi-panel reception, since multi-panel implementation is targeted for spherical coverage. Observation 2 (Supplemented): According to simulation, the gain introduced by simultaneous operation and combination of 2 panels for spherical coverage can be negligible, compared to active one panel at a time. Even in more favourable (not likely typical) condition e.g. plastic back/side, the performance difference can still be small, e.g. 1dB etc. Observation 3: It is also safe to reuse current minimum requirements for multi-panel reception for EIS spherical coverage since it is the baseline performance. Observation 4: Other receiver requirements are not impacted by multi-panel reception. Observation 5 (New): Multi-panel simultaneous Rx is non-configurable, and even likely not to be reported which is still not confirmed yet. Thus make the testability quite challenging even if some merits of testing can be found. Proposal: Do not define requirements for UE supporting multi-panel simultaneous reception. |
| R4-2118289 MPE | vivo | Observation 1: RAN1 has extend the number of P-MPR from 1 to N (4 maximum) that can be reported by a UE. Observation 2: One P-MPR would be reported with one corresponding SSBRI/CRI, this means the reported P-MPR is actually a per-beam value. Observation 3: No per-panel or per-beam P-MPR was explicit mentioned, but neither implementations are precluded. Observation 4: The criteria of selection of SSBRI(s)/CRI(s) is still not concluded in RAN1. Observation 5: Conceptually the newly introduced multiple P-MPRs can be extended to current RAN4 requirements equation. Observation 6: P-MPR range is likely to be the same to Rel-16. Observation 7: No need to verify P-MPR selection criteria since it is pretty much implantation dependent. Proposal: RAN4 requirements is not to be impacted by MPE scheme, apart from possible editorial/concept clarifications. |
| R4-2118782 Multi-panel reception | Qualcomm | Observation 1: RF requirements for dual AoA reception are needed to guarantee a minimum usability of the feature. Observation 2. A statistical requirement considering pairs of angles should be able to guarantee a certain level of minimum performance while not imposing implementation constraints. Observation 3: RAN4 will need to introduce a metric to ensure that the correlation between beam pairs used to receive simultaneously is low enough. |
| R4-2119091 Multi-panel reception, MPE, SRS | Huawei, HiSilicon | Proposal 1: No need to specify simultaneous multi-panel reception requirements in Rel-17. Proposal 2: RAN4 could hold the discussion on the potential impact on MPE mitigation until the final clear scheme is provided by RAN1. Proposal 3: Wait until new WI on the whole package of core requirements for 8 Rx UE. |
| R4-2119119 Multi-panel reception | ZTE | Proposal 1: Not need to enhance receiver requirement for multi-panel reception unless the necessity can be found. |
| R4-2119120 MPE | ZTE | Observation 1: For Rel-16 P-MPR report, which defers to the current UL beam used to transmit the actual UL transmission. Proposal 1: Whether all FR2 UE support the enhanced proximity detection capability, which should be discussed. Proposal 2: Maybe a UE capability is needed unless all FR2 UE can support enhanced proximity detection. |
| R4-2119121 SRS | ZTE | Proposal 1: For RF enhancement, considering for SRS switching for up to 8 antennas, only possible IL enhancement should be further determined. |

Open issues summary

Based on the latest approved WF and submitted contributions, each sub-topic to conclude the impact to RF requirements can be organized as follows:

- Sub topic 1-1: Additional requirement for multi-panel reception

- Sub topic 1-2: Impact of MPE enhancements

- Sub topic 1-3: SRS related impact

Sub-topic 1-1: Additional requirement for multi-panel reception

At the last meeting, RAN4 continued to check whether the multi-panel reception due to the multi-TRP operation has an impact to the current reception requirements of RAN4. Although companies discussed two different options during the meeting, on neither was an agreement reached. Hence, it was agreed that RAN4 would further determine if additional requirement is required for the multi-panel reception as follows [1].

- RAN4 will further determine if additional requirement is required for the multi-panel reception - The RF impact for the multi-panel UE will be discussed in in RAN4#101-e based on the contribution - Aim is to decide on the impact in RAN4#101-e given the time limitation

Following proposals are provided given the input contributions to in this meeting.

Issue 1-1-1: Need for additional requirements

- Proposals

  - Option 1: Conclude the discussion without additional requirement in Rel-17

  - Option 2: Continue the discussion to derive additional requirement in Rel-17

- Recommended WF

  - Collecting companies view

  - Postpone the discussion to future releases if consensus cannot be reached between the options

Sub-topic 1-2: Impact of MPE enhancements

On the impact of the MPE enhancement, at the last meeting, the common understanding was that RAN4 should wait for the concrete MPE solution in RAN1 before making a decision on the UE RF impact. After the meeting, RAN1 made the agreement on the MPE mitigation enhancement on top of the RAN4 scheme of Rel-16. In this meeting, moderator suggests discussing the possible impact to the existing RAN4 specs as proposed in input contributions.

Issue 1-2-1: Need for updates of per-beam or per-panel P-MPR

- Proposals

  - Option 1: P-MPR definition update

  - Option 2: Nothing

- Recommended WF

  - Collecting companies view

Issue 1-2-2: Need for additional capabilities

- Proposals

  - Option 1: Proximity detection

  - Option 2: Nothing

- Recommended WF

  - Collecting companies view

Sub-topic 1-3: SRS related impact

Discussion about the impact on the SRS enhancement was also continued whether RAN4 should wait until the full set of requirements for 8 antenna ports before having a requirement only about the SRS assumption. Since the consensus was not achievable in the last meeting, following agreements were made for further discussion.

- RAN4 will further discuss if there is any SRS related impact in Rel-17 - It will be concluded in RAN4#101-e based on the discussion given the time limitation

Sub-topic 1-3 is to continue the discussion for the consensus on the impact for the SRS enhancement.

Issue 1-3-1: Impact of SRS switching for up to 8 antennas

- Proposals

  - Option 1: Conclude the discussion in Rel-17

  - Option 2: Continue for IL enhancement

- Recommended WF

  - Collecting companies view

  - Postpone the discussion to future releases if consensus cannot be reached between the options

Companies views’ collection for 1st round

Open issues

Sub topic 1-1: Additional requirement for multi-panel reception (Issue 1-1-1)

| Company | Comments |
| --- | --- |
| Huawei | Option 1 is our preference. |
| vivo | Option 1. Dual AoA test considering for multi-TRP scenario for FR2 have testability issues, and have a great impact for existing RF test system and method. No RF tests have considered multiple AoA, and even for RRM the existing consideration was simple. It is proposed that multiple AoA would not be considered at least for Rel-17. |
| Qualcomm | Option 2, we should at least discuss what requirements are needed. So far, companies supporting Option 1 have not provided any technical arguments, only argument so far is that this is too complicated. It seems that companies are just interested in closing the WI without doing the actual work needed to have a useful feature. We fully agree that there is impact also on the test system, this is the reason that we should start discussing also testing related issues so the problems can be solved. |
| MediaTek | Option1. The key components of multi-panel reception are re-used and tested. |
| Apple | We are OK not to specify new requirements in R17. However, we also concern to directly extend the existing requirements to multi-panel reception either. Since the related work is currently considered under R18 scope discussion, we can postpone it to future release. |
| Nokia | Option 2: Continue the discussion to derive additional requirement in Rel-17 Apart from requirements to be specified in Rel-17 or not, at least it is necessary to continue and conclude to clarify what test(s) and performance requirements we need to specify to make sure that UE are simultaneously receiving different beams from different TRPs. |
| Samsung | Our proposal is Option 1 that no additional requirement is necessary given the background of Rx requirement in FR2. We see that it also makes sense to reuse the current Rx requirements from the RF requirement’s point of view. However, as QC/Nokia mentioned, we are OK to continue the discussion on the impact like the requirement enhancement or 2 AoA testability for the future release as long as the requirement is not specified in Rel-17. |
| ZTE | Support Option 1. The operation of UE is very similar between multi-panel receiving for multi-TRP scenario and for co-located inter-band CA with IBM case. So the discuss of EIS spherical coverage requirement enhancement during Rel-16 inter-band CA can be as a good reference to our discussion here. If single AoA is still assumed in test for multi-panel receiving, similar as inter-band CA in Rel-16, no need to enhance the requirement of EIS sphere coverage. |
| OPPO | Option 1. |
| Intel | Option 1. Prefer to postpone to Rel-18. Support of multi-panel reception would require cross-area work for RF/RRM/Demod and also support of corresponding OTA test methods. Given the late stage of Rel-17 we do not think it is realistic to complete it within Rel-17 timeframe. |

Sub topic 1-2: Impact of MPE enhancements (Issue 1-2-1 and 1-2-2)

| Company | Comments |
| --- | --- |
| Huawei | For Issue 1-2-1, we feel no need to add beam/panel index into the definition of PCMAX,f,c, maybe just some clarification is enough. Because for the UE which is capable of reporting per-beam/panel P-MPR, the actual beam that such UE will use is indicated by gNB, then such UE can follow the current spec to configure the PCMAX,f,c using the P-MPR of that beam. For Issue 1-2-2, we think such additional UE capability on proximity detection is not needed. From gNB perspective, per-beam/panel P-MPR reporting is enough for gNB to decide the best beam. From UE perspective, it is an implementation issue regardless whether the UE is capable of per beam/panel P-MPR report or not. Thus we prefer Option 2. |
| vivo | Issue 1-2-1: Option 2. The P-MPRf,c current used in PCMAX,f,c does not need to be extended and assuming worst case P-MPR would still well serve the equation. In addition, RAN1 actually did not introduce specific “per-panel” or “per-beam” concept, thus also making it difficult to make concept extension for P-MPR. Furthermore, P-MPR does not have specific requirements, and multiple sets of equations and requirements for P-MPR adaptation seems useless since no way for verification in conformance tests. Anyway, network can still use the reported RS and corresponding P-MPR in the field without further RAN4 requirements. Issue 1-2-2: Since the whole scheme was discussed in RAN1 and still continuing, RAN4 should not discuss this and this topic should leave to RAN1. |
| Qualcomm | Issue 1-2-1: Option 2. We do not see any need to update the P-MPR definition because even now P-MPR will change depending on the beam that the UE is using and it can change anytime without the gNB knowing about it. Issue 1-2-2: We do not see any need to do anything, proximity detection or any other technique is purely implementation dependent and outside 3GPP specs. |
| Apple | Issue 1-2-1: as what RAN1 has agreed, UE may need to report multiple PMPR with associated SSBRI/CRI. We need to check if the existing definition/signaling can enable the new enhancement. Issue 1-2-2: proximity detection capability is under discussion in FR2 enhancement WI, e.g. UL gap for BPS. If issues are identified, a joint discussion can be done. At this stage, we don’t additional capability is needed specifically for multiple PMPR feature. |
| Nokia | Issue 1-2-1 Option 1. The title of Issue 1-2-1 may look odd. In our understanding, RAN1 agreement says P-MPR per beam while it does not say anything P-MPR per panel. Hence, how virtual P-MPR per beam is utilized in configured power formula shall be clarified. Issue 1-2-2 Neither. We would define a capability if a UE can report virtual PHR or not, I guess. The capability based on a hardware capability is not needed for the purpose of MPE for Rel-17 FeMIMO since to report virtual PHR, not only hardware but also BB needs to deal with it. We don’t think proximity detection capability is not related to this discussion. |
| Samsung | Issue 1-2-1: Option 2. We see no reason for the change of the P-MPR related definition given that the current wording itself does not limit the UE operation of Rel-17 MPE enhancement. Even though a UE has multiple P-MPR numbers, it does not have an impact to the configured power. Issue 1-2-2: We do not believe that the proximity detection capability is necessary for the Rel-17 MPE enhancement since it is essential one for P-MPR as noted in the spec already. The additional capabilities for the N P-MPRs might be needed in the future but we think it is up to other WGs. |
| ZTE | Issue 1-2-1: Support Option 2. Even the per-UE P-MPR would be enhanced to per-beam/panel P-MPR, RAN1 would give concrete solution of determining the mapping between P-MPRs and beams/panels. From RAN4’s perspective, no need any enhancement. Issue 1-2-2: Proximity detection is under discussion in FR2 UL gap topic. Based on the discussion under that topic, UE can execute proximity detection with or without specific UL gap configuration. So we believe proximity detection is fundamental and essential for the UE operation in FR2, not need any UE capability. |
| OPPO | Option 2 for both issues. |

Sub topic 1-3: SRS related impact (Issue 1-3-1)

| Company | Comments |
| --- | --- |
| Huawei | We feel a general 8Rx requirements related WI should be established first, then 8Rx SRS issue could be discussed after. Considering the limited time in Rel-17, we prefer Option 1 on this issue. |
| vivo | Option 1. 8Rx SRS should be discussed after general 8Rx requirements are set. No further work proposed for Rel-17. |
| Qualcomm | We believe none of the options is really suitable. WE can at most conclude that nothing will be done in Rel.17 but requirements will be added when 8Rx requirements are introduced. It would be useful to have a discussion about what exact requirements should be defined so that we know what to do in a future release when 8Rx reqs will be defined. |
| Apple | Agree to postpone this work until 8Rx feature/requirements becomes available. |
| Nokia | Neither. Apart from requirements to be specified in Rel-17 or not, at least it is necessary to continue and conclude to clarify what we need to specify to utilize SRS enhancement defined in RAN1 in Rel-17. And the outcome should be included in a summary of Rel-17 Fe-MIMO WI when it is completed. |
| Samsung | We prefer Option 1. We do not see the benefit to continue the discussion on this topic even though we will concentrate our efforts on finding the impact more. No proposal is submitted on the impact except IL so far. However, we are OK to continue the discussion only for the future release discussion. |
| OPPO | Option 1. Can be discussed when 8Rx UEs are introduced. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | 1st round summary: <Issue 1-1-1: Need for additional requirements> o Option 1: Conclude the discussion without additional requirement in Rel-17 (Huawei, vivo, MediaTek, Apple, Samsung, ZTE, OPPO, Intel) o Option 2: Continue the discussion to derive additional requirement in Rel-17 (Qualcomm, Nokia) No consensus has been made given the comments provided during the 1st round. However, most companies support Option 1, not to specify the additional requirement for the multi-panel reception in Rel-17. They also prefer to postpone the discussion to Rel-18 with other impacts like test system. On the other hand, the proponents of Option 2 also share that the discussion can be continued to find out possible implications for the future discussions at least. From moderator’s perspective, it would be meaningless to take up the time of Rel-17 by repeating arguments about whether to introduce the requirement or not. Instead, it is better to concentrate our efforts on finding the impact for the requirement while making a decision on the requirement in this meeting. Based on the WF of the last meeting and the situation above, moderator would like to suggest compromised options to move forward. Tentative agreements: None Candidate options: RAN4 will not specify additional RF requirements for the multi-panel reception in Rel-17. For the future release, discussion can be continued to find out the problem/impact to verify the simultaneous reception from multi-TRPs. For example: - Test system and environment - Type of the requirement (e.g., R4-2118782) Recommendations for 2nd round: Draft WF will be shared for the 2nd round discussion. Companies are encouraged to provide comments to the WF directly. |
| Sub-topic #2 | 1st round summary: <Issue 1-2-1: Need for updates of per-beam or per-panel P-MPR> o Option 1: P-MPR definition update (Apple, Nokia) o Option 2: Nothing (Huawei, vivo, Qualcomm, Samsung, ZTE, OPPO) <Issue 1-2-2: Need for additional capabilities> o Option 1: Proximity detection (none) o Option 2: Nothing (Huawei, vivo, Qualcomm, Apple, Nokia, Samsung, ZTE, OPPO) Most companies support Option 2, not to make changes to the existing P-MPR related definition/capabilities for the Rel-17 MPE enhancements. On the other hand, the proponents of Option 1 share that RAN4 needs further check if the current definition/capability could accommodate the enhancement such as multiple P-MPR, virtual P-MPR. Based on the WF of the last meeting and the situation above, moderator would like to encourage companies to provide the detailed solution if changes are required to the next meeting. Tentative agreements: None Candidate options: RAN4 will further check if the current definition/capability could accommodate the enhancement until the next meeting. For the next meeting, companies are encouraged to provide the detailed solution for the RAN4 specs if changes are required. Recommendations for 2nd round: Draft WF will be shared for the 2nd round discussion. Companies are encouraged to provide comments to the WF directly. |
| Sub-topic #3 | 1st round summary: <Issue 1-3-1: Impact of SRS switching for up to 8 antennas> o Option 1: Conclude the discussion in Rel-17 (Huawei, vivo, Apple, Samsung, OPPO) o Option 2: Continue for IL enhancement (Qualcomm, Nokia) No consensus has been made given the comments provided during the 1st round. However, companies supporting Option 2 also share that the discussion at least can be continued to find out possible implications for the future discussion even though the requirement is postponed until general 8Rx requirements are introduced. From moderator’s perspective, it would be meaningless to take up the time of Rel-17 by repeating arguments about whether to introduce the requirement or not. Instead, it is better to concentrate our efforts on finding the impact for the SRS related requirement while making a decision on the requirement in this meeting. Based on the WF of the last meeting and the situation above, moderator would like to suggest compromised options to move forward. Tentative agreements: None Candidate options: RAN4 will not consider the SRS enhancements, i.e., xT8R, in Rel-17. For the future release, discussion can be continued to find out the impact to utilize the SRS enhancement. For the next meeting, companies are encouraged to provide the detailed requirement, which will be specified with general 8Rx requirements. Recommendations for 2nd round: Draft WF will be shared for the 2nd round discussion. Companies are encouraged to provide comments to the WF directly. |

Discussion on 2nd round

Summary for 2nd round

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on FeMIMO RF | Samsung | WF is supposed to capture the agreements and what to be discussed in the next meeting |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117844 | Views on RF requirements for further enhancements on MIMO | Samsung | Noted |  |
| R4-2118288 | Further discussion on impact of multi-panel reception requirements | vivo | Noted |  |
| R4-2118289 | Further discussion on impact of MPE requirements | vivo | Noted |  |
| R4-21118782 | RF Requirements for Multi-panel in FR2 | Qualcomm | Noted |  |
| R4-2119091 | RF requirements for further enhancements on MIMO | Huawei, HiSilicon | Noted |  |
| R4-2119119 | Discussion on Additional requirement for multi-panel reception | ZTE | Noted |  |
| R4-2119120 | Discussion on Impact of MPE enhancements | ZTE | Noted |  |
| R4-2119121 | Discussion on SRS related impact | ZTE | Noted |  |

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |