3GPP TSG RAN WG4 Meeting #101-e	R4-2119812

Online, 1st – 12th Nov. 2021

Agenda item:	7.14.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-21-11: EN-DC_8-41_n3

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_8-41_n3 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC. The MSD exception value needs to be updated to complete the combo.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

3.  Co-existence

When Uplink EN-DC configuration is DC_8A_n3A, (1) IMD2/3 of (B8 - n3) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n3A, (2) IMD2/3 of (B41 - n3) will fall into Rx band of Band 8.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B8+B41 | B8+n3 | B41+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.3 | 0.3 |  | 0.3 |
| B41 | 0.3 |  | 0.33 | 0.33 |
|  |  |  | 0.84 | 0.84 |
| n3 |  | 0.3 | 0.5 | 0.5 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |  |  |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B8+B41 | B8+n3 | B41+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0 | 0 |  | 0 |
| B41 | 0 |  | 03 | 03 |
|  |  |  | 0.54 | 0.54 |
| n3 |  | 0 | 0 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |  |  |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0.3 |
|  | 41 | 0.33/0.84 |
|  | n3 | 0.5 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0 |
|  | 41 | 03/0.54 |
|  | n3 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

5.  Reference sensitivity exceptions

As mentioned in section 3, REFSENS exceptions due to IMD2/3 are expected. Based on the MSD of CA_n3_n18_n41 and DC_3_n28-n41, the following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n3A DC_8A-41C_n3A | 8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | 41 | 2665 | 5 | 25 | 2665 | 27.4 | TDD | IMD21 |
| DC_8A-41A_n3A DC_8A-41C_n3A | 41 | 2665 | 5 | 25 | 2665 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 28.9 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.  Conclusion

This paper proposes study results of EN-DC 8-41_n3.

TP is shown in Annex.

7.  Reference

[1]  RP-212174  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_8-41_n3

### 5.x.1	Configurations for DC_8-41_n3

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n3A, (1) IMD2/3 of (B8 - n3) will fall into Rx band of Band 41. When Uplink EN-DC configuration is DC_41A_n3A, (2) IMD2/3 of (B41 - n3) will fall into Rx band of Band 8.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0.3 |
|  | 41 | 0.33/0.84 |
|  | n3 | 0.5 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n3 | 8 | 0 |
|  | 41 | 03/0.54 |
|  | n3 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. |  |  |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD2/3 of B8 and n3 to Band41 Rx and IMD2/3 of B41 and n3 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n3A DC_8A-41C_n3A | 8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | 41 | 2665 | 5 | 25 | 2665 | 27.4 | TDD | IMD21 |
| DC_8A-41A_n3A DC_8A-41C_n3A | 41 | 2665 | 5 | 25 | 2665 | N/A | TDD | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 28.9 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]