3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2119819

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-11: DC_n77A_1A/ DC_n77(2A)_1A

Agenda Item:	7.13

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR band n3 and LTE band1 for TR 37.717-11-11[1].

# Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.4.0

# Text Proposal

----- Start of TP -----

### 6.1.x	DC_n77_1

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n77_1 | 1 | n77 | DC_n77_1 |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A | 1A | n77A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n77A_1A | 23 | +2/-3 |

#### 6.1.x.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n77A_1A are the same with DC_1A_n77A, which have been already defined in the TS38.101-3 spec.

#### 6.1.x.4	MSD analysis for DC

The MSD analysis for DC_n77A_1A are the same with DC_1A_n77A, as given in TR 37.863-01-01_V2.0.0:

- 2nd harmonics of band 1 fall on n77 DL range.

- IMD2 and IMD4 fall on band 1 DL range.

#### 6.1.x.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n77A_1A are the same with DC_1A_n77A, which have already been defined in the TS38.101-3 spec.

#### 6.1.x.6		Self-interference analysis

The analysis for DC_n77A_1A (and DC_n77(2A)_1A) are the same with DC_1A_n77A, as given in TR 37.863-01-01_V2.0.0.

#### 6.1.x.7	OOB blocking exception requirements

No OOB blocking exception for DC_n77A_1A.

----- End of TP -----