3GPP TSG-RAN WG4 Meeting # 101-e                                                            R4-2119879

Electronic Meeting, 1– 12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1 to specify MSD due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BCS4-Core |  |  |  |  |  | Date: |  |  | 2021-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN4#99 meeting, the MSD due to Tx non-linearity interference in 1st and 2nd adjacent channel of UL band is identified. - Case 1: Tx ACLR1 is overlapping with the Rx DL channel. - Case 2: Tx ACLR2 is overlapping with the Rx DL channel without Tx ACLR1 overlapping. Besides, CA_n1-n3_BCS1 has been introduced into spec. The MSD in Rx band n3 for UL band n1 50MHz need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To specify Reference sensitivity exceptions due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band. - The examplary band combination is CA_n1-n3_BCS1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no definition for REFSENS exceptions due to Tx non-linearities interference in 1st and 2nd adjacent channel of UL band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change1>>

7.3A.7	Reference sensitivity exceptions due to Tx non-linearity interference in 1st or 2nd adjacent channel of UL band for CA

Sensitivity degradation is allowed for a band if it is impacted by Tx non-linearity interference in 1st or 2nd adjacent channel from another UL band part of the same NR CA configuration. Reference sensitivity exceptions for the victim band (DL band) are specified in Table 7.3A.6-1 with uplink configuration of the aggressor band (UL band).

Table 7.3A.7-1: Reference sensitivity exceptions (MSD) due to Tx non-linearity interference in 1st or 2nd adjacent channel of UL band for CA

| UL band | DL band | UL Fc | UL BW | UL RB Allocation | DL Fc | DL BW | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | LCRB | (MHz) | (MHz) | (dB) |
| n1 | n3 | 1945 | 50 | 270 (RBstart=0) | 1877.5 | 5 | [22.9] |
| n40 | n1 | 2350 | 100 | 270 (RBstart=0) | 2167.5 | 5 | [32.7] |

<<End of Change1>>