3GPP TSG-RAN WG4 Meeting #101-e	R4-2119889

Electronic Meeting, 1st – 12th November 2021

Source:	AT&T

Title:	TP for TR 38.842 Addition of CA_n5-n14-n77

Agenda item:			7.31.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.842 to add CA_n5-n14-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n5-n14-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n14A-n77A | - | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.X.2	Maximum output power

Table 5.X.2-1 UE Power Class 2 for One UL Carrier

| CA power class | Carrier n5 power class | Carrier n14 power class | Carrier n77 power class |
| --- | --- | --- | --- |
| 26dBm | 23dBm | 23dBm | 26dBm |

### 5.X.3	REFSENS requirements

MSD requirements for single uplink PC2 are captured in lower order combinations.

---Skipping to Dual Uplink Carrier---

## 6.X	CA_n5-n14-n77

### 6.X.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n14A-n77A | CA_n5A-n77A, CA_n14A-n77A | n5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n5-n77 | CA power class | Carrier n5 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied.

Table 6.x.2-2 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n14-n77 | CA power class | Carrier n14 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n14A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied.

### 6.X.3	REFSENS requirements

According to the PC3 CA_n5A-n14A-n77A study captured in TR 38.717-03-02, the following co-existence issues due to dual uplink were identified:

-	IMD4 and IMD5 products are produced by Band n5 and n14 that might fall in Rx of band n77.

-	IMD4 and IMD5 products are produced by Band n5 and n77 that might fall in Rx of band n14.

-	IMD5 products are produced by Band n14 and n77 might fall in Rx of band n5.

For the IMD products produced by Band n5 and n14 UL, the MSD values of the corresponding PC3 CA case can be applied since this uplink configuration does not support PC2.

#### 6.X.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n5A-n14A-n77A are defined in Table 6.X.3.1-1.

Table 6.X.3.1-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n14A-n77AZZ | n5 | 835 | 5 | 25 | 880 | 14.0 | FDD | IMD5 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 20.3 | FDD | IMD4yy |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
| NOTE YY: This band is subject to IMD5 also which MSD is not specified. NOTE ZZ: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.X.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n5A-n14A-n77A are same as the Case A defined in Table 6.X.3.1-1.

---End of changes---

# References