3GPP TSG-RAN WG4 Meeting # 101-e 														R4-2119925

Electronic Meeting, 01st – 12th November, 2021

Agenda item:			8.15.1, 8.15.2, 8.15.3

Source:	Moderator (LG Electronics)

Title:	Email discussion summary for [101-e][125] NRSL_enh_Part1

Document for:	Information

Introduction

In this paper, RAN4 treats the SL_enh_part1 in Rel-17 for operating bands, system parameters, and UE transmitter/Receiver requirements for SL enhancement including 5G V2X enhancement and Public safety usage with PC5 operation.

The provided technical docs list of email discussion are shown in the existing Tdocs in recommenations for Tdocs Part.

Candidate target of email discussion for 1st round is listed as follows.

- 1st round: Discuss NR SL enhancement UE RF requirements to support PS service and other SL operation.

  - Topic #1: SL enhancement UE RF requirements

  - Sub-Topic #1-1: Configured Tx power

    - Sub-Topic #1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17

    - Sub-Topic #1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16

    - Sub-Topic #1-1-3: Pcmax definition for intra-band V2X UE in TS38.101-3 in Rel-16

    - Sub-Topic #1-1-4: Pcmax definition for inter-band V2X UE in TS38.101-3 in Rel-16

  - Sub-Topic #1-2: REFSENS requirements

    - Sub-Topic #1-2-1: REFSENS requirements in n14

  - Sub-Topic #1-3: Other RF requirements

    - Sub-Topic #1-3-1: Editorial correction for TR38.785

    - Sub-Topic #1-3-2: Updated TR 38.785 v0.4.0

- 2nd round: The following issues will be further discussed

  - Topic #1: SL enhancement UE RF requirements

The following Tdocs are further discussed in 2nd round.

| Tdoc number | Title | Source | Note |
| --- | --- | --- | --- |
| R4-211xxxx | WF on Pcamx revision in NR V2X UE in Rel-16 | Xiaomi |  |
| Rev_R4-2119244 | CR for TS 38.101-1 PEMAX correction | Xiaomi |  |
| R4-2119254 | CR for TS 38.101-1 PEMAX correction _catA | Xiaomi | Cat. A CR will be treated based on main CR |
| Rev_R4-2117402 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT |  |
| R4-2117403 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Cat. A CR will be treated based on main CR |
| Rev_R4-2117404 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT |  |
| R4-2117405 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Cat. A CR will be treated based on main CR |
| R4-211xxxx | WF on REFSENS requiremetns in SL enh. UE in n14 | LG Electronics |  |
| Rev_R4-2118114 | TP on Updating REFSENS requirements for NR SL enhancement | LG Electronics |  |
| Rev_R4-2118278 | TP for SL enhancements | vivo |  |
| R4-2118081 | TR38.785 v0.4.0 TR Update for SL enhancement in Rel-17 | LG Electronics France |  |
| R4-2120047 | Draft LS on Pemax for NR V2X | Huawei, CATT |  |

- Topic #1: SL enhancements UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117402 | CATT | CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) Provide draft CR to change the Pcmax for intra-band V2X UE and inter-band V2X con-current UE in Rel-16 - 6.2E.4.1 UE configured output power for Intra-band V2X For intra-band V2X operating UE in Band n47, each UE configured output power specified in clause 6.2.5G [4] and in clause 6.2E.4 [2] apply, respectively. 6.2E.4.2 UE configured output power for Inter-band V2X PCMAX_H (p,q) = MIN{10 log10 [pCMAX_H,c,Uu (p) + pCMAX_H,c,V2X (q)], PPowerClass, V2X} where - pCMAX_H,c,V2X and pCMAX_H,c,Uu are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,Uu (p) expressed in linear scale. - PPowerClass, V2X is the maximum intra-band V2X con-current UE power specified in Table 6.2E.1.3-1 without taking into account the tolerance; |
| R4-2117404 | CATT | CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) Provide draft CR to remove serving cell information in TS38.101-1 for the Pcmax for single carrier V2X UE UE in Rel-16 - 6.2E.4 Configured transmitted power for V2X - 6.2E.4.1 General - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IEsl-maxTxPower, defined by TS 38.331. 6.2E.4.2 Configured transmitted power for inter-band V2X con-current operation When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation and for intra-band con-current operation in Band n79, the UE is allowed to set its configured maximum output power PCMAX,c,NR and PCMAX,c,V2X for the configured NR uplink carrier and the configured NR V2X carrier, respectively, and its total configured maximum output power PCMAX,c. PCMAX_H (p,q) = MIN{10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,V2X (q)], PPowerClass, V2X} where - pCMAX_H,c,V2X and pCMAX_H,c,NR are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,NR (p) expressed in linear scale. - PPowerClass, V2X is the maximum intra-band V2X con-current UE power specified in Table 6.2E.1.2-1 without taking into account the tolerance; |
| R4-2117406 | CATT | Discussion and TP on configured transmitted power Same contents in Draft CR in R4-2117404 and R4-2117402 |
| R4-2118988 | Ericsson | CR on Pcmax for NR V2X in Rel-17 (TS38.101-1) Provide draft CR to change the Pemax for single carrier V2X UE in Rel-17 - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE P-Max for serving cell c, defined by [7], when UE is associated with a serving cell on the NR V2X carrier; PEMAX,c is the value given by IEsl-maxTxPower, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier . |
| R4-2119244 | Xiaomi | CR for TS 38.101-1 PEMAX correction in Rel-16 Provide draft CR to change the Pemax for single carrier V2X UE in Rel-16. Use sl-TxPower instead of sl-maxTxPower in both licensed/unlicensed band. - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE sl-TxPower, defined by TS 38.331. When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is the smaller value given by IE sl-TxPower and IE p-max of that serving cell. |
| R4-2119247 | Xiaomi | Further discussion on PEMAX issue Observation 1: The IE sl-MaxTxPower is optional and depends on the value of sl-TxPower-r16. Observation 2: The IE sl-MaxTransPower indicates the max value of the whole resource pool. Observation 3: IE sl-TxPower and p-max are used to limit the UE power for sidelink and NR uplink on a carrier frequency respectively. Proposal 1: To agree to use IE sl-TxPower instead of current IE sl-maxTxPower in TS 38.101-1 sub-clause 6.2E.4.1 for sidelink Pemax,c for V2X SA mode for un-licensed band. Observation 4: The RRC signalling as sl-TxPower is still used even for sidelink licensed band. Observation 5：The value of sl-TxPower should be no larger than the value of IE p-max. Proposal 2: Still use sl-TxPower for UE configured power for licensed band but to limit the value of sl-TxPower no larger than the value of IE p-max. |
| R4-2119536 | Huawei, HiSilicon | draft CR for TS 38.101-3 correction of output power for intra-band V2X operation (R16) Provide draft CR to change the Tx part for intra-band V2X UE in Rel-16. - 6.2E.2.1 UE maximum output power reduction for Intra-band V2X For intra-band V2X operating UE in ITS band, maximum output power reduction specified in clause 6.2.3G [4] and in clause 6.2E.2 [2] apply, respectively. - 6.2E.3.1 UE additional maximum output power reduction for Intra-band V2X For intra-band V2X operating UE in ITS band, additional maximum output power reduction specified in clause 6.2.4G [4] and in clause 6.2E.3 [2] apply, respectively. - 6.2E.4.1 UE configured output power for Intra-band V2X For intra-band V2X operating UE in ITS band, each UE configured output power specified in clause 6.2.5G [4] and in clause 6.2E.4 [2] apply, respectively. |
| R4-2117407 | CATT | Discussion and TP on REFSENS for band n14 Table 2: Reference sensitivity for NR SL enhancement (PC5) NR Operating band / SCS / Channel bandwidth / Duplex-mode. V2X Band; SCS kHz; 5MHz (dBm); 10MHz (dBm); 20MHz (dBm); 30MHz (dBm); 40MHz (dBm); Duplex Mode. n14; 15; -98.0; -95.5; FDD. 30; -95.1. 60. Proposal 1: To adopt the principle defined in TR 38.886 with an extra 1dB relaxation and to specify SL REFSENS for Band n14 in Table 2. |
| R4-2117647 | Qualcomm Incorporated | n14 REFSENS for PS in licensed band Proposal 1: The difference in insertion loss of the UL filter compared to the DL filter of the band-duplexer, ∆ILUL-DL =0.1 dB Proposal 2: Use the REFSENS (RefSensNRn14) values presented in table 1 for NR n14 PS |
| R4-2118114 | LG Electronics France | TP on Updating REFSENS requirements for NR SL enhancement Proposal 1: RAN4 can reuse the REFSENS equation of D2D UE for NR SL enh. UE in NR refarming band. Proposal 2: For the number of RB size for REFSENS of SL enhancement UE in sidelink Tx configuration, RAN4 can consider the allowed LCRB allocation for NR SL UE that is same or close to uplink configuration for NR UE. NR Operating band / SCS / Channel bandwidth / Duplex-mode. V2X Band; SCS kHz; 5MHz (dBm); 10MHz (dBm); 20MHz (dBm); 30MHz (dBm); 40MHz (dBm); Duplex Mode. n14; 15; -96.4; -93.2; HD. 30; -93.5. 60. |
| R4-2118278 | vivo | TP for SL enhancements Propose some editorial changes were made on the latest version of TR 38.785 See the detail changes in R4-2118278 |
| R4-2118081 | LGE | TR38.785 v0.4.0 TR update for SL enh. In Rel-17 It will be reserved to capture the all approved TPS and WF in this meeting |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on provided contributions, RAN4 discusses the detailed NR UE RF Tx/Rx requirements for SL enhancement service.

  - Topic #1: SL enhancement UE RF requirements

  - Sub-Topic #1-1: Configured Tx power

    - Sub-Topic #1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17

    - Sub-Topic #1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16

    - Sub-Topic #1-1-3: Pcmax definition for intra-band V2X UE in TS38.101-3 in Rel-16

    - Sub-Topic #1-1-4: Pcmax definition for inter-band V2X UE in TS38.101-3 in Rel-16

  - Sub-Topic #1-2: REFSENS requirements

    - Sub-Topic #1-2-1: REFSENS requirements in n14

  - Sub-Topic #1-3: Other RF requirements

    - Sub-Topic #1-3-1: Editorial correction for TR38.785

    - Sub-Topic #1-3-2: Updated TR 38.785 v0.4.0

    - Sub-topic #1-1

Sub-topic description: Configured Tx power

Open issues and candidate options before e-meeting:

Discussion points for GTW session

- Use IE sl-MaxTransPower instead of IE sl-maxTxPower for NR SL

- In rel-16, RAN4 used IE sl-maxTxPower to define the configured Tx power for V2X UE. But the IE is to indicate the maximum transmission power for transmission on PSSCH and PSCCH for Channel Busy Ratio (CBR) condition.

- The IE sl-MaxTransPower is considered in RAN1 and RAN2 to limit the maximum out power for SL UE.

  - Proposal : Update the configured Tx power with IE sl-MaxTransPower for SL UE.

Discussion:

Xiaomi: in our proposal, we propose sl-Tx-Power to reflect the power on carrier.

Ericsson: we agree with xiaomi. sl-Tx-power needs be used. Such parameter should be pre-configured. Otherwise, there is no power limitation for SL UE in out-of-coverage.

Qualcomm: sl-MaxTransPower should be used according to our reading of the spec.

OPPO: agree with moderator proposal. Checking with RAN1, the max Tx is for congestion control. We should use MaxTrans power.

Huawei: ls-MaxTransPower is correct one. RAN1 has already corrected this error. We should align with RAN1.

CATT: Agree with moderator proposal. In our understanding this IE is designed for SL power control.

LGE: same with with Qualcomm,Oppo…

Ericsson: there are some different understanding. In pcmax the factor pemax in the formula. The regulation.

Xiaomi: RAN1 does not use sl-MaxTransPower. sl-Tx-power is more general IE which can cover both in-coverage and out-of-coverage. sl-Tx-power is different from sl-MaxTrans there will be interference.

Huawei: for in-coverage scenario, this IE can be configured by network. If UE is out-of-coverage, the pre-configured value can be used. We should keep consistent between WGs.

- Whether remove the “when the UE is not associated with a serving cell on the NR V2X carrier” for n47 SL UE

- In rel-16, RAN4 define the configured Tx power for NR SL UE in n47 with above sentence.

  - Proposal : Keep or remove the highlighted sentence

Discusions:

Ericsson: this is LTE V2X behavior. The regulatory requirement will be applied Pemax. When NR is configured with BWP within the cell, this is the same as NR Uu. They should use the same p-max. We propose to add the other condition but keep this condition.

Xiaomi: we want to check if there is common understanding if UE is in licensend band if UE is associated with a serving cell.

Qualcomm: we should keep this sentence. When associated with serving cell, we should use p-Max.

CATT: p-max can be applied to sidelink no matter whether sidelink in in-coverage or out-of-coverage. The sentence should be removed.

LGE: this is about in NR sidelink can support both licensed and unlicensed band. We can modify the sentence to add licensed operation and use p-Max.

Huawei: even for band n47 UE can work in in-coverage scenario. The serving cell can work..

Xiaomi: we propose to use sl-Tx-power and limit it < p-max

Ericsson: sidelink is configured as BWP within the cell when we are talking about association with cell.

OPPO: we should align the understanding about the associate with cell.

Agreements:

- For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE A, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier .

  - FFS on IE A

    - Option 1: sl-Tx-Power

    - Option 2: sl-MaxTransPower

- When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is given by IE B of that serving cell.

  - FFS on IE B

    - Option 1: sl-Tx-Power

    - Option 2: sl-MaxTransPower

    - Option 3: p-Max

- IE A and IE B can be the same.

- Is it necessary to update the configured Tx power for intra-band V2X UE rel-16?

- The intra-band V2X UE in Rel-16 is operated with TDM manner between LTE SL and NR SL in n47.

  - Proposal : To clear understanding, RAN4 decide whether add n47 or ITS band in the sentence or not.

- Whether consider PPowerClass, V2X for inter-band V2X UE in Rel-16?

- In Rel-16, only PC3 V2X UE supported. The PC2 inter-band V2X UE will be introduced in Rel-17

  - Proposal: RAN4 need further discuss whether to consider the PPowerClass, V2X for configured Tx power in Rel-16.

Issue 1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17

- Proposals

  - Option 1: Based on Ericsson paper (R4-2118988), RAN4 update the configured Tx power of V2X UE with ‘IE P-Max’ for single carrier from Rel-17.

  - Option 2: Based on Ericsson paper (R4-2118988), RAN4 update the configured Tx power of V2X UE with ‘IE P-Max’ for single carrier from Rel-16.

  - Option 3: Based on Xiaomi paper (R4-2119244), RAN4 update to use ‘sl-TxPower’ for configured Tx power of V2X UE for single carrier from Rel-16.

  - Option 4: Based on CATT paper (R4-2117404), RAN4 remove the serving information sentence in configured Tx power of V2X UE for single carrier from Rel-16.

  - Option 5: Keep the current configured Tx power of V2X UE for single carrier

- Recommended WF

  - TBA

Issue 1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16

- Proposals

  - Option 1: Based on CATT paper (R4-2117404), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16.

  - Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16.

- Recommended WF

  - TBA

Issue 1-1-3: Pcmax definition of V2X UE for intra-band V2X UE in TS38.101-3 in Rel-16

- Proposals

  - Option 1: Based on CATT paper (R4-2117402), RAN4 update the configured Tx power for intra-band V2X UE in Rel-16.

  - Option 2: Based on Huawei paper (R4-2119536), RAN4 update the configured Tx power for intra-band V2X UE in Rel-16.

  - Option 3: Keep the current configured Tx power for intra-band V2X UE

- Recommended WF

  - TBA

Issue 1-1-4: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-3 in Rel-16

- Proposals

  - Option 1: Based on CATT paper (R4-2117402), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16.

  - Option 2: For the inter-band V2X UE in TS38.101-3 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16.

- Recommended WF

  - TBA

    - Sub-topic #1-2

Sub-topic description: REFSENS requirements

Open issues and candidate options before e-meeting:

Issue 1-2-1: REFSENS requirements for n14 NR SL UE

- Proposals

  - Option 1: REFSENS (R4-2117407, CATT) requirements are acceptable for n14 NR SL UE.

  - Option 2: REFSENS (R4-2117647, Qualcomm) requirements are acceptable for n14 NR SL UE.

  - Option 3: REFSENS (R4-2118114, LGE) requirements are acceptable for n14 NR SL UE.

  - Option 4: Others

- Recommended WF

  - TBA.

    - Sub-topic #1-3

Sub-topic description: Other RF requirements

Open issues and candidate options before e-meeting:

Issue 1-3-1: Editorial correction for TR38.785

- Proposals

  - Option 1: Based on TP (R4-2118278, vivo), RAN4 can update the TR contents.

  - Option 2: TP (R4-2118278, vivo) will be updated based on 1st comments from interested companies

- Recommended WF

  - TBA

Issue 1-3-2: Updated TR 38.785 v0.4.0

- Proposals

  - Option 1: updated TR will be treated in 2nd round to capture the approved TPs in this meeting.

- Recommended WF

  - Option 1 is acceptable

Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1: SL enhancement UE RF requirements

Issue 1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17

| Company | Comments |
| --- | --- |
| Xiaomi | Option 3. We have analyzed the three different IE as sl-TxPower, sl-MaxTxPower and sl-MaxTransPower and shows that the IE sl-TxPower is the appropriate IE to be used. Furthermore, for licensed band, still same IE as sl-TxPower should be used but set a upper limit as p-max. If we use p-max directly, if the sl-TxPower is smaller then p-max, then it will cause ambiguity. |
| Qualcomm | Our preferred wording is: For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE sl-MaxTransPower-r16, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier .When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is given by IE p-max of that serving cell. |
| LGE | Option 2 |
| CATT | Option 4. In our understanding, there is no need to indicate two different cases (associated with cell and not associated with cell) as clause 6.2E.4 is dedicated for V2X UE. When Uu interface is used, Pemax should follow Uu dedicated IE. Even in case that UE is associated with a serving cell, PEMAX,c of Uu carrier and V2X carrier can be separately signalled with each dedicated IE, i.e. IEP-max for Uu and IEsl-maxTxPower for SL. |
| Huawei, HiSilicon | The meaning of sl-maxTxPower is different from that of maxTxPower for Uu. The IE of sl-maxTxPower is to indicate the maximum transmission power for transmission on PSSCH and PSCCH for Channel Busy Ratio (CBR) condition. Instead, sl-MaxTransPower-r16 is used for the purpose to limit the max output power for SL in the SL resource pool. The parameter can be get via RRC configuration or SIB12 for in coverage scenario or via pre-configuration in out of coverage scenario. Thus the parameter sl-MaxTransPower can be used for cases with or w/o a serving cell, “when the UE is not associated with a serving cell on the NR V2X carrier” should be removed. RAN1 is aware of the wrongly used parameter, and the correction CR for TS 38.213 was agreed in R1-2107221. The proposed changes in TS 38.101-1 for SL Pemax part are: For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE sl-MaxTransPower, defined by TS 38.331. We agree part of the change proposed by CATT for the general part in R4-2117404, but not the changes for the inter-band co-current operation. |
| Ericsson | Option 2. This issue partly overlap with [127] issue 2-2. The Pcmax has two issues, first is that whether or not to add the associated cell and the other is whether IE sl-maxTxPower is correct or not. For the issue of IE sl-maxTXpower, we think proposal 1 in Xiaomi paper (R4-2119244) seems ok and in our paper R4-2118990, such parameter actually also need to be pre-configured, so I think if this parameter is believed to be correct one to set the upper limit on V2X transmission, then it will be good to discuss further whether it also need to be pre-configured otherwise there is no upper limit to control NR V2X power in mode 2 if there is regulatory requirement on it. For the issue of the associated cell, when the NR V2X is configured as one BWP within the cell, the upper limit of the V2X tranmit power should follow the IE P-max as the same as the NR Uu, we donot see the need to introduce different upper limit for Pemax when NR v2x is configured as one BWP within the cell. We also agree that in Xiaomi paper (R4-2119244), there is potential interference issue if P-max and sl-TxPower are different, rather than limit this value, we think this is coexisting issue and further is discussed in [127]issue 2-1. |
| CATT | For the specific IE, we think IE sl-MaxTransPower should be used for SL instead of IE sl-TxPower. sl-TxPower is the value of IE sl-MaxTxPower. As mentioned by Huawei, IE sl-MaxTxPower is optional for transmission on PSSCH and PSCCH for CBR condition. It seems IE sl-MaxTxPower and IE sl-MaxTransPower are overlapped in some cases. If the CBR condition is met, the smaller vaule of IE sl-MaxTxPower and IE sl-MaxTransPower coul be more appropriate for SL. |
| Vivo | Prefer Option 4 as the starting point. We need to check how to update the IE ‘sl-maxTxPower’. |
| OPPO | IE sl-MaxTransPower should be used for SL, this is used for power control purpose, while sl-MaxTxPower-r16 is defined for congestion control which has already been covered in RAN1 power calculation. Removing the sentence of “UE is not associated with a serving cell on the NR V2X carrier” is also needed since the above Pmax (sl-MaxTransPower) can be applied in coverage or out of coverage. |

Issue 1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16

| Company | Comments |
| --- | --- |
| Xiaomi | Option 2. We can keep current inter-band definition. |
| Qualcomm | Option 2. Keep the current configured TX power for inter-band V2X UE in Rel-16 |
| LGE | Option 2 |
| CATT | Option 1. We also notice that only PC3 is supported for inter-band V2X con-current operation. Seems PC3 is not sufficient to support Uu and V2X con-current operation. In principle, Pcmax should be limited by a upper value when the maximum power is defined for inter-band con-current operating UE. If companies think PC3 is not sufficient, higher power class, e.g. PC2, PC1, can be added for specific bands on a case by case basis. So the current specs needs updation to avoid misalignment between maximum output power and Pcmax for inter-band con-current operation. |
| Huawei, HiSilicon | Option 2. |
| vivo | Option 2. |
| OPPO | Option 1. Pcmax should be bounded by the total power class of uu+SL inter band combination. Not sure how this can be kept open just because there is only PC3 defined in Rel-16. |

Issue 1-1-3: Pcmax definition of V2X UE for intra-band V2X UE in TS38.101-3 in Rel-16

| Company | Comments |
| --- | --- |
| Xiaomi | Option 3. For intra-band Pcmax definition in TS 38.101-3, we observe that both 7402 and 9536 have correction on specific ITS band. As currently no other bands captured in TS 38.101-3, we prefer to keep current spec unchanged. |
| Qualcomm | Option 4: We believe that the Pcmax_H equations given in R4-2119535 are correct for intra-band V2X UE After further consideration we select option 2 |
| LGE | We are same view with Xiaomi. In rel-16, the intra-band V2X UE between LTE V2X and NR V2X only supported in n47. |
| CATT | We prefer to add Band n47 or ITS band for intra-band operation because Band n79 also has intra-band operation. It is necessary to give clarification as to which requirements apply for which band though Band n47 and Band n79 are specified in different specs (38.101-1 and 38.101-3) . For Pcmax_h, we have the same comments as Issue 1-1-2. |
| Huawei, HiSilicon | Option 2. The CR is to change TS 38.101-3, which is for inter-operation for LTE and NR, but for LTE, we only have ITS specified for LTE-V, which is not similar to the discussion as in thread [126] for intra-band con-current operation in licensed band. Clarification is needed in 38101-3 for intra-band case. |
| Ericsson | Option 2. Clarification on Rel-16 spec is good to have though the feature of intra-band V2X and Uu operation is for Rel-17. |
| vivo | Option 2. |
| OPPO | Option 2 or Option 3. |

Issue 1-1-4: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-3 in Rel-16

| Company | Comments |
| --- | --- |
| Xiaomi | Option 2. Similar to issue 1-1-2. |
| Qualcomm | Option 2 |
| LGE | Option 2 |
| CATT | Option 1. For Pcmax_h, we have the same comments as Issue 1-1-2. |
| Huawei, HiSilicon | Option 2. There is no need to make change of existing Pcmax requirement for the inter-band case. |
| Ericsson | Option 1. RAN1 spec TS 38.213 clause 16.2.4.3 specifies the UE transmit power behavriou and opton 1 just follow the RAN1 spec. - 16.2.4.3 Simultaneous SL and UL transmissions/receptions If a UE - is capable of simultaneous transmissions on the UL and on the SL in two respective carriers, - would transmit on the UL and on the SL in the two respective carriers, - the transmission on the UL would overlap with the transmission on the SL over a time period, and - the total UE transmission power over the time period would exceed PCMAX the UE - reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX - reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX |
| vivo | Option 2. |
| OPPO | Option 1. To keep total power under power class limitation. |

Sub topic 1-2: REFSENS requirements

Issue 1-2-1: REFSENS requirements for n14 NR SL UE

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2 |
| LGE | We are fine with option 2 and option3. The difference is full RB allocation or same RB configuration with NR Uu in n14. In this pahse, we slightely prefer option 3. |
| CATT | To make progress, we can accept the relaxation for n14 REFSENS. The full RB allocation is preferred. |
| AT&T | We would like to confirm that the RMC will be equivalent to the Uu case, full RB allocation in DL and no retransmissions. Given that the QC and LGE REFSENS values are so close between Option 2 and Option 3, maybe we can take the average in dB. Although, full allocation in UL option does allow for future relay cases, we slightly prefer to use the same UL configuration as Uu for conformance testing. |

Sub topic 1-3: Other RF requirements

Issue 1-3-1: Editorial correction for TR38.785

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2 |
| LGE | Maybe some editorial corrections are applied in this TP. |
| CATT | Option 2. |
| Huawei, HiSilicon | Option 2. Besides the editiorial correction, other parts can also be updated based on the discussion. |
| vivo | Option 2. |
| AT&T | Option 2. At a minimum, clause 4.3.2 shall be updated to reflect that NR sidelink operation in band n14 is not restricted to out-of-coverage. It should include both out-of-coverage and in-coverage case as noted in R4-2111535 at RAN4 #99e. |

Issue 1-3-2:  Updated TR38.785 v0.4.0

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1 is acceptable |
| LGE | Option 1 |
| CATT | Option 1. |
| vivo | Option 1. |

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117402 | Company A |
|  | Company B |
|  | … |
| R4-2117404 | Company A |
|  | Company B |
|  | … |
| R4-2118988 | Company A |
|  | Company B |
|  | … |
| R4-2119536 | Company A |
|  | Company B |
|  | … |
| R4-2119244 | Company A |
|  | Company B |
|  | … |
| R4-2117407 | Company A |
|  | Company B |
|  | … |
| R4-2118114 | Company A |
|  | Company B |
|  | … |
| R4-2118278 | Company A |
|  | Company B |
|  | … |

Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1: Configuired Tx power | Issue 1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17 In GTW session, there were some arguement to use the sl-MaxTransPower since the RAN1 did not agree to use the IE and the IE sl-Tx-power is more general IE which can cover both in-coverage and out-of-coverage. Based on this argurement point, RAN4 needs further discussion which IE is the best to define the configured Tx power for NR SL UE for both in-coverage and out-of coverage. - Candidate options - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE A, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier . - FFS on IE A - Option 1: sl-Tx-Power - Option 2: sl-MaxTransPower - When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is given by IE B of that serving cell. - FFS on IE B - Option 1: sl-Tx-Power - Option 2: sl-MaxTransPower - Option 3: p-Max - IE A and IE B can be the same. - Recommendations for 2nd round: In the 2nd round, RAN4 will further discuss which IE is correct to update the configured Tx power for SL UE in licensed and unlicensed bands. Also the IE usage should be aligned with other WGs. Based on this agreement, the Xiaomi draft CR(R4-2119244) will be revised. |
|  | Issue 1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16 In the 1st round, majority view is to keep the existing configured Tx power for inter-band V2X UE in Rel-16. Two companies worried the existing Pcmax would exceed the total power of NR Uu+NR SL inter band combination. - Candidate options - Option 1: Based on CATT paper (R4-2117402), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16. - Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16. - Recommendations for 2nd round: In the 2nd round, RAN4 will further discuss whether the current Pcmax defition of inter-band V2X UE is sufficient and reasonable not to violate the power class for each band. |
|  | Issue 1-1-3: Pcmax definition of V2X UE for intra-band V2X UE in TS38.101-3 in Rel-16 In the 1st round, 5 companies wanted to update the sentence based on HW paper. 2 companies preferred to keep the existing sentence because the intra-band V2X UE is operated in only n47 band. And one company wanted to add PPowerClass, V2X in the Pcmax,H definition. Based on this preference, RAN4 can make the tentative agreement as option 2 based on majority view. - Tentative agreements - Option 2: Based on Huawei paper (R4-2119536), RAN4 update the configured Tx power for intra-band V2X UE in Rel-16. - Recommendations for 2nd round: RAN4 can endorse the CR for intra-band V2X UE in TS38.101-1 in Rel-16. |
|  | Issue 1-1-4: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-3 in Rel-16 In the 1st round, majority view (5 companies) is to keep the existing configured Tx power for inter-band V2X UE in Rel-16. 3 companies worried the existing Pcmax would exceed the total power of LTE Uu+NR SL or NR Uu + LTE SL inter band combination. Based on this, the following two candidate options will be further discussed in the 2nd round. - Candidate options - Option 1: Based on CATT paper (R4-2117404), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16. - Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16. - Recommendations for 2nd round: In the 2nd round, RAN4 will further discuss whether current Pcmax defition of inter-band V2X UE in TS38.101-3 is sufficient and reasonable not to violate the power class for each band. |
| Sub-Topic#1-2: REFSENS requirements | Issue 1-2-1: REFSENS requirements in n14 NR SL UE In the 1st round, majority view was to reuse the REFSENS of D2D equation and reflect the different points e.g. SNR Uu-SL and allocation RB size. For the allocatied RB size, two companies preferred to use full RB allocation and two companies preferred to use same RB size of NR Uu. Based on this, the following candidate options will be further discussed. - Candidate options - Option 1: Full RB for SL transmission configuration will be considered to derive REFSENSE requirements in n14 SL enh. UE. - Option 2: Same RB of NR Uu for SL transmission configuration will consider to derive REFSENS requirements in n14 SL enh. UE. - Recommendations for 2nd round: In the 2nd round, WF and TP will be treated to cover the Issue 1-2-1 |
| Sub-topic#1-3: Other RF requirements | Issue 1-3-1: Editorial correction TR38.785 In the 1st round, majority view is to update the editorial correction in TR38.785. some additional revision can be added in 2nd round. - Tentative Agreements: - Option 2: TP (R4-2118278, vivo) will be updated based on 1st comments from interested companies - Recommendations for 2nd round: The detail updating contents will be proposed and discussed in the 2nd round. |
|  | Issue 1-3-2: Updated TR38.785 v0.4.0 In the 1st round, majority view was draft TR will be treated in the 2nd round as following tentative agreements. - Tentative Agreements: - Option 1: updated TR will be treated in 2nd round to capture the approved TPs in this meeting. - Recommendations for 2nd round: Option 1 is acceptable. The updated TR will capture all the approved TPs in this meeting. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117402 | The draft CR can be revised based on issue 1-1-4 results for inter-band V2X UE in TS38.101-3 in rel-16. |
| R4-2117404 | The draft CR can be revised based on issue 1-1-2 results for inter-band V2X UE in TS38.101-1 in rel-16. |
| R4-2119536 | The draft CR can be endorsed based on 1st round discussion. (cover issue 1-1-3) |
| R4-2118988 | The draft CR will be not pursued and the issue will be covered in R4-2119244. |
| R4-2119244 | The CR can be revised based on issue 1-1-1 results for single carrier V2X UE in TS38.101-1 in rel-16. |
| R4-2117407 | This TP can be noted. |
| R4-2118114 | The TP will be revised to capture the REFSENS and uplink configurations for n14 SL enhancement UE. (cover issue 1-2-1) |
| R4-2118278 | The TP will be revised to capture the all editorial correction in TR38.785 in 2nd round. (cover 1-3-1) |
| R4-2118081 | The draft TR will be treated in 2nd round to capture all the approved TPs in this meeting. (cover 1-3-2) |

Discussion on 2nd round (if applicable)

    - Open issues (if applicable)

Issue 1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 & Rel-17

- Candidate options

- For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE A, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier .

  - FFS on IE A

    - Option 1: sl-Tx-Power

    - Option 2: sl-MaxTransPower

- When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is given by IE B of that serving cell.

  - FFS on IE B

    - Option 1: sl-Tx-Power

    - Option 2: sl-MaxTransPower

    - Option 3: p-Max

- IE A and IE B can be the same.

Based on this agreement, the Xiaomi draft CR(R4-2119244) will be revised.

| Company | Comments for issue 1-1-1 |
| --- | --- |
| LGE | For IE A, we still prefer sl-MaxTransPower For IE B, we only consider the P-max. do not consider IE of SL operation since same power class will be applied in a licensed bnad. RAN4 need to send LS to RAN1/RAN2 for the clarify the correct IE of the power limitation for SL transmission. |
| Qualcomm | For IE A our option is sl-MaxTransPower For IE B our option is p_Max |
| CATT | This can be decided based on RAN1/RAN2 feedback. |
| Huawei | For both with and w/o associated serving cell, we think that the parameter should be given by sl-MaxTransPower, but that also depends on further feedback from RAN1 and RAN2. |
| Vivo | We can wait the feedback from RAN1/2. |
| Xiaomi | We are OK with the draft LS and this issue can be further discussed after the reply LS received. |

Issue 1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16

- Candidate options

- Option 1: Based on CATT paper (R4-2117402), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16.

- Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16.

- Recommendations for 2nd round:

- In the 2nd round,  RAN4 will further discuss whether current Pcmax defition of inter-band V2X UE in TS38.101-1 is sufficient and reasonable not to violate the power class for each band.

| Company | Comments for issue 1-1-2 |
| --- | --- |
| LGE | Option 2. For the inter-band V2X UE, RAN4 do not support high power UE capability for inter-band con-current V2X UE operation. The reason is that regulatory requirements will be applied for each band. So maximum Tx power of inter-band V2X UE will not restricted the total inter-band V2X power class. In Rel-16, RAN4 only allowed that inter-band con-current V2X UE is only support with PC3. |
| Qualcomm | Oprion 2: As one link is communicating with a gNB and another with a vehicle and the 2 are not co-located it is expected that each link can transmit at max power for a given power class. |
| CATT | Our comments provided in the reflector are pasted here: Concerning the Pcmax,h for inter-band BC, normally it should be limited by the maximum power defined for each BC. Otherwise, RAN4 does not have to defined the MOP requirements for each inter-band BC. As mentioned by Xiaomi, the sum of current per band maximum power for Uu and PC5 could exceed per BC maximum power (PC3+PC3=PC2). In this case, it would cause a lot of misunderstanding and confusion for readers and developers given such inconsistency. The appropriate way I assume is to increase the MOP for each BC if request identified rather than to leave configured maximum power for BC without a upper bound. We checked 36.101 and 38.101-1, and as indicated by Note 3, the maximum power requirement apply to total transmitted power over all CC. So from the perspective of spec consistency, we also prefer to add the upper bound for each BC. |
| Huawei | Option 2. Similar issue has been discussed in RAN4#98bis, and the conclusion is that there is no need to change the upper bound for inter-band operation as it is in the spec. |
| vivo | Option 2. We understand the intention of Pcmax,H and we brought the same issue several meetings ago. It was agreed that no change is needed. |
| Xiaomi | It seems the majority view is option 2, I will update the WF accordingly. |
| CATT | We checked vivo’s paper (R4-2104531) submitted in RAN4#98bis meeting. In our understanding, the case here is different from the proposal in vivo’s paper. In this meeting we are proposing to add upper power bound for inter-band concurrent operation. The upper power bound can be either PC3 or PC2 (once defined) instead of only PC3. If PC2 would be defined for each BC, the total power of Uu ans SL can definitely reach PC2. Proposal 1: For inter-band con-current HPUE, the maximum outpower could not exceed PC3 for each Uu and SL transmission. If no agreement in this meeting, we would like to keep it open and to further discuss it in next meeting. |

Issue 1-1-4: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-3 in Rel-16

In the 1st round, majority view (5 companies) is to keep the existing configured Tx power for inter-band V2X UE in Rel-16. 3 companies worried the existing Pcmax would exceed the total power of LTE Uu+NR SL or NR Uu + LTE SL inter band combination. Based on this, the following two candidate options will be further discussed in the 2nd round.

- Candidate options

- Option 1: Based on CATT paper (R4-2117404), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16.

- Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16.

- Recommendations for 2nd round:

- In the 2nd round,  RAN4 will further discuss whether current Pcmax defition of inter-band V2X UE in TS38.101-1 is sufficient and reasonable not to violate the power class for each band.

| Company | Comments for issue 1-1-4 |
| --- | --- |
| LGE | Same comment within 1-1-2 |
| Qualcomm | Option2 |
| CATT | Our comments provided in the reflector are pasted here: Concerning the Pcmax,h for inter-band BC, normally it should be limited by the maximum power defined for each BC. Otherwise, RAN4 does not have to defined the MOP requirements for each inter-band BC. As mentioned by Xiaomi, the sum of current per band maximum power for Uu and PC5 could exceed per BC maximum power (PC3+PC3=PC2). In this case, it would cause a lot of misunderstanding and confusion for readers and developers given such inconsistency. The appropriate way I assume is to increase the MOP for each BC if request identified rather than to leave configured maximum power for BC without a upper bound. We checked 36.101 and 38.101-1, and as indicated by Note 3, the maximum power requirement apply to total transmitted power over all CC. So from the perspective of spec consistency, we also prefer to add the upper bound for each BC. |
| Huawei | Option 2. See comments for above issue. |
| Vivo | Option 2. |
| Xiaomi | It seems the majority view is option 2 and I will update the WF accordingly, |

Issue 1-2-1: REFSENS requirements in n14 NR SL UE

In the 1st round, majority view was to reuse the REFSENS of D2D equation and reflect the different points e.g.  SNR Uu-SL and allocation RB size. For the allocatied RB size, two companies preferred to use full RB allocation and two companies preferred to use same RB size of NR Uu. Based on this, the following candidate options will be further discussed.

- Candidate options

- Option 1: Full RB for SL transmission configuration will be considered to derive REFSENSE requirements in n14 SL enh. UE.

- Option 2: Same RB of NR Uu for SL transmission configuration will consider to derive REFSENS requirements in n14 SL enh. UE.

- Recommendations for 2nd round:

In the 2nd round, WF and TP will be treated to cover the Issue 1-2-1

| Company | Comments for issue 1-2-1 |
| --- | --- |
| LGE | Prefer option 2. The WF will cover the issue |
| Qualcomm | Option 2 |
| AT&T | Option 2. However, in the WF, the REFSENS seems to be further relaxed by 0.5dB due to the smaller Tx RB allocation. Our understanding is that the REFSENS should be 0.5dB better than the full RB allocation case. |

Issue 1-3-1: Editorial correction for TR38.785

| Company | Comments |
| --- | --- |
| LGE | We are fine with the revision. |
| Qualcomm | Option 2 |
| CATT | Fine with the revision. |
| AT&T | OK with the revision. |

Issue 1-3-2:  Updated TR38.785 v0.4.0

| Company | Comments |
| --- | --- |
| XXX |  |

    - Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

|  | Status summary |
| --- | --- |
| Sub-topic#1-1: Configuired Tx power | Issue 1-1-1: Pcmax definition of V2X UE for single carrier in TS38.101-1 in Rel-16 In the 2nd round, RAN4 agreed to send LS to RAN1/RAN2 to confirm the proper IE parameters to specify the power limitation of NR V2X UE in both licensed band and ITS spectrum. So the following IE parameters will be further discussed in the next RAN1 meeting based on the reply LS from other WGs. - Candidate options - For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IE A, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier. - FFS on IE A - Option 1: sl-Tx-Power - Option 2: sl-MaxTransPower - When the UE is associated with a serving cell on the NR V2X carrier, PEMAX,c is given by IE B of that serving cell. - FFS on IE B - Option 1: sl-Tx-Power - Option 2: sl-MaxTransPower - Option 3: p-Max - IE A and IE B can be the same. - Tentative agreements - Send LS to RAN1 and RAN2 to check which signaling is the proper one to be used. Draft LS will be approved. - The detailed IE parameter will be further discussed in the next RAN4 meeting based on the reply LS from other WGs. |
|  | Issue 1-1-2: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-1 in Rel-16 In the 2nd round, one company needs to further discuss the upper power limitation of Pcmax,H. Most companies prefer to keep the current configured Tx power in TS38.101-1. Also, RAN4 agreed to specify the PC2 inter-band con-current V2X UE in Rel-17. Since RAN4 is open to revise the configured Tx power for PC2 inter-band con-current V2X UE in Rel-17. However, in Rel-16, we can keep the current RAN4 agreements. - Candidate options - Option 1: Based on CATT paper (R4-2117404), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16. - Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE is only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16. - Tentative agreements Option 2 is agreeable in Rel-16. Further discuss whether to introduce PC2 inter-band concurrent operation in Rel-17 and if the Pcmax needs to be revisited correspondingly. |
|  | Issue 1-1-4: Pcmax definition of V2X UE for inter-band V2X UE in TS38.101-3 in Rel-16 This issue is the same as for issue 1-1-2. It just covers the inter-band con-current V2X UE in TS38.101-3. So most companies are comments as same to issue 1-1-2. - Candidate options - Option 1: Based on CATT paper (R4-2117402), RAN4 update PCmax,H with Power class of V2X in the configured Tx power for inter-band V2X UE in Rel-16. - Option 2: For the inter-band V2X UE in TS38.101-1 in Rel-16, the power class for inter-band V2X UE only supported with PC3, so RAN4 can keep the current configured Tx power for inter-band V2X UE in Rel-16. - Tentative agreements Option 2 is agreeable in Rel-16. Further discuss whether to introduce PC2 inter-band concurrent operation in Rel-17 and if the Pcmax needs to be revisited correspondingly. |
| Sub-Topic#1-2: REFSENS requirements | Issue 1-2-1: REFSENS requirements in n14 NR SL UE In the 2nd round, there are two open issues. One is to determine the SL transmission RB size between Full RBs and the same RBs size of NR Uu. The other is the correct REFSENS equation for n14 PS UE. All companies support aligning the RB size with NR Uu. For the REFSENS equation, RAN4 revised the D2D REFSENS equation since the 10log10(LCRB/NRB) parameters are duplicated to define the NR SL REFSENS. Based on the observation, RAN4 agreed to change the REFSENS equation as follow NR SLREFSENS = NR UuREFSENS + ∆SNRSL-Uu + ∆ILUL-DL + 10log10(∆LCRB/NRB) - Candidate options 1) RB size of NR SL in n14 - Option 1: Full RB for SL transmission configuration will be considered to derive REFSENS requirements in n14 SL enh. UE. - Option 2: The same RB of NR Uu for SL transmission configuration will consider deriving REFSENS requirements in n14 SL enh. UE. 2) REFSENS equation - Option 1: NR SLREFSENS = NR UuREFSENS + ∆SNRSL-Uu + ∆ILUL-DL + 10log10(LCRB/NRB) - Option 2: NR SLREFSENS = NR UuREFSENS + ∆SNRSL-Uu + ∆ILUL-DL + 10log10(∆LCRB/NRB) - Tentative agreements 1) RB size of NR SL in n14 - Option 2 is chosen for REFSENS in n14 PS UE. 2) REFSENS equation - Option 2 is chosen since option 1 is only sufficient when the Full RB was allocated in NR Uu. RAN4 further discuss whether allow the additional 0.5 dB IM when small RB (RB size ≤ 24) is allocated in n14 or not. |
| Sub-topic#1-3: Other RF requirements | Issue 1-3-1: Editorial correction TR38.785 All companies support the revised TP contents. - Tentative agreements The revised TP can be approved. |
|  | Issue 1-3-2: Updated TR38.785 v0.4.0 - Tentative agreements The updated TR38.785 v0.4.0 will capture the all approved TP in [125][126][127] e-mail thread. The final version is agreeable. |

- Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on Pcamx revision in NR V2X UE in Rel-16 | Xiaomi |  |
| WF on REFSENS requiremetns in SL enh. UE in n14 | LG Electronics |  |
| Draft LS on Pemax for NR V2X | Huawei, CATT |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117402 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT | Will be revised to |  |
| R4-2117403 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Cat. A CR will be treated based on main CR |  |
| R4-2117404 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT | Will be revised to | It was moved from [126] e-mail thread |
| R4-2117405 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Cat. A CR will be treated based on main CR | It was moved from [126] e-mail thread |
| R4-2117406 | Discussion and TP on configured transmitted power | CATT | The contents of intra-band con-current V2X UE will be treated in [126] e-mail thread | It was moved from [126] e-mail thread |
| R4-2117407 | Discussion and TP on REFSENS for band n14 | CATT | Noted |  |
| R4-2117408 | Discussion on time mask for Uu and SL switching | CATT | treated in [126] e-mail thread | It was moved into [126] e-mail thread |
| R4-2117647 | n14 REFSENS for PS in licensed band | Qualcomm Incorporated | Noted |  |
| R4-2118081 | TR38.785 v0.4.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | It will be discussed after 1st round |  |
| R4-2118114 | TP on Updating REFSENS requirements for NR SL enhancement | LG Electronics France | Will be revised to |  |
| R4-2118278 | TP for SL enhancements | vivo | Will be revised to |  |
| R4-2118988 | CR on Pcmax for NR V2X | Ericsson | Not pursued |  |
| R4-2119244 | CR for TS 38.101-1 PEMAX correction | Xiaomi | Will be revised to |  |
| R4-2119247 | further discussion on PEMAX issue | Xiaomi | Noted |  |
| R4-2119254 | CR for TS 38.101-1 PEMAX correction _catA | Xiaomi | Cat. A CR will be treated based on main CR |  |
| R4-2119536 | draft CR for TS 38.101-3 correction of output power for intra-band V2X operation (R16) | Huawei, HiSilicon | The CR will be endorsed |  |
| R4-2119537 | draft CR for TS 38.101-3 correction of output power for intra-band V2X operation (R17) | Huawei, HiSilicon | The Cat. A CR will be endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2119981 | WF on Pcamx revision in NR V2X UE in Rel-16 | Xiaomi | Will be approved |  |
| R4-2119987 | CR for TS 38.101-1 PEMAX correction | Xiaomi | Not pursed |  |
| R4-2119254 | CR for TS 38.101-1 PEMAX correction _catA | Xiaomi | Not pursed | Cat. A CR will be treated based on main CR |
| R4-2119983 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT | Not pursed |  |
| R4-2117403 | Draft CR for TS 38.101-3, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Not pursed | Cat. A CR will be treated based on main CR |
| R4-2119984 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-16) | CATT | Will be endorsed |  |
| R4-2117405 | Draft CR for TS 38.101-1, Correction on configured transmitted power for inter-band V2X concurrent operation (Rel-17) | CATT | Will be endorsed | Cat. A CR will be treated based on main CR |
| R4-2119982 | WF on REFSENS requiremetns in SL enh. UE in n14 | LG Electronics | Will be approved |  |
| R4-2119985 | TP on Updating REFSENS requirements for NR SL enhancement | LG Electronics | Will be approved |  |
| R4-2119986 | TP for SL enhancements | vivo | Will be approved |  |
| R4-2118081 | TR38.785 v0.4.0 TR Update for SL enhancement in Rel-17 | LG Electronics France | Will be agreed |  |
| R4-2120047 | Draft LS on Pemax for NR V2X | Huawei, CATT | Will be approved |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| LG Electronics | Suhwan Lim | Suhwan.lim@lge.com |
| CATT | Yuan Gao | gaoyuan@catt.cn |
| vivo | Shuai Zhou | shuai.zhou@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)