3GPP TSG-WG RAN4 Meeting #101-e	R4-2119953

Electronic Meeting, November 01 - 12, 2021

WF on UL-MIMO coherence for Rel-17 Tx switching

China Telecom

Way forward

UL-MIMO coherence for 3CC 1Tx-2Tx switching in Rel-17

- Background:

  - RAN4 #100e agreement in R4-2114957

    - For 3CC 1Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the Rel-16 per BC UE capability can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching.

  - Rel-16 agreements in LS R4-2107765

    - If the per BC UL-MIMO capability for Tx switching is absent, the existing per band UE capability pusch-TransCoherence is applicable to the scenario with Tx switching.

Agreement:

- WF in RAN4 #101e: For 3CC (within 2 bands) 1Tx-2Tx switching

  - The Rel-16 per BC UE capability for 2CC 1Tx-2Tx switching can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching.

  - How to handle if the UL-MMO coherence capability for Tx switching is absent:

    - If the per BC UL-MIMO capability for 1Tx-2Tx switching is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable bands for 1Tx-2Tx switching.

UL-MIMO coherence for 2CC and 3CC 2Tx-2Tx switching in Rel-17

- Background: RAN4 #100e agreement in R4-2114957

  - Candidate options for 2CC 2Tx-2Tx switching:

    - Option 1: For 2CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, further clarify whether the Rel-16 per BC UE capability can be applied for both CCs in different bands.

    - Option 2: The Rel-16 signaling cannot be reused

  - Agreements for 3CC 2Tx-2Tx switching:

    - For 3CC 2Tx-2Tx switching, if UE capability on UL-MIMO coherence is needed, the capability for 2CC 2Tx-2Tx switching can be applied, i.e., the same capability applies to both 2CC and 3CC 2Tx-2Tx switching.

Agreement

- WF in RAN4 #101e: For 2CC and 3CC (within 2 bands) 2Tx-2Tx switching

  - Introduce new per band per BC UL-MIMO coherence capability, and the same capability applies to both 2CC and 3CC (within 2 bands) 2Tx-2Tx switching.

  - How to handle if the per band per BC UL-MMO coherence capability for Tx switching is absent:

    - If the per band per BC UL-MIMO coherence capability is absent, the per BC UL-MIMO coherence capability for 1Tx-2Tx switching is applicable to 2Tx-2Tx switching.

    - If both the per band per BC UL-MIMO coherence capability and per BC UL-MIMO coherence are absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the bands for 2Tx-2Tx switching.

Agreement:

Send LS out to RAN2 to inform them the agreements above.