3GPP TSG-RAN WG4 Meeting#101      	R4-2119967

E-meeting, 1st – 12th Nov, 2021

Title:	WF on FR2 CA BW classes

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.4.5

Document for:	Approval

# 1	Introduction

This WF is to track down discussions on CR[1] especially as there were proposal to reconsider earlier agreements [2].

From [2]

We make the following

Observation 1: adoption of Option 2 implies less flexible fallback with new restrictions on release of carriers, less flexible assignment of UEs within the operator block, less flexible UL assignment and less flexible adaptation to different operator block sizes.

It is recognized that the number of contiguous CC required for a 1600 MHz bandwidth is limited to 12 for Option 2, the number of CCs supported is subject to network and/or UE capability. This should also be consider in conjunction.

Proposal 1: deployment aspects should be considered further before agreeing on new CA BW classes for extension up to 1600 MHz and sending an LS to RAN2 asking for ASN.1 implementation.

Other options for Rel-17 can also be considered.

CR can be found from here

Draft CR for 38.101-2 for FR2 CA BW classes

# 2	Discussion

Do you see the need to reconsider the agreement [3] in because of deployment aspects [2]. CR is [1].

| Company | Comment |
| --- | --- |
| Qualcomm | We do not oppose creation of new BW classes, for example by extending FBG3. In the absence of a concrete alternative or supplementary proposal to do so, we would like to complete definition of previously agreed composite contiguous BW classes. |
| Verizon | We don’t see an immediate need other than the agreed CA bandwidth classes (FBG 3+2) at this time. In the absence of clear requirements, RAN4 should prioritize and complete the definition of FBG 3+2 before other proposals. One of shortage from possible extending FBG3 only is it would imply to not support larger than 100MHz carrier in the BW CA. This eliminates flexible adaptation of wider bandwidth carriers (200 or 400MHz), flexibility of HW utilization and possible overrated implementations. |
| MediaTek | We think RAN4 shall implement current valuable agreements to TS38.101-2 firstly, i.e. FBG 3+2. And then, we can further discuss other potential flexibility discussion based on demand. |
| CHTTL | If our understanding is correct, the Alt. 2 in R4-2117541 can allow better flexibility, and considering the legacy devices are using the blue part of 8x100MHz, the new UE with Alt.2 can have more flexibility on controlling the network load between 8x100MHz and additional 4x200MHz. But we are fine to go with majority view |
| Samsung | We think Alt-1 of R4-2117541 is exactly aligned with the agreement of last meeting. We need to focus on Alt-1 so that the FBG3+2 agreements can be implemented in TS38.101-2 with no variation. |
| ZTE | We agree with the current working assumption that we should first have a baseline to implement the agreement of last meeting, i.e. to agree a 38.101-2 CR as the first step. However, other alternatives with more flexible fallback should not be excluded. Companies are encouraged to provide further analysis on the issues raised in R4-2118136 at least for 1~2 additional meeting cycles. |
| Verizon | We would like to clarify our positon on the Alt. 2 in R4-2117541. We agree this alternative proposal and would suggest RAN4 considering it further. Still, we would like RAN4 to complete definition of previously agreed composite contiguous BW classes (FBG3+2). |

# 3	Conclusion

CR was discussed in GTW 11th Nov but was not yet agreeable. We need to comeback to this CR in next RAN4 meeting.

# 4 	References

[1] R4-2118447	Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classes MF,ME, MD and MA, RAN4#101e, Xiaomi

[2] R4-2118136	FR2 bandwidth class with multiple fall-back groups, RAN4#101e, Ericsson

[3] R4-2114963, WF on FR2 new CA BW Class, RAN4#101e, Nokia