3GPP TSG-RAN WG4 Meeting #101-e	R4-2120019

E-meeting, 1st – 12th Nov, 2021

Title:	WF on approaches for delta TIB and RIB optimization

Source:	ZTE Corporation

Agenda item:	9.3

Document for:	Approval

WF on approaches for delta TIB and RIB optimization.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic 3-1: Optimization to delta TIB and RIB | Issue 3-1A: Is a mix of rule-based and table-based approach for delta TIB and RIB optimization acceptable? Issue 3-1B: If the rule-based or mix approach is selected, from the perspective of protocol stability do we need to keep the rule set stable? Issue 3-1C: If the table-based or mix approach is selected, can option 2b be used as the optimization template of delta TIB and RIB? Issue 3-1D: When to apply the optimization of delta TIB and RIB if the optimization approach is agreed? Company views during 1st round discussion: - Issue 3-1A: - Option 1 (Yes): Huawei, Qualcomm, Skyworks, ZTE - Option 3 (Others): - Samsung: Given no further input on rule-based approach this meeting, maybe the rules (still with many exceptions) could be included in TR for reference but not included in TS. - Nokia: One option is that rules are used to derive the values but values itself are put into table as now. - Issue 3-1B: - Option 1 (Yes): Samsung, Skyworks, Qualcomm, ZTE - Option 3 (Others): - Huawei: I suppose we can specify the general requirements with some exceptions as REFSENS. These rules can be considered as general requirements. - Nokia: Stable rules is of course the goal but to say that those never need “maintenance” is hard. - Issue 3-1C: - Option 1 (Yes): Huawei, Nokia, Skyworks, Qualcomm, ZTE - Option 3 (Others): - Samsung: Prefer Option 2a proposed at last meeting. Option 2b is also acceptable. - CHTTL: Would like to have some clarification on the order of bands in configuration here. For example, DC_2-48_(n)5 the order of delta T/R from left to right is 2, 5, 48, n5 ? - Issue 3-1D: - Option 1 (Yes): Samsung, Huawei, Nokia, Qualcomm, ZTE - Option 3 (Others): - Skyworks: TBD with MCC. Recommendations for 2nd round: A WF to capture the above tentative agreements is suggested to be discussed in the 2nd round, including the new added note for (Issue 3-1C) and possible coordination for (Issue 3-1D). |

After the first round discussion, the following tentative agreements have been achieved:

(Issue 3-1A): It is suggested to use a mix of rule-based and table-based approach for delta TIB and RIB optimization.

(Issue 3-1B): Stable rules are the goal and beneficial to a stable specification, however it is hard to say no need “maintenance” for the rules.

(Issue 3-1C): Select Option 2b as the template for delta TIB and RIB table. A note should be added to the template to make the “band order” more clear for the configuration which includes intra-band contiguous EN-DC part.

(Issue 3-1D): It depends on the progress of rule-based approach. More coordination with basket WI rapporteurs and MCC is needed. If progress smooth, It can be applied at the last meeting of Rel-17 and implement in Rel-18.

Way Forward

- A mix of rule-based and table-based approach will be used for delta TIB and RIB optimization.

- Select Option (2b’) as the template for delta TIB and RIB table.

  - Option (2b’):

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |  |  |
| DC_1-3-7-40_n78 | 0.6 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_1-3-8-11_n28 | 0.3 | 0.8 | 0.6 | 0.9 | 0.6 |
| Note 1: “-” denotes ΔTIB,c = 0. Note 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-48_(n)5 the band order from left to right is 2, 5, 48 and n5. |  |  |  |  |  |

- Companies are encouraged to provide further analysis based on the following rules, but are not limited to the following rules.

  - L-bands in above are below 1GHz

  - H-bands above 1 GHz and below 3GHz

  - VH-bands are above 3 GHz

  - H-L dTib=0.3 and dRib=0

  - L-L and H-H dTib=0.5 and dRib=0 (exception 1+3 dTib=0.3 and dRib=0)

  - VH bands dTib=0.8 and dRib=0.5

  - L-VH: L dTib=0.3 and dRib=0 and VH band dTib=0.8 and dRib=0.5

  - H-VH: H dTib=0.5 and dRib=0 and VH band dTib=0.8 and dRib=0.5

  - Carriers in bands with a harmonic falling onto one of the DLs: dTIB = 0.6 dB

  - n79 with L and/or H bands dTib=0

  - if not specified above then dRib = dTib-0.5 dB

  - L-L-L and H-H-H case by case study

  - Combinations with notes are handled case by case

- Stable rule-set are our goal and beneficial to a stable specification, however it is hard to say no need “maintenance” for the rules in future.

- It is suggested to postpone to RAN4#101-bis-e/RAN4#102-e meeting to decide when to apply delta TIB/RIB optimization so as to have more time to evaluate the rule-based approach.