3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120197

Electronic Meeting, November 1-12, 2021

Agenda item:			4.1.7

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [101-e][202] NR_RRM_maintenance_R15_Perf

Document for:	Information

# Introduction

The documents in agenda item 4.1.7 contains CRs to correct test configuration or test cases. Following is the main topic:

- Topic #1: Correction to RRM test configuration and test cases

# Topic #1: Correction to RRM test configuration and test cases

## Companies’ contributions summary

| T-doc number | Proposals / Observations | Company | CR cat |
| --- | --- | --- | --- |
| R4-2117718 | Discussion on NE-DC RRM test cases | CMCC | N/A |
| R4-2117719 | Draft CR to TS 38.133 on test cases of NE-DC active BWP switch | CMCC, Apple, Intel | F |
| R4-2117720 | Draft CR to TS 38.133 on test cases of NE-DC active BWP switch | CMCC, Apple, Intel | A |
| R4-2117721 | Draft CR to TS 38.133 on test cases of NE-DC active BWP switch | CMCC, Apple, Intel | A |
| R4-2117732 | CR for test configuration correction of CSI-RS reference measurement channel for TDD SCS=120kHz | CMCC | F |
| R4-2117733 | CR for test configuration correction of CSI-RS reference measurement channel for TDD SCS=120kHz | CMCC | A |
| R4-2117734 | CR for test configuration correction of CSI-RS reference measurement channel for TDD SCS=120kHz | CMCC | A |
| R4-2117779 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-15) | Anritsu Corporation | F |
| R4-2117780 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-16) | Anritsu Corporation | F |
| R4-2117781 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-17) | Anritsu Corporation | F |
| R4-2117782 | FR2 Inter-frequency Relative SS-RSRP accuracy | Anritsu Corporation | N/A |
| R4-2117783 | Correction on the FR2 inter-frequency relative RSRP accuracy | Anritsu Corporation, MediaTek Inc. | F |
| R4-2117784 | Correction on the FR2 inter-frequency relative RSRP accuracy | Anritsu Corporation, MediaTek Inc. | A |
| R4-2117785 | Correction on the FR2 inter-frequency relative RSRP accuracy | Anritsu Corporation, MediaTek Inc. | A |
| R4-2117786 | Testability issue with FR2 CSI-RS based RLM TCs | Anritsu Corporation | N/A |
| R4-2118070 | Test cases for SFTD measurement delay under NE-DC | Intel Corporation, CMCC, Apple | F |
| R4-2118071 | Test cases for SFTD measurement delay under NE-DC | Intel Corporation, CMCC, Apple | A |
| R4-2118072 | Test cases for SFTD measurement delay under NE-DC | Intel Corporation, CMCC, Apple | A |
| R4-2118073 | Test cases for SFTD measurement accuracy under NE-DC | Intel Corporation, CMCC, Apple | F |
| R4-2118074 | Test cases for SFTD measurement accuracy under NE-DC | Intel Corporation, CMCC, Apple | A |
| R4-2118075 | Test cases for SFTD measurement accuracy under NE-DC | Intel Corporation, CMCC, Apple | A |
| R4-2118098 | Discussion on FR2 inter-frequency relative RSRP accuracy | MediaTek Inc. | N/A |
| R4-2118111 | CR for RRC Re-establishment test case in R15 | MediaTek Inc. | F |
| R4-2118112 | CR for RRC Re-establishment test case in R16 | MediaTek Inc. | A |
| R4-2118113 | CR for RRC Re-establishment test case in R17 | MediaTek Inc. | A |
| R4-2118787 | Correction to interruption test cases_R15 | Huawei, Hisilicon | F |
| R4-2118788 | Correction to interruption test cases_R16 | Huawei, Hisilicon | A |
| R4-2118789 | Correction to interruption test cases_R17 | Huawei, Hisilicon | A |
| R4-2118854 | Discussion on FR2 inter-frequency relative RSRP accuracy | Huawei, Hisilicon | N/A |
| R4-2119239 | Test case for LTE PSCell addition and release in NE-DC (R15) | Apple, Intel, CMCC | F |
| R4-2119240 | Test case for LTE PSCell addition and release in NE-DC (R16) | Apple, Intel, CMCC | A |
| R4-2119241 | Test case for LTE PSCell addition and release in NE-DC (R17) | Apple, Intel, CMCC | A |
| R4-2119259 | Draft CR to TS 38.133: Corrections to radio link monitoring test cases (Rel 15) | Rohde & Schwarz | F |
| R4-2119260 | Draft CR to TS 38.133: Corrections to radio link monitoring test cases (Rel 16) | Rohde & Schwarz | A |
| R4-2119261 | Draft CR to TS 38.133: Corrections to radio link monitoring test cases (Rel 17) | Rohde & Schwarz | A |
| R4-2119571 | Rel-15 Cat-F CR to RMC CORESET reference channel for test cases A.5.3.2.2.x | Qualcomm Incorporated | F |
| R4-2119572 | Rel-16 Cat-A CR to RMC CORESET reference channel for test cases A.5.3.2.2.x | Qualcomm Incorporated | A |
| R4-2119573 | Rel-17 Cat-A CR to RMC CORESET reference channel for test cases A.5.3.2.2.x | Qualcomm Incorporated | A |
| R4-2119576 | Rel-15 Cat-F CR to E-UTRAN - NR FR2 interruptions at transitions between active and non-active during DRX in Xsynchronous EN-DC A.5.5.2.x | Qualcomm Incorporated | F |
| R4-2119577 | Rel-16 Cat-A CR to E-UTRAN - NR FR2 interruptions at transitions between active and non-active during DRX in Xsynchronous EN-DC A.5.5.2.x | Qualcomm Incorporated | A |
| R4-2119578 | Rel-17 Cat-A CR to E-UTRAN - NR FR2 interruptions at transitions between active and non-active during DRX in Xsynchronous EN-DC A.5.5.2.x | Qualcomm Incorporated | A |

## Open issues summary

### Sub-topic 1-1: NE-DC RRM TCs

According to R4-2117718 (CMCC):

- Proposal 1: It is propose to following RRM test cases from Rel-15 specification.

| Test case | Core requirements |
| --- | --- |
| SFTD delay under NE-DC | 38.133 section 9.6.2.2 |
| SFTD measurement accuracy for NE-DC | 38.133 section 10.1.21.1 |
| LTE PSCell addition in NE-DC | 38.133 section 8.8.2 38.133 section 8.2.3.2.3 |
| NE-DC active BWP switch | 36.133 section 7.36.2.6 |

- Proposal 2: It is propose to introduce NE-DC RRM test cases only for FR1 PCell+E-UTRAN PSCell.

- Recommended WF

- Discuss the above proposed test cases for NE-DC.

| Company | Comments |
| --- | --- |
| Nokia | We do not agree the proposal on test cases for NE-DC. In RAN4#91 meeting, it was agreed that no additional test cases will be defined for NE-DC which captured in AI 6.2.3 in chairman notes. Refer to Ericsson’s paper R4-1906451: the reason is to avoid excessive testing burden on UE. RRM requirements for NE-DC are very similar to RRM requirements for EN-DC, verification of most requirements in an EN-DC configuration would already give a high degree of confidence that the UE would operate correctly in an NE-DC configuration. |
| CMCC | We disagree with Nokia’s comments on the excessive testing burden. For NEDC test cases, our proposal is only to add additional 4 test cases, which are essential functions including PSCell addition and release, BWP switching interruption on LTE and SFTD. In our understanding, NE-DC UE will support SA, so most requirements can be guaranteed by SA test cases. But we cannot say the NE-DC UE will all support EN-DC mode. So these test cases are necessary. |
| Huawei | We are wondering whether it is an appropriate timing point to introduce new branch of test cases in Rel-15 at current stage (late stage of Rel-17). It is suggested to discuss whether to have test cases for NE-DC in later release (e.g. Rel-17) . |
| Ericsson | We have same view as Nokia. R4-1906451 was not agreed, but RAN4#91 had the following agreements: Agreement: - No additional tests are developed for NE-DC - Develop tests in the table below for NR-DC - Whether to introduce TC V.2 needs be revisited, if the feasibility problem is identified for this test. TC V.1; FR2 NR-DC PSCell addition delay test; Analagous to SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 in non-DRX”. UE is configured with an FR1 PCell and the delay requirement to add an FR2 PSCell is verified in non DRX. TV V.2; NR-DC FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA; Analagous to “NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA “ BWP switch is performed on FR1. BWP switching delay on FR1 and interruptions to FR2 PSCell are verified.. We suggest to follow the agreements. |
| CMCC3 | To Huawei: regarding the release issue, NE-DC band combinations can be release independent from Rel-15. In order to make the feature complete, the RRM test cases should also be captured in Rel-15 38.133 To Ericssona/Nokia: At that time, no operator showed interests in NEDC deployment. However, currently, many operators have plan to deploy NEDC network as discussed in previous RAN plenary, and RAN5 is also working on speeding up the NEDC test cases. 3GPP should specify requirements aimed to satisfy operators’ 5G deployment. From technical perspective, NEDC PSCell addition/release, NR BWP switching interruption on LTE and SFTD cannot be verified if no test cases are defined since we cannot assume NEDC UE always support ENDC From test burden perspective, only 4 additional test cases are introduced. I do not believe these 4 test cases will increase the test efforts significantly. And the test burden should not be a problem for network vendor; |
| Intel | We agree with CMCC comments. One extra reason that we have to introduce NE-DC test cases is that NE-DC UE which is not capable of EN-DC has no verification for certain functionality if we do not introduce proper test cases. Regarding the agreements in RAN4#91, things change apparently over this quite long period of time and now we would like to ask the group to consider seriously the requests from operators. |

### Sub-topic 1-2: FR2 inter-frequency relative RSRP accuracy

According to R4-2117782 (Anritsu):

- TS 38.133 Table 10.1.5.1.2-1 refers to a relative accuracy relaxation Ginter for the inter-band case, to be defined in a new clause B.2.1.5.2. An example markup is shown in section 4.1.

- Misalignment between the Rough Beam peak direction and the Fine Beam peak direction D is defined in a new clause B.2.1.5.3. An example markup is shown in section 5.

- TS 38.133 Test requirements in Tables A.5.7.1.2.3-2 and A.7.7.1.2.3-2 use the new parameters “Ginter” and “D” to set the test requirements, and refer to the new clauses B.2.1.5.2 and B.2.1.5.3. An example markup is shown below.

According to R4-2118098 (MediaTek):

- Observation 1: In the FR2 inter-frequency RSRP relative accuracy test, the beam peak AoA2 for Cell 3 is based on fine beam, while SS-RSRP measurement can be conducted by rough beam.

- Observation 2: UE may achieve worse rough beamforming gain at AoA2 (i.e. at the fine beam peak) than at AoA1 (selected from spherical coverage).

- Observation 3: Current SS-RSRP relative accuracy test requirement mandates UE to equalize the antenna gain difference between 2 frequencies or even 2 bands, which seems not well-discussion in Rel-15.

- Proposal 1: For the relative inter-frequency accuracy requirement, the following two additional margins should be considered:

- Mis-alignment between fine beam and rough beam

- Different antenna gain on different bands

- Proposal 2: For the test case of FR2 inter-frequency relative RSRP accuracy, to add 5.5 (D) dB and 8.5 (D+ Ginter) dB margin in the lower bound for intra-band and inter-band, respectively, where

- D (Mis-alignment between fine beam and rough beam) = 5.5

- Ginter (Different antenna gain on different bands) = 3

- Proposal 3: For the test case of FR2 inter-frequency relative RSRP accuracy, to 3 (Ginter) dB margin in the upper bound for inter-band only, where

  - Different antenna gain on different bands (Ginter) = 3 dB

According to R4-2118854 (Huawei):

  - Observation 1: Misalignment of rough beam peak and fine beam peak and gain difference among different frequencies may lead to that the received power from AoA2 is lower that the power from AoA1.

  - Observation 2: The maximum RSRP difference (SSB_RP2 – SSB_RP1) could be larger than – X as defined in the test requirements.

  - Proposal 1: Both the lower bound and upper bound of the reported RSRP difference in the test requirements should be reconsidered.

  - Proposal 2: RAN4 to discuss adding additional margin in the upper bound of test requirements for FR2 inter-frequency RSRP accuracy. Consider additional margin of Y dB as starting point.

- Issue 1-2-1: Type of margins in FR2 inter-frequency relative RSRP accuracy TCs

  - Proposal 1: Anritsu, MediaTek

    - Type of margin needed in FR2 intra-band inter-frequency relative RSRP accuracy TCs:

      - Margin due to misalignment between fine beam and rough beam (D)

    - Type of margin needed in FR2 inter-band inter-frequency relative RSRP accuracy TCs:

      - Margin due to misalignment between fine beam and rough beam (D), and

      - Margin due to different antenna gain on different bands (Ginter)

  - Recommended WF

    - Discuss if above proposal 1 is acceptable?

| Company | Comments |
| --- | --- |
| Qualcomm | In the previous meeting we agreed that, in principle, both margins are needed. i.e. a first margin to account for misalignment between peak directions of fine beam and rough beam and a second margin to account for variation in antenna gain across frequency. Proposal 1 further suggests that the second margins should be zero in the case of intra-band measurements. We can support the proposal. However, we think issue 1-2-3 raised by Huawei should be addressed as well before finalizing these margins, so that we end up with meaningful requirements. |
| MediaTek | Support proposal 1. We prefer to add this margin in lower bound first and further discuss whether and how to capture the margin for upper bound. Or maybe we can have a way forward to capture all the options. |
| Huawei | We would like clarify that proposal 1 is the margin for lower bound. For the margin for upper bound as analyzed in our paper, it should include: - Margin due to gain difference between rough peak direction and rough spherical coverage direction. - Margin due to gain difference on different frequency. For lower bound. We are generally fine with the value, but share same views as QC that the margin to lower band and upper bound cannot be decoupled completely. |
| Ericsson | We can accept the proposal 1. We don’t want to mix up issue 1-2-1 and issue 1-2-3. These issues should be discussed separately. |
| Anritsu | We share the same view with Ericsson and we think that 1) Proposal 1 still can be applied to the left side of the test requirement, and 2) Ginter can also be added as the gain difference due to the different bands at the right side of the requirement. Because the error in the issue 1-2-3 is caused by the antenna gain difference between beam peak and spherical coverage and it does not contain the frequency factor, we believe we can split the discussion of 1-2-3 with issue 1-2-1 and 1-2-2. |

- Issue 1-2-2: How to derive margins in FR2 inter-frequency relative RSRP accuracy TCs?

  - Proposal 1: Anritsu, MediaTek

    - D depends on UE power class:

      - D=5.5 dB for UE power class 3.

    - Ginter depends on UE power class:

      - Ginter =3 dB for UE power class 3.

- Recommended WF

  - Discuss if above proposal 1 is acceptable?

| Company | Comments |
| --- | --- |
| Qualcomm | Given that Huawei has identified a new issue 1-2-3, we think it may be better to find a comprehensive solution for how to specify the requirements before deciding on the proposed margin values. |
| MediaTek | Support proposal 1. For the derivation, please check our discussion paper R4-2118098. |
| Huawei | Similar views as QC. |
| Ericsson | We can accept the proposal 1. |
| Anritsu | Support proposal 1. |

- Issue 1-2-3: Any additional margin needed in FR2 inter-frequency relative RSRP accuracy TCs?

- Proposal 1: Huawei

  - Additional margin needed in the upper bound/right side of test requirements for FR2 inter-frequency RSRP accuracy

- Recommended WF

  - Discuss if above proposal 1 is acceptable?

| Company | Comments |
| --- | --- |
| Qualcomm | We understand that there is a valid argument behind proposal 1. i.e. -X is the difference between the RefSens and Spherical coverage requirements but in actuality a compliant UE may outperform each of these requirements by a different amount. A larger margin may need to be allowed for UEs that achieve a larger dynamic range between beam peak and spherical coverage. However, it’s not clear how to determine/derive the additional margin from existing requirements. |
| MediaTek | Support proposal 1. RAN4 can further discuss how much is the margin should be added. |
| Huawei | Support option 1. We think it is unreasonable to use–X to represent the maximum RSRP difference. –X is derived as Refsens (fine beam) – spherical coverage (fine peak). If UE has better gain at peak direction or rough peak gain is better than the minimum required difference “Y” dB. It is easily that RSRP difference from rough peak and rough spherical coverage exceed –X. Theoretically, the fine beam difference between peak direction and spherical coverage direction could be larger than –X. And considering difference between fine beam peak and rough beam peak, the difference between rough peak and rough spherical coverage could be even larger. |
| Ericsson | It is not clear for us why we need additional margin since we have already had margin X (X is the Spherical coverage gain difference) to account for it |

- Issue 1-2-4: If proposal 1 in 1-2-3 is agreeable, then how to derive additional margins in FR2 inter-frequency relative RSRP accuracy TCs?

  - Proposal 1: Huawei

    - Additional margin = Y dB.

- Recommended WF

  - Discuss if above proposal 1 is acceptable?

| Company | Comments |
| --- | --- |
| Anritsu | Though we understand the idea that a gain variety may exist between the direction of beam peak and spherical coverage, and that may require additional margins, we do not think Y can be applied to the right side of the test requirement. Since Y is the gain difference between fine beam and rough beam at the beam peak direction, we suppose the newly proposed additional margin cannot reach that level as far as the test requirement is assuming rough beams for both beam peak direction and spherical coverage direction. (i.e. additional margin should be smaller than Y.) |
| Qualcomm | We agree that the additional margin may be upper bounded by Y but we should see if there are alternative approaches that would avoid excessive relaxation of the requirements. Needs more discussion. |
| MediaTek | To our understanding, for upper bound, Ginter may be reused but the beam mis-alignment issue in upper bound can be FFS. |
| Huawei | We support option 1 to take Y dB starting point if –X is kept in the test requirements. And we think there is no need to add Ginter for the upper bound. The reason is that for -X, it already reflects the gain difference from different bands as –X is calculated by the operation bands. |
| Ericsson | See our comment on Issue 1-2-3. It is not clear to us why we need additional margin on top of the existing margin. |
| Anritsu2 | To Huawei, for upper bound, we believe that Ginter is still necessary since X is derived assuming the same frequency, and Ginter is for the gain difference due to the different band. |

### Sub-topic 1-3: Testability issue with FR2 CSI-RS based RLM TCs

According to R4-2117786 (Anritsu):

- Observation 1: With the current parameter of CSI-RS based RLM test cases, there is a case that estimated Io exceeds the power level that the test equipment can provide when we consider a test tolerance.

- Observation 2: It is challenging to carry out the CSI-RS-based RLM test under the current test conditions also with 2AoA at frequencies for band n261 and higher.

- Observation 3: It is not allowed to configure CSI-RS for tracking with 24 RB allocation also with 66 RB BWP setting, which makes the total power at symbols where CSI-RS exists bigger than other symbols..

- Observation 4: One possible solution is to reduce the BWP size to 24 RBs.

- Observation 5: If BWP size is changed to 24 RBs, then the FR2 CSI-RS based RLM test cases will become testable with 2 AoA condition at least for band n257, n258, n260 and n261.

- Observation 6: If test setup is changed to 1AoA (beam peak), then the FR2 CSI-RS based RLM test cases will become testable for FR2-1 (up to band n259). FFS for band n262.

- Proposal 1: For CSI-RS based RLM test cases in FR2, reduce the BWP size setting to 24 RBs.

- Recommended WF

  - Further discuss if the following is acceptable?

    - For CSI-RS based RLM test cases in FR2, reduce the BWP size setting to 24 RBs

| Company | Comments |
| --- | --- |
| Qualcomm | We understand the issue and agree that we should keep 2 AoA test setup. In principle, we are okay with the recommended WF. And it would be safer if companies can re-evaluate the current requirement can be still met with this side condition chance, i.e. narrower TRS may degrade UE performance which may or may not lead to RLM performance impact. |
| Nokia | The core requirements are based on 48 RBs, if we reduce to 24RBs, how to guarantee the core requirements are verified correctly? |
| Ericsson | We are ok with the recommended WF. |
| R&S | We agree keep the TC as 2AoA with the necessary changes. |
| Anritsu | We are ok to set the re-evaluation period to see the impact to the RLM performance. But since there is no associated 2 AoA RRM test system available yet, we would like to ask a help from UE vendors and chipset vendors. |

### Sub-topic 1-4: CRs on RRM tests and test configurations

- Directly provide comments on the cat-F CRs in section 1.3.2

## Companies views’ collection for 1st round

### Open issues

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117719 | Nokia: NOK, see our comments in sub-topic 1-1 for NE-DC test cases. |
|  | Huawei: Depends on the conclusion of sub-topic 1-1. |
|  | Ericsson: Depending on the conclusion on Sub-topic 1-1. |
| R4-2117732 | Ericsson: OK |
| R4-2117779 | Qualcomm: Changes#2-5: Okay Change#1: Reducing AL to 2 will make it harder for the UE to pass as it will increase the misdetection probability. Can we alternatively consider increasing the BW of DLBWP.1.3 above 25RBs yet smaller than the full channel? |
|  | Huawei: Can Anritsu explain more about how T2 = 1.5 seconds (A.4.5.7.1) are derived to cover measurement +reprot + RRC reoncfiguration? |
|  | Ericsson: OK |
|  | Anritsu to Qualcomm: We are fine to keep the AG level as 4 and alternatively increasing the BW of DLBWP above 25RBs to cover whole CORESET. However since we have no background information on the reason why 25 RBs was chosen for 15 kHz SCS in DLBWP.1.3, it is appreciated that companies could advise if 31 RBs (25+6 RBs) is OK as the new setteing, perhaps for DLBWP.1.4 (new). To Huawei: Actually the measurement report delay from the UE may take 922 ms based on the description at 8.4.2 and clause 12 in TS 38.331. And after that the test system sends the RRC message. But the test system faces in short to send the RRC message for gap release by the end of T2 (currently 1.0 s). So we proposed to extend another 500 ms as T2 period. |
| R4-2117780 | Ericsson: OK |
| R4-2117781 | Ericsson: OK |
| R4-2117783 | Qualcomm: pending issue on Sub-topic 1-2 |
|  | Huawei: it depends on the conclusion of sub-topic 1-2 |
|  | Ericsson: OK |
| R4-2118070 | Qualcomm: There is no EN-DC test for SFTD, we want to understand why we want to have it for NE-DC? Is the E-UTRA to NR inter-RAT test not sufficient? |
|  | Nokia: NOK, see our comments in sub-topic 1-1 for NE-DC test cases. |
|  | Huawei: Depends on the conclusion of sub-topic 1-1. |
|  | Ericsson: Depending on the conclusion on Sub-topic 1-1 |
|  | Intel: One compromise could be to note in the spec that the UE will only need to pass one of the SFTD test cases between E-UTRA SA and NE-DC. |
| R4-2118073 | Qualcomm: In A.4.7.5 and A.8.4.1, EUTRA cell configurations are using Table A.3.7.2.1, which is different from Table A.4A.X.1.1.2-3. If there is no particular reason for changing parameters, we should follow legacy tests |
|  | CMCC: The parameters in A.4A.X.1.1.2-3 actually are the same as Table A.3.7.2.1. In our CR R4-2117719, we add a new section A.3A.7 for NE-DC test setup, and the parameters are referred to table A.3.7.2.1. So maybe the CR can be revised to use table A.3.7.2.1 for EUTRA cell configuration. |
|  | Nokia: NOK, see our comments in sub-topic 1-1 for NE-DC test cases. |
|  | Huawei: Depends on the conclusion of sub-topic 1-1. |
|  | Ericsson: Depending on the conclusion on Sub-topic 1-1 |
| R4-2118111 | Anritsu: As for the 2nd bullet descriptions at the reason for change “Thus, evaluation period for OOS can be 5 s with 200 ms margins.” could MediaTek provide the rationale of the 200 ms margin? But from the TC implementation point of view, we do not have an issue in either way of adding the margin or not. |
|  | Qualcomm: Why P sharing factor is 3 in the test? We don't see SSB is shared with other measurement in the configuration |
|  | MediaTek: To Anritsu: According to clause 8.1.5 in TS 38.133, UE needs to turn off transmitter within 40ms. Thus, to our understanding it should be captured in T2. Initially, to be simple, the 200ms is proposed. But we are ok to specify 40ms in test case. Besides, we also found that, in A.6.3.2.1, the additional margin 40 ms should also be added. The revised T2 is provided as follows: - T2 in A.6.3.2.1.3: 6.24 s - T2 in A.7.3.2.1.1: 4.84 s - T2 in A.7.3.2.1.2: 4.84 s - T2 in A.7.3.2.1.3: 10.84 s To Qualcomm: For A.7.3.2.1.1, A.7.3.2.1.2 and A.7.3.2.1.3, the SSB and SMTC configurations are provided as follows: TC; A.7.3.2.1.1, A.7.3.2.1.2 and A.7.3.2.1.3. SSB configuration; SSB.1 FR2 - SCS: 120kHz - SSB periodicity 20ms - Slot num containing SSB: 0. SMTC configuration; SMTC pattern 1 - SMTC periodicity: 20ms - SMTC offset: 0 ms - SMTC duration 1ms. According to the above table, the SSB is fully overlapped with SMTC. Thus, according to the clause 8.1.2.2 (RLF) in TS 38.133, the P will be 3. |
|  | Anritsu 2: To MediaTek, thank you for the explanation, understood. We prefer the 40 ms delay. |
| R4-2118787 | Anritsu: In Table A.4.5.2.4.1-3, a unit of the parameter “Time offset to Cell1“ should also be corrected from ms to us. |
|  | Nokia: since the test case cover both inter-band EN-DC and intra-band EN-DC, it seems not reasonable with the change of cell offset to 3us for inter-band test. We need to test for both scenarios with different suitable settings. |
|  | Huawei: Thank very much for Nokia's comments. Yes, we agree using 3us and 33us for intra-band and inter-band case respectively is also OK. However, We're not sure whether it is neccessary. In our opinion, the parameter "Cell offset" is only used to ensure timing difference requirements for sync EN-DC is satisfied. And 3us can satisify the requirement for both inter-band and intra-band case at one time. Furthermore, using single value is also simpler for TE implementation. So we still suggest using 3us for both cases. |
|  | Ericsson: We are fine with the changes. |
| R4-2119239 | Qualcomm: When we checked the reference test cases from LTE and NR for LTE PSCell addition, some parameters do not look identical. Please check T2 and power level configurations Table A.4A.x.1.1-4, and provide explanations on where and how those numbers are derived. Apple: power level for E-UTRAN cell in Table A.4A.x.1.1-4 is duplicated from Table A.3.7.2.1-1. Power level for NR cell in Table A.4A.x.1.1-3 is duplicated from EN-DC PSCell addition test in A.4.5.7.1.1-3. We found that there is discrepancy between TS38133-ff0 and TS38133-h30 regarding power level in Table A.4.5.7.1.1-3. In R15 spec -88dBm is used while in R17 spec -85dBm is used. We are not sure if this is reason why QC raise this issue. This CR is based on TS38133-ff0. Technically, we only need to make sure E-UTRAN power level is higher than B1 threshold starting from T2. In our CR B1 is -96dBm and E-UTRAN power level is -88dBm. There is 8dB margin, which shall be larger enough to cover measurement uncertainty. |
|  | Nokia: NOK, see our comments in sub-topic 1-1 for NE-DC test cases. |
|  | Huawei: Depends on the conclusion of sub-topic 1-1. |
|  | Ericsson: Depending on the conclusion on Sub-topic 1-1 |
| R4-2119259 | Ericsson: OK |
| R4-2119571 | Ericsson: OK |
|  | R&S: For the change, can we use the terminology “Dedicated CORESET Reference Channel”? |
| R4-2119576 | Huawei: Can companies explain more about the logic behind this why two consecutive DRXs shall not be detected during 1600xT1? |
|  | Ericsson: OK |

## Summary for 1st round

### Open issues

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic 1-1 | Issue 1-1-1: NE-DC RRM TCs Tentative agreements: None Candidate options: Option 1: - Develop following NE-DC RRM test cases for FR1 PCell+E-UTRAN PSCell. Test case; Core requirements. SFTD delay under NE-DC; 38.133 section 9.6.2.2. SFTD measurement accuracy for NE-DC; 38.133 section 10.1.21.1. LTE PSCell addition in NE-DC; 38.133 section 8.8.2 38.133 section 8.2.3.2.3. NE-DC active BWP switch; 36.133 section 7.36.2.6. Option 2: - Do not develop any NE-DC RRM tests upholding previously RAN4 agreement (section 6.2.3 in RAN4#91 meeting minutes in R4-1907901). Recommendations for 2nd round: Further discuss the options |
| Sub-topic 1-1 | Issue 1-1-2: Release for NE-DC RRM TCs if they are agreed? Tentative agreements: New issue Candidate options: Under the assumption that Option 1 in issue 1-1-1 is agreed, the release in which the NE-DC RRM TCs should be defined? - Option 1; Release 15 - Option 2: Release 17 - Option 3: Other Release ? Recommendations for 2nd round: Further discuss the options |
| Sub-topic 1-2 | Issue 1-2-1: Type of margins in FR2 inter-frequency relative RSRP accuracy TCs Tentative agreements: None Candidate options: - Proposal 1: Agree the following as lower bound margin: - Type of margin needed in FR2 intra-band inter-frequency relative RSRP accuracy TCs: - Margin due to misalignment between fine beam and rough beam (D) - Type of margin needed in FR2 inter-band inter-frequency relative RSRP accuracy TCs: - Margin due to misalignment between fine beam and rough beam (D), and - Margin due to different antenna gain on different bands (Ginter) - Proposal 2: Defer the agreement on lower bound until upper bound margin issue is resolved. Recommendations for 2nd round: Further discuss the options |
| Sub-topic 1-2 | Issue 1-2-2: How to derive margins in FR2 inter-frequency relative RSRP accuracy TCs? Tentative agreements: None Candidate options: - Proposal 1: Agree the following values for lower bound margin: - D depends on UE power class: - D=5.5 dB for UE power class 3. - Ginter depends on UE power class: - Ginter =3 dB for UE power class 3. - Proposal 2: Defer the agreement on values of D and Ginter until upper bound margin issue is resolved. Recommendations for 2nd round: Further discuss the options |
| Sub-topic 1-2 | Issue 1-2-3: Any additional margin needed in FR2 inter-frequency relative RSRP accuracy TCs? Tentative agreements: None Candidate options: - Proposal: - Additional margin needed in the upper bound/right side of test requirements for FR2 inter-frequency RSRP accuracy? - Option 1: Yes. - Option 2: No. Recommendations for 2nd round: |
| Sub-topic 1-2 | Issue 1-2-4: If proposal 1 in 1-2-3 is agreeable, then how to derive additional margins in FR2 inter-frequency relative RSRP accuracy TCs? Tentative agreements: None Candidate options: - Proposal 1: - Additional margin = Y dB. Recommendations for 2nd round: Further discuss the options |
| Sub-topic 1-3 | Sub-topic 1-3: Testability issue with FR2 CSI-RS based RLM TCs Tentative agreements: CSI-RS based RLM test cases in FR2 are conducted with 2 AoA Candidate options: Further discuss impact of reducing BWP size setting from 48 RBs to 24 RBs in the CSI-RS based RLM tests on core requirements. Any impact of meeting CSI-RS based RLM core requirements? - Option 1: Yes. In this case proponents suggest solution. - Option 2; No Recommendations for 2nd round: Further discuss the options |

### CRs/TPs

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117719 | Draft CR to TS 38.133 on test cases of NE-DC active BWP switch | CMCC, Apple, Intel | Return to | Due to sub-topic 1-1. |
| R4-2117732 | CR for test configuration correction of CSI-RS reference measurement channel for TDD SCS=120kHz | CMCC | Agreeable |  |
| R4-2117779 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-15) | Anritsu Corporation | Revised |  |
| R4-2117780 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-16) | Anritsu Corporation | Revised |  |
| R4-2117781 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-17) | Anritsu Corporation | Revised |  |
| R4-2117783 | Correction on the FR2 inter-frequency relative RSRP accuracy | Anritsu Corporation, MediaTek Inc. | Return to | Due to sub-topic 1-2. |
| R4-2118070 | Test cases for SFTD measurement delay under NE-DC | Intel Corporation, CMCC, Apple | Return to | Due to sub-topic 1-1. |
| R4-2118073 | Test cases for SFTD measurement accuracy under NE-DC | Intel Corporation, CMCC, Apple | Return to | Due to sub-topic 1-1. |
| R4-2118111 | CR for RRC Re-establishment test case in R15 | MediaTek Inc. | Revised |  |
| R4-2118787 | Correction to interruption test cases_R15 | Huawei, Hisilicon | Revised |  |
| R4-2119239 | Test case for LTE PSCell addition and release in NE-DC (R15) | Apple, Intel, CMCC | Return to | Due to sub-topic 1-1. |
| R4-2119259 | Draft CR to TS 38.133: Corrections to radio link monitoring test cases (Rel 15) | Rohde & Schwarz | Agreeable |  |
| R4-2119571 | Rel-15 Cat-F CR to RMC CORESET reference channel for test cases A.5.3.2.2.x | Qualcomm Incorporated | Revised |  |
| R4-2119576 | Rel-15 Cat-F CR to E-UTRAN - NR FR2 interruptions at transitions between active and non-active during DRX in Xsynchronous EN-DC A.5.5.2.x | Qualcomm Incorporated | Return to | Need clarification |

## Discussion on 2nd round (if applicable)

### Sub-topic 1-1: NE-DC RRM TCs

### Sub-topic 1-2: FR2 inter-frequency relative RSRP accuracy

### Sub-topic 1-3: Testability issue with FR2 CSI-RS based RLM TCs

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117719 (return to) |  |
| (Rev of R4-2117779) |  |
| (Rev of R4-2117780) |  |
| (Rev of R4-2117781) |  |
| R4-2117783 (return to) |  |
| R4-2118070 (return to) |  |
| R4-2118073 (return to) |  |
| (Rev of R4-2118111) |  |
| (Rev of R4-2118787) |  |
| R4-2119239 (return to) |  |
| (Rev of R4-2119571) |  |
| R4-2119576 (return to) |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on Rel-15 NR RRM test cases | Ericsson | To capture agreements |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117719 | Draft CR to TS 38.133 on test cases of NE-DC active BWP switch | CMCC, Apple, Intel | Return to | Due to sub-topic 1-1. |
| R4-2117732 | CR for test configuration correction of CSI-RS reference measurement channel for TDD SCS=120kHz | CMCC | Agreeable |  |
| R4-2117779 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-15) | Anritsu Corporation | Revised |  |
| R4-2117780 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-16) | Anritsu Corporation | Revised |  |
| R4-2117781 | Draft CR to TS 38.133: NR_newRAT-Perf maintenance (Rel-17) | Anritsu Corporation | Revised |  |
| R4-2117783 | Correction on the FR2 inter-frequency relative RSRP accuracy | Anritsu Corporation, MediaTek Inc. | Return to | Due to sub-topic 1-2. |
| R4-2118070 | Test cases for SFTD measurement delay under NE-DC | Intel Corporation, CMCC, Apple | Return to | Due to sub-topic 1-1. |
| R4-2118073 | Test cases for SFTD measurement accuracy under NE-DC | Intel Corporation, CMCC, Apple | Return to | Due to sub-topic 1-1. |
| R4-2118111 | CR for RRC Re-establishment test case in R15 | MediaTek Inc. | Revised |  |
| R4-2118787 | Correction to interruption test cases_R15 | Huawei, Hisilicon | Revised |  |
| R4-2119239 | Test case for LTE PSCell addition and release in NE-DC (R15) | Apple, Intel, CMCC | Return to | Due to sub-topic 1-1. |
| R4-2119259 | Draft CR to TS 38.133: Corrections to radio link monitoring test cases (Rel 15) | Rohde & Schwarz | Agreeable |  |
| R4-2119571 | Rel-15 Cat-F CR to RMC CORESET reference channel for test cases A.5.3.2.2.x | Qualcomm Incorporated | Revised |  |
| R4-2119576 | Rel-15 Cat-F CR to E-UTRAN - NR FR2 interruptions at transitions between active and non-active during DRX in Xsynchronous EN-DC A.5.5.2.x | Qualcomm Incorporated | Return to | Need clarification |
| R4-2117718 | Discussion on NE-DC RRM test cases | CMCC | Noted |  |
| R4-2117782 | FR2 Inter-frequency Relative SS-RSRP accuracy | Anritsu Corporation | Noted |  |
| R4-2117786 | Testability issue with FR2 CSI-RS based RLM TCs | Anritsu Corporation | Noted |  |
| R4-2118098 | Discussion on FR2 inter-frequency relative RSRP accuracy | MediaTek Inc. | Noted |  |
| R4-2118854 | Discussion on FR2 inter-frequency relative RSRP accuracy | Huawei, Hisilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Anritsu corp. | Osamu Yamashita | Osamu.Yamashita@anritsu.com |
| Qualcomm | CH Park | chparkqck@qti.qualcomm.com |
| CMCC | Zhang Xiaoran | zhangxiaoran@chinamobile.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)