3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120277

Electronic Meeting, November 1-12, 2021

Agenda item:			5.1.5

Source:	CATT

Title:	WF on NR CSI-RS based L3 measurement requirements maintenance

Document for:	Approval

Introduction

This is the WF to capture all the agreements and open issues in [101-e][210] Maintenance_NR_CSIRS_L3meas_NWM email discussion for RAN4#101e meeting.

UE behavior when the timing offset exceeds the threshold with single FFT assumption

Issue 1-1-1 UE behavior when the timing offset exceeds the threshold

- Agreement:

      - When the timing offset exceeds the threshold  X

        - X = CP + Y, where Y is the extra margin. Y is FFS.

        - UE is not required to report CSI-RS based L3 measurements.

        - If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold.

        - Do not define test cases to verify UE behavior for the case when the timing offset is above the threshold.

Reference

- R4-2120353 Email discussion summary for [101-e][210] Maintenance_NR_CSIRS_L3meas_NWM, CATT, RAN4#101e