3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120285

Electronic Meeting, November 01-12, 2021

Agenda item:			8.8.1&8.8.2

Source:	CMCC

Title:	WF on RRM for FR1 HST

Document for:	Approval

Introduction

This is the WF to capture all agreements and open issues in [101-e][215] NR_HST_FR1_enh_RRM_1 for RAN4#101e meeting

Topic #1: intra-frequency measurements

Sub-topic 1-1 NSCC_SSB for CSSFoutside_gap,i

Issue 1-1-1: NSCC_SSB for CSSFoutside_gap,i

- Agreements in 1st round

  - From Rel-17 to have NSCC_SSB clarification/correction, the NSCC_SSB clarification/correction is “only SCell(s) measured without MG are counted in NSCC_SSB for the calculation of CSSFoutside_gap,i”.

Issue 1-1-2: whether to consider the case where SMTCs of SCells are not fully overlapped in the enhancements of Nscc_ssb

- Agreements in 2nd round

  - Conclusion in issue 1-1-1 is enough, i.e. only to consider whether SMTCs of SCells are measured within gaps or outside gaps.

Topic #2: inter-frequency measurements

Sub-topic 2-1: inter-frequency measurement in idle state

Issue 2-1-1: how to perform the enhancement for inter-frequency measurement in idle mode for HST

- Agreements in GTW

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.2 x M2 (10 x M2)] | [0.32 x M3 (1 x M3)] | 0.96 x M4 (3 x M4) |
| 0.64 | [6.4 (10)] | [0.64 (1)] | 1.92 (3) |
| 1.28 | [10.24 (8)] | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

Issue 2-1-2: For idle mode, FFS on the principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured

- Agreements in 1st round

  - For idle mode, following principle on the requirements will be used if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured:

NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf

Sub-topic 2-2: inter-frequency measurement with MG, connected state

Issue 2-2-1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

- Agreements in 2nd round:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | FFS |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Topic #3: other

Sub-topic 3-1: L1-SINR

Issue 3-1-2: how to specify the requirements when L1-SINR is applied in high speed scenario

- Option 1 (HW, QC, Ericsson, MTK, ZTE, Intel, CMCC): Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

- Option 2 (Nokia, vivo): Current L1-SINR measurement requirement can be reused in HST, no upper bound of side condition

Sub-topic 3-2: SCell link recovery

Issue 3-2-1: for Scell link recovery, whether it is necessary to introduce additional limitation of 5second, i.e. “Performance degradation may occur when total evaluation period is longer than 5 secs in HST”

- Agreements in 1st round:

  - For Scell link recovery, no need to introduce the cap in evaluation period

Sub-topic 3-3: CSSF

Issue 3-3-1: for CSSF, whether it is necessary to introduce additional limitation of 5second, i.e. “Performance degradation may occur when total evaluation period is longer than 5 secs in HST”

- Agreements in 1st round:

  - For CSSF, no need to introduce the cap in evaluation period

Sub-topic 3-4: signalling

Issue 3-4-1: for idle state, if the answer to Issue 2-1-2 is Yes (NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf), whether NW shall indicate UE for which inter-frequency carrier, the enhanced inter-frequency measurement requirements shall apply

- Agreements in GTW

  - For IDLE mode, introduce network signalling to inform UE whether the enhanced inter-frequency HST measurement requirements shall be applied per each inter-frequency carrier. The signalling design is up to RAN2.

Issue 3-4-2: for connected state, whether NW shall indicate UE for which inter-frequency carrier, the enhanced inter-frequency measurement requirements shall be applied

- Agreements in GTW

  - For connected state, NW shall NOT indicate UE for which inter-frequency carriers, the enhanced inter-frequency measurement requirements shall be applied.

    - It is RAN4 understanding that the requirements will be defined under assumption that all carriers in the connected mode are high-speed train carriers and enhanced inter-frequency measurement requirements shall be applied

Issue 3-4-3: for HST CA, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced CA requirements

- Agreements in GTW

  - The common understanding is to inform RAN2 that Rel-16 signalling already covers CA case and inform that Rel-17 we have new feature for RRM CA. It is up RAN2 decision on how to define signalling.

Issue 3-4-4: for inter-frequency measurement, whether the network signalling for CA enhancement can be reused for inter-frequency measurement

- Q1: whether the network signalling for CA enhancement can be reused for inter-frequency measurement

  - Option 1 (Ericsson): Yes, highSpeedMeasFlag-r16 can be reused for inter-frequency measurement

  - Option 2 (MTK, CMCC, Apple, MTK, QC, HW, CATT, vivo): No, introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency

- Q2: if new signalling is agreed, whether it is per cell level or per frequency layer level (pending on Issue 3-4-2)

  - Option 1 (CMCC, QC): the signalling is a cell-level signalling

  - Option 2 (HW): the indication is per inter-frequency layer configuration

Issue 3-4-5: for CA, whether to introduce UE capability to indicate whether the enhanced requirements for HST CA are supported or not

- Agreements in 1st round

  - For CA, introduce UE capability to indicate whether the enhanced requirements for HST CA are supported or not

Sub-topic 3-5: release independent

Issue 3-5-1: release independent

- FFS whether Rel-17 NR HST RRM enhancement can be release independent from Rel-15

Topic #4: work split for draft CR

|  | Responsibility |
| --- | --- |
| Enhancement on intra-frequency without MG (9.2.5.1&9.2.5.2), including activated SCells and deactivated SCells | CMCC |
| Enhancement on intra-frequency with MG (9.2.6.2 &9.2.6.3), including activated SCells | CATT |
| Enhancement on cell reselection, inter-frequency measurement (4.2.2.4) | Ericsson |
| Enhancement on inter-frequency measurement with measurement gaps (9.3.4&9.3.5) | Huawei |
| Enhancement on inter-frequency measurement without measurement gaps (9.3.9.1 &9.3.9.2) | Nokia, Nokia Shanghai Bell |
| L1-RSRP (9.5.4) | Apple |

- Reference

R4-2103678 WF on RRM for NR HST, RAN4#98-e

R4-2105793 WF on RRM for NR HST, RAN4#98-e-bis

R4-2108341 WF on RRM for NR HST, RAN4#99-e

R4-2115439 WF on RRM for FR1 HST, RAN4#100-e