3GPP TSG-RAN WG4 Meeting # 100-e	R4-2120340

Electronic Meeting, Nov. 1-12, 2021

Agenda item:	10.9.4

Source: 	Huawei, HiSilicon

Title: 		WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC

Document for:	Approval

- Introduction

This document contains the agreements and discussion in RAN4#101-e in email thread [101-e][241] NB_IOTenh4_LTE_eMTC6_RRM_NWM [1].

- Topic

Issue 1-1-2: Scenarios prioritization

Deprioritize defining CONNECTED mode neighbour cell measurement requirements when the target cell is in enhanced coverage

FFS: Deprioritize defining CONNECTED mode neighbor cell measurement requirements when the serving cell is in enhanced coverage.

Issue 1-2-1: Carriers measured in IDLE mode

FFS for following options:

○ Option 1: The UE may continue measuring on neighbour cells detected in RRC_IDLE/INACTIVE modes continuously (e.g. every 5 seconds) after transition to RRC_CONNECTED state provided the carrier of the detected cell is one of the configured carriers for RRC connection reestablishment.

○ Option 2: Carriers to perform neighbour cell measurement in connected mode is up to UE implementation, which could be the carriers detected in IDLE mode.

Issue 1-2-2: Minimum interval between two measurements/samples for inter-frequency measurement

FFS:

- Option 1: The minimum length between two measurement occasions can be set 80 ms for normal and enhanced coverage respectively.

- FFS whether to considering filtering for neighbour cell measurement in connected mode?

- If the answer to above question is Yes

  - FFS whether the filtering is applied among samples to derive one measurement or among multiple measurements?

  - FFS values of minimum interval/separation

Issue 1-2-3: Minimum length of single measurement occasions for inter-frequency measurement

FFS:

Minimum length of occasion for measurement is X ms, where X is FFS

Minimum length of occasion for cell detection is Y ms, where Y is FFS

Issue 1-3-1: Intra-frequency requirements

FFS:

When DRX is not configured:

- Tdetect_intra = 1400 ms

- Tmeasure_intra = 800 ms for NRS-based measurement

- Tmeasure_intra = 1600 ms for NSSS-based measurement

When DRX is configured:

- Tdetect_intra = 5 DRX cycles

- Tmeasure_intra = 5 DRX cycles.

Issue 1-3-2: Inter-frequency Measurement requirements

FFS:

- Option 1:

      - RRM measurement requirements on a target carrier different from the serving carrier is defined as:

      - Tmeasure_inter = 5*min(5000, Ta)

      - Where Tmin ≤ Ta ≤ 5000 ms

      - Tmin=80 ms is the minimum time interval between two successive measurement occasions.

      - Assuming DL subframes containing NRS signals of the measured cell is available within (5, Ta) at the UE assuming the measured cell is an identified cell.

- Option 2:

- Tmeasure_inter =   Ceil(Tmeasure_basic/Ninter_NB_occasion)* Tmeasure_NB_interval

- Where Tmeasure_basic is 800ms for NRS-based measurement and 1600ms for NSSS-based measurement, Ninter_NB_occasion is the minimum length of available occasions during measurement period, and Tinter_NB_interval is the maximum interval between two available occasions during measurement period.

Issue 1-3-3: Inter-frequency Detection requirements

FFS:

- option 1:

  - Cell detection requirements on a target carrier different from the serving carrier is defined as:

  - Tdetect_inter = Nd*min(5000, Ta)

  - Where Tmin ≤ Ta ≤ 5000 ms and Nd = number of detection attempts needed for successful cell detection

  - Tmin=80 ms is the minimum time interval between two successive measurement occasions.

  - Assuming that at least subframes # 0, #4, #5 or #9 containing NPSS/NSSS of the measured cell is available within min(5, Ta)

- Option 2:

  - Tdetect_inter =  Ceil(Tdetect_basic/Ninter_NB_occasion)* Tinter_NB_interval

  - Where Tdetect_basic is 1400ms, Ninter_NB_occasion is the minimum length of available occasions during detection period, and Tinter_NB_interval is the maximum interval between two available occasions during detection period.

Issue 1-3-4: Detection and measurement delay when UE is required to monitor multiple carriers

FFS:

Tdetect = Tdetect _intra+Nfreq* Tdetect_inter

Tmeasure = Tmeasure _intra+Nfreq* Tmeasure_inter

Where Nfreq is number of inter-frequency layers to be measured according to the measurement capability.

3. References

[1] R4-2120383	Email discussion summary for [101-e][241] NB_IOTenh4_LTE_eMTC6_RRM_NWM