3GPP TSG-RAN WG4 Meeting # 101-e 												       R4-2120363

Electronic Meeting, 1st – 12th Nov, 2021

Agenda item:			8.11.1 and 8.11.2.2

Source:	Moderator (MediaTek inc.)

Title:	Email discussion summary for [101-e][221] NR_MG_enh_1

Document for:	Information

# Introduction

This document is the email discussion summary for [101-e][221] NR_MG_enh_1 with the following topics covered

- Topic 1:	General (AI 8.11.1)

- Topic 2: Multiple concurrent and independent MG patterns (AI 8.11.2.2)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Decide on the scope, priority, options and tentative agreement to be discussed in the 2nd round. Conclude issues with strict consensus, if any.

- 2nd round:

  - Conclude the issues identified in the 1st round.

  - Conclude the CR work partition

# Topic #1: General (AI 8.11.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117602 | MediaTek inc., Intel Cooperation, Apple | Observation 1: There is no agreement on whether RAN4 shall start the discussion of joint requirements. Observation 2: According to Plenary guidance, individual requirements needs to be stable before starting the discussions of joint requirements. Proposal 1: RAN4 to focus on individual requirements in this meeting and leave the joint requirement to later meetings or releases. Proposal 2: For MG enh WI, each Email moderator for the separated objective to take above the spec impact analysis in RP-2117602 as a starting point to agree on the CR work split for TS38.133 in the 2nd round. Proposal 3: Only the measurement delay impacts on TS 38.133 need to be extended to sections 8.17 and 8.19 of TS36.133. |
| R4-2117352 | CATT | Proposal 15: Postpone the joint discussion of pre-MG, concurrent MG and/or NCSG to next release. |
| R4-2118400 | Ericsson | Proposal 21: RAN4 will start the discussion on the joint features after having agreements on each MG enh. topic, such as from Rel-18. |
| R4-2118775 | Nokia, Nokia Shanghai Bell | Proposal 16: Initiate the discussion of how concurrent MGP and pre-configured MGP shall operate combined. |
| R4-2119350 | Huawei, Hisilicon | Proposal 11: Start the 2nd phase of the WI from RAN4 Jan. meeting (if feasible and pending on the outcome of RAN4#101-e), and the scope of the 2nd phase includes - Joint working of concurrent MGs and NCSG - Joint working of concurrent MGs and pre-MG |
| R4-2117460 | Apple | Proposal 7: RAN4 shall conclude whether hybrid operation among pre-MG, concurrent gaps and NCSG is supported in R17 in this meeting. Proposal 8: ask RAN2 to take hybrid operations into account in RRC signalling design for forward compatibility. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General isues

#### Issue 1-1-1: Whether and when to start the discussion of joint requirements among different objectives in this WI

- Background:

  - Note in WID: The joint requirements for objectives (1), (2) and (3) can be discussed in the WI stage after stabilizing individual requirements.

- Proposals

  - Option 1: Nokia, OPPO

    - Initiate the discussion [in Rel-17], e.g.,

      - (Nokia) Concurrent pre-MG and legacy MG.

      - (OPPO) Multiple concurrent pre-MGs

  - Option 2: Huawei

    - Start the 2nd phase of the WI from RAN4 Jan. meeting (if feasible and pending on the outcome of RAN4#101-e), and the scope of the 2nd phase includes

      - Joint working of concurrent MGs and NCSG

      - Joint working of concurrent MGs and pre-MG

  - Option 3: MTK, Intel, Apple

    - Focus on individual requirements in this meeting

  - Option 4: CATT, Ericsson

    - Postpone to next release

  - Option 5: Apple

    - Make decision in this meeting

- Recommended WF

  - Company inputs are welcomed.

#### Issue 1-1-2: Whether to ask RAN2 to allow simultaneous configurations of pre-configured gap, concurrent gap and NCSG in RRC signalling design for forward compatibility

- Proposals

  - Option 1: Apple

    - RAN4 shall ask RAN2 to take hybrid operations (among Pre-MG, NCSG and concurrent gaps) into account in RRC signalling design for forward compatibility.

- Recommended WF

  - Company inputs are welcomed.

#### Issue 1-1-3: Spec impact

- Proposals

  - Option 1: MTK, Intel, Apple

    - TS38.133: Take the spec impact analysis in RP-2117602 as a starting point to agree on the CR work split for TS38.133 in the 2nd round

    - TS36.133: Only the measurement delay impacts on TS 38.133 need to be extended to sections 8.17 and 8.19 of TS36.133.

- Recommended WF

  - Company inputs are welcomed.

  - Try to agree the principle and start the CR work split discussion in the 2nd round

## Companies views’ collection for 1st round

### Open issues

Example 2

Issue 1-1-1: Whether and when to start the discussion of joint requirements among different objectives in this WI

| Company | Comments |
| --- | --- |
| vivo | Ok with option 3 or 4 |
| Huawei | First we support option 3, i.e. focus on individual requirements in this meeting. For the joint working of individual objectives, we support option 2, i.e. to define some combinations of high interests among companies in Rel-17, e.g. joint working of concurrent MGs and NCSG. Of course, this is subject to the status of the discussion on each individual objective, and if they are not stable enough at the end of this meeting, we are also fine to postpone the joint working to next release as in option 4. |
| Ericsson | Option 4 and 3. We suggest to focus on individual requirements firstly. RAN4 can further consider the joint requirements in the later release. If the progress for each individual requirements is very good(for example, finishing the main issues of individual requirements in this meeting), RAN4 can discuss whether to further consider limited combination of requirements and identify the most important feature combinations in Rel-17. |
| MTK | Support Option 3 (or 4) The most important issue in this meeting is to finish the individual requirements which can be used as the foundation for more complicated joint requirements. Leave the joint requirements to Rel-18 is also fine to us. |
| Xiaomi | Option 3, prefer to start the joint discussion after the requirements for concurrent MGs are stable, and RAN4 should focus on individual requirements in this meeting. |
| ZTE | Support Option 4 and 3. After individual requirements determined, then RAN4 can consider the joint requirements in WI. |
| OPPO | Option 3 is fine. We can focus on individual requirements in this meeting. Whether to shift to Rel-18 should be FFS. |
| LG Electronics | Support Option 3 & Option 4. |
| Intel | Support Option 3. And how we proceed the next step on the joint works among these objects can be decided in 2nd round at least. |
| CATT | Support option 4 and option 3. Suggest to focus on individual requirements firstly. |
| Nokia | Due to WI timeline, it seems that RAN4 either need to make some agreements or there is a need to down scope the WI in general. Not only regarding joint operations. If RAN4 agree to down scope the work for this WI this should be discussed as an overall down scoping discussion including down selecting other options listed open in RAN4#101. We are fine to take an overall discussion on down scoping the WI. We do not see a problem discussing joint requirements in this meeting. Hence, we support option 1 and option 2 to work on joint requirements for MG enhancements in Rel-17. In our view, the discussion on individual objectives has considerably progressed and we have achieved to stabilize them including sending LSs to RAN2 on Pre-MG and Concurrent MGs. This work clearly needs to be continued by starting the phase of normative work at the present meeting with CR work split. However, this does not preclude to start the investigation on joint requirements in the 2nd WI phase. We agree with Huawei that the scenarios - joint operation of Concurrent MGs with Pre-MG - joint operation of Concurrent MGs with NCSG are the major important use cases to work on in the 2nd WI phase. In our view, this work can be done over remaining meetings by addressing the specification impact of the joint operation for a subset of collision scenarios (for instance FNO and selected partial overlap scenarios). |

Issue 1-1-2: Whether to ask RAN2 to allow simultaneous configurations of pre-configured gap, concurrent gap and NCSG in RRC signalling design for forward compatibility

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 |
| Huawei | Option 1 is fine. |
| Ericsson | Option 1 is fine. |
| MTK | Option 1 is fine. |
| Xiaomi | Option 1 is fine |
| ZTE | Fine with Option 1. |
| OPPO | Option 1 is fine. |
| LG Electronics | Fine with Option 1. |
| Intel | Option1 is fine. But if the signaling is based on MAC-CE based for RAN2, the backward compatibility could be difficult |
| CATT | Fine with option 1. |
| Nokia | Firstly, we support this, but we need to understand what ‘hybrid’ operation means such it can be clearly explained to RAN2. Secondly, it depends on Issue 1-1-1. But in essence RAN4 can work on the gap enhancements and inform RAN2 about the progress once agreements are reached. This should be independent of related MG enhancements agreed for Rel-18. |

Issue 1-1-3: Spec impact

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 |
| Huawei | We are fine with the spec impact analysis for 38.133. For 36.133 impact, we suggest FFS as it may depend on the outcome of the discussions on the support of EN-DC and NE-DC of individual objectives. |
| Ericsson | Option 1 is fine. We also support Huawei’s observation to FFS 36.133. |
| MTK | We are fine with Huawei’s suggestion. |
| ZTE | Support Option 1, and agree with Huawei’s suggestion. |
| OPPO | Option 1 is ok. |
| LG Electronics | RP-2117602 seems typo of R4-2117602. For 38.133, OK with option 1. For 36.133, we have same view with Huawei. |
| Intel | Option 1. |
| CATT | Fine with option 1. |
| Nokia | The specification impact and CR split will depend on the further agreements and possible down scoping discussing in Issue 1-1-1. We support agreeing of CR work split in this meeting to ensure we have final CRs ready in time. Further we have specific comments to the spec impact analysis in R4-2117602: We do not agree to put MAC CE indication / activation as FFS, since there was recent RAN1 agreement that MAC CE will be used for MG request and MG activation for positioning in Rel-17. We support using MAC CE based MG activation / deactivation also for RRM measurements such as SSB and CSI-RS L3 inter-RAT measurements, since it is network controlled and faster than RRC based activation/deactivation. Thus, it should be considered for Rel-17. Measurement delay requirements will also be needed for Pre-MG. Given that MG activation/deactivation signalling may be based on RRC or MAC CE, we observe an impact to the measurement delay, which needs further consideration. Regarding the measurement delay impact, we agree, impacts need to be extended to TS 36.133 for EN-DC and NE-DC cases, in that measurement delay requirements in terms of RRC processing delay and measurement reporting delay need to be captured. In case of NE-DC the requirements apply due to enhanced MG operation in NR PCell, for EN-DC the requirements apply due to enhanced MG operation in NR PSCell. |

### CRs/TPs comments collection

No CR/TP submitted in this agenda

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1 | Whether and when to start the discussion of joint requirements among different objectives in this WI Status: - Option 1, Nokia - Option 2: HW, Nokia - Option 3: vivo, HW, E///, MTK, Xiaomi, ZTE, OPPO, LGE, Intel, CATT, Apple - Option 4: vivo, HW, E///, MTK, ZTE, LGE, CATT - Option 5, Apple Recommendations for 2nd round: - Moderator suggests to go with the majority view: - Focus on the individual requirements in this meeting. |
| Issue 1-1-2 | Whether to ask RAN2 to allow simultaneous configurations of pre-configured gap, concurrent gap and NCSG in RRC signalling design for forward compatibility Status: - Option 1 is supported by all companies - Intel pointed out the potential issue for MAC-CE based pre-MG - Nokia suggested to clearly explain to RAN2 about the definition of ‘hybrid’. Recommendations for 2nd round: - Moderator suggests to agree on Option 1. The followings are to be captured in the draft WF. - Ask RAN2 to take hybrid operations (among Pre-MG, NCSG and concurrent gaps) into account in RRC signalling design for forward compatibility. - Detail wording to be discussed in the LS draft. |
| Issue 1-1-3 | Spec impact Status: - 9 companies are fine with the suggestion for TS38.133 in Option 1 (R4-2117602) - 5 companies suggests to FFS the impact on TS 36.133 - Nokia suggested to consider MAC-CE based indication/activation also for other non-positioning measurements as well as the impact on measurement delay due to pre-MG, Recommendations for 2nd round: - Moderator suggests to take the analysis of spec impact on TS38.133 in R4-2117602 as a starting point and FFS for TS36.133. Details and any further adjustments are subjected to the technical discussions and will be left to the individual email thread discussion. |

### CRs/TPs

No CR/TP submitted in this agenda

## Discussion on 2nd round (if applicable)

Moderator: The discussions are copied from the WF discussion

## Issue 1-1-1: Whether and when to start the discussion of joint requirements among different objectives in this WI - Agreement - RAN4 focuses on the individual requirements in this meeting ## Issue 1-1-2: Whether to ask RAN2 to allow simultaneous configurations of pre-configured gap, concurrent gap and NCSG in RRC signalling design for forward compatibility - Agreement - Ask RAN2 to take joint operations (among Pre-MG, NCSG and concurrent gaps) into account in RRC signalling design for forward compatibility. - Detail wording to be discussed in the LS draft. ## Issue 1-1-3: Spec impact - Agreement - Take the analysis of spec impact on TS38.133 in R4-2117602 as a starting point and FFS for TS36.133. - Details and any further adjustments are subject to further technical discussions and will be left to the individual email thread discussion

# Topic #2: Multiple concurrent and independent MG patterns (AI 8.11.2.2)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117352 | CATT | Proposal 1: Concurrent gaps are allowed in the case when only non-NR RAT measurement objectives are configured. Proposal 2: It is feasible that PRS measurement for positioning is associated with only one of the instance of multiple gaps. Proposal 3: When UE supports per-FR gap, allow simultaneous configuring per-UE gap and per-FR gap. Proposal 4: The max number of supported concurrent gaps across all FRs for per-FR gap capable UE is 4. Proposal 5: The following combinations of gap configuration for per-FR gap capable UE should be supported: Index; # of simultaneous MG; RAN4 conclusion. Per-FR1; Per-FR2; Per-UE. 3; 1; 0; 1; Supported. 4; 0; 1; 1; Supported. 5; 1; 1; 1; Supported. 6; 2; 2; 0; Supported. Proposal 6: For the gap collision handling, the priority rule and gap sharing factors can be defined jointly i.e. UE follow the defined priority rule of gap patterns to perform measurements and when the priority of two gap patterns are the same, UE will use the defined gap sharing factor to perform measurement in each gap occasion. Proposal 7: For defining the priority rule, there can be the following approaches: - Option 1: Pre-define the priority rule. - Option 2: NW indicates the priority rule to UE based on UE capability. Proposal 8: For gap sharing factor, there can be the following approaches: - Option A: Define a fixed sharing factor between 2 gaps with same priority - Option B: NW indicates sharing factor based on scenario Proposal 9: Define the proximity conditions to differentiate overlapping and fully non-overlapping case i.e. when the minimum time distance between two gaps is smaller than [X]ms, the two gaps can be regarded as fully non-overlapping. In which the time distance between two gaps is defined from the end of the first gap to the start of the second gap. Proposal 10: The data can be scheduled on the non-overlapped part in the dropped gap occasion. Proposal 11: Not to define overhead cap for concurrent gap. Proposal 12: No need to additionally define that each reference signal can only be measured in one MG pattern. Proposal 13: CSSF should be calculated separately for each gap and only the frequency layers sharing this gap should be counted in. Proposal 14: Do not introduce a transition period for gap configuration/deconfiguration. Proposal 15: Postpone the joint discussion of pre-MG, concurrent MG and/or NCSG to next release. |
| R4-2117459 | Apple | Proposal 1: not allow concurrent gap in the case when only non-NR RAT measurement objectives are configured. Proposal 2: Simultaneous configuring per-UE gap and per-FR gap is only allowed when the per-UE gap is associated to PRS measurement. Proposal 3: without considering NTN and MUSIM, up to 3 concurrent gap patterns across all FRs for per-FR gap capable UEs is supported in this release. Proposal 4: additional concurrent gap patterns can be considered in NTN or MUSIM operation. Proposal 5: for gap collision handling on UE’s measurement behavior, define a sharing factor between 2 gaps, e.g., given X% gap sharing, the measurement w.r.t. one gap will share roughly X% of the time, while the other gap shares the remaining. Proposal 6: the proximity conditions to apply gap collision handling, e.g., a time domain minimal distance [2]ms between the two gap instances. Proposal 7: the same gap collision handling can be applied to all of the FO/FPO/PFO/PPO/FNO cases. Proposal 8: more justification is needed if RAN4 decides to introduce gap cancelling due to NW scheduling. A simple solution is to avoid such concurrent MG configuration. Proposal 9: it is necessary to introduce an overhead cap for concurrent gaps. RAN4 can introduce a UE capability indicating the supported maximum overhead. Proposal 10: CSSF is separately calculated for each MG. If RAN4 decides to support overlapped scenario, then additional delay due to gap dropping can be reflected in the measurement latency requirements in section 9. Proposal 11: Do not introduce a transition period if it’s agreed the RRC processing time is sufficient for gap configuration/deconfiguration. Proposal 12: existing L1 measurement requirements need to be revisited. Take FR1 for example: - For FO and FPO, existing P can apply. - For FNO: P=11-TSSBMGRP => P=11-TSSBMGRP1-TSSBMGRP2 - For PFO and PPO: P=11-TSSBMGRP => P=11-TSSBmin⁡(MGRP1, MGRP2 ) Proposal 13: RAN4 to study if L1 measurement degradation due to multiple concurrent gaps needs to be addressed in this WI. |
| R4-2117495 | Qualcomm Incorporated | Proposal 1: For a UE capable of supporting multiple concurrent MG, the network can configure one of the MG exclusively for NR positioning measurements, i.e. the MG would not be shared with any other types of measurements. Proposal 2: No need to support configuring multiple concurrent MG for scenarios where only non-NR RAT measurements are configured by the network. Proposal 3: For UEs capable of per-FR gaps and multiple concurrent gaps, support simultaneous configuration of per-UE gap and per-FR gap when the per-UE gap is used for positioning measurements. Proposal 4: Support a maximum of 3 concurrent MG. Specifically, support concurrent MG combinations in rows 0-4 in the table below. Index; # of simultaneous MG; RAN4 conclusion. Per-FR1; Per-FR2; Per-UE. 0; 2; 1; 0; Supported. 1; 1; 2; 0; Supported. 2; 0; 0; 2; Supported. 3; 1; 0; 1; FFS. 4; 0; 1; 1; FFS. 5; 1; 1; 1; FFS. 6; 2; 2; 0; FFS. 7; 0; 0; 1; Supported. 8; 1; 1; 0; Supported. 9; 1; 0; 0; Supported. 10; 0; 1; 0; Supported. Proposal 5: If it is agreed to support overlapping concurrent measurement gaps, any rules to resolve collisions between overlapping measurement gaps would only apply between - two per-FR1 gaps, - two per-FR2 gaps, - one per-UE gap and one per-FR (FR1 or FR2) gap. Proposal 6: Do not support FO or FPO concurrent MG. Proposal 7: Support the following rules for handling gap collisions for PFO and/or PPO configurations (if agreed): - For each colliding instance, select the gap with the longest MGRP. - Take into account the proximity condition between gap instances when declaring a collision. - In each case the UE performs measurements only within the selected/prioritized gap and data traffic is expected outside the selected/prioritized gap. Proposal 8: X = 5 ms in the MG proximity condition. Proposal 9: The rules for handling gap collisions would apply to FO, FPO, PFO and PPO, if any of them are agreed to be supported. No need to discuss each case separately. Observation 1: Calculating MG overhead would need to account for any rules for resolving gap collisions and per-FR vs. per-UE gaps. Proposal 10: Do not introduce a hard limit on MG overhead. It would be up to the network to control MG overhead by choosing efficient MG configurations. Proposal 11: No need to impose the limitation that each reference signal can only be measured in one MG pattern. Proposal 12: At least for non-overlapping concurrent MGs, CSSF within gap is calculated separately for each MG. Only the frequency layers/MOs associated with each MG would be counted when calculating CSSF within gap. Proposal 13: FFS how to calculate CSSF for overlapping concurrent MGs, if such MG are agreed to be supported. Proposal 14: Do not introduce a transition period if it’s agreed that the RRC processing time is sufficient for gap configuration/deconfiguration. Proposal 15: The network should configure concurrent MG so that RLM is not adversely impacted. |
| R4-2117604 | MediaTek inc. | Proposal 1: No limitation to concurrent gap in the case when only non-NR RAT measurement objectives are configured. Proposal 2: Leave the support of 2G/3G in concurrent gap work as low priority in Rel-17. RAN4 to finalize 4G/5G measurement requirements first. Proposal 3: It is left to network implementation on whether to associate PRS measurement exclusively to a measurement gap. Proposal 4: For per-FR gap capable UE, simultaneous configuring per-UE gap and per-FR gap is only allowed when the per-UE gap is associated to PRS measurement. Proposal 5: Without considering MU-SIM, the max # of gap to be supported across all FRs for per-FR gap capable UEs is up to UE’s capability. The value can be reported between 3 and 4. FFS how to jointly consider MU-SIM. Proposal 6: Two measurement gaps are considered as colliding only if they are physically collide in time, i.e., X<0ms. Proposal 7: Adopt the priority rule when gaps collide. Network can signal the priority level together with the gap configuration. Proposal 8: The baseline UE supports the overhead cap no larger than the max overhead that it can support in Rel-15/16. An advanced UE capability can be added for the UE which does not need this overhead cap. Proposal 9: Do not introduce any limitation on the RS to be associated to a measurement gap. Proposal 10: CSSF is separately calculated for each MG, e.g., for a particular gap, only the dedicated frequency layers /use cases share this gap should be counted in. Proposal 11: For intra-frequency measurement delay, RAN4 to further study the following 3 cases for concurrent gaps: • All SMTC occasions are non-overlapped with any of the 2 MGs, • All SMTC occasions are fully-overlapped with one of the 2 MGs (including both MGs), • Some SMTC occasions are non-overlapped with MGs and some are not Proposal 12: For inter-frequency measurement delay, add a clear indication in the requirement on which MGRP/CSSF value should be adopted. FFS on how to handle the gap colliding case. Proposal 13: Do not introduce a transition period for gap configuration/de-configuration. Existing RRC processing time is sufficient. Proposal 14: For impact to L1 measurements, RAN4 to follow the same high-level principle in Rel-15. Priority will be given to measurement gap, followed by L3 measurement and then L1 measurement. A sharing factor between L3 and L1 measurement can be introduced if there is no remaining occasions for pure L1 measurements. |
| R4-2117696 | CMCC | Proposal 1: it is not necessary to limit that only PRS is measured for one of the measurement gaps. The MG used for PRS measurement may or may not be used for other measurement, which is up to network implementation. Proposal 2: For the per-FR gap capable UE, it is proposed to allow the simultaneous configuring per-UE gap and per-FR gap. Proposal 3: for the possible combinations for per-FR gap capable UE, it is proposed as following: Index; # of simultaneous MG; RAN4 conclusion. Per-FR1; Per-FR2; Per-UE. 0; 2; 1; 0; Supported. 1; 1; 2; 0; Supported. 2; 0; 0; 2; Supported. 3; 1; 0; 1; Supported. 4; 0; 1; 1; Supported. 5; 1; 1; 1; Supported. 6; 2; 2; 0; Supported. 7; 0; 0; 1; Supported. 8; 1; 1; 0; Supported. 9; 1; 0; 0; Supported. 10; 0; 1; 0; Supported. Proposal 4: the max number of concurrent gap across all FRs for per-FR gap capable UEs is proposed to be 4. Proposal 5: it is proposed to consider partially and fully-overlapped concurrent gaps, which could reduce the impact on the data loss. Proposal 6: it is not necessary to define an overhead cap for concurrent gaps, which can be left to network implementation. Proposal 7: it is not necessary to define transition period for gaps configuration/ reconfiguration |
| R4-2117796 | vivo | Proposal 1: Allow concurrent gap in the case when only non-NR RAT measurement objectives are configured for LTE. Do not consider 2G/3G in concurrent MG work. Proposal 2: Use option 2 or 2a for whether allow simultaneous configuring per-UE gap and per-FR gap for per-FR gap capable UEs issue. Proposal 3: For the scenario where only per-FR is configured, the max number of gaps across all FRs could be 4, assuming the maximum number of gaps per FR is 2. The maximum number of gaps when per-UE gap and per-FR gap are configured simultaneous could be 4 whereas the maximum number of gaps is 2 for per FR and per UE configuration, respectively. Proposal 4: For all identified overlapping scenarios, in order to define performance requirement, rules based on either priority or sharing principles should be investigated, i.e., fine with option 1/2/3. Proposal 5: For the FO case, support option 1, i.e., defining requirements, at least for the right hand side scenario in the figure. Proposal 6: Prefer not to define gap cancelling rule for FNO Proposal 7: Investigate how to define a suitable MGRP when multiple measurement gaps are configured for related measurement performance requirements such as RLM. Proposal 8: suggest to define an overhead cap for concurrent gap. Principles on how to define the overhead cap can be FFS |
| R4-2117824 | Xiaomi | Proposal 1: UE is allowed to be configured with concurrent MG to perform only non-NR RAT measurements. Proposal 2: RAN4 clarifies that the gap pattern #24 and #25 can be configured as per-UE gap or –FR gap for per-UE capable UE or per-FR capable UE respectively. Proposal 3: For per-FR gap capable UE, it is not allowed to be configured simultaneously per-UE gap and per-FR gap. Proposal 4: For per-FR capable UE, the maximum number of the concurrent measurement gap across all FRs is 3. Proposal 5: The combinations of simultaneous gaps for per-UE capable UE and per-FR gap capable UE are supported in Table 1. Table 1: The supported combinations of simultaneous gaps Index; # of simultaneous MG. Per-FR1; Per-FR2; Per-UE. 0; 0; 0; 1. 1; 1; 0; 0. 2; 0; 1; 0. 3; 1; 1; 0. 4; 0; 0; 2. 5; 2; 1; 0. 6; 1; 2; 0. Proposal 6: RAN4 is deprioritized to define requirements for fully-overlapped (FO) and fully-partial overlapped (FPO) concurrent gaps. Proposal 7: RAN4 is to define the requirements for partial fully-overlapped (PFO) or partial partial-overlapped (PPO) concurrent gaps. Proposal 8: Either the priority rules or gap sharing rules is adopted for partial fully-overlapped (PFO) or partial partial-overlapped (PPO) concurrent gaps. Proposal 9: The CSSF with gap should be defined based on the carriers to be measured with the same measurement gap pattern. Proposal 10: One reference signal can only be measured in one MG pattern. |
| R4-2117835 | LG Electronics | Proposal 1: Do not define simultaneous configurations of per-UE gap and per-FR gap for UE supporting per-FR gap in Rel-17. Proposal 2: Decide whether to allow both 2 MGs for FR1 and 2 MGs for FR2 simultaneously after investigating both benefits and UE complexity. Proposal 3: Consider priority when measuring only in one MG on occasions where the two MGs are overlapped. Proposal 3-1: Consider gap sharing if each priority for two MGs is same. Proposal 4: Configure priority considering that any overlapped MG is not precluded during all overlapped duration. Proposal 5: UE is not required to conduct reception/transmission of data during MGL of MG with high priority if priority between MGs is different in case of overlapped MGs. Proposal 6: UE is not required to conduct reception/transmission of data during entire MGLs of multiple MGs if priority between MGs is same in case of overlapped MGs. Proposal 7: UE is required to conduct reception/transmission of data outside of MGL of MG with higher priority in case of overlapped MGs. Proposal 8: Consider overhead cap with m=1NMGLmMGRPmMGLrMGRPr when configuring multiple MG patterns. - m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K) - N : number of multiple MG patterns - MGLr : MGL of referenced MG - MGRPr : MGRP of referenced MG - K is FFS |
| R4-2118014 | Intel Corporation | Observation 1: The concurrent MGs can be any of - all per-UE, - all per-FR (for the same FR), or - a combination of per-UE and per-FR MG patterns, with at least one per-UE and at least one per-FR Proposal 1: The maximum number of supported concurrent gaps across all FRs can be 3. Proposal 2: An overhead cap for the concurrent MG shall be defined. Observation 2: How to define the limitation of the total concurrent gap patterns activated can be FFS, e.g. - The static number (e.g., a cap as the applicability condition) - The adaptive limitation based on the gap instances within the concurrent gap pattern Proposal 3: The concurrent MGs shall be NOT allowed in the case when only non-NR RAT measurement objectives are configured. Proposal 4: PRS measurement for positioning is exclusively associated with only one of instances of multiple gaps at least for Rel17 Observation 3: As the network can know the priority of the concurrent gaps which may be overlapping before granting them, the serving gNB can enable/disable either of them. Proposal 5: Defining the requirements on the minimum proximity among the multiple concurrent gap instances is the more feasible way to avoid the overlapping issue for the multiple concurrent gaps. Observation 4: UE processing capability shall be taken count into the proximity of two adjacent gap instances in a concurrent measurement gap configuration. Observation 5: There are several alternatives to resolve such problem: - To define the new candidates of k1, k2 when concurrent MG applied - Or limited to the concurrent MGs for some use cases with higher tolerance of latency. Proposal 6: The measurement delay requirement in case of multiple gaps shall be revisited. The non-overlapping scenarios can be studied as a start point. Observation 6: When non-overlapping concurrent measurement gap patterns, the measurement requirements for SSB/CSI-RS/PRS in Rel15/Rel16 without the gap sharing can be applicable for them independently. |
| R4-2118365 | OPPO | Proposal 1: Concurrent MGs are not allowed when the UE is configured to perform only non-NR RAT measurement. Proposal 2: When the association is not provided, RAN4 can consider to define default UE behavior of measurement, e.g., fallback to legacy gaps. Proposal 3: For Per-FR gap capable UE, it is allowed to be configured with only per-FR or per-UE concurrent gaps, but not allowed for per-UE gap and per-FR gap to be configured simultaneously. Proposal 4: Define max number of concurrent gaps across all FRs for per-FR gap capable UEs as 4. Proposal 5: Consider priority when measuring only in one MG in occasions where the two MGs are overlapped. Consider gap sharing if each priority for two MGs is same. Proposal 6: open to discuss overhead issues after conclusion of overlapping issues. |
| R4-2118400 | Ericsson | Observation 1: When introducing concurrent gaps, UE may not to receive the DL or/and transmit the UL during a long period which may be intolerable by some low latency service, such as URLLC. Observation 2: UE may not transmit the HARQ feedback due to the length of aggregated gaps larger than K1. Observation 3: Compared with priority rule and sharing rule, the advantages of gap cancel rule are as follows. • Both NW and UE will have the clear understanding in each gap collision occasion if gap cancel rule will be applied. • Gap cancel rule is a general type of sharing rule and priority rule and can transform to sharing rule and priority rule easily. • Compared with sharing rule which have uncertain UE behaviours on gaps, gap cancel rule can maximize on resource utilization and efficient data scheduling. • Compared with priority rule which always prioritize one gap, gap cancel rule can bring more measurement flexibility and maximize gap utilization. Proposal 1: It’s up to NW to decide whether to configure only non-NR RAT measurement objectives. Proposal 2: It’s up to NW to decide whether to configure an exclusive gap for positioning. Proposal 3: UE can be configured with per-UE gap and per-FR gap when UE is capable of per-FR gap and concurrent gaps. Proposal 4: When UE supports both per-FR gap and concurrent gaps, except the legacy gap combination, the combination of the per-UE gap and/or per-FR gap to be configured can be as follow. Index; # of simultaneous MG; RAN4 conclusion. Per-FR1; Per-FR2; Per-UE. 0; 2; 1; 0; Supported. 1; 1; 2; 0; Supported. 2; 0; 0; 2; Supported. 3; 1; 0; 1; Supported. 4; 0; 1; 1; Supported. 5; 1; 1; 1; Supported. 6; 2; 2; 0; Supported. 7; 0; 0; 1; Supported. 8; 1; 1; 0; Supported. 9; 1; 0; 0; Supported. 10; 0; 1; 0; Supported. Proposal 5: The max number of supported concurrent gap can be 4 when UE supports both per-FR gap and concurrent gaps. Proposal 6: The pre-condition to apply the overlapping rule is to compare the time difference between ending point of one gap and the starting point of the other gap with a threshold equaling 4ms in FR1 and 1ms in FR2. Proposal 7: The example of signalling table for three collision rules is shown as follow. Signalling indication; Signalling Transformation among Three Rules. Cancel rule; Sharing rule(Y%); Priority rule. #0; 0000; 0%; Gap #1. #1; 0001; 25%; NA. #2; 0010; 25%; NA. #3; 0011; 50%; NA. #4; 0100; 25%; NA. #5; 0101; 50%; NA. #6; 0110; 50%; NA. #7; 0111; 75%; NA. #8; 1000; 25%; NA. #9; 1001; 50%; NA. #10; 1010; 50%; NA. #11; 1011; 75%; NA. #12; 1100; 50%; NA. #13; 1101; 75%; NA. #14; 1110; 75%; NA. #15; 1111; 100%; Gap #0. Proposal 8: Introduce the gap cancel rule to indicate which gap will be disabled when overlapping happens. The detail signalling is up to RAN2. Proposal 9: The gap cancel rule can be extended to further support new type of gaps by indicating the gaps based on the usage, such as MU-SIM gaps. Proposal 10: RAN4 can further study the new gaps, such as MU-SIM gaps after agreeing current concurrent gaps overlapping rule. Proposal 11: Data scheduling is expected on the dropping gap occasions after introducing the gap cancel rule. Proposal 12: RAN4 to skip the FO/FPO/PFO/PPO/FNO discussion and applies the gap cancel rule to all of these scenarios. Proposal 13: RAN4 not to define an overhead cap. Proposal 14: Only the dedicated frequency layers or the frequency layers related to the configured use cases will be counted in the CSSF calculation for each gap. Proposal 15: Additional scaling factor Kgap shall be introduced on top of CSSF when gap collision happens. Proposal 16: Do not introduce a transition period due to current RRC processing time is sufficient for concurrent gaps reconfiguration. Proposal 17: The UE will continue and complete the ongoing measurement on MO1 using MGP1 and meet the corresponding measurement requirement based on MGP1 during this measurement period even if the MO1 is reconfigured to be measured using MGP2. Proposal 18: UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or if gap length is not enough. Proposal 19: After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions. Proposal 20: When overlapping rule won’t applied, extend the P scaling factor by replacing T_SSB/MGRP with ∑_(i=1)^N▒T_SSB/MGRP_i for L1-RSRP measurements. Where, N is the number of MGPs which are non-overlapped with serving cell’s SMTC. Proposal 21: RAN4 will start the discussion on the joint features after having agreements on each MG enh. topic, such as from Rel-18. |
| R4-2118413 | Ericsson | Draft CR on skeleton change |
| R4-2118775 | Nokia, Nokia Shanghai Bell | Proposal 1: Allow concurrent gap also when only non-NR RAT measurement objectives are configured. Proposal 2: No need to keep ‘exclusively’ in ‘PRS measurement for positioning is [exclusively] associated with only one of the instances of multiple gaps at least for R17’ Proposal 3: Support Index 6, 2 MGs per FR when UE support per-FR and concurrent MGs and in total of 4 MGPs. Proposal 4: Support simultaneous configuring of per-UE gap and per-FR gap (for per-FR gap capable UE). Proposal 5: Simultaneous MG combinations Index 3 – 5 are supported. Proposal 6: Support the 2 additional combinations (Index 11 and Index 12). Proposal 7: There are no limitations on which MGPs that can be configured as concurrent MGPs, provided that the MGPs are MGPs supported by the UE. Proposal 8: Define requirements for fully overlapped (FO) Proposal 9: Define requirements for fully partial overlapped (FPO) Proposal 10: No need to define gap collision handling rules for FNO scenario. Proposal 11: Measurements performed based on a concurrent MG configuration shall have highest priority. Proposal 12: measurement performed based on a concurrent MG configuration use all available MG of the concurrent MGP to perform the measurements according to the concurrent MG configuration. Proposal 13: Option 2. There is no need for RAN4 to define a measurement gap overhead. Proposal 14: Any measurement gap overhead limitations need to be justified. Proposal 15: There is no need to introduce a transition period for gap configuration and deconfiguration Proposal 16: Initiate the discussion of how concurrent MGP and pre-configured MGP shall operate combined. |
| R4-2119114 | ZTE Corporation | Proposal 1: Once the association of one concurrent MG is not provided through configuration signaling by network, which means the concurrent MG is applicable for all MOs, i.e. Option 1 is preferred. Proposal 2: For the case that only non-RA RAT MO are configured, no need to further discuss. Proposal 3: We prefer Option 2a, Simultaneous configuring per-UE gap and per-FR gap is only allowed when the per-UE gap is associated to PRS measurement. Proposal 4: The motivation of introducing priority rule is that specific use case exists. Proposal 5: Sharing rule should be supported as baseline. Proposal 6: It is no need to define an overhead cap for concurrent gaps. NW can fully control the concurrent gaps overhead when configuring. Proposal 7: The achieved agreements for Issue 2-4 and Issue 7-1 can support Option 1 and Option 2 here. No need to further discuss. Proposal 8: Based on the latest revised WID, concurrent gap introduced by MU-SIM may be out of the WID scope. |
| R4-2119350 | Huawei, Hisilicon | Proposal 1: Remove “exclusively” in the following agreement: - PRS measurement for positioning is [exclusively] associated with only one of the instance of multiple gaps at least for R17 Proposal 2a: Support simultaneous per-UE MG and per-FR MG for per-FR MG capable UEs. Proposal 2b: Max number of concurrent MGs across all FRs for per-FR MG capable UE is 3. Proposal 2c: Combination Index 3 and 4 in the Table for Issue 3-3 in [1] are supported, and Index 5 and 6 are not supported. Proposal 3: The proximity condition for two concurrent MGs is defined as: If the distance between two MGs (Dmin) is smaller than a threshold (X) then the condition is met, otherwise the condition is not met, where X is up to UE capability and Dmin is the minimum between - D1: the end of an occasion of MG#1 to the beginning of next closest occasion of MG#2, and - D2: the end of an occasion of MG#2 to the beginning of next closest occasion of MG#1 The two occasions from the two MGs that give Dmin are considered as colliding. Proposal 4: Apply the same collision handling for all FO/FPO/PFO/PPO cases and FNO case when the proximity condition is met: - UE will only do the measurement w.r.t. the MG with higher priority on all colliding occasions, and which MG has higher priority is indicated by the NW. - The data scheduling is expected during the dropped MG occasions. Proposal 5: When concurrent MGs are configured, the MGRP for each MG cannot be smaller than 40ms. Proposal 6a: Do not support 2G/3G measurement with concurrent MGs. Proposal 6b: Support LTE measurement with concurrent MGs as a separate UE capability. Proposal 6c: LTE measurement with concurrent MGs can be supported no matter if there is no NR measurement configured. Proposal 7: The limitation that each reference signal can only be measured in one MG is not needed. Proposal 8: For defining measurement requirements with concurrent MGs, CSSF is separately calculated for each MG accounting for the frequency layers associated with the concerned MG. Proposal 9: Do not introduce transition period for configuration, reconfiguration or de-configuration of concurrent MGs. Proposal 10: Do not define effective MGRP for the L1 measurement requirements in case of concurrent MGs, but define P factor by counting the number of L1 measurement occasions not overlapped with any occasion of the concurrent MGs. Proposal 11: Start the 2nd phase of the WI from RAN4 Jan. meeting (if feasible and pending on the outcome of RAN4#101-e), and the scope of the 2nd phase includes - Joint working of concurrent MGs and NCSG - Joint working of concurrent MGs and pre-MG |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 Applicability and configurations

Note: Regarding the case when association is not provided, OPPO and ZTE have proposals. But according to the agreemetn in last meeting, it is pending on RAN2’s feedback on the feasibility. Therefore, this issue is not add here by Moderator.

#### Issue 2-1-1: Whether concurrent gaps are allowed in  the case when only E-UTRAN measurement objectives are configured.

- Note: 2G/3G will be separately discussed.

- Proposals

  - Option 1: CATT, MTK, vivo, Xiaomi, Ericsson, Nokia

    - Yes

  - Option 2: Apple, QC, Intel, OPPO

    - No

  - Option 3: ZTE

    - No need to further discuss

  - Option 4: Huawei

    - Yes, provided that UE supports LTE measurement with concurrent MGs, which is Up to UE capability

- Recommended WF

  - Company inputs are welcomed.

#### Issue 2-1-2: Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17

- Proposals

  - Option 1: [CATT], [QC], MTK, CMCC, Ericsson, Nokia, Huawei

    - No

  - Option 2: Intel

    - Yes

- Recommended WF

  - Agree on Option 1, i.e., left it to network implementation

#### Issue 2-1-3: Whether to support 2G/3G measurement with concurrent gap

- Proposals

  - Option 1: vivo, Huawei

    - No

  - Option 2: MTK

    - Leave it low priority

- Recommended WF

  - Company inputs are welcomed.

### Sub-topic 2-2: UE capability related issues

Note:

- On MU-SIM, let’s wait for the LS reply and also RAN Plenary guidance in Dec. Moderator will postpone the discussion to next meeting.

- On NTN, RAN2 is still discussion how to modify the measurement gap. Moderator will postpone the discussion to next meeting.

- Nokia’s proposal 7 was already discussed in May meeting and addressed in the page 5 of WF R4-2108346. Moderator will skip the discussion in this meeting.

- Agreement: - No separate UE capability is needed for the gap patterns supported for concurrent gap - Revisit it in the future based on the conclusion in overhead cap discussion, if needed

- Current status on supported cases for information

| Index | # of simultaneous MG |  |  | RAN4 conclusion |
| --- | --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | FFS |
| 4 | 0 | 1 | 1 | FFS |
| 5 | 1 | 1 | 1 | FFS |
| 6 | 2 | 2 | 0 | FFS |
| 7 | 0 | 0 | 1 | Supported |
| 8 | 1 | 1 | 0 | Supported |
| 9 | 1 | 0 | 0 | Supported |
| 10 | 0 | 1 | 0 | Supported |
| 11 | 2 | 0 | 0 | FFS |
| 12 | 0 | 2 | 0 | FFS |

#### Issue 2-2-1: Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs

- Proposals

  - Option 1: Xiaomi, LGE, OPPO

    - No

  - Option 2: CATT, CMCC, vivo, Ericsson, Nokia, Huawei

    - Yes

  - Option 2a: Apple, QC, MTK, vivo, ZTE

    - Simultaneous configuring per-UE gap and per-FR gap is only allowed when the per-UE gap is associated to PRS measurement

- Recommended WF

  - Can we take Option 2a as a middle ground for compromise?

  - Note: Huawei has a proposal on not to support index 5, which is pending on this discussion.

#### Issue 2-2-2: Max number of concurrent gap across all FRs for per-FR gap capable Ues (without considering MU-SIM and NTN)

- Proposals

  - Option 1: Apple, QC, Xiaomi, Intel, Huawei

    - 3

  - Option 2: CATT, CMCC, vivo, OPPO, Ericsson, Nokia

    - 4

  - Option 3: MTK

    - Up to UE capability

  - Option 4: LGE

    - Up to the study on both benefits and UE complexity

- Recommended WF

  - Any idea on compromise is welcomed.

#### Issue 2-2-3: Support of indices 11 and 12

- Proposals

  - Option 1: Nokia

    - Support the 2 additional combinations (Index 11 and Index 12)

| Index | # of simultaneous MG |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 | Per-FR2 | Per-UE |
| 11 | 2 | 0 | 0 |
| 12 | 0 | 2 | 0 |

- Recommended WF

  - Agree on Option 1.

  - As they are the fall-backs of index 0 and index 1 respectively, there should be no concern to support 11 and 12.

#### Issue 2-2-4: Clarification on gap pattern #24 and #25

- Proposals

  - Option 1: Xiaomi

    - RAN4 clarifies that the gap pattern #24 and #25 can be configured as per-UE gap or –FR gap for per-UE capable UE or per-FR capable UE respectively.

- Recommended WF

  - Company inputs are welcomed.

#### Issue 2-2-5: Clarification on configuration limitations or restrictions regarding the GPs supported by the UE

- Proposals

  - Option 1: Nokia

    - Besides the potential outcome of UE capability and overhead cap discussions, no additional configuration limitations or restrictions regarding which of the GPs supported by the UE that can be configured as concurrent gap.

- Recommended WF

  - Company inputs are welcomed.

### Sub-topic 2-3: Overlapping

Moderator will focus on the general rules for defining the requirements in this meeting. Regarding FO, FPO, PFO, PPO scenarios, let’s just collect the preference from each company in the 1st round

#### Issue 2-3-1: Proximity condition for overlapping

- FFS in last meeting: the proximity conditions to apply gap collision handling, e.g., a time domain minimal distance [X] ms between the two gap instances

- Proposals

  - Option 1: MTK

    - X < 0, i.e., physically collided

  - Option 2: Apple

    - X = 2

  - Option 3: Ericson

    - 4 in FR1 and in 1 FR2

  - Option 4: QC

    - X = 5

  - Option 5: Huawei

    - Up to UE capability

- Recommended WF

  - Company inputs are welcomed.

#### Issue 2-3-2: UE behavior during colliding gap occasion

- Proposals

  - Option 1: QC, MTK, vivo, Xiaomi, OPPO, [Nokia], [ZTE], Huawei

    - Priority rule

      - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions

      - The priority can be configurable or fixed

      - FFS whether to resume data scheduling during dropped gap occasions

  - Option 2: Apple, vivo, Xiaomi, ZTE

    - Gap-sharing rule

      - Given X% gap sharing, the measurement w.r.t. one gap will share roughly X% of the time, while the other gap shares the remaining

      - The sharing factor can be configurable or fixed

      - [Data schedule is not expected during both gaps]

  - Option 3: CATT, LGE

    - A hybrid priority and gap-sharing rule

      - Sharing is only considered between equal priority gaps

      - Data scheduling is expected during the dropped gap occasions

  - Option 4: Ericsson,

    - Gap cancellation rule

      - A pattern telling UE how to drop gaps in each colliding occasion.

      - Data scheduling is expected during the dropped gap occasions

- Recommended WF

  - Any idea to compromise is welcomed.

#### Issue 2-3-3: Company preference on introducing FO, FPO, PFO, PPO scenarios

- A table will be used to collect preference. If the general rule can be concluded in the 1st round, we can try to agree some scenarios which are less controversial.

- Recommended WF

  - Please provide your company preference

### Sub-topic 2-4: Overhead

#### Issue 2-4-1: Whether to define the overhead cap

- Proposals

  - Option 1: Apple, vivo, LGE, Intel, HW

    - Yes

  - Option 2: CATT, QC, CMCC, Ericsson, Nokia, ZTE

    - No

  - Option 3: MTK

    - Introduce a UE capability for those UE who does not need cap

  - Option 4: OPPO

    - Postpone after overlapping issues get concluded

- Recommended WF

  - Any idea to compromise is welcomed

#### Issue 2-4-2: how to define overhead cap

- Proposals

  - Option 1: MTK

    - The max overhead that UE can support in Rel-15/16.

  - Option 2: LGE

    - m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K)

      - N : number of multiple MG patterns

      - MGLr : MGL of referenced MG

      - MGRPr : MGRP

  - Option 3: Huawei

    - When concurrent MGs are configured, the MGRP for each MG cannot be smaller than 40ms

- Recommended WF

  - Company inputs are welcomed.

### Sub-topic 2-5: Measurement requirements

Sub-topic description

Open issues and candidate options before e-meeting:

#### Issue 2-5-1: Whether to additionally consider the limitation that each reference signal can only be measured in one MG pattern

- Proposals

  - Option 1: CATT, QC, MTK, Huawei

    - No

  - Option 2: Xiaomi

    - Yes

- Recommended WF

  - Can we agree on Option 1?

#### Issue 2-5-2: CSSF calculation

- Proposals

  - Option 1: CATT, Apple, QC, MTK, [Xiaomi], Ericsson, Huawei

    - CSSF should be calculated separately for each gap and only the frequency layers sharing this gap should be counted in

    - Note how to deal with overlapping concurrent MGs is up to Sub-topic 2-3.

  - Option 2: Nokia

    - Measurement performed based on a concurrent MG configuration use all available MG of the concurrent MGP to perform the measurements according to the concurrent MG configuration.

- Recommended WF

  - Can we agree on Option 1?

#### Issue 2-5-3: Measurement delay outside gap

- Note: This issue is pending on the conclusion of overlapping

- Proposals

  - Option 1:  MTK

    - RAN4 to further study the following 3 cases for concurrent gaps

      - All SMTC occasions are non-overlapped with any of the 2 MGs,

      - All SMTC occasions are fully-overlapped with one of the 2 MGs (including both MGs),

      - Some SMTC occasions are non-overlapped with MGs and some are not

- Recommended WF

  - Collect view in 1st round

#### Issue 2-5-4: Measurement delay within gap

- Proposals

  - Option 1: MTK

    - Add a clear indication in the requirement on which MGRP/CSSF value should be adopted. FFS on how to handle the gap colliding case

  - Option 2: Ericsson

    - Additional scaling factor Kgap shall be introduced on top of CSSF when gap collision happens.

- Recommended WF

  - Collect view in 1st round

### Sub-topic 2-6: Impact to other L1 measurements

#### Issue 2-6-1: How to capture the impact on L1 measurements due to concurrent gap

- Note: Including RLM, BFD, CBD, L1-RSRP, L1-SINR

- Proposals

  - Option 1: Apple

    - For FO and FPO, existing P can apply.

    - For FNO:                 P=11-TSSBMGRP       =>      P=11-TSSBMGRP1-TSSBMGRP2

    - For PFO and PPO:   P=11-TSSBMGRP       =>      P=11-TSSBmin⁡(MGRP1, MGRP2 )

  - Option 2: QC

    - The network should configure concurrent MG so that RLM is not adversely impacted

  - Option 3: MTK

    - RAN4 to follow the same high-level principle in Rel-15. Priority will be given to measurement gap, followed by L3 measurement and then L1 measurement. A sharing factor between L3 and L1 measurement can be introduced if there is no remaining occasions for pure L1 measurements

  - Option 4: Ericsson

    - When overlapping rule won’t applied, extend the P scaling factor by replacing  TSSBMGRP with i=1NTSSBMGRPi for L1-RSRP measurements. Where, N is the number of MGPs which are non-overlapped with serving cell’s SMTC.

  - Option 5: HW

    - Do not define effective MGRP for the L1 measurement requirements in case of concurrent MGs, but define P factor by counting the number of L1 measurement occasions not overlapped with any occasion of the concurrent MGs.

- Recommended WF

  - Moderator is thinking that for forward compatibility, maybe we should avoid creating more and more scenarios and using exactly equation(s) in each scenario because later we may need to work on even more complicated problem, e.g., MU-SIM and NCSG. With more and more gaps to be considered, the difficulty to identify all scenarios grows exponentially. Maybe a generic way to define the requirement is better, e.g., try to count the number of remaining available samples for L1 measurements

  - Companies are welcomed to provide on how to consider forward compatibility

### Sub-topic 2-7: Others

#### Issue 2-7-1: Whether to introduce a transition period for gap configuration/deconfiguration

- Proposals

  - Option 1: CATT, Apple, QC, MTK, CMCC, Ericsson, Nokia, Huawei

    - No

- Recommended WF

  - Agree on Option 1

#### Issue 2-7-2: UE measurement behavior after transition

- Proposals

  - Option 1: Ericsson

    - The UE will continue and complete the ongoing measurement on MO1 using MGP1 and meet the corresponding measurement requirement based on MGP1 during this measurement period even if the MO1 is reconfigured to be measured using MGP2.

    - UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or  if gap length is not enough.

    - After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions.

- Recommended WF

  - Company views are welcomed.

## Companies views’ collection for 1st round

### Open issues

Issue 2-1-1: Whether concurrent gaps are allowed in  the case when only E-UTRAN measurement objectives are configured

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 |
| Huawei | Option 4. We think concurrent MGs for LTE measurement should be a separate capability from concurrent MGs for NR measurement. For UE supporting this capability, concurrent gaps can be allowed in the case when only E-UTRAN measurement objectives are configured. |
| Ericsson | Option 1 It’s up to NW to configure one gap for non-NR RAT measurements or two gaps. If UE supports concurrent gaps, there is no technical reason to NOT support two inter-RAT measurements. |
| MTK | Support Option 1. At least from our side we do not see the problem to allow this configuration nor any UE implementation difficulty. But we are open to accommodate different UE implementations if clarified. |
| Xiaomi | Option 1, it is benefit to configure concurrent gaps for LTE measurement when UE move from NR coverage to LTE coverage. |
| ZTE | We are open for this issue, we just want to emphasize it is not an important issue, no need to spend much time here. |
| OPPO | Option 2. Concurrent MGs are not expected when the UE is configured to perform only non-NR RAT measurements, because we cannot see the necessity of such measurement. It can follow current network configuration. |
| LG Electronics | Support Option 2. For Option 1, it is not clear why/when NR UE is configured to measure only LTE with LTE MOs. And, benefit is not clear because LTE measurement is based on CRS. |
| Apple | Support option 2. We don’t see many practical use cases of such configuration. In LTE there is PSS, SSS and CRS every 5ms, which is denser than SMTC in NR. Therefore, it is easier to use one gap to cover all the LTE layers. Besides, this is more like an enhancement on LTE. If there is real demand from the field, this should have been raised in LTE design. But no one ever claimed such need in LTE design. On the other hand, allowing such configuration would result in extra standardization work, which is something doable but definitely with low priority (much lower than joint discussion with pre-MG and NCSG). |
| Intel | Option 2 since such aspect is for NR only. It doesn’t make sense enable the advanced NR UE to measurement non-NR RATs only. |
| CATT | Support option 1. The use case configuration for concurrent gap is NW implementation. There is not necessity to limit the configuration. And we don’t understand what is the issue for this configuration if UE support concurrent gaps. |
| Nokia | Option 1. We would like to understand the views on 2 aspects: 1) is it allowed to configure concurrent MGs if only E-UTRAN measurements are configured (using non concurrent MGs)? 2) is it allowed to configure E-UTRAN measurements only using concurrent measurement gaps? It would be good to get clarification on both aspects. It is not clear to us what would be technical reason why there would not be allowed to configure only E-UTRAN measurements for either of 1) or 2). However, if there are reasons why this would not be possible it can be discussed. |
| Qualcomm | We prefer option 2. Is there a need to enhance non-NR RAT measurements by configuring concurrent MGs? To address Nokia’s questions: - No, concurrent MG would not be allowed if only E-UTRAN measurements are configured. - If only E-UTRAN measurements are configured then only one MG should be configured. |

Issue 2-1-2: Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF |
| Ericsson | Support the Recommended WF |
| MTK | Support the Recommended WF |
| ZTE | Support the Recommended WF |
| OPPO | Support the Recommended WF |
| LG Electronics | Support the recommended WF |
| Apple | Fine with the recommended WF. |
| CMCC | Support the recommended WF. |
| Intel | Fine with the recommended WF. |
| CATT | Fine with the recommended WF. |
| Nokia | We conditionally support the recommended WF. However, it still needs to be agreed whether the agreement should include or exclude the word ‘exclusively’. Our view here is that it was not clear what ‘exclusively’ would mean, but if it refer to that ONLY ONE GP can be associated to PRS we support removing the word ‘exclusively’. |
| Qualcomm | Support the recommended WF. Not sure why we’re discussing this question. The important point was to agree that the network can configure one concurrent MG to be used for positioning measurements, not shared with RRM measurements. We understand this has been agreed already. |

Issue 2-1-3: Whether to support 2G/3G measurement with concurrent gap

| Company | Comments |
| --- | --- |
| vivo | Support option 1 |
| Huawei | Option 1 |
| Ericsson | Option 2. |
| MTK | Either Option 1 or 2 is fine to us. |
| Xiaomi | Option 1 |
| ZTE | Prefer Option 2. |
| OPPO | Support option 1 |
| LG Electronics | Support Option 1 |
| Apple | Option 1. |
| Intel | Option 1 |
| CATT | Support option 2. |
| Nokia | Option 3: Support 2G/3G measurements with concurrent gap We are wondering why it would not be allowed to configure 2G/3G measurements using concurrent measurement gaps. It would technically be the same as configuring 2G/3G measurement only using existing (non-concurrent) measurement gaps. Additionally, this seems against the agreement in last meeting that there is no additional capability related to concurrent GPs? Hence, neither option1 nor option 2. |
| Qualcomm | Some clarification is needed. Does option 1 mean that if he network configures any 2G/3G measurements together with NR measurements then only one MG could be configured? In our view, this issue can be addressed in the same way as issue 2-1-1. |

Issue 2-2-1: Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs

| Company | Comments |
| --- | --- |
| vivo | Ok with option 2 and 2a |
| Huawei | Option 2. On option 2a, if UE could support simultaneous per-UE gap and per-FR gap when the per-UE gap is associated to PRS measurement, we do not see clear reason why UE cannot support it when the per-UE gap is used for RRM measurement. |
| Ericsson | Option 2. We have the same view with Huawei. In addition, we don’t see any technique obstacle for UE to support per-UE gap and per-FR gap simultaneously when UE supports both per-FR gap and concurrent gaps. |
| MTK | We suggest to go for Option 2a as a compromise, given this issue has been discussed for several meetings. Option 2a seems the only use case that has been mentioned in previous meetings. If other important use cases are later identified, we are also fine to have more discussion. |
| Xiaomi | The clarification on gap pattern#24 and #25 is needed, if gap pattern #24 and #25 can be configured as per-UE gap or –FR gap for per-UE capable UE or per-FR capable UE respectively, then option 1 is supported, if gap pattern#24 and #25 are considered as per-UE gap, then option 2a is fine. |
| ZTE | Support Option 2a. |
| OPPO | Share the similar concern as Xiaomi’s. For option 2a, when the per-UE gap is associated to PRS measurement, is there any restriction on gap patterns to be configured for per-UE gap, e.g., only gap pattern#24 and #25 are allowed? |
| LG Electronics | Support Option 1. As compromise, Option 2a can be supported also. |
| Apple | Support option 2a. Some companies are asking why UE cannot support it when the per-UE gap is used for RRM measurement. Technically, we agree that this can be supported from UE perspective. However, we are looking into this issue from a different angle. We mentioned several meetings ago that there is no benefit of such configuration, compared with configuring per-FR gap separately in different FR, except for PRS measurement. For instance, configuring GP#0 as per-UE gap would result in 0.5ms loss in FR2 compared with configuring GP#0 in FR1 and GP#13 in FR2 as per-FR gaps. |
| Intel | Fine with option 2 and 2a. |
| CATT | Support option 2. Same view as Huawei. If it can be supported for PRS measurement, there is no reason to preclude the support for RRM measurement. |
| Nokia | Option 2. It is not clear to us the reasoning for the limitation why the Per-UE gap has to be limited PRS only? For example, if the network configures a Per-UE capable UE with GP#1 in FR1 and FR2 and additionally GP#0 in FR2 or FR1 it would essentially be the same as configuring 1 Per UE GP (GP#1) and one Per-FR GP (GP#0)? However, without limitation. |
| Qualcomm | We support option 2a because that is the use case that motivates supporting concurrent per-UE + per-FR gaps. To manage feature scope, we are asking for support of this essential use case. To Xiaomi and OPPO: MGP #24 and #25 are per-UE only. |

Issue 2-2-2: Max number of concurrent gap across all FRs for per-FR gap capable Ues (without considering MU-SIM and NTN)

| Company | Comments |
| --- | --- |
| vivo | Support option 2. Having the maximum number as 3 means only 1 gap can be allocated for a particular FR when 2 gaps are preferred. |
| Huawei | Option 1. We are also fine with option 3 as a compromise. |
| Ericsson | Option 2. |
| MTK | Support Option 3 Again, this issue has been discussed for many meetings. To move forward, we suggest to add a UE capability. |
| Xiaomi | Option 1 or option 3, UE capability can be considered as a compromised option. |
| ZTE | We are fine with Option 2 or 3. |
| OPPO | Option 2, maximum as 4 with 2 perFR1 gaps + 2 perFR2 gaps. It also depends on issue 2-2-1. |
| LG Electronics | Support Option 1. If UE complexity is not issue, Option 2 can be fine. |
| Apple | Option 1or 3. |
| CMCC | Option 2. In previous meetings, it was agreed to assume max 2 MGs in an FR as a starting point. And in our view, each FR shall be treated equally for the per-FR gap capable UEs. Based on this understanding, the max number of concurrent gap across all FRs is 4 without considering the simultaneous configuration of per-UE gap and per-FR gap for per-FR gap capable UEs. |
| Intel | Option 1 & 3. We are also fine with option 3 as a compromise. |
| CATT | Support option 2. It has been agreed the maximum number in each FR is 2. If having the limit number in option 1, it means even for the UE supporting concurrent gap, NW cannot configure two gaps in a certain FR. It is conflicted with the previous agreement. For option 3, we would like to understand more what is the UE implementation issue for supporting the configuration that 2 gaps are configured for each FR? We think only if some implementation problems are identified, we would go for UE capability. |
| Nokia | Option 2. We do not support introducing a UE capability for this. Our preference is to support option 2 but we can discuss option 1. However, we would like to understand the reasoning why 2 GPs would not be possible as the UE would have 2 searchers in this case (FR1 and FR2). |
| Qualcomm | We support options 1 and 3. |

Issue 2-2-3: Support of indices 11 and 12

| Company | Comments |
| --- | --- |
| vivo | Ok with the recommended WF |
| Huawei | Support the Recommended WF |
| Ericsson | Ok with the recommended WF |
| MTK | Support the Recommended WF |
| Xiaomi | Option 1 is fine. |
| ZTE | Fine with the recommended WF |
| OPPO | Fine with the recommended WF |
| LG Electronics | Support the recommended WF. |
| Apple | Option 1. |
| CMCC | Support the recommended WF. |
| Intel | Support the recommended WF |
| CATT | Support the recommended WF. |
| Nokia | Support recommended WF |
| Qualcomm | Support the recommended WF |

Issue 2-2-4: Clarification on gap pattern #24 and #25

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We do not support option 1. Based on Rel-16 MG applicability, MGP #24 and #25 are only used as per-UE gap. We suggest to not change the MG applicability in this WI. |
| Ericsson | We do not support option 1. Same view as Huawei. |
| MTK | Huawei’s comment aligns with our understanding. |
| Xiaomi | Option 1, the clarification on gap type for GP#24 and #25 is needed. |
| ZTE | Same view as Huawei, not change Rel-16 MG capability. |
| OPPO | Same comments as for issue 2-2-1. The restriction on gap pattern for per UE gap should be clarified, |
| LG Electronics | Same view with Huawei. |
| Apple | Do not support option 1. This is more like an enhancement on positioning which shall not be triggered in this WI. |
| Intel | Do not support option 1. Same view as Huawei. |
| CATT | Same view as Huawei. Also it has been agreed in R16 POS that the gap used for positioning measurement can only be per-UE gap which is applied for all gaps. Gap#24 and #25 is introduced for positioning, obviously no need to be configured as per-FR. |
| Nokia | No support of Option 1. Currently PRS GP are only supported as Per-UE GP. Whether PRS GPs (#24 and #25) need to be supported as Per-FR measurement gaps would depend on whether the UE is required to be able to support Per-FR PRS measurements or if PRS measurements are only defined for when configured with Per-UE GPs |
| Qualcomm | We do not support option 1. |

Issue 2-2-5: Clarification on configuration limitations or restrictions regarding the GPs supported by the UE

| Company | Comments |
| --- | --- |
| vivo | Fine with option 1 |
| Huawei | Fine with option 1 |
| Ericsson | Fine with option 1 |
| MTK | Fine with option 1 |
| ZTE | Fine with Option 1 |
| LG Electronics | Fine with option 1 |
| Apple | Ok with option 1. |
| CMCC | OK with option 1. |
| Intel | Option 1 |
| CATT | Fine with option 1. |
| Nokia | Option 1 This is related to Issues 2-1-1 and Issue 2-1-3. To ensure aligned discussions we prefer to clarify any limitations on applicability of the concurrent gap. Hence, are there any applicability rules related to which measurements can be configured using concurrent measurement gaps compared to when configured using non concurrent measurement gaps. In our view, for the measurements performed in a gap, it will make no difference whether the gap is a gap configured as a concurrent measurement gap or a non-concurrent measurement gap. As long as the GP is applicable for the configured measurements and GP is supported by the UE. In RAN4#99 it was agreed ‘No separate UE capability is needed for the gap patterns supported for concurrent gap’. Hence, if the UE supports GP#0-3 it will support these GPs as allowed GPs also for concurrent gaps. Current applicability table state that these gaps can be used also for non-NR RAT including E-UTRAN. If RAN4 limit the RAT that can be measured when using concurrent gaps it seems the same as different gap capability. Hence, the currently defined measurement purpose in the applicability tables in 38.133 applies also for concurrent measurement gaps. |
| Qualcomm | We understand the proponent of option 1 may be trying to avoid unnecessary discussions in RAN4. However, there are other considerations besides the ones mentioned in the proposal. E.g. applicability of existing measurement gap patterns. See issue 2-2-4. |

Issue 2-3-1: Proximity condition for overlapping

| Company | Comments |
| --- | --- |
| vivo | Support option 1 |
| Huawei | Option 5. It may be difficult to agree on a single value considering different UE implementations, so we prefer to define a UE capability. |
| Ericsson | Option 3 When we firstly raised this issue, we had already mentioned that the issue is not only a UE implementation issue, but also a NW scheduling issue. To avoid a long interruption for two gap instances close each other, one of the gap instance shall be disabled. |
| MTK | We support Option 1 to make the spec simple. To Ericsson. With GP#24 and 25, we already have interruptions as long as 10ms and 20ms, respectively. We are not sure why we need extra handling for 2 ‘almost-colliding’ gaps with total interruption far shorter than 20ms. Would like to understand more about the reason. |
| Xiaomi | Option 4, similar way for 2 consecutive SSB signal. |
| ZTE | Introducing the time domain minimum distance is necessary, we are open for the detailed value. |
| LG Electronics | Preference is Option 1 based on intuitive and simple aspect. For other value of X > 0, if X=2 and the time difference between MGs is 1, it is not overlapped physically however it can be considered with overlapped. It can be strange. |
| Apple | Option 5 can be a good compromise to move forward. |
| Intel | Option 5 based Observation 4 in our TDoc “Observation 4: UE processing capability shall be taken count into the proximity of two adjacent gap instances in a concurrent measurement gap configuration. “ |
| CATT | Prefer option 1. |
| Nokia | We can support option 1. However, if there are technical reasons why there is a need for a longer fixed minimum distance between measurement gaps and concurrent measurement gaps we are open for discussion. We suggest not introducing a UE capability as it only increases network complexity. Due to time limits we suggest agreeing on one proximity length. |
| Qualcomm | Option 4. |
| Ericsson | We have the same view as Nokia. Additional capability will increase unnecessary network complexity. At the same time, the total concurrent gap WI is an optional feature, we don’t think additional capability is needed. We suggest agreeing only one proximity length. As a compromise, we can partially support option 4 for FR1, and 5slots for FR2. From our understanding, UE processing time is much faster in FR2. 5ms will be 40 slots in FR2 which is too long. |

Issue 2-3-2: UE behavior during colliding gap occasion

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 and 2 |
| Huawei | Option 1. With priority rule it would be clear which of the colliding MG occasions is used for measurement and which is dropped, and it can enable data scheduling the dropped MG occasions. This is a clear benefit compared to option 2 and 3. On option 4, it may be overkilling to define occasion level prioritization because in most cases, the colliding case can be avoided by NW implementation, e.g. by configuring proper periodicity and offset for concurrent MGs. Also, it may be difficult for NW to determine such a priority pattern. |
| Ericsson | Option 4. We suggest companies to further check the Pros and Cons of each method. Compared with priority rule and sharing rule, gap cancel rule is a more general rule which can avoid the shortcoming for priority rule and sharing rule. Gap cancel rule can transform to sharing rule and priority rule easily. - Priority rule Priority rule will always prioritize one gap when overlapping happens. However, if measurements are always prioritized for one gap, there is no benefits for configuring concurrent gaps. For example, if two SMTCs with 160ms periodicity are partially overlapped, it’s better to support half-half measurements for both SMTCs other than always measured only one SMTC. Furthermore, if the NW just wants to always prioritize one gap, it’s also easy to indicate to the UE by setting all ‘1’ or all ‘0’. The gap cancel rule can transform to gap priority rule. - Sharing rule Sharing rule can be seen as cancel rule without clear indication which means data scheduling is impossible for both gap instances even which don’t be used. On the contrary, after clear indication, data scheduling on the unused gap duration can be expected to avoid the additional gap waste. From our understanding, if UE can support sharing rule, it can also support the cancel rule. There is no essential difference. |
| MTK | Support Option 1. Option 1 simplifies the spec. As long as the rule is clear, network knows how to configure the best gap offset and MGRPs for the measurement jobs. Option 2 (and also 3) is more complicated in terms of UE implementation. Furthermore, it is difficult to be further extended to more complicated scenarios with MU-SIM, NTN or Rel-17 Positioning. If there are total 5 gaps configured, we may have 2 or 3 or 4 or 5 gaps colliding on a particular gap occasion. We are not very clear on how gap sharing can be extended to the case with more than 2 gaps. Option 4 has the same issue as we mentioned for Option 2. Plus, it requires more complicated UE implementation. The impact on gap cancellation is not only on the data scheduling. If a gap occasion is cancelled, an intra-frequency SMTC (which was punctured by the gap) could become available again. This also means some impact to other measurements outside gap, e.g., the P factor needs to be revised. |
| Xiaomi | Option 1 or option 2, either priority rule or sharing rule is adapted for colliding cases. |
| ZTE | Support Option 2. For sharing rule, since which has been supported in Rel-15/16, so no additional standardization workload increased, and the sharing rule can overcome the overlapping issue well, so the sharing rule should be certainly a baseline. For priority rule, we are not sure whether specific use case exists. The proponent can give the detailed use case analysis. |
| OPPO | Option 1. Priority rule is preferred. For the case of same priority, gap sharing can also be considered. |
| LG Electronics | Fine with option 1 & Option 3. For Option 1, we think data scheduling can be resumed during dropped gap occasions. |
| Apple | Support option 2. Compared with option 1, option 2 can cover option 1 by setting 100% sharing for the ‘higher priority’ GP. Besides, if UE always do the measurement w.r.t. the GP with higher priority, then the possible use cases of configuring concurrent gaps would be quite limited, probably the only use is for PPO with high priority on the GP with longer MGRP: For other cases, option 1 is effectively saying no need to configure concurrent gaps since the GP with lower priority would be always skipped. For option 3, it is technically doable in case there are many concurrent GPs. However, we only support 2 GPs in each FR (if MUSIM, NTN is not considered). On the other hand, extra standard effort is needed to support option 3. But we don’t see significant gain compared with option 2. In our view, option 4 is even more complicated than option 3 by introducing configurable gap cancellation patterns, which is not preferred. |
| CMCC | Both option 1 and option 2 are OK for us. |
| Intel | Option 2 since if the NW know the priority rules it is more efficient to not assign any gap for the lower priority measurements. |
| CATT | Support option 3 or option 2 in principle. But for option 2, further clarification is needed that the gap sharing is only for the overlapped gap occasion not for the gap repetition period. The intention is to clarify the how to calculate the X% of the time when the two gap periodicities are different. |
| Nokia | We see similarities between all proposals. However, due to time limitation we believe RAN4 should focus on a solution with low complexity. We support an approach where the measurements configured using concurrent measurement gaps is prioritized to the level that those measurements are always performed whenever there is a gap collision (if RAN4 decide to discuss other than FNO scenario in Rel-17). Whether this is named priority, sharing, hybrid or cancellation rule is a matter of opinion. In a sense it is priority (100%) where the gap sharing is that sharing is 0% which leads to gap cancellation when there is collision. The rule can be applied also for when proximity is detected (2 gaps are too close). Hence, if an option is to be chosen it would be: Option 1 conditioned fixed priority with concurrent having 100% priority. RAN4 can discuss further improving the scheme in later releases. |
| Qualcomm | We support option 1 for simplicity and efficiency. It is understood here that in each collision only one MG is selected based on priority and corresponding measurements are performed during the selected MG. Data transfer can take place during the MG that was not selected. Option 2 is inefficient because data transfer cannot occur during both gaps (even though only one of the gaps is used for measurements). Option 4 would provide additional flexibility w.r.t. option 1 but at the cost of higher complexity. |
| Ericsson | Firstly, we would emphasis that regardless of which options would be chosen, data scheduling will be expected during the disabled MG. Compared with option 1, option 4 is more flexibility but higher complexity. To move forward, we can compromise to NOT define so many patterns, but at least to include 50% priority indication from network side. As we mentioned before, in some cases, 100% priority for one gap isn’t expected, such as two SMTCs = 160ms with partially overlapping. |

Issue 2-3-3: Company preference on introducing FO, FPO, PFO, PPO scenarios

| Whether to introduce corresponding requirements for the following scenarios |  |  |  |  |
| --- | --- | --- | --- | --- |
| Company | FO | FPO | PFO | PPO |
| Company A | [Yes / No / FFS] | [Yes / No / FFS] | [Yes / No / FFS] | [Yes / No / FFS] |
| vivo | Yes | Yes | Yes | Yes |
| Huawei | Yes | Yes | Yes | Yes |
| Ericsson | Yes | Yes | Yes | Yes |
| MTK | No/FFS | No/FFS | Yes | Yes |
| Xiaomi | No | No | Yes | Yes |
| ZTE | FFS | FFS | Yes | Yes |
| OPPO | Yes | Yes | Yes | Yes |
| LG Electronics | Yes | Yes | Yes | Yes |
| Apple | Yes | Yes | Yes | Yes |
| CMCC | Yes | Yes | Yes | Yes |
| Intel | NO/FFS | NO | Yes | Yes |
| CATT | Yes | Yes | Yes | Yes |
| Nokia | FFS | No | Yes | No |
| Qualcomm | No | No | FFS | FFS |

Issue 2-4-1: Whether to define the overhead cap

| Company | Comments |
| --- | --- |
| vivo | Support option 1. |
| Huawei | Option 1 |
| Ericsson | Option 2 On the one hand, network can manage this cap and tradeoff between the throughput loss and measurement gaps’ configuration. On the other hand, with the gap cancelling rules for UE, there is no significant throughput loss for UE compared with the legacy MG |
| MTK | Support Option 3. Again, this issue has been discussed for many meetings without conclusion. We suggest to add capability as a compromise to move forward. |
| ZTE | Option 2. The overhead can fully controlled by NW. Further more, with the help of collision handling, which can alleviate the throughput deterioration caused by too heavy overhead of gaps. |
| LG Electroncis | Support Option 1. |
| Apple | Support option 1. Also fine with option 3 to move forward. |
| CMCC | Option 2. It is up to network implementation. |
| Intel | Can be compromised to Option 2 to make progress |
| CATT | Support option 2. This issue is about NW configuration, we don’t understand why to define UE capability. Also we think it is not a good way to resolve every issue by introducing UE capability. There already been several issues suggested to define UE capability. |
| Nokia | Option 2 We have not seen any technical reason from UE side why there would be a need to have a cap on the overhead. We are open to discuss if such are provided. But this discussion depend on the Issue 2-3-1, Issue 2-3-2 and Issue 2-3-3. |
| Qualcomm | No strong view on this issue. |
| Ericsson | We have concern on option 3. Currently, concurrent gaps is an optional feature. If UE has concerns on the overhead, UE can just announce not to support this feature. If too many capabilities are introduced, it will increase more complex for the overall system design and the real product implementation. We encourage companies to further discuss the essentiality of the overhead other than simply introducing such UE capability. |

Issue 2-4-2: how to define overhead cap

| Company | Comments |
| --- | --- |
| vivo | Propose option 4: consider this issue after the rules for solving gap colliding issue are cleared. |
| Huawei | Option 3, and we are also open to option 1. |
| Ericsson | Postpone the discussion after agreeing 2-4-1. |
| MTK | Support Option 1 to align with UE’s legacy capability. Also fine to wait the conclusion of Issue 2-4-1. |
| ZTE | Postpone the discussion after agreeing 2-4-1. |
| LG Electroncis | Support Option 2. However, OK to postpone it. For Option1, if one MG is configured with GP#4{MGRP of 20ms, MGL of 6ms}, multiple gap cannot be configured. Because GP#4 corresponds to the max overhead in Rel-15/16. It can restrict to configure multiple MGs. With Option2, multiple MGs can be configured with K threshold. |
| Apple | Support option 1. Also fine with leaving it to UE capability. |
| Intel | Can be FFS. the adaptive rules by Option 2 can be fine for us. |
| CATT | Depend on issue 2-4-1, and we think no overhead cap is needed. |
| Nokia | RAN4 would need to settle Issue 2-4-1 first |
| Qualcomm | If an overhead cap rule is introduced, it should take into account per-UE vs. per-FR and MG collisions (if agreed to be supported). |

Issue 2-5-1: whether to additionally consider the limitation that each reference signal can only be measured in one MG pattern

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 |
| Huawei | Option 1. |
| Ericsson | Option 1 |
| MTK | Support Option 1. No limitation, but if network wants to do so, it can still be configured. |
| Xiaomi | Option 2, our intention to have this limitation is to simplify the delay requirements, since the CSSF is calculated for each MG pattern separately, if one reference signal, e.g. SSB or CSI-RS can be measured in multiple MG pattern, which MG pattern is used for defining requirements? As shown in figure 1, does CC1 should be accounted in CSSF calculation for both MG#1 and MG#2 and how to define measurement delay requirement for CC1? |
| ZTE | Support Option 1. |
| OPPO | Option 1. |
| LG Electronics | Prefer Option 1 |
| Apple | Support option 1. To Xiaomi: we agreed in RAN4#100e and sent an LS to RAN2 that RAN4 expects association between MO and MG. To answer the question, even though CC1 can be covered by both MG#1 and MG#2, UE shall only CC1 by using the associated MG. |
| Intel | Option 1 |
| CATT | Support option 1. It has been agreed one layer can only be associated with one gap which can include this issue. |
| Nokia | Support recommended WF (Option 1) |
| Qualcomm | Option 1. |

Issue 2-5-2: CSSF calculation

| Company | Comments |
| --- | --- |
| vivo | Ok with option 1 |
| Huawei | Option 1. In our understanding, option 2 is related to the collision handling of concurrent MGs, which could be discussed in Issue 2-3-2. |
| Ericsson | Option 1. |
| MTK | Support Option 1, which eases significantly the spec and UE implementation complexities. This issue was discussed in last meeting with a clear majority view. Hope we can converge this time. |
| Xiaomi | Option 1 |
| ZTE | Support Option 1. |
| OPPO | Option 1. |
| LG Electronics | Prefer Option1. |
| Apple | Option 1. |
| Intel | Option 1 |
| CATT | Option 1. |
| Nokia | In the end the CSSF will depend on agreements related to Issues 2-3-1 and 2-3-2. However, we agree on the principles of option 1. Only layers sharing the MG (and the sharing is the open aspect) will be used when calculating CSSF. This would be aligned with current CSSF approach. Option 1. We would see that there is a need to agree on the sharing aspect for: 1) sharing of gaps among carriers within a concurrent MG when the concurrent measurement gap is not overlapping with a measurement gap. 2) sharing of gaps among carriers within an overlapping concurrent MG when the concurrent measurement gap is overlapping with a measurement gap. |
| Qualcomm | Option 1 |

Issue 2-5-3: Measurement delay outside gap

| Company | Comments |
| --- | --- |
| vivo | OK with the scenarios suggested to be studied. Agree that this issue is pending on the overlapping issue. |
| Huawei | Fine with option 1 to further study the identified cases. |
| Ericsson | Fine with option 1 to further study the identified cases. |
| MTK | Support Option 1 for FFS. |
| Xiaomi | One simple way is to define the delay requirement provided that one SMTC is associated with one MG. |
| ZTE | Fine with option 1 to further study the identified cases. |
| LG Electronics | OK option 1 with further study. |
| Apple | Fine with option 1. |
| Huawei | Option 1. In our understanding, option 2 is related to the collision handling of concurrent MGs, which could be discussed in Issue 2-3-2. |
| Intel | It is up to issue 2-3-2/3 |
| CATT | Fine with option 1. |
| Nokia | We are fine discussing further and discuss the listed issues. We prefer to discuss once more agreements are clear. We prefer to stay to the currently defined rules for measurement outside gaps and measurements within gaps. |
| Qualcomm | Question about option 1. Do the scenarios include the case where one SMTC occasion is partially overlapped with a MG instance? |

Issue 2-5-4: Measurement delay within gap

| Company | Comments |
| --- | --- |
| Vivo | Slight prefer option 1 |
| Huawei | The two options are not exclusive. On option 1, with the association between frequency layer and MG to be provided by NW (as agreed in last meeting), it should be clear which MGRP/CSSF value is to be used to derive the requirements for each frequency layer. Is this same as the “clear indication” in option 1? On option 2, we are fine with the principle, and the details could be discussed after concluding Issue 2-3-2. |
| Ericsson | Option 1 and option 2. We support Huawei’s observations. |
| MTK | We are fine with both options. To Huawei. The intention is to add a clarification in 38.133 spec, assuming that the association will be implemented in 38.331. |
| Xiaomi | One simple way is to define the delay requirement provided that one SMTC is associated with one MG. |
| ZTE | Support Option 2, which is common mechanism. Open to Option 1. |
| LG Electronics | Open with Option 1 & Option 2. |
| Apple | Related to how to handle the overlapped issue. In current stage, both options are feasible in principle. |
| Intel | It is up to issue 2-3-2/3 |
| CATT | Fine with both principles. But how to reflect them can be further studied when defining the measurement requirements. |
| Nokia | We prefer to discuss once more agreements are clear. We prefer to stay to the currently defined rules for measurement outside gaps and measurements within gaps. Please also see our comment to Issue 2-5-2 We would see that there is a need to agree on the sharing aspect for: 1) sharing of gaps among carriers within a concurrent MG when the concurrent measurement gap is not overlapping with a measurement gap. 2) sharing of gaps among carriers within an overlapping concurrent MG when the concurrent measurement gap is overlapping with a measurement gap. |
| Qualcomm | Same observations as Huawei. The details on option 2 are FFS. |

Issue 2-6-1: How to capture the impact on L1 measurements due to concurrent gap

| Company | Comments |
| --- | --- |
| vivo | Suggest add one more option (Option 6: Investigate how to define a suitable MGRP when multiple measurement gaps are configured for related measurement performance requirements such as RLM.) based on our discussion tdoc. Fine to consider forward compatibility |
| Huawei | We support option 3 and option 5, which are similar and both aligned with the Recommended WF. We see it difficult and non-forward compatible to define an effective MGRP when considering concurrent MGs, so we suggest to adopt a generic and scalable approach. |
| Ericsson | We’re fine to consider option 3 and 5 as a general rule to define P scaling factor. We still think it’s better to have some equations to clearly indicate the requirements. Otherwise, it’s necessary to verify all the necessary scenarios by test cases. |
| MTK | Both Option 3 and 5 are fine to us. We see the difficulty to maintain the current equations if later we also need to handle more gaps added by MU-SIM and NTN. Also the introduction of NCSG may also complicated the calculations. Therefore, we suggest RAN4 to work on a generic requirements for forward compatibility. |
| ZTE | Support Option 5, which is aligned with the Recommended WF. For Option 3, we still query the priority rule. |
| OPPO | Option 3 and 5 are fine. |
| Apple | When developing option 1, we have a similar principle in our mind as option 3. We were using equation just because usually RAN4 uses exact equation to precisely describe our requirements. however, we are also fine with moderator’s suggestion to work on a generic way to handle this for forward compatibility, considering more and more features may have impact on gap design. |
| Intel | Support Option 3 |
| CATT | Fine with option 3 or option 5. |
| Nokia | We agree with moderator. Initially RAN4 would need to reach some more (principle) agreements and potentially even try to down scope the work by only focusing on the simpler scenarios. Also for this Issue we suggest starting point will be the existing L3 and L1-RSRP measurement rules and adapt those to account for the combined impact from both measurement gaps. |
| Qualcomm | Agree with the comments from the moderator. Suggest to keep it FFS for now. |

Issue 2-7-1: Whether to introduce a transition period for gap configuration/deconfiguration

| Company | Comments |
| --- | --- |
| vivo | Ok with the recommended way forward |
| Huawei | Support the Recommended WF |
| Ericsson | Support the Recommended WF |
| MTK | Support the Recommended WF |
| Xiaomi | Option 1 is fine |
| ZTE | Support the Recommended WF |
| OPPO | Support the recommended WF |
| LG Electronics | Support the recommended WF |
| Apple | Option 1. |
| CMCC | Support the recommended WF. |
| Intel | Support the recommended WF. |
| CATT | Support the recommended WF. |
| Nokia | support the recommended WF |
| Qualcomm | Support the recommended WF. |

2.2.7.2	Issue 2-7-2: UE measurement behavior after transition

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We are fine with the second and third bullet of option 1 which are same as single MG case. We do not support the first bullet of option 1. During such a transition case, UE cannot use MGP1 anymore after reconfiguration, so it may not be able to meet the requirements based on MGP1. The longer requirements before and after the transition should apply, or UE should be allowed to re-start the measurement. |
| Ericsson | Support option 1. @Huawei, could you help to further clarify your comments for bullet 1? From our understanding, MGP1 is still there, UE can still perform measurements in MGP1 after MGP2 configuration. It’s better to continue the on-going measurements in MGP1 other than re-start the measurements in MGP2. |
| MTK | Actually, we have a very basic question: Do we need to define the transition behavior? In our view, measurement configurations (MO, gap) are expected to be applied for a very long time. The transition period (maybe several hundreds of ms) is not a very important issue to us. |
| Xiaomi | For the 1st bullet, UE can use MGP2 for the ongoing measurement, and a related delay requirement can be applied between MGP1 and MGP2. |
| ZTE | We support the 3rd sub-bullet of Option 1. |
| Apple | We don’t see the necessity to define UE measurement behavior after transition. There is no much difference from legacy. For instance, UE is doing intra-frequency measurement and no gap is configured. Then NW configures some inter-freq MO and also the measurement gap. Assuming gap and SMTC for intra-freq is partially overlapped, according to RAN4 requirement, UE shall perform intra-freq measurement outside the gap (skip some the SMTC which can be used for measurement before gap is configured). But RAN4 didn’t define any specific UE behaviour for this. |
| Intel | We also have same views as MTK and Apple, We need not to define such behavior. |
| CATT | We don’t see the necessity to define this UE behavior. And for the first bullet, we share the same view as Huawei that when MO1 is reconfigured to be measured with MGP2, it cannot be measured in MGP1. Although MGP1 still exists, it is not associated with MO1 anymore. UE should perform measurement based on the association information configured by NW. |
| Nokia | We do not see a strong reason to introduce detailed rules related to these transition scenarios. We support that when the UE is configured with concurrent MG for a layer the UE shall apply the concurrent MG immediately and measure the layer accordingly – no additional conditions. |
| Qualcomm | We are not sure this is needed. Similar issues can occur with legacy gaps. Do we have requirements now for transient behavior? |
| Ericsson | To MTK, Apple, Intel, CATT, QC, #### In current spec. for single MG, the transition period requirement is clearly captured. ### 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition. When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.. At the same time, many UE transition behaviour are also defined in different sections - Idle mode(4.2.2.8 Minimum requirement at transitions) - RLM(8.1.4 Minimum requirement at transitions) - BFD(8.5.10 Minimum requirement at transitions for beam failure detection) Network needs to clearly know UE’s behaviour in such transition period, especially after new gap is configured or deconfigured. When concurrent gaps are introduced, the clear UE behaviours shall also be defined. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118413 Ericsson | MTK: Thanks so much for the contribution. We suggest to postpone the CR to next meeting, as we expect the CR work split will be discussed in the 2nd round in this meeting. |
|  | Apple: similar view as MTK. |
|  | Nokia: Same view as MTK – wait further agreements and CR split |
|  | Ericsson: We’re fine to postpone the CR after agreeing the CR split |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1 | Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured. Status: - Option 1: CATT, MTK, vivo, Xiaomi, Ericsson, Nokia - Option 2: Apple, QC, Intel, OPPO, LGE - Option 3: ZTE - Option 4: Huawei Recommendations for 2nd round: - Continue discussion in the 2nd round. Companies are encourage to provide reasons for the benefits for this configuration and any difficulty to support this configuration. - Moderator will capture this issue in the draft WF. |
| Issue 2-1-2 | Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17 Status: - All companies are fine with the recommended WF. - Nokia suggests to further clarify the meaning of ‘exclusively’. - To Nokia: In last meeting, we already agreed in the LS R4-2115343 that each frequency layer can be associated with only one MG Note: - It is RAN4 common understanding that frequency layer includes positioning frequency layer. - Each frequency layer can be associated with only one MG (leave it for RAN2 on how to implement the association) - SSB, CSI-RS and PRS are treated as different frequency layers - One MG can be associated with multiple frequency layers, while one frequency layers can only be associated to a single MG.. Recommendations for 2nd round: - Moderator suggests to agree on the recommended WF. The followings are to be captured in the draft WF for 2nd round discussion - PRS measurement for positioning is associated with only one of the concurrent gaps - It is up to network whether to associate a gap only to PRS measurement |
| Issue 2-1-3 | Whether to support 2G/3G measurement with concurrent gap Status: - Option 1: vivo, HW, MTK, Xiaomi, OPPO, LGE, Apple, Intel - Option 2: E///, MTK, ZTE, CATT - (New) Option 3: Nokia - Support 2G/3G - QC wants to clarify the consequence if Option 1 is agreed. Recommendations for 2nd round: - As there is a clear majority to preclude 2G/3G, moderator suggest to take Option 2 as a middle ground to proceed, i.e., RAN4 to focus on NR and EUTRAN measurements before considering 2G/3G. The followings are to be captured in the draft WF for 2nd round discussions - RAN4 to focus on NR and EUTRAN measurements with concurrent gaps before considering 2G/3G. - Note: The understanding of “2G/3G is not supported with concurrent gap” is that UE expects network to configure only one MG if any 2G/3G measurements are configured, regardless whether NR or EUTRAN measurements are configured. |
| Issue 2-2-1 | Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs Status: - Option 1: Xiaomi, LGE, OPPO - Option 2: CATT, CMCC, vivo, Ericsson, Nokia, Huawei, Intel - Option 2a: Apple, QC, MTK, vivo, ZTE, LGE, Intel - To Xiaomi and OPPO, please check if QC’s reply answers your question. Recommendations for 2nd round: - As this issue has been discussed for many meetings, moderator suggest to take a compromise based on Option 2a, but still open a door if any new use cases are identified in the future. The followings are to be captured in the draft WF for 2nd round discussion. - Simultaneous configuring per-UE gap and per-FR gap is only allowed when the per-UE gap is associated to PRS measurement. FFS to include other use cases, if identified. |
| Issue 2-2-2 | Max number of concurrent gap across all FRs for per-FR gap capable UEs (without considering MU-SIM and NTN) Status: - Option 1: Apple, QC, Xiaomi, Intel, Huawei, LGE, [Nokia] - Option 2: CATT, CMCC, vivo, OPPO, Ericsson, Nokia, ZTE, [LGE] - Option 3: MTK, HW, Xiaomi, ZTE, Apple, Intel, QC Recommendations for 2nd round: - Continue discussion in the 2nd round. Same options to be captured in the draft WF. |
| Issue 2-2-3 | Support of indices 11 and 12 Status: - All companies support the WF. Recommendations for 2nd round: - The followings are to be captured as agreed in the draft WF. - Support the 2 additional combinations (Index 11 and Index 12) Index; # of simultaneous MG. Per-FR1; Per-FR2; Per-UE. 11; 2; 0; 0. 12; 0; 2; 0. |
| Issue 2-2-4 | Clarification on gap pattern #24 and #25 Status: - 11 companies do not support Option 1 to re-discuss whether GP#24 and #25 are per-UE or per-FR Recommendations for 2nd round: - Moderator suggests not the change the Rel-16 conclusion as preferred by the majority. The followings are to be captured as an agreement in the draft WF - It is RAN4’s understanding that GP#24 and #25 are only used as per-UE gap. No change of this conclusion is expected in this WID. |
| Issue 2-2-5 | Clarification on configuration limitations or restrictions regarding the GPs supported by the UE Status: - 11 companies are fine with Option 1. - QC pointed out that Issue 2-2-4 may also bring some impact to gap applicability. Recommendations for 2nd round: - Given the current status in Issue 2-2-4, Moderator believe that it is OK to agree on Option 1. The followings are to be captured as agreed in the draft WF in 2nd round. - Besides the potential outcome of UE capability and overhead cap discussions, no additional configuration limitations or restrictions regarding which of the GPs supported by the UE that can be configured as concurrent gap. |
| Issue 2-3-1 | Proximity condition for overlapping Status: The outcome of GTW session - Agreements - Two measurement gap occasions are defined as colliding (overlapping) if at least one of the following conditions apply - Condition #1: The gaps are physically fully or partially overlapping in time domain - Condition #2: The gaps are not physically overlapping in time domain but the minimal distance between the two gap instances is equal or less to X - X = [1, 2, or 4] ms - FFS if split between FR1/FR2 is needed Recommendations for 2nd round: - The agreements are to be captured in the WF. |
| Issue 2-3-2 | UE behavior during colliding gap occasion Status: - Option 1: QC, MTK, vivo, Xiaomi, OPPO, Nokia, Huawei, LGE, CMCC - Option 2: Apple, vivo, Xiaomi, ZTE, CMCC, Intel, CATT - Option 3: CATT, LGE, OPPO - Option 4: Ericsson Recommendations for 2nd round: - This issue is still very controversial, and unfortunately we do not have time to discuss it during the GTW session. Moderator has a compromised idea to merge Option 1, 2 and 4: - Option 5: Introduce gap sharing rule. - Request RAN2 to reserve some RRC signaling for different sharing factors. - The signalling design may consider the possibility of resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%. - The requirements for other sharing factors are FFS in later releases. - FFS whether the resume scheduling on those dropped gaps as well as the impact to other intra-frequency measurements - All options 1 – 5 will be captured in the draft WF for 2nd round discussion (including the GTW) |
| Issue 2-3-3 | Company preference on introducing FO, FPO, PFO, PPO scenarios Status: - Companies have provided inputs on the preference for different scenarios Recommendations for 2nd round: - Moderator thanks all companies for the inputs, which are to be used as the reference if any prioritization discussion is needed in the future. - As there is not much progress in Issues 2-3-1 and 2-3-2 in the 1st round, the plan is to postpone the discussion to the next meeting. |
| Issue 2-4-1 | Whether to define the overhead cap Status: - Option 1: Apple, vivo, LGE, Intel, HW - Option 2: CATT, QC, CMCC, Ericsson, Nokia, ZTE, Intel - Option 3: MTK, Apple - Option 4: OPPO Recommendations for 2nd round: - Continue discussion in 2nd round |
| Issue 2-4-2 | how to define overhead cap Status: - Option 1: HW, MTK, Apple - Option 2: LGE, Intel - Option 3: HW - Option 4: vivo - 5 companies are fine to postpone it before conclusion in Issue 2-4-1 - QC commented that it should take into account per-UE vs. per-FR and MG collisions (if agreed to be supported) Recommendations for 2nd round: - Continue discussion in 2nd round |
| Issue 2-5-1 | Whether to additionally consider the limitation that each reference signal can only be measured in one MG pattern Status: - 12 companies support Option 1. - 1 company support Option 2. Recommendations for 2nd round: - Moderator suggest to agree on Option 1. Note that this is also an information that is important for RAN2, as they are also discussing this at the same time. The followings are to be captured in the draft WF as an agreement (@ Xiaomi, please check if you still have serious concern) - Measurements for different frequency layers but with the same reference signal can be associated to different concurrent MGs. |
| Issue 2-5-2 | CSSF calculation Status: - All companies agree with Option 1. - Nokia pointed out some aspects to be further studied. Recommendations for 2nd round: - The followings are to be captured in the draft WF as an agreement: - CSSF should be calculated separately for each gap and only the frequency layers sharing this gap should be counted in - Note: how to deal with overlapping concurrent MGs is up to Sub-topic 2-3. |
| Issue 2-5-3 | Measurement delay outside gap Status: - Most of the companies are fine with Option 1 to further study the identified cases. - Nokia preferred to wait until more agreements are clear. - QC question whether the case where one SMTC occasion is partially overlapped with a MG instance - To QC: In moderator’s understanding this case is already precluded in the requirement, as addressed in Section 9.1.5.2.1 of TS38133 An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR carriers, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.. Recommendations for 2nd round: - Moderator suggests to at least capture the issue in the WF to be further studied. The detail can FFS. |
| Issue 2-5-4 | Issue 2-5-4: Measurement delay within gap Status: - Most of the companies are fine with Options 1 and 2 to further study the identified cases. - Nokia preferred to wait until more agreements are clear. Recommendations for 2nd round: - Moderator suggests to at least capture the issue in the WF to be further studied. The detail can FFS |
| Issue 2-6-1 | How to capture the impact on L1 measurements due to concurrent gap Status: - Most of companies are fine with Option 3 and 5. - Vivo suggested Option 6 for defining a suitable MGRP - ZTE mentioned the discussion is pending on the conclusion of UE behavior during gap collision. - Nokia suggested to capture some high-level principle as a starting point. Recommendations for 2nd round: - Moderator suggest to take Rel-15 principle as a starting point, while trying to work on a generic rule for forward compatibility. The followings are to be captured in the draft WF for 2nd round discussion. - Take Rel-15 principle as a starting point. - L1 measurements are only expected to be performed outside gap. - In FR1, L3 and L1 measurements can be performed at the same time. - In FR2, L3 and L1 measurements are not expected to be perform at the same time. - FFS how to specify the impact of concurrent gap on L1 measurement period in a generic manner. |
| Issue 2-7-1 | Whether to introduce a transition period for gap configuration/de-configuration Status: - All companies agree with Option 1 Recommendations for 2nd round: - The followings are to be captured as an agreement in the draft WF: - Do not introduce a transition period for gap configuration/de-configuration |
| Issue 2-7-2 | UE measurement behavior after transition Status: - 6 companies do not see the need to define a requirement for transition. - HW, Xiaomi, CATT raised a question to bullet #1. - E/// provided examples in current spec on transition behavior. Recommendations for 2nd round: - From Moderator’s point of view, the current spec has some transition requirement for particular scenarios, but there are also other scenarios without transition requirements (as mentioned by Apple). RAN4 can of course have a case-by-case discussion on whether the transition requirements are needed. - The issue will be captured in the draft WF for 2nd round discussion. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2118413 Ericsson | Postponed. |

## Discussion on 2nd round (if applicable)

Moderator: The discussions are copied from the WF discussion

## Issue 2-1-1: Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured - Open issue - Option 1: Yes - Option 2: No - Option 3: No need to further discuss - Option 4: Yes, provided that UE supports LTE measurement with concurrent MGs, which is Up to UE capability - Companies are encourage to provide reasons for the benefits or difficulty to support this configuration Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) CATT; we still think there is no need to limit the NW configuration. Huawei; We support option 4. In our view, using concurrent MGs for NR measurement and LTE measurement should be considered as different UE capabilities since they may require different UE implementations. If a UE can support using concurrent MGs for LTE measurement, we do not see the reason why it should not be allowed when only LTE MOs are configured for measurement.. Ericsson; Up to the NW configuration. @Huawei, Could you further explain the restriction from UE side on supporting concurrent gaps on LTE only?. Apple; Support option 2. We agree that if UE can support using concurrent MGs for LTE measurement, UE may also support the case wherein only LTE Mos are configured for measurement. However, our concern is more about use case and potential extra standardization work. Sometimes we don’t specify requirements just because UE can support it. We don’t see many practical use cases of such configuration. In LTE there is PSS, SSS and CRS every 5ms, which is denser than SMTC in NR. Therefore, it is easier to use one gap to cover all the LTE layers. Besides, this is more like an enhancement on LTE. If there is real demand from the field, this should have been raised in LTE design. But no one ever claimed such need in LTE design. On the other hand, allowing such configuration may result in extra standardization work, which is something doable but with low priority (at least lower than joint discussion with pre-MG and NCSG).. Intel; We prefer Option 2. Such scenario will be rarely happened in practical. Regarding to the timeline issue of this WI, we need focus on the other more typical and useful cases.. vivo; Considering the case is rare, we can compromise to option 2.. OPPO; Support option 2. Do not see the necessity for such case.. LG Electronics; Support option 2. As mentioned in 1st round, benefit is not clear.. ZTE; Prefer Option 2 and Option 3. Same view with Intel.. MTK; We support Option 1 and believe this is up to network configuration. We have provide the use cases that network can still try to prioritize particular frequency layers through imbalanced association between 2 gaps. A compromise proposal is to allow this signalling in Rel-17 but RAN4 Rel-17 requirement will not consider the scenario. Whether to extend the requirement in later release can be FFS.. Xiaomi; Support option 1. Qualcomm; Prefer option 2.. Nokia; Option 1, we see this up to network configuration. It is not clear which technical aspects would prevent this.. ## Issue 2-1-2: Whether PRS measurement for positioning can only be exclusively associated with only one of the instance of multiple gaps at least for R17 - Agreement - PRS measurement for positioning including all positioning frequency layers is associated with only one of the concurrent gaps - It is up to network whether to associate a concurrent gap only to PRS measurement Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) CATT; Our understanding on this issue is to clarify all the positioning frequency layers should be associated with the same gap. We suggest to explicitly included in the conclusion to avoid confusion of RAN2. MTK; Agree with CATT.. ## Issue 2-1-3: Whether to support 2G/3G measurement with concurrent gap - Agreement - RAN4 to focus on NR and EUTRAN measurement requirements with concurrent gaps before considering 2G/3G. - It is up to RAN2 to decide whether to support gap association to 2G/3G from signalling perspective - Note: The understanding of “2G/3G is not supported with concurrent gap” is that UE expects network to configure only one MG if any 2G/3G measurements are configured, regardless whether NR or EUTRAN measurements are configured. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; the agreement may be sufficient for RAN4, but it may not be fully clear to RAN2 when they define the signalling, e.g. whether to consider 2G/3G measurement when defining the association between concurrent MGs and frequency layers or dedicated use cases. We hope we can make it more clear during the second round, and our preference is not to support 2G/3G measurement with concurrent MGs.. Ericsson; Up to the NW configuration. From our understanding, NW can configure one gap for NR and another gap for inter-RAT including 2G/3G. @Huawei, Could you further explain the restriction from UE side on supporting concurrent gaps on 2G/3G only?. Apple; Share similar view with Huawei. “focus on NR and EUTRAN measurements with concurrent gaps before considering 2G/3G” means no RAN4 requirements. RAN2 may interpret this as 2G/3G can be supported from signalling point of view. Enabling concurrent gaps when 2G/3G measurement is configured would result in extra effort in UE baseband implementation, which is unnecessary considering 2G/3G are no longer the critical deployment in reality.. Intel; In our understanding, such 2G/3G measurement may be not necessary to be considered. If so, the exact wording to exclude them can be better.. vivo; Similar view as Huawei and Apple. OPPO; Prefer to clearly inform RAN2 of not supporting 2G/3G measurement with concurrent MGs.. LG Electronics; Same view with Huawei & Apple. MTK; Regarding the issue raised by Huawei, we think it is up to RAN4 discussion. Given that RAN4 already de-prioritized 2G/3G, RAN4 can continue discussing whether the signalling is still needed. If no conclusion is achieved, we can also let RAN2 to decide. To be clear at this stage, in the LS to RAN2 we can clarify that RAN4 is still discussing whether the signalling is needed.. CATT2; Same view as Ericsson. We would like to understand more why NW cannot configure one gap for NR measurement and the other gap for other RAT including E-UTRAN/2G/3G.. Qualcomm; In our view, this issue can be addressed in the same way as issue 2-1-1.. Nokia; We understand that there are many companies that prefer not to support but we believe this would be up to network configuration. It would technically be the same as configuring 2G/3G measurement only using existing (non-concurrent) measurement gaps. Hence, we do not see any technical issues raised. Additionally, this seems against the agreement in last meeting that there is no additional capability related to concurrent GPs.. ## Issue 2-2-1: Whether to allow simultaneous configuration of per-UE gap and per-FR gap to FR gap capable UEs - Open issue - FFS the use case of simultaneous configuring per-UE gap and per-FR gap. Consider the identified use cases to make decision in RAN4#101b-e meeting. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) CATT; We still think if simultaneous configuration of per-UE and per-FR cap is allowed for PRS measurement, then there is no reason to preclude that for RRM requirements. Since it is still open for other cases, we suggest to remove ‘only’ in the conclusion.. Ericsson; Support CATT.. Apple; Do not support to remove ‘only’. We raised our concern several times. Again, there is no benefit of such configuration, compared with configuring per-FR gap separately in different FR, except for PRS measurement. For instance, configuring GP#0 as per-UE gap would result in 0.5ms loss in FR2 compared with configuring GP#0 in FR1 and GP#13 in FR2 as per-FR gaps. NW is encouraged to make use of this 0.5ms, rather than simply configure per-UE gap, to improve system throughput.. OPPO; Removing ‘only’ seems fine to us.. ZTE; Fine with the Agreement given by moderator originally. ‘only’ is necessary.. MTK; We disagree with removing ‘only’. If ‘only’ is removed, it is an open-ended agreement. The ‘FFS’ is added so that companies can provide use cases to justify the need of such a configuration. We tend to believe that RAN4 need to make the agreement based on real use cases or scenarios to avoid over-designing the feature functionality.. Xiaomi; Do not support to remove ‘only’. The per-UE gap (only GP#24 and #25) can be only configured for PRS measurement exclusively. Without such clarification, we cannot accept such agreement.. Qualcomm; We support the original wording (including ‘only’) proposed by the moderator.. Nokia; We can agree having this as open issue for further discussion. That would give companies a chance to discuss the reasoning for the limitation why the Per-UE gap has to be limited PRS only? For example, if the network configures a Per-UE capable UE with GP#1 in FR1 and FR2 and additionally GP#0 in FR2 or FR1 it would essentially be the same as configuring 1 Per UE GP (GP#1) and one Per-FR GP (GP#0)? However, without limitation.. ## Issue 2-2-2: Max number of concurrent gap across all FRs for per-FR gap capable Ues (without considering MU-SIM and NTN) - Open issue - Option 1: 3 - Option 2: 4 - Option 3: Up to UE capability Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; we support option 1 and can compromise to option 3. CATT; Based on issue 2-2-1, index 5 in the following table is supported. If so, then index 6 should be also supported which is equivalent to index 5. Since per-UE gap is applied for both FR1 and FR2. It can be regarded as configuring one per-FR1 gap plus one per-FR2 gap.. Ericsson; Option 2 and can compromise to option 1, but we don’t support to introduce any UE capability for max numbers.. Apple; Support option 1 considering UE complexity. Not very clear about comment from CATT.. Intel; Our preference is Option 1. Option 3 can be a compromise option.. Vivo; Prefer option 2. Can compromise to option 3.. OPPO; Prefer option 2 and can compromise to option 1.. LG Electronics; Prefer option 1. If UE complexity is not issue, option 2 is also fine.. ZTE; Both Option 2 and 3 are OK for us.. MTK; We do not have a strong view between Option 1 and 2. But if no consensus can be achieved, we would suggest to go for Option 3 as a compromised solution.. Xiaomi; Our preference is Option 1. Option 3 can be a compromise option.. CATT2; Support option 2. To Apple, our point is that one per-UE gap is equivalent to configuring per-FR1 gap and per FR2 gap simultaneously. So if we support the configuration (per-UE gap + per FR1 gap + per FR2 gap), it is equivalent to the configuration (2 per-FR1 gap + 2 per-FR2 gap).. CMCC; Option 2. In our understanding, each FR shall be treated equally for the per-FR gap capable UEs. Based on this understanding, the max number of concurrent gap across all FRs is 4.. Apple; @CATT, Thanks for clarification. However, per-UE gap + per FR1 gap + per FR2 gap is still open. At least from our perspective we don’t see the necessity to support this configuration.. Qualcomm; We support options 1 and 3.. Nokia; Option 2. We do not support introducing a UE capability for this. Our preference is to support option 2 but we can discuss compromising to option 1.. ## Issue 2-2-3: Support of indices 11 and 12 - Agreement - Support the 2 additional combinations (Index 11 and Index 12) Index; # of simultaneous MG. Per-FR1; Per-FR2; Per-UE. 11; 2; 0; 0. 12; 0; 2; 0. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ZTE; Fine with the Agreement given by moderator.. ## Issue 2-2-4: Clarification on gap pattern #24 and #25 - Agreement - It is RAN4’s understanding that GP#24 and #25 are only used as per-UE gap. No change of this conclusion is expected in this WI. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-2-5: Clarification on configuration limitations or restrictions regarding the GPs supported by the UE - Agreement - Besides the potential outcome of UE capability, applicability of existing MG patterns, and overhead cap discussions, no additional configuration limitations or restrictions regarding which of the GPs supported by the UE that can be configured as concurrent gap. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Qualcomm; We assume this agreement would take into account existing applicability of MG patterns 0-25. E.g. MG patterns are only per-UE and can be configured only when positioning measurements are requested. It would be good to clarify.. Moderator; To QC: This is also our understanding. Maybe the agreement in Issue 2-2-4 can already address your concern?. Qualcomm; To moderator: Would the following clarification be agreeable? - Besides the potential outcome of UE capability, applicability of existing MG patterns, and overhead cap discussions, no additional configuration limitations or restrictions regarding which of the GPs supported by the UE that can be configured as concurrent gap.. ## Issue 2-3-1: Proximity condition for overlapping - Agreements (from GTW session on Nov 4th) - Two measurement gap occasions are defined as colliding (overlapping) if at least one of the following conditions apply - Condition #1: The gaps are physically fully or partially overlapping in time domain - Condition #2: The gaps are not physically overlapping in time domain but the minimal distance between the two gap instances is equal or less to X - X = 1 or 4 ms for FR1 - X = [1, 2, or 4] ms for FR2 - FFS if split between FR1/FR2 is needed Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; we support X=4ms for both FR1 and FR2. From UE side this values is related to the time for measurement scheduling, so FR2 delay is not necessarily smaller than FR1. Also, having one single value can be simpler for both UE and NW implementation, e.g. it can be difficult to diff FR1 and FR2 for a per-UE gap. Ericsson; From our understanding, UE will always perform processing faster in FR2 than FR1. Thus, we suggest to define values as X=4ms for FR1 and 1ms for FR2.. Apple; We are fine with 2 or 4ms. A question for clarification regarding different values for FR1/FR2 for per-UE gap only UE. For instance, UE which only supports per-UE gap is working in FR1 + FR2 CA. What is the correct understanding of different values between FR1/FR2? Assuming 4ms in FR1 and 1ms in FR2 as proposed by some companies, does it mean the proximity condition is still 4ms in FR1 and 1ms in FR2? Or shall it be the longer one (4ms)?. Intel; Can be [4]ms.. LG Electronics; For split between FR1/FR2, we need to clarify what is considered.. MTK; Although we do not see a need to introduce non-zero X, we can compromise to 4ms for both FR1 and FR2. We tend to believe that a company who want a smaller value can also support a larger value. The question mentioned by Apple can be resolved by setting the same value for both FR1 and FR2.. Xiaomi; Support define single value, X=4ms for both FR1 and FR2.. Qualcomm; Support 4 ms for both FR1 and FR2.. Nokia; As we see it, the longer inter-gap distance is required to be accounted by the network in the configuration of the concurrent gap (and legacy gaps) can limit the use of concurrent gaps due to restricting the location of GPs. Based on the discussion and because X=0 is no longer an option, we prefer following: X=1 for FR1 X=1 for FR2. Ericsson; Our original intention is to disable one of gaps and allow the data scheduling on the disabled gaps. If no data scheduling is permitted, our preference value is the same as Nokia, X=1 for both FR1 and FR2. ## Issue 2-3-2: UE behavior during colliding gap occasion - Open issue - Option 1: Priority rule - UE will only do the measurement w.r.t. the gap with higher priority on all colliding occasions - The priority can be configurable or fixed - FFS whether to resume data scheduling during dropped gap occasions - Option 2: Gap-sharing rule - Given X% gap sharing, the measurement w.r.t. one gap will share roughly X% of the time, while the other gap shares the remaining - The sharing factor can be configurable or fixed - [Data schedule is not expected during both gaps] - Option 3: A hybrid priority and gap-sharing rule - Sharing is only considered between equal priority gaps - Data scheduling is expected during the dropped gap occasions - Option 4: Gap cancellation rule - A pattern telling UE how to drop gaps in each colliding occasion. - Data scheduling is expected during the dropped gap occasions - Option 5: Compromised proposal from moderator - Introduce gap sharing rule. - Request RAN2 to reserve some RRC signaling for different sharing factors. - The signalling design may consider the possibility of resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%. - The requirements for other sharing factors are FFS in later releases. - FFS whether the resume scheduling on those dropped gaps as well as the impact to other intra-frequency measurements Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; We support option 1 which enables a basic framework for Rel-17. In our understanding, there can be multiple options if we want to consider sharing factors other than 0% and 100%, and they may require different signaling designs, e.g. option 3 and 4 can both enable 50%-50% sharing between two MGs, but the signaling are different. Without concrete mechanism design in RAN4, it may be difficult for RAN2 to design the signaling as in option 5. Considering the time line of Rel-17, we suggest to focus on the most basic mechanism, so we support option 1, and further enhancement can be considered in future releases.. Ericsson; We support option 4 which is a dynamic scheduling for concurrent gaps. We can compromise to an update version of option 5 to further consider other sharing ratios except 0% and 100% in the later release, but RAN4 shall guarantee data scheduling for dropped gaps. Thus, we propose option 6: Compromised proposal - Request RAN2 to reserve some RRC signaling to indicate different sharing factors. - The signalling design shall consider resuming data scheduling on dropped gaps - Rel-17 requirements will only consider sharing ratios 0% and 100%. - The requirements for other sharing factors are FFS in later releases. - Data scheduling on those dropped gaps is expected - The requirement for other intra-frequency measurements is FFS. Apple; Support option 2. Compared with option 1, option 2 can cover option 1 by setting 100% sharing for the ‘higher priority’ GP. Besides, if UE always do the measurement w.r.t. the GP with higher priority, then the possible use cases of configuring concurrent gaps would be quite limited, probably the only use case is for PPO with high priority on the GP with longer MGRP. For other cases, option 1 is effectively saying no need to configure concurrent gaps since the GP with lower priority would be always skipped. For option 3, it is technically doable in case there are many concurrent GPs. However, we only support 2 GPs in each FR (if MUSIM, NTN is not considered). On the other hand, extra standard effort is needed to support option 3. But we don’t see significant gain compared with option 2. In our view, option 4 is even more complicated than option 3 by introducing configurable gap cancellation patterns, which is not preferred.. Intel; Option 2.. vivo; Ok with either option 1 and 2. OPPO; Prefer option 1.. LGE; Support Option 1. In our paper (R4-2117835), we provide one example of how not to skip the lower priority. For detail, we can discuss further.. ZTE; Support Option 2.. MTK; We support Option 1 and 5. Option 1 remains the simplest solution among all proposals. The point raised by Apple is valid, but we think it is eventually up to network whether to configure in those ways. Option 5 is a compromise between Option 1, 2 and 4. The intention is to focus on the simplest case and consider complicated cases in later releases. We understand there is a conflict between gap sharing and resuming data scheduling on dropped gap. The final solution may not be as perfect as everyone expects.. Xiaomi; Ok with either option 1 and 2. CATT2; Fine with option 2.. Qualcomm; Support option 1 with data traffic during the dropped gap instance.. Nokia; To move forward we appreciate the moderator compromise in option 5. This seems close to many of the options and what we also discussed in 1st round: We support an approach where the measurements configured using concurrent measurement gaps is prioritized to the level that those measurements are always performed whenever there is a gap collision (if RAN4 decide to discuss other than FNO scenario in Rel-17). And option 5 seem to support such an approach.. ## Issue 2-3-3: Company preference on introducing FO, FPO, PFO, PPO scenarios - Postpone this decision to next meeting Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-4-1: Whether to define the overhead cap - Open issue - Option 1: Yes - Option 2: No - Option 3: Introduce a UE capability for those UE who does not need cap Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; Option 1. Ericsson; Option 2 Considering the parallel discussion on MU-SIM, we don’t think the companies who support to use legacy gap for MU-SIM has any overhead issue in concurrent gaps.. Apple; Support option 1. We don’t think RAN4 is ready to mix MUSIM and concurrent gaps discussion at current stage. But if that is the concern, we can clarify that the overhead cap only applies when UE is not operating with MUSIM.. Intel; Option 1. Share same view as Apple, we needed NOT couple MUSIM aspects in this WI.. LG Electronics; Support Option 1.. ZTE; Option 2. The overhead can fully controlled by NW.. MTK; We propose Option 3 as a compromise, if no consensus can be reached. We also agree with Apple’s suggestion to clarify that MUSIM, NTN, Positioning are not considered in this issue.. CATT2; Support option 2.. Qualcomm; No strong view. A UE capability could be introduced as a compromise.. Nokia; Option 2. For the same reasons as in 1st round. We do not see UE capability as a solution to this.. ## Issue 2-4-2: how to define overhead cap - Open issue - Option 1: The max overhead that UE can support in Rel-15/16. - Option 2: m=1NMGLmMGRPmMGLrMGRPr≤1+ threshold(K) - N : number of multiple MG patterns - MGLr : MGL of referenced MG - MGRPr : MGRP - Option 3: When concurrent MGs are configured, the MGRP for each MG cannot be smaller than 40ms Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Huawei; option 1 or option 3.. Ericsson; Postpone. Apple; Support option 1. We can also compromise to new UE capability.. Intel; Can be FFS upon 2-4-1. LG Electronics; Support option 2.. MTK; Either Option 1 or 3 is fine to us.. Qualcomm; FFS.. Nokia; Agree with Ericsson and postpone. ## Issue 2-5-1: Whether to additionally consider the limitation that each reference signal can only be measured in one MG pattern - Agreement - Measurements for different frequency layers but with the same reference signal can be associated to different concurrent MGs Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-5-2: CSSF calculation - Agreement - CSSF should be calculated separately for each gap and only the frequency layers sharing this gap should be counted in - Note: how to deal with overlapping concurrent MGs is up to Sub-topic 2-3 Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-5-3: Measurement delay outside gap - Open issue - Companies are encouraged to provide proposals on how to modify the measurement delay requirements for concurrent gap Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-5-4: Measurement delay within gap - Open issue - Companies are encouraged to provide proposals on how to modify the measurement delay requirements for concurrent gap Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-6-1: How to capture the impact on L1 measurements due to concurrent gap - Agreement - Take Rel-15 principle as a starting point, e.g., - L1 measurements are only expected to be performed outside gap. - In FR1, L3 and L1 measurements can be performed at the same time. - In FR2, L3 and L1 measurements are not expected to be perform at the same time. - FFS how to specify the impact of concurrent gap on L1 measurement period in a generic manner. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Apple; Fine with the tentative agreement. However, seems it doesn’t cover the case wherein all RLM-RS are covered by gaps. For instance, RLM-RS is partially overlapped with MG1 or MG2, but fully overlapped with MG1+MG2. To move forward, RAN4 is to: Option 1: preclude such configuration (or not define requirements for this case) Option 2: further discuss how to share measurement opportunity between L1 and MG.. vivo; Ok with the moderator’s summary. MTK; Apple raised a valid point, which is not only issue for L1 measurement but also for L3 intra-frequency measurement. In light of the principle in Rel-15, we would suggest to preclude this case for L1 measurement. In other words, all L1 measurements are outside gap.. Apple; @MTK, we agree that this can also happen to L3 measurement. The difference between L1 and L3 here is that L3 intra-frequency measurement can be done within gap, while L1 measurement within gap is not supported yet. We are open to further discussion.. ## Issue 2-7-1: Whether to introduce a transition period for gap configuration/deconfiguration - Agreement - Do not introduce a transition period for gap configuration/de-configuration Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) ## Issue 2-7-2: UE measurement behavior after transition - Open issue - Option 1: - The UE will continue and complete the ongoing measurement on MO1 using MGP1 and meet the corresponding measurement requirement based on MGP1 during this measurement period even if the MO1 is reconfigured to be measured using MGP2. - UE will perform the measurement on MO2 using MGP2 immediately after the concurrent gaps reconfiguration, if MO2 can’t be measured by MGP1 due to gap offset or if gap length is not enough. - After one of concurrent gaps deconfiguration, data scheduling is expected on this disabled MG’s time occasions. - Option 2: - FFS whether/how to define UE measurement behaviour after transition. Addition comments (to be moved to moderator’s summary and removed in the formal Tdoc) Xiaomi; According to 1st round discussion, it is propose to add option 2 which reflects majority companies’ views.. Huawei; We can support option 2. Technically, some bullets in option 1 make sense to us, but we are also open to discuss whether and how to define the UE behaviour and requirements.. Ericsson; We’re fine to further discuss these issues. From our understanding, it’s important for NW to understand UE’s behaviour on transition period. Thus, the UE’s behaviour shall be clearly defined.. Apple; Support option 2. We don’t see significant difference from legacy situation, wherein NW can also configure or de-configure a gap pattern.. MTK; We are fine to FFS. Qualcomm; FFS. Nokia; Can be discussed further. # CR work split Spec; Section; Detail; Responsible company. TS38.133; (New) 9.1.2B; - Applicability - The condition where the con-MG requirements applied, e.g., UE capability, network configure, CA/DC scenarios - Association - How each gap is associated to dedicated use cases - UE Capability related - Max # of supported gaps for both with and without pre-FR gap capability - Including whether UE supports simultaneous per-UE/FR gap. - [Overhead cap]; Ericsson. (New) 9.1.2B; - Gap colliding rules - Definition of gap colliding and the corresponding UE behavior, e.g., gap priority/sharing and how effective MGRP should be revised in the delay requirement - Gap interruption - Extension from 1 gap to multiple gaps; Huawei. 9.1.5; - Association - Update the CSSF based on the new association between gap and dedicated use cases.; Apple. 9.2.5, 9.2.6, 9.2A.5, 9.2A.6; - Intra-frequency measurement - Impact due to concurrent gap; CATT. 9.3.4, 9.3.5, 9.3.9, 9.3A.4, 9.3A.5, 9.3A.9; - Inter-frequency measurement - Impact due to concurrent gap; Xiaomi. 9.4.2, 9.4.3, [9.4.6], 9.6.2; - Inter-RAT measurement and SFTD - Impact due to concurrent gap; vivo. 9.9.2, 9.9.3, 9.9.4,; - Positioning measurement - Impact due to concurrent gap; Intel. 9.10.2.5, 9.10.3; - CSI-RS based L3 measurement - Impact due to concurrent gap; OPPO. 8.1, 8.1A, 8.5, 8.5A 9.5, 9.5A, 9.8; - Impact to L1 measurement - How to determine the factor P in RLM, BFD, CBD, L1-RSRP… etc. requirements.; MTK. TS36.133; 8.17, 8.19; - Impact on measurement delay under EN-DC and NR-DC - Note: The detail is up to RAN2’s decision on EN-DC, NR-DC; Nokia.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on R17 NR MG enhancements – General issues and Multiple concurrent MGs | MediaTek inc. | Also include CR work split |
| LS on R17 NR MG enhancements – Concurrent MG | Huawei | RAN_2; Cc: RAN_1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2118413 | draftCR on concurrent gaps | Ericsson | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120303 | WF on NR MG enhancements – General issues and Multiple concurrent MGs | MediaTek inc. | Approved |  |
| R4-2120304 | LS on multiple concurrent MGs | Huawei | Approved | RAN_2; Cc: RAN_1 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Zhixun Tang | Zhixun.tang@ericsson.com |
| ZTE | Chenchen Zhang | zhang.chenchen@zte.com.cn |
| Apple | Qiming Li | Li_qiming@apple.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Nokia | Lars Dalsgaard | lars.dalsgaard@nokia.com |
| Qualcomm | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)