3GPP TSG-RAN WG4 Meeting # 101-e 						R4-2120416

Electronic Meeting, 1st – 12th Nov, 2021

Agenda item:			8.9.4

Source:	Moderator (Samsung)

Title:	WF on FR2 HST RRM requirements (part 2)

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FR2 HST RRM part 2 in RAN4 #101 meeting

Way-forward

For one shot large uplink timing adjustment

Moderator note: Highlight part is agreed during RAN4 GTW session

  - It is up to network configuration to enable one shot large uplink timing adjustment mechanism

    - RAN4 will further study if additional flag, e.g., unidirectional flag on top of general FR2 HST scenario flag is needed to enable one shot large uplink timing adjustment

    - RAN4 will further study the network configuration means to disable one shot large uplink timing adjustment.

    - If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed

  - Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq

    - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment.

    - The following options can be considered for triggering condition and network assistance

      - Option 1: No condition except DL timing difference:

        - UE will apply one shot large timing adjustment if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

      - Option 2: TCI switching without network assistance:

        - UE will apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

      - Option 3: TCI switching with network assistance of indication of inter-RRH and UE large DL timing change detection

        - UE will apply one shot large timing adjustment on TCI switching between RRH occasion if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

        - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH.

      - Option 4: TCI switching with network assistance of indication of inter-RRH but without UE large DL timing change detection

        - UE will apply one shot large timing adjustment on TCI switching between RRH occasions

        - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH.

  - Performance degradation and impact to signalling design shall be discussed for above procedures

  - RAN4 will further discuss the accuracy performance and testing issues based on conclusion of above procedures

For RLM/BFD requirements

  - The existing 1280ms duration for known condition is applied for FR2 HST scenario

  - RAN4 will further study the RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios

    - Option 1: following RX beam factor for set-1 and set-2

    - Option 2: The existing RLM/BFD requirements for DRX <=80ms is applied

For CBD requirements

  - No enhancement on CBD requirements for DRX <=80ms

For PSS/SS and intra-freq measurement

  - Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST with the same SMTC periodicity bound of 40ms unless technical issues identified

  - FFS if a different scaling factor is needed for scenario-B with two-side RRH

For L1-RSRP measurement

  - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified

  - Separate sets of requirements for deployment Scenarios A and B

    - For scenario A

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NA)*TSSB) |
| DRX cycle ≤ [80ms] | max(TReport, ceil([1.5]*M*P*NA)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NA = 2

    - For scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle < [80ms] | max(TReport, ceil([1.5]*M*P*NB)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NB = 6