3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120620

Electronic Meeting, 1st  – 12th Nov, 2021

Agenda item:			8.17.1, 8.17.2

Source:	Moderator (Samsung)

Title:                       Email discussion summary for [101-e][313] NR_eIAB

Document for:	Information

Introduction

According to discussion in previous meeting, it’s expected to discuss further on RF impact due to timing enhancement and simultaneous operation of IAB node’s child and parent links within this meeting in RF session. It’s suggested to collect view on each topic in 1st round and seek for consensus to be captured in WF in 2nd round.

Besides the discussion on RF requirement there is another draft CR related to update on FR2-2 definition which is highly dependent on [101-e][128] NR_ext_to_71GHz_Part_1. It’s proposed to collect view in 1st round on specific issue to incorporate this from IAB perspective. But the timeline on when frequency range definition update to be implemented in specifications should be decision in [128] as package in general. And the further work split if any is supposed to be determined and captured in [128] as well.

Topic #1: RF requirement impact due to NR eIAB

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118232 | Samsung | Observation 1: According to RAN1 agreement, the procedure on power control can be applied to allow TX power configured for simultaneous transmission within transmitter capability. Observation 2: RAN1 agreements on power control applicable to mitigate power imbalance issue are dependent on many factors which makes it may not be straightforward to derive related universal minimum requirement other than existing ones. Observation 3: as long as the RAN1 design contain enough margin on top of current baseline for desired PSD range, forward compatible and implementation flexibility could be ensured. Proposal 1: existing EVM requirement should not be impact due to simultaneous transmission. Proposal 2: It’s suggested to take further discussion based option2 with agnostic to implementation solution for simultaneous operation on MT TX/DU TX. |
| R4-2118233 | Samsung | Proposal 1: 3us is agreeable as maximum timing error between intra-node IAB-DU and IAB-MT in case# 6 timing operation. Proposal 2: Timing error between parent IAB-DU and child node IAB-DU transmission is the same as 3us and there is no specification impact for this. Proposal 3: Timing case#7 has no RF requirement impact on IAB. |
| R4-2118436 | ZTE Corporation | Proposal 1: The MT RX/DU RX simultaneous operation in FDM mode based on proper power control, which can ensure such operation mode will not be assumed to exceed legacy requirement of ICS. Proposal 2: Intra-system minimum requirements can be a baseline or start point for case 2, and option 2 is preferred for case 1. |
| R4-2118437 | ZTE Corporation | Observation 1: Parent node will not control UL Tx timing of its child node when the child node operates in Case 6 timing. Observation 2: Te is selected to guarantee the UL Rx of the parent node when child node operates in Case 6 timing. Proposal 1: The following option can be considered for Te determination. Option 1: Define Te for one SCS which may be called reference SCS and Te can be scaled base on u. Option 2: Define an absolute Te which is smaller than 3us without considering SCS. |
| R4-2118736 | Nokia, Nokia Shanghai Bell | Observation 1: Isolation between beams can be expected to be guaranteed to be at most 13 dB in FR2, assuming beams are pointing in clearly different directions. Observation 2: Same 13 dB isolation can be assumed for FR1. Observation 3: Implementation specific methods are available for both FR1 and FR2 to improve the isolation. Observation 4: FDM of IAB-MT and IAB-DU transmissions may impact RF performance, especially if IAB-MT transmission is power controlled. Observation 5: With 13 dB beam isolation, different power levels of IAB-DU and IAB-MT transmissions are not problematic for signal quality. This may not be the case when transmission power capabilities are different. Proposal 1: Specify exception for relative ACLR so that emission level is relative to higher power transmission out of IAB-MT and IAB-DU output power capabilities, at maximum output power. Proposal 2: For simultaneous IAB-MT and IAB-DU transmission in FDM manner, define EVM requirement only for the higher power transmission. |
| R4-2118912 | Nokia, Nokia Shaghai Bell | Proposal 1: IAB timing error between IAB-DU and IAB-MT is needed for timing case #6 to ensure IAB will provide satisfactory performance in real-life deployment. Proposal 2: Consider IAB timing error between IAB-DU and IAB-MT as the average frame timing difference between any two transmissions on IAB-DU and IAB-MT on different transmit antenna connectors or different physical antenna ports. Proposal 3: Consider 3us and/or cyclic prefix length of largest supported SCS for IAB timing error requirement between IAB-MT and IAB-DU transmissions for both SDM and FDM operation when case #6 timing is used. Proposal 4: Specify clearly new test configuration(s) and test model(s) to verify IAB timing error between IAB-DU and IAB-MT transmission for both SDM and FDM operation when case #6 timing is used. |
| R4-2118979 | Ericsson | Observation#1: Inter-beam isolation depend on the azimuth and elevation angel separation. Observation-2: There is no issue for the parent IAB-DU receiving of child IAB-MT in the presence of the child IAB-DU transmission. Observation-3: There is no issue for the NR UE receiving of child IAB-MT in the presence of the child IAB-MT transmission. Observation-4: The unbalanced EIRP/PSD power difference reduce the inter-beam isolation and it should be implementation specific on how many dB the IAB product allowed to tolerate a certain SNR degradation caused by unbalanced EIRP/PSD power. Observation-5: The inter-beam isolation principle is the same irrespective the same or different panel. Proposal-#1: Introduce the new declaration for the IAB-MT and IAB-DU carrier power for the simultaneous transmission. Proposal-#2: Follow the NR BS approach on the multiple carrier operation for IAB-MT and IAB-DU simultaneous transmission. |
| R4-2118980 | Ericsson | - The TAE between IAB-MT and IAB-DU in more general terms is the TAE between an IAB-MT transmit in downlink time slot in one IAB node and IAB-DU simultaneously transmit in DL in another co-located IAB node. - Parent IAB-DU does not need to be aware about the TAE between its DL timing and the DL timing of child IAB-DU for Case-6 timing operation. - For the case of child IAB-MT synchronizing with co-located child IAB-DU, Parent IAB-DU needs to be aware about the TAE between its DL timing and the DL timing of child IAB-DU for case#6 timing operation. so the correct setting of the receiving timing on parent IAB-DU will be possible - Parent IAB-DU set its receiving timing differently depending on the child IAB-MT synchronization implementation. - Proposal-1: For shared hardware architecture, the parent IAB node should tolerate the maximum 3 us timing error uncertainty between its child IAB node and its own DL timing. Observation-5: Additional tolerance on TAE between child IAB-MT and parent IAB-DU DL timing would be needed if TAE requirement is set on the child IAB-MT and its co-located IAB-DU. Proposal#2: Send LS to RAN1 asking if it is feasible for parent IAB to tolerate the 3 us time error between child IAB-MT and parent IAB-DU DL timing with below LS wording: RAN4 has made working assumption that there could be maximum 3 us time uncertainty between the received IAB-MT timing and expected IAB-MT timing due to the cell phase synchronization accuracy is 3 us between the TDD cell (and thus between the parent IAB-DU DL timing and child IAB-DU DL timing). The extend cyclic prefix could be configured on child IAB-MT so parent IAB could tolerate such time error for 60kHz SCS on IAB-MT receiving in DL time slot but it would cost additional system overhead of 18% and thus RAN4 would like to ask RAN1 if parent IAB node can tolerate the 3us time uncertainty without introducing additional system overhead and if it could be implementation specific. Proposal#3: IAB-MT when transmit simultaneously with co-located IAB-DU, the transmit timing should follow the TS 38.133 cell phase sync requirement between the different cell. |

Open issues summary

Sub-topic 1-1: Simultaneous operation

In last RAN4 meeting we have below WF regarding NR eIAB simultaneous operation. In this meeting four contributions provide views regarding the remaining issue

Simultaneous operation of IAB-node’s child and parent links by SDM

RAN4 RF specification impact due to Simultaneous MT TX/DU TX for FDM operation - No RF core requirement impact for MT TX/DU TX simultaneous operation in FDM mode in point of view of coexistence between IAB and existing NR network - There is no need for additional coexistence simulation. - No additional effort is needed on RF impact except issues listed below - FFS on intra-node (i.e. transmissions from the same node interfering each other) interference considering MT/DU simultaneous transmission operation with unbalanced transmitting power with below cases: Case 1: MT and DU using the same antenna panel Case 2: MT and DU using different antenna panes Case 3: other possibility is not excluded. - Based on the investigation of the intra-node (i.e. transmissions from the same node interfering each other) interference, RAN4 could decide whether exception on unwanted emission or restriction on scenario can be addressed in core spec, conformance spec or TR with below options - Option 1: exception on unwanted emission: EVM, relative and absolute ACLR are not applied for power controlled link - Option 2: restriction on scenario: FDM operation with shared beam case is assumed/considered for the same class and/or similar power capability between IAB-DU and IAB-MT only in RAN4 spec. - Option 3: other options are not excluded. - Further study on conformance testing detail on this case is not precluded in perf. part such as testability, test coverage and test configuration

Issue 1-1-1: RAN4 RF specification impact due to Simultaneous MT TX/DU TX with imbalance power

- Proposals: Companies are encouraged to provide the comments/view on below options

  - Option 1: Follow the NR BS approach on the multiple carrier operation for IAB-MT and IAB-DU simultaneous transmission and to introduce dedicated declaration on simultaneous TX carrier power for MT TX/DU TX (R4-2118979;R4-2118436_for case2 )

  - Option 2: Exception allowed for relative ACLR and EVM for FDM case(R4-2118736)

  - Option 3: scenario exclusion (R4-2118232; R4-2118436_for case 1)

- Recommended WF

  - TBA

Sub-topic 1-2: timing enhancement

In Aug RAN4 meeting we have agreement regarding timing enhancement on case#6 in R4-2115645. In this meeting four contributions provide views regarding the open issues on:

- Timing error between IAB-DU and IAB-MT transmission within one node for timing case#6

- Timing error between parent IAB-DU and child node IAB-DU transmission

Besides that timing case#7 the last WF agreed in May meeting as below. One contribution provides further proposal on this case.

Timing case#7

Agreement: No RF requirement impact identified at least for IAB node which supports timing case#7 by separated RF chains between its own MT and DU. For IAB node supports timing case#7 with shared RF chain solution, regarding RX power imbalance no LS to RAN1 needed. Note: the RX power imbalance is merged in discussion on Simultaneous operation of IAB-node’s child and parent links by FDM.

Issue 1-2-1: Timing error between own MT TX and DU TX for case#6

- Proposals: Companies are encouraged to provide the comments/view on below options

  - Option 1: No need to define RF requirement since cell phase synchronization covers this(R4-2118980)

  - Option 2: Timing error needs to be defined with below Alts on limitation

    - Alt1: 3us(R4-2118233, R4-2118912)

    - Alt2:  cyclic prefix length of largest supported SCS(R4-2118912, R4-2118437)

    - Alt3:  value as SCS dependent(R4-2118437)

- Recommended WF

  - TBA

Issue 1-2-2: Timing error between parent IAB DU and Child IAB -DU

- Proposals: Companies are encouraged to provide the comments/view on below option

  - Option 1: the parent IAB node could tolerate the maximum 3 us timing error uncertainty between its child IAB-DU and its own DL timing. And no RAN4 specification impact identified.

  - Option 2: time error uncertainty between Child and parent IAB DL timing to be tolerated by parent IAB node is related to conclusion on timing error between child node  MT TX and DU TX for timing case#6

- Recommended WF

  - TBA

Issue 1-2-3: Necessity on LS to RAN1

- Proposals: Companies are encouraged to provide the comments/view on below option

  - Option 1: Send LS to RAN1 asking if it is feasible for parent IAB to tolerate the 3 us time error between child IAB-MT and parent IAB-DU DL timing(R4-2118980)

- Recommended WF

  - TBA

Issue 1-2-4: finalization on case#7 timing

- Proposals: Companies are encouraged to provide the comments/view on below option

  - Option 1: no IAB RF impact due to enhancement to support case#7 timing (R4-2118233)

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 1-1-1: simultaneous operation

| Company | Comments |
| --- | --- |
| Nokia | Option 2 from RAN4#100-e WF means FDM DU and MT carriers need to be in TCs together. RAN4 needs to specify a test case where both MT and DU simultaneous transmission is configured and then it is for further discussion if the power difference is declared by manufacturer and which requirements are measured (e.g. measure EVM only from the higher power transmission while both transmissions are on-going simultaneously). WF in RAN4#100-e agreed on no specification impact from simultaneous DU RX/MT RX. This discussion should take place in performance part where possible test case for simultaneous DU RX/MT RX can be discussed. |
| Ericsson | Option 1. As NR BS type 1-O and 2-O also has multiple carrier configuration simultaneously so we think maybe we could follow NR BS approach here also. Performance wise, there should be no EVM degradation if inter-beam isolation is good enough as shown in our paper. This is however deployment specific. |
| ZTE | Option 1 for Case 2 (different panel). We think the existing requirement such as Intra-system minimum requirements can be discussed for Case 2. And for Case 1(same panel), if there is no requirement for reference the power imbalance needs to be determined not to interfere the weaker one. |
| Samsung | First of all we prefer to agree the principle on how to incorporate the case of simultaneous transmission on IAB-MT and IAB-DU with imbalance power level with implementation agnostic way. Secondly, we share the similar view as Ericsson on EVM core requirement for which the impact should be avoided. Consequently, we are OK to option3 as proponent. However, we could be compromise to option 1 for which we believe the existing RF requirement should be applied for simultaneous operation case with permission for IAB with such capability to declare the power imbalance condition to be verified to meet all related requirement. |

Sub topic 1-2-1:  timing case#6 timing error of intra-node

| Company | Comments |
| --- | --- |
| Nokia | We disagree with option 1. RAN4 agreed in previous meeting that “Timing error between IAB-DU and IAB-MT simultaneous transmission is to be considered as new dedicated RAN4 requirement to be decided to capture it in which section in TS38.174”. The timing uncertainty depends on the error between IAB-DU and IAB-MT, but the absolute time difference is also dependent on propagation delay. Thus we support option 2, with regards to alternative on value, we proposed in our Tdoc R4-2118912 that either Alt1 or Alt2 can be agreed, specified and later on tested. |
| Ericsson | Issue 1-2-1: Option 1. As our paper indicated, the additional TAE between MT and co-located DU will add on to the existing 3us which make parent IAB-DU more difficult to tolerate. Above all, the IAB-MT may transmit in DL time slot in separate sector with IAB-DU transmission, which make it even difficult to set the requirement as there is no IAB-DU timing to compare with. |
| ZTE | We think the Rx timing of parent node should be ensured when we discuss Te between IAB-DU Tx and co-located IAB-MT Tx. And we think it better to discuss UL Rx timing of parent node together with issue 1-2-2. |
| Samsung | Option 1 can be interpreted as zero timing error assumed for IAB under timing case#6 or 3us assumed between donor IAB-DU TX and child IAB-MT TX. The former one is not agreed since it’s not realistic. And the later one could be somehow equal to define a timing error between intra-node IAB-MT and IAB-DU. With this the IAB-MT timing aligned with own IAB-DU could be verified. Hence we prefer the option 2. Regarding whether RX timing of parent node should be ensured by child node MT UL timing it may dependent on whether parent can make own adjustment for such reception case by case or solely rely on child IAB side. |

Sub topic 1-2-2: Timing error between parent IAB DU and Child IAB –DU

| Company | Comments |
| --- | --- |
| Nokia | Support option 2, this should be related to timing case #6. |
| Ericsson | Issue 1-2-2: Option 2. One thing to note is that any number x will add on to the 3us and parent IAB need tolerate total 3us + x us I |
| ZTE | The timing error uncertainty of 3us is not reasonable for all cases, take 120 kHz SCS as an example, the duration of CP is about 0.6 us which is much less than 3us. Also as pointed in issue 1-2-1, the time error uncertainty between IAB and its parent node will also put constraint on the Te between IAB-DU Tx and co-located IAB-MT Tx |
| Samsung | We believe option 1 is the assumption since rel-16 on timing case#1. And the mechanism on timing case#1 can be applied for timing case#6 directly. |

Sub topic 1-2-3: necessity on RAN1 LS

| Company | Comments |
| --- | --- |
| Nokia | LS to RAN1 is unnecessary, RAN4 is the expert group on performance topics. We have also proposed the option to tighten the requirement to match with CP length. |
| Ericsson | Option 1. The case#6 timing is feature specified in RAN1 group and whatever requirement we set on timing, maybe it is good to check with RAN1. Parent tolerance issue is key issue relating to the timing aspect. Any time difference between IAB-MT and its co-located DU will add more uncertainty acc. To below picture. |
| ZTE | We think it better to discuss it within RAN4 to conclude a value we think reasonable. |
| Samsung | No LS needed |

Sub topic 1-2-4: finalization on timing case#7

| Company | Comments |
| --- | --- |
| Nokia | OK with option 1. |
| Ericsson | Option 1 is ok for now, seems also depending how case#6 timing discussion goes. |
| ZTE | We want to clarify that the ICS can be the requirement for power imbalance for simultaneous Rx. |
| Samsung | Ok for option 1 as proponent |

CRs/TPs comments collection

N/A

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Candidate options: According to feedback only one company supports option 2 as requirement exception solution. The other 3 companies prefer the solution(s) to avoid core RF requirement impact, as the proposals can be recognized as equivalent as avoid the scenario which violates existing requirement. Among 3 companies, 2 companies support the solution should be applicable for all possible implementations, 1 company wants different solution dependent on antenna implementation. It’s believed that at least baseline principle of applicable requirement in condition of simultaneous transmission of IAB node should be clearly stated in specification.[Moderator note: it’s believed that for simultaneous RX case which agreed as no RF requirement impact the similar statement on RF requirement applicability should be addressed as well.] With respect to this, two questions should be considered: Question 1: whether it’s agreeable to relax existing RF requirement for individual interface for simultaneous transmission condition? Question 2: whether the final solution to be agreed should be applied as implementation agnostic way? Based on common understanding to those, whether the new declaration is to be introduced for IAB-MT and IAB-DU power in condition of simultaneous transmission operation for IAB node with such capability can be discussed further. Recommendations for 2nd round: Discuss further in 2nd round |
| Issue 1-2-1: Timing error between own MT TX and DU TX for case#6 | Candidate options: Three companies support to define timing error for intra-node IAB-MT and IAB-DU in timing case#6.One companies still have concern to define this. Hence the option 2 should be majority view which is already agreed WF in last meeting. On the other hand, there is different understanding on whether parent IAB-DU reception can be adjusted for case#6 reception or not, which has impact on the what limit should be defined for such timing error Recommendations for 2nd round: Discuss further in 2nd round |
| Issue 1-2-2: Timing error between parent IAB DU and Child IAB –DU | Candidate options: 3companies believe this related issue 1-2-1 discussion. And 1 company believe option1 is valid for both timing case#1 and timing case#6. Recommendations for 2nd round: To clarify issue 1-2-1 first in 2nd round. |
| Issue 1-2-3: Necessity on LS to RAN1 | Candidate options: Three companies believe related discussion should be within RAN4 and no LS needed Recommendations for 2nd round: Prioritize the discussion on issue 1-2-1/2 and defer the discussion on LS. . |
| Issue 1-2-4: finalization on case#7 timing | Tentative agreements: No objection on candidate proposal. Hence it should be agreeable. And there is clarification request regarding the RX requirement to be verified for power imbalance case which can be discussed further on how to reflect it as general statement on requirement applicability. Recommendations for 2nd round: Capture the agreement in WF on timing case#7. Moderator note: The needed specification update on requirement applicability should follow similar principle of outcome of issue 1-1 if any. |

CRs/TPs

N/A

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #2: FR2-2 implication on IAB specification

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2119148 | Huawei | This Draft CR provides implementation of FR2-1 and FR2-2 frequency sub-ranges for the General parts of the TS 38.174 specification, as per discussion in NR_ext_to_71GHz-Core WI. |

Open issues summary

Related WF is agreed in R4-2114986 in which the IAB TS38.174 is not covered for FR2 definition update. However, since the frequency range definition is also captured in IAB specification, the IAB specification impact should be considered.

Originally in Rel-16 IAB discussion, it was agreed to support all existing Rel-15 FR2 bands defined in NR specification. Later, it was extended to all FR2, i.e., FR2-1 according to latest agreement, bands defined in TS38.104. Since the FR2 is extended to 71GHz, it should be determined that whether FR2-2 band(s) is to be supported by IAB by default as FR2-1 bands or not.

Sub-topic 2: FR2 definition update for IAB

Sub-topic description: Companies are encouraged to provide the comments/view on applicability of FR2-2 to IAB node. It should be noted that RF requirement enhancement due to support of newly defined FR2-2 band is not captured in latest WID for Rel-17 enhancement to NR IAB. Hence, it’s suggested to exclude this if no specific request identified during this meeting from moderator perspective.

Issue 2-1: FR2-2 applicability to IAB

- Proposals from moderator:

  - Proposal 1: Update on RF2 definition is needed to TS38.174 with WI code of NR_ext_to_71GHz-Core

  - Proposal 2: operating band for FR2-2 is not applied for rel-17 IAB according to current WID on NR eIAB.

- Recommended WF

  - Company’s view on above proposals is welcome

  - Company’s view on other aspect regarding FR2-2 impact on IAB is welcome.

Companies views’ collection for 1st round

Open issues

Issue 2-1: FR2-2 applicability to IAB

| Company | Comments |
| --- | --- |
| Nokia | This CR should be implemented only when FR2-2 specification is introduced. It should be also reviewed that term FR2 is not used in the specification as otherwise this CR accidentally extends those requirements to cover also 52.6 - 71 GHz. Thus, FR2 should be replaced with FR2-1 in the specification where necessary. We agree with proposal 2. |
| Ericsson | We should align with NR BS specification in this aspect. But we donot have FR2-2 frequency for IAB so maybe for now , option 2 is ok. |
| Samsung | As the this issue is under discussion in thread[128], it seems majority view should be OK to both proposal 1 and proposal 2 as: -include TS38.174 for specification impact due to FR2 definition extension only -FR2-2 is not supported for IAB in rel-17. => based on above feedback, it should be OK to confirm FR2-2 to be excluded in TS38.174 with only update on 5.5-1 with the definition. However, there are many places refer to FR2 requirement defined in TS38.104 and TS38.101-2 in TS38.174. It’s proposed to make it clear in specification with generic approach. The simple way as indicated by Nokia we tend to agree is to replace “FR2” by “FR2-1”. |

CRs/TPs comments collection

| CR/TP number | CRs/TPs comments collection |
| --- | --- |
| R4-2119148 | Samsung: according to comment shared under issue 2-1 we believe the draft CR should be treated in WI of Extending current NR operation to 71GHz as package with CRs to other impacted specification. Except the update on FR2 definition, the original FR2 should be modified as FR2-1. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2 | Tentative agreements: According to feedback below proposal should be agreeable: - Frequency range definition in TS38.174 will be impacted. - FR2-2 operating band is not supported by IAB in Rel-17. Candidate options: There are also comments could be taken into account in further discussion on final CR: - Comment to postpone the implementation of FR2 definition when corresponding specification is ready. - Comment to ensure the FR2 in spec will be updated as FR2-1 to align with the original intention as frequency range in 24.25-52.6GHz. Recommendations for 2nd round: It is suggested to terminate discussion on RF2 definition update for IAB spec in this thread as there is common understanding as FR2-2 is not supported for IAB node at least in current release. Above tentative agreements and handling on draft CR in future discussion to be captured in thread [128] 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2119148 | Based on 1st round of comments collection, it’s recommended to be postponed as this is not impacted due to Rel-17 IAB enhancement. |

Discussion on 2nd round (if applicable)

N/A

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RF specification impact for eIAB | Samsung |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2118232 | Simultaneous operation on IAB-node’s child and parent links | Samsung | Noted |  |
| R4-2118233 | Timing enhancement on Rel-17 IAB | Samsung | Noted |  |
| R4-2118436 | Discussion on power imbalance for simultaneous operation of IAB | ZTE Corporation | Noted |  |
| R4-2118437 | Discussion on timing issues for simultaneous operation of IAB | ZTE Corporation | Noted |  |
| R4-2118736 | RF requirements for simultaneous IAB-MT and IAB-DU operation | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2118912 | On IAB timing error | Nokia, Nokia Shaghai Bell | Noted |  |
| R4-2118979 | RF impact analysis for simultaneous DU and MT operation | Ericsson | Noted |  |
| R4-2118980 | IAB MT /DU case 6 timing | Ericsson | Noted |  |
| R4-2118232 | Draft CR to TS 38.174:implementation of FR2-1 and FR2-2 frequency sub-ranges for the General parts of the specification | Huawei | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Bartlomiej Golebiowski | bartlomiej.golebiowski@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)