3GPP TSG-RAN WG4 Meeting #101-e	R4-2120675

# Electronic meeting, November 1st – 12th, 2021

Source:		Ericsson

Title:			NTN - WF on Rx RF requirements for satellite access node

Agenda item:		8.13.3

Document for:		Approval

# Rx RF requirements for satellite access node

## REFSENS

Sub-topic 1-3 Rx requirements for Satellite BS

Issue 1-3-1: FRC

- GTW Nov. 2nd:

Agreement: taking option 1 as starting point.

  - Option 1: The FRCs specified in TS 38.104 shall be re-used to specify the satellite node Rx requirements. Specifically the following FRCs and DMRS pattern is re-used.

|  | BS Channel BW | Subcarrier spacing | Number of PRBS |
| --- | --- | --- | --- |
| G-FR1-A1-1 | 5 MHz | 15 kHz | 25 |
| G-FR1-A1-2 | 5 MHz | 30 kHz | 11 |
| G-FR1-A1-3 | 10 MHz | 60 kHz | 11 |
| G-FR1-A1-4 | 20 MHz | 15 kHz | 106 |
| G-FR1-A1-5 | 20 MHz | 30 kHz | 51 |
| G-FR1-A1-6 | 20 MHz | 60 kHz | 24 |

Issue 1-3-2: Noise figure

- GTW Nov. 2nd:

Agreements: Option 1:

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

Issue 1-3-3: Implementation margin

- GTW Nov. 2nd discussion:

Agreement: Reusing margin from TN as [2dB].

## Dynamic Range

WF: Further check the following proposals for next meeting:

    - define IoT level as 18dBc for LEO600KM NTN BS;

    - define IoT level as 12dBc for LEO1200KM NTN BS;

    - not to define Rx dynamic range requirements for GEO NTN BS;

## ACS

Agreement: Wait for co-existence simulation conclusion.

## Blocking

WF: FFS, waiting also fo co-existence simulation conclusion.

## Co-location

WF: FFS, companies are encouraged to check if the presence of multiple wireless systems and their impact for next meeting.

## Rx intermodulation

Agreement: This requirement is not needed considering the scenario of satellite BS.

## In-channel selectivity

WF: Further investigate whether to keep ICS requirement for NTN BS considering the calibration results, where the CL CDF is within 5dB difference.

## Rx requirements for type 1-O

WF: FFS

# References

- R4-2117382, Further discussion on Rx requirements for NTN BS, CATT

- R4-2118161, NTN - Satellite Node - Rx requirements, Erisson

- R4-2119142, TP to TR 38.863: node class, RF RX (6.2), Huawei

- R4-2119206, Discussion on Rx requirements of satellite gNB, ZTE Corporation