3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2120703

Electronic Meeting, November 1-12, 2021

Agenda item:			8.9.5.3

Source:	Nokia, Nokia Shanghai Bell, Samsung

Title:	WF on BS demodulation requirement for FR2 HST

Document for:	Approval

# Background

This contribution is to capture the agreements and the directions for the future studies from the email discussion for the topic of Rel-17 NR HST FR2 BS demodulation requirements in RAN4 #101, with the email thread identifier “[101-e][321] NR_HST_FR2_Demod”.

This e-mail thread captures the e-mail discussions for the following sub-agenda items for FR2 HST RRM

- 8.9.5.3	BS demodulation requirements

  - 8.9.5.3.1	PUSCH requirements

  - 8.9.5.3.2	PUSCH with UL timing adjustment requirements

  - 8.9.5.3.3	PRACH requirements

The detailed discussions can be found in the e-mail discussion summary [R4-2120613], Topic#3.

# WF on PUSCH requirements

## Scope of PUSCH requirement with Uni-directional RRH scenario A/B and Bi-directional RRH scenario B

Agreements: - Define only one set of requirements for PUSCH - Define HST FR2 model based on Bi-directional scenario-B model. - No test applicability rules are needed

## RS configurations for PUSCH and UL timing adjustment

Agreement: - Define requirement with 1 DMRS + PT_RS (L=1, K=2) configuration - Define FRC for 1 DMRS + PT_RS (L=1, K=2) - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS+ PT_RS (L=1, K=2) and 3 DMRS + PT_RS (L=1, K=2) - Define FRC for 2 DMRS + PT_RS (L=1, K=2) - Define FRC for 3 DMRS + PT_RS (L=1, K=2) Way forward - FFS, definition of test applicability rules based on BS manufacturer declaration

## FOC implementation and MCS

Agreements: - FOC method is up to BS implementation - Do not capture assumptions on FOC scheme in specification - Define PUSCH requirements with MCS(s) between MCS16-20 that is/are feasible and testable. Way forward: - Companies are encouraged to further check and align the results because a significant span among the initial submitted results is observed ([R4-2117591]). - FFS, the ways to agree on the requirements if the significant span in the results persists - Companies are encouraged to identify suitable MCS(s) for requirements between MCS16-20 in the next meeting

## Phase noise modelling

Agreements: - Phase noise impact can be included in the impairment results, but it is left up to companies - No explicit phase noise modelling in the alignment result

# WF on UL timing adjustment requirements

Agreements: - GTW agreement on channel model for UL timing adjustment requirement: - Follow same approach as existing PUSCH UL timing adjustment requirement demodulation requirement specified in TS 38.104 - On test parameters for timing offset: - Use A=1.25us, Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements - On Timing different between moving UE and stationary UE: - Apply the timing difference between moving UE and stationary UE as Δτ-(TA-31)x16*8Tc for UL timing adjustment requirement - On SRS transmission configuration: - The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2

# WF on PRACH requirements

Agreements: - One timing offset configuration - Configure the maximum timing offset (i.e., the end of the tested range) in FR2 HST testing setup equal to 4.8us - Value of Timing offset: 0 - Step of Timing offset increase: 0.48us

# Simulation assumption for PUSCH requirement

| Parameter |  | Value |
| --- | --- | --- |
| Channel Model |  | HST FR2 mode based on Bi-directional scenario B, the details can be referred as R4-2115726 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos0, Pos1, Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | 50MHz and 200MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

# Simulation assumption for UL timing adjustment requirement

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

# Simulation assumption for PRACH requirement

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Timing offset | 4.8us with TO range 0:0.48:4.8 |

# CR work split for FR2 HST BS demod (Information)

The companies have indicated their interest to be responsible for (draft)CRs to the following section of TS 38.104 and TS 38.101-4

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Huawei , [Samsung] |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia, [Samsung] |
| 4.6 | Manufacturer's declarations |  |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements |  |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, [Samsung] |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | [Ericsson] |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Editor (Nokia, Nokia Shanghai Bell) | Dmitry Petrov | dmitry.a.petrov@nokia-bell-labs.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Samsung | Yunchuan Yang | yc0301.yang@samsung.com |
| Intel | Artyom Putilin | artyom.putilin@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)