3GPP TSG-RAN WG4 Meeting # 101-e 													R4-2120738

Electronic Meeting, 1st – 12th November, 2021

Agenda item:			8.12.2.1, 8.12.2.2

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [101-e][323] NR_perf_enh2_Demod_Part2

Document for:	Information

# Introduction

The scope of this email thread is the following topics of Rel-17 Further enhancement on NR demodulation performance WI:

- MMSE-IRC receiver for inter-cell interference

- MMSE-IRC receiver for intra-cell inter-user interference

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Discussion on open issues

  - Collection of comments on MMSE-IRC MU-MIMO TPs

  - Collection of comments on CR work split

- 2nd round:

  - Discussion on open issues

  - Revision of MMSE-IRC MU-MIMO TPs

  - WFs preparation

# Topic #1: MMSE-IRC receiver for inter-cell interference – Demodulation requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117432 | Apple | Proposal #1: Only consider synchronized network for ICI requirements. Proposal #2: Configure non-overlapping SSBs on target and interfering cells. Proposal #3: Down select the MCS based on operating SINR > -6dB and SNR > INR. Proposal #4: Only consider homogeneous network deployment for requirements with inter-cell interference. Observation #1: Performance with 1 cell and 2 cells interference with MMSE-IRC is very comparable. Proposal #5: Do not consider INR values of 13.91 and 3.34 dB for requirements definition with ICI. Proposal #6: Define requirements with 1 interference cell. Proposal #7: Define requirements with INR 3.1 dB in case of 1 interference cell. Observation #2: With 1 cell and 2 cells interference with option 1 INR levels, SINR at 70% max TP with 4RX and MCS 4 is < -6dB. Observation #3: Performance gain with MMSE-IRC if slightly better with TDLA channel compared to TDLC. Proposal #8: Define PDSCH demod requirements for ICI with MCS 13 and TDLA channel model. |
| R4-2117642 | Nokia, Nokia Shanghai Bell | Proposal 1: Do not deviate from the synchronized frequency offsets and limit the frequency offset of each basestation to within ±0.1 ppm of carrier frequency Observation 1: IRC receiver gain is only slightly larger with TDLA30-10 channel model and requires slightly lower SNR to achieve 70% throughput Proposal 2: Choose channel model that better fits the propagation conditions at cell edge (delay spread, etc.), i.e. TDLC300-100. Observation 2: Option 3 interference profile gives the highest gain for the IRC receiver. Proposal 3: Define requirements based on option 3 in addition to baseline option 1. Observation 3: Gain of IRC receiver in HetNet deployment is similar to that observed with homogenous deployment. Observation 4: TDLA30-10 channel model gives higher gain of IRC receiver with HetNet deployment using option 1. Proposal 4: Not to define requirements for HetNet deployment for 2 Rx antenna configuration. Proposal 5: In case HetNet is agreed we prefer TDLA30-10 channel model. |
| R4-2117735 | CMCC | Proposal 1: Include FDD asynchronized network type in FR1. Proposal 2: Reuse LTE time and frequency offset configuration as the starting point. Proposal 3: In the case of similar PDSCH performance between Option 1 and Option 2, configure all SSBs (serving cell and interference cell(s)) in the same time/frequency resources. Proposal 4: At least cover MCS 13 for all MIMO configuration. Proposal 5: Include HetNet deployment in R17 inter-cell interference suppressing work item. Proposal 6: For HetNet, use INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. |
| R4-2117736 | CMCC | Observation 1: Performance gain from MMSE-IRC can be observed for both MCS 4 and MCS 13. Observation 2: For MCS 4 configuration, the SNR at 70% throughput is lower than INR. |
| R4-2117994 | NTT DOCOMO, INC. | Proposal 1: For SSB configuration, use Option 1 configuration ( i.e. serving cell and interference cells SSBs are in the same time/frequency resources). |
| R4-2117998 | Intel Corporation | Proposal 1: Use the following common test parameters for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario: - SSB configuration: - 2 interference cells modelling: Serving cell SSB and first dominant interference cell SSB are in the different time/frequency resources - 1 interference cell modelling: Different SSB configurations for Demodulation and CSI tests - Propagation condition: TDLC300-100 Proposal 2: Further discuss the following options on interference modelling for different deployment assumptions: - Option 1: Use same deployment assumptions (Homogeneous or HetNet) for Demodulation and CSI requirements - Option 2: Use different deployment assumptions for Demodulation and CSI requirements (i.e. HetNet for Demodulation and Homogeneous for CSI or vice versa). Proposal 3: Consider INRs 7.77 and 2.29 dB (2 interference cells) or INR 5.49 dB (1 interference cell) for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario in case Homogeneous deployment assumptions will be used Observation #1: If 1 interference cell is explicitly modelled then the contribution of the total receive signal power from dominant interference cell to the total receive signal power from all interference cells is 50% or less for the 50% of user. Observation #2: In 2 interference cells are explicitly modelled then the contribution of the total receive signal power from dominant interference cells to the total receive signal power from all interference cells is 73% or less for the 50% of user Proposal 4: Use explicit modelling of 2 interference cells for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario. Proposal 5: Use MCS 13 for the definition of MMSE-IRC PDSCH demodulation requirements for inter-cell interference scenario. Observation #3: MMSE-IRC performance benefits are rather same for different channel models. Observation #4: MMSE-IRC performance benefits for scenario with INRs 5.43 and -1.50 dB, 2 Rx and 16QAM are less or equal to 1 dB. Observation #5: MMSE-IRC performance benefits for scenario with INR 3.10 dB, 2 Rx and 16QAM are less than 1 dB. Observation #6: MMSE-IRC performance benefits are higher than 1 dB for all considered scenarios with INR values: 7.77 and 2.29 dB; 5.49 dB; 13.91 and 3.34 dB; 10.59 dB. |
| R4-2118406 | Ericsson | Observation 1: Async. test case was introduced in LTE IRC WI. Observation 2: All SSBs (serving cell and interference cell(s)) in the same time/frequency resources had already agreed in RAN4 to define RRM requirements. Observation 3: MIB decoding requirement is defined based on multiple trials other than one shot. Proposal 1: RAN4 to consider FDD asynchronous test case with interf. cell whose time offset relative to serving cell is 0.5 slot + half a OFDM symbol. Proposal 2: RAN4 to define the test cases with the same SSB time/frequency resources for interfering inter-cells. Proposal 3: RAN4 to define the test cases for both TDLA30-10 and TDLC300-100 based on the following down-selection rule. - Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC - such as TDLA30 for heterogeneous scenario and TDLC300 for homogeneous scenario Proposal 4: RAN4 to select MCS based on the following rules. - Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC - SINR is not lower than -6 dB - Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB) Proposal 5: RAN4 can evaluate both homogeneous and heterogenous scenarios with different interference power settings. - Consider INRs 5.43 dB and -1.5 dB in homogeneous scenario - Consider INRs 11.39 dB and 5.45 dB in heterogenous scenario |
| R4-2118408 | Ericsson | Observation 1: Compared with MRC, IRC receiver can have about 2dB or even more gain in 70% of maximum throughput SNR point. Observation 2: IRC receiver have gains in both QPSK and 16QAM scenarios. Observation 3: IRC receiver have gains in both TDLA and TDLC channels. Observation 4: The higher the inference from the interferer cell(s) the higher the gain with IRC receiver. |
| R4-2118861 | China Telecom | Proposal 1: For the requirement definition for MMSE-IRC for inter-cell interference for synchronized network, reuse the same time and frequency offsets for CRS-IM: - Time offset: The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively - Frequency shift: The serving cell is 300 Hz and -100 Hz for interfering cell 1 and cell 2 respectively. Observation 1: For async networks, UE will suffer from completely different interference within one slot. In such case, UE should implement different IRC processing in the slot. Proposal 2: Additionally include async FDD scenario for MMSE-IRC receiver for suppressing inter-cell interference. Proposal 3: For SSB, support Option 1 to use same time/frequency resource for different cells. Proposal 4: Use TDLC300-100 channel model for the target and interference cells. Proposal 5: Cover 16QAM and QPSK in different new test cases to balance the test coverage and test case number. Proposal 6: For QPSK, use higher MCS index with higher code rate instead of MCS 4 to have higher SINR point, i.e., MCS 9. Proposal 7: For the INR values for HomNet scenario, use INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell. Proposal 8: We support to cover both 1 cell and 2 cells. Proposal 9: Additionally cover HetNet scenarios and the interference profile from LTE NAICS can be used for initial simulation, i.e., INRs 11.39 and 5.45 dB (DIPs -1.23 and -7.16 dB). Proposal 10: Both considering the test coverage and not duplicate the IRC test with purely increasing the interference power, ok to cover HomNet and HetNet scenarios in different test cases with different antenna configuration, e.g., define HomNet IRC receiving requirements for 2Rx test case and define HetNet receiving requirements for 4Rx test case. Proposal 11: If there are companies’ simulation results show that the interfered TRS will impact the UE synchronizing, and leads to PDSCH demodulation performance degradation, we are ok to add such clarification only in the simulation assumptions. Proposal 12: The UE demodulation requirements under inter-cell interference should be released independent from Rel-15. |
| R4-2119043 | Huawei, HiSilicon | Proposal 1: Not consider asynchronized scenario. Observation 1: Modelling two interference cells will lead to larger INR level and the performance gain for IRC processing will be more obvious. Observation 2: There is only 1.1dB performance gain for one interference cell with INR=3.1dB, MCS 13, TDLC and 2RX Proposal 2: Use two modelled interference cells. Observation 3: In RRM cell handover test, the power difference between interference and serving cell is 3dB for known target cell and 0dB for unknown target cell. Proposal 3: Consider option 1, option 2 and option 4 as conditions for MCS and INR value selection. Proposal 4: Use INR1=7.77dB, INR =2.29dB for INR values. Proposal 5: Consider no overlapping PBCH resources for serving cell and interference cells. Proposal 6: Consider TDLC300-100 Proposal 7: Use SNR for performance measurement and INR for interference profile configuration. Proposal 8: Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition |
| R4-2119044 | Huawei, HiSilicon | Simulation results for PDSCH demodulation requirements for inter-cell MMSE-IRC receiver |
| R4-2119403 | MediaTek inc. | Proposal 1: Suggest to set serving cell SSB and interference cell(s) SSB(s) in the different time/frequency resources. If it is agreed to set SSB(s) of serving cell and interference cell(s)) in the same time/frequency resources, the configuration should be properly chosen to avoid the case that the performance of SSB is the bottleneck. Observation 1: From the simulation results, the gain of MMSE-IRC over MMSE-IRC is larger than 1dB for all cases. Observation 2: From the simulation results, the corresponding SINR for “SNR for 70% of Max T-put” are smaller than -6 dB in many cases. Proposal 2: Suggest to apply SINR < -6dB as the first criterion for down selection. |
| R4-2119545 | Qualcomm Incorporated | Proposal 1: Only consider 10MHz CBW for 15kHz SCS and 40MHz CBW for 30kHz SCS. Proposal 2: Serving cell SSB and interference cell(s) SSB(s) are specified in the different time/frequency resources. Proposal 3: Do not define test cases for HetNet deployment. Proposal 4: Only consider 1 interfering cell for defining the requirements. Proposal 5: Use same INR levels as in LTE for defining Intercell Interference requirements. Proposal 6: Use MCS13 for defining Intercell Interference requirements. Proposal 7: Use TDLA30-10 propagation condition for defining Intercell Interference requirements. |

## Open issues summary

### Sub-topic 1-1: Common test parameters

Issue 1-1-1: Network type

- Background

  - Synchronized for FDD and TDD

  - FFS asynchronized for FDD

  - Interested companies are encouraged to provide view on test setup for asynchronized FDD scenarios (especially, time and frequency offsets)

- Proposals

  - Option 1 (Apple, Huawei): Only consider synchronized network

  - Option 2 (Nokia): Do not deviate from the synchronized frequency offsets and limit the frequency offset of each basestation to within ±0.1 ppm of carrier frequency

  - Option 3 (CMCC, Ericsson, China Telecom): Include FDD asynchronized network type in FR1.

    - Option 3A (CMCC): Reuse LTE time and frequency offset configuration as the starting point (i.e. 0.33 ms and 0 Hz for cell 1 and 0.67 ms and 0 Hz for cell 1)

    - Option 3B (Ericsson): Time offset relative to serving cell is 0.5 slot + half a OFDM symbol.

- Recommended WF

  - Collect detailed companies views on pros and cons to include asynchronized scenario in the scope

Issue 1-1-2: SSB configuration

- Background

  - Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

  - Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

  - Interested companies can check the PBCH performance for both options

    - Use simulation assumptions from Section 5.4 of TS 38.101-4 for serving cell

    - Use the following SSB configuration for Option 2

      - SSB of serving cell is transmitted in PRB 0~19 in first occasion

      - For case with 1 interference cell (if agreed), SSB of interference cell is transmitted in PRB 0~19 in second occasion.

      - For case with 2 interference cell, SSB of interference cell 1 is transmitted in PRB 20~39 in first occasion, SSB of interference cell 2 is transmitted in PRB 0~19 in second occasion.

    - Other simulation assumptions are FFS

- Proposals

  - Option 1 (CMCC, DOCOMO, Ericsson, China Telecom): All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

    - CMCC: In the case of similar PDSCH performance between Option 1 and Option 2

    - MediaTek: If it is agreed to set SSB(s) of serving cell and interference cell(s)) in the same time/frequency resources, the configuration should be properly chosen to avoid the case that the performance of SSB is the bottleneck.

  - Option 2 (Apple, Huawei, MediaTek, Qualcomm): Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

  - Option 3 (Intel):

    - 2 interference cells modelling: Serving cell SSB and first dominant interference cell SSB are in the different time/frequency resources

    - 1 interference cell modelling: Different SSB configurations for Demodulation and CSI tests

- Recommended WF

  - Collect views on options above.

  - Check whether any impact on PDSCH testing can be expected in case Option 1 will be used.

Issue 1-1-3: Propagation condition

- Background

  - Consider TDLA30-10 and TDLC300-100 channel models for evaluation purpose and select only one for requirements definition

  - Interested companies are encourage to provide views on down-selection criteria

- Proposals

  - Option 1 (Apple, Ericsson for HetNet, Qualcomm): TDLA30-10

    - Nokia: In case HetNet is agreed we prefer TDLA30-10 channel model.

  - Option 2 (Nokia for HomoNet, Intel, Ericsson for HomoNet, China Telecom, Huawei): TDLC300-100

- Recommended WF

  - Check whether we can consider TDLC300-100 for scenario with homogenies network

  - Further discuss channel model for heterogenous network is case it will be agreed for requirements definition

### Sub-topic 1-2: Target PDSCH parameters for scenario 1

Issue 1-2-1: MCS

- Background

  - Down selection between MCS 4 and MCS 13 based on results for agreed INR values based on the following criteria

    - Option 1: Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC

    - Option 2: SINR is not lower than -6 dB

    - Option 3: Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB)

    - Option 4: SNR > INR

    - Other options are not precluded

    - Using of multiple options is not precluded

  - Interested companies are encouraged to provide views on criteria for MCS down selection

- Proposals on criteria

  - Option 1 (Apple): Down select the MCS based on operating SINR > -6dB and SNR > INR.

  - Option 2 (Ericsson): Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC; SINR is not lower than -6 dB; Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB)

  - Option 3 (Huawei): Consider option 1, option 2 and option 4 as conditions for MCS and INR value selection.

  - Option 4 (MediaTek): Suggest to apply SINR > -6dB as the first criterion for down selection.

- Proposals on MCS

  - Option 1 (Apple, CMCC, Intel, Qualcomm): MCS 13

  - Option 2 (China Telecom): Cover 16QAM and QPSK in different new test cases to balance the test coverage and test case number. For QPSK, use higher MCS index with higher code rate instead of MCS 4 to have higher SINR point, i.e., MCS 9.

- Recommended WF

  - Criteria: Check whether we can consider at least Option 1 (Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC) and Option 2 (SINR is not lower than -6 dB) and further discuss Option 3 (SNR-INR > -3dB) and Option 4 (SNR > INR)

  - MCS:

    - Check whether we can use at least MCS 13 and further discuss proposal from China Telecom.

    - Check companies’ views on list of PDSCH tests cases

### Sub-topic 1-3: Interference model for scenario 1

Issue 1-3-1: Deployment

- Background

  - Consider Homogeneous deployment assumptions

  - FFS whether for consider HetNet deployment assumptions

- Proposals

  - Option 1 (Apple, Qualcomm): Only consider homogeneous network deployment

  - Option 2 (Nokia): Not to define requirements for HetNet deployment for 2 Rx antenna configuration

  - Option 3 (CMCC, Ericsson, China Telecom): Include HetNet deployment

    - China Telecom: Both considering the test coverage and not duplicate the IRC test with purely increasing the interference power, ok to cover HomNet and HetNet scenarios in different test cases with different antenna configuration, e.g., define HomNet IRC receiving requirements for 2Rx test case and define HetNet receiving requirements for 4Rx test case.

  - Option 4 (Intel): Further discuss the following options on interference modelling for different deployment assumptions:

    - Option 1: Use same deployment assumptions (Homogeneous or HetNet) for Demodulation and CSI requirements

    - Option 2: Use different deployment assumptions for Demodulation and CSI requirements (i.e. HetNet for Demodulation and Homogeneous for CSI or vice versa).

- Recommended WF

  - Collect views on options above

Issue 1-3-2: INR values for Homogeneous deployment assumptions

- Background

  - Further discuss the following options for PDSCH requirements definition for synchronous network

    - Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

    - Option 2: INRs 7.77 and 2.29 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Option 3: INRs 13.91 and 3.34 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Other options are not precluded

  - FFS assumptions for asynchronous network

- Proposals

  - Option 1 (Apple): Do not consider INR values of 13.91 and 3.34 dB for requirements definition with ICI. Define requirements with INR 3.1 dB in case of 1 interference cell.

  - Option 2 (Nokia): INRs 13.91 and 3.34 dB

  - Option 3 (Intel): Consider INRs 7.77 and 2.29 dB (2 interference cells) or INR 5.49 dB (1 interference cell)

  - Option 4 (Ericsson, China Telecom, Qualcomm): INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

  - Option 5 (Huawei): Use INR1=7.77dB, INR =2.29dB for INR values.

- Recommended WF

  - Collect views on options above

Issue 1-3-3: INR values for HetNet deployment assumptions (if HetNet will be agreed for Issue 1-3-1)

- Background

  - Option 1: INRs 11.39 and 5.45 dB (DIPs -1.23 and -7.16 dB)

  - Other options are not precluded

- Proposals

  - Option 1 (CMCC): For HetNet, use INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

  - Option 2 (Ericsson, China Telecom): INRs 11.39 and 5.45 dB

- Recommended WF

  - Check whether we can consider INRs 11.39 and 5.45 dB (2 interference cells) and INR 4.84 dB (1 interference cell) as baseline in case requirements for HetNet will be agreed

Issue 1-3-4: Number of explicitly modeled interference cells

- Background

  - Companies are encouraged to check performance with 1 and 2 interference cells for initial simulations

  - Further discuss the assumptions for requirements definition

- Proposals

  - Option 1 (Apple, Qualcomm): Define requirements with 1 interference cell.

  - Option 2 (Intel, Huawei): Use explicit modelling of 2 interference cells

  - Option 3 (China Telecom): Cover both 1 cell and 2 cells.

- Recommended WF

  - Collect views on options above

Issue 1-3-5: Time and frequency offsets for synchronized network

- Proposals

  - Option 1 (China Telecom):

    - Time offset: The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively

    - Frequency shift: The serving cell is 300 Hz and -100 Hz for interfering cell 1 and cell 2 respectively.

- Recommended WF

  - Collect views on option above

### Sub-topic 1-4: Receiver assumptions

Issue 1-4-1: TRS-IC/IM

- Background

  - Further discuss whether to add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

  - No such clarification will be added in the TS 38.101-4

  - Interested companies are encouraged to check with performance for scenarios with and without TRS-IC/IM

- Proposals

  - Option 1 (China Telecom): If there are companies’ simulation results show that the interfered TRS will impact the UE synchronizing, and leads to PDSCH demodulation performance degradation, we are ok to add such clarification only in the simulation assumptions.

  - Option 2 (Huawei): Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

- Recommended WF

  - Check views on options above

### Sub-topic 1-5: Release independency of requirements

Issue 1-5-1: Release independency of Demodulation requirements

- Background

  - RAN4 discuss whether the UE demodulation with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption

- Proposals

  - Option 1 (China Telecom): The UE demodulation requirements under inter-cell interference should be released independent from Rel-15.

- Recommended WF

  - Follow previous meeting agreement

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 1-1: Common test parameters

| Company | Comments |
| --- | --- |
| Apple | Issue 1-1-1: Network type Option 1. We would like to understand the motivation for introducing requirements for async network. We would like to understand if the UE is aware of the time offset for any advanced processing-we would like clarification from China Telecom on the following: Observation 1: For async networks, UE will suffer from completely different interference within one slot. In such case, UE should implement different IRC processing in the slot. Also, are we assuming the same simulation parameters for interferer as sync case for parameters other than time offset? Issue 1-1-2: SSB configuration We support option 2. We don’t see why the SSBs have to be overlapping. We are defining PDSCH demod requirements in inter-cell interference with MMSE-IRC. We are not sure if the effect on performance if any can be captured in link levels simulations for PDSCH demod, but in actual testing might lead some issues with tracking loops. Issue 1-1-3: Propagation condition We support option 1. Based on companies’ preference of defining requirements where we see more measurable gain with MMSE_IRC over MMSE, TDLA seems to be a better choice as we observe more gain with IRC in TDLA than TDLC. We would like to understand the reason for choosing TDLC. |
| CMCC | Issue 1-1-1: Network type We prefer Option 3. Option 3A can be the starting point (0.33 ms and 0 Hz for cell 1 and 0.67 ms and 0 Hz for cell 2), we are open to have more discuss about specific value. Issue 1-1-2: SSB configuration Option 1, which is also common configuration in our practical network. Issue 1-1-3: Propagation condition We propose to use TDLC300-100 for HomoNet, TDLA30-10 for HetNet. Because the ISD in HetNet is smaller than HomoNet, and usually UE in serving cell (such as pico) is in low mobility. In this way, the test coverage can be guaranteed, and the number of test cases will not increase. |
| China Telecom | Issue 1-1-1: Network type We support to cover FDD async network. FDD sync cannot always be guaranteed, and under async scenario, if the time offset between target and interference cells is as large as 0.33ms, UE will observe different interference on the different DMRS symbols, which will impact the MMSE-IRC processing including Rnn estimation. Therefore, similar with the test case design in LTE, we think a test for MMSE-IRC under FDD async is necessary to verify the PDSCH demodulation performance. As CMCC proposed, we are ok to take the LTE’s assumption for FDD async as a start point, i.e., Option 3A is our preference: Interference Cell 1; Interference Cell 2. Time offset relative to target cell; 0.33(ms); 0.67(ms). Frequency shift relative to target cell; 0(Hz); 0(Hz). Clarification to Apple: In our initial thinking, we are ok to reuse the simulation assumptions for FDD sync scenario other than time/frequency offset. We have agreed to use random precoding for the interference cell whose precoder will be updated per slot, therefore, the interference power at the target cell will be different in time domain. Issue 1-1-2: SSB configuration Support Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources. We do not think option 2 is a practical SSB configuration in the real network. And in LTE, tracking RS such as PSS SSS are naturally collided, which did not cause big problem. There are companies provided simulation results for PBCH demodulation performance degradation due to ICI. However, it is still not clear whether PBCH demodulation performance degradation will truly impact the time/frequency tracking accuracy and leads to PDSCH performance degradation. To try to address Huawei’s concern, we are ok to use option 1 plus an additional note that ‘Assume no SSB-IM/IC is considered for inter-cell MMSE-IRC requirements definition’ in the chairman note or the simulation assumption, which is similar with issue 1-4-1. Issue 1-1-3: Propagation condition Support Option 2 TDLC300-100 to better verify the Rnn estimation accuracy under frequency selective condition. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1: Network type We are fine with options 3A or 3B for the time offset. Concerning frequency offset, we can compromise to 3A, if companies confirm that there is no practical performance difference between 0Hz and 0.1ppm fc. Issue 1-1-3: Propagation Conditions We agree with proposed WF for homogenous network Option 1 in heterogenous networks gives slightly higher gain and mirrors the propagation conditions more appropriately. |
| Intel | Issue 1-1-1: Network type Before further discussion on this topic, we would like to check whether asynchronous network is practical scenarios and what is the benefit to consider such scenario. Based on our understanding, in case all cells, for example, are used GNSS as reference sync source, we don’t expect to have the asynchronous network. Issue 1-1-2: SSB configuration Based on our analysis, PBCH decoding for Option 1 can have impact on PDSCH testing only for scenario with QPSK modulation. Same time, we think that it is not the practical use case where PDSCH performance is better than PBCH performance. Therefore, in case 16QAM modulation is used for PDSCH, we don’t expect any issue with testing for any of SSB mapping option. To cover different SSB mapping options, we suggest to consider the following assumption for scenario with 2 interference cell: - Serving cell SSB and first dominant interference cell SSB are in the different time/frequency resources - Serving cell SSB and second dominant interference cell SSB are in the same time/frequency resources Also, in case HetNet will be considered, we can consider Option 1 for HomoNet and Option 2 for HetNet. Issue 1-1-3: Propagation condition We support to consider TDLC300-100 for scenario with homogenies network Based on our analysis, the MMSE-IRC gains over MMSE-MRC are same for different channel models. SNR operating point for scenarios with TDL-C channel model is higher in comparison to TDL-A channel model. Therefore, it allows to consider more scenarios in case we consider the criteria from Issue 1-2-1 |
| Qualcomm | Issue 1-1-1: Network type Prefer Option 1. Issue 1-1-2: SSB configuration Prefer option 2. In general, system will have better performance if UE can at least get one reference signal which it can rely on for timing/frequency tracking. We already agreed to have colliding TRS. So, we prefer to have non-colliding SSB. |
| MediaTek | Issue 1-1-1: Network type Prefer Option 1. Issue 1-1-2: SSB configuration Prefer Option 2. We are now defining the PDSCH requirement under the scenario of inter-cell interference. It should prevent the case that UE fails the test due to the poor performance of PBCH or poor tracking accuracy rather than improper interference handling. Hence, we prefer to have non-overlapping SSB for serving cell and interference cells. Also, we would like to raise a question that do we need to also consider the SSB configuration for CQI testing? In the CQI testing, UE also need to perform PDSCH demodulation under the scenario of inter-cell interference. |
| Docomo | Issue 1-1-2: SSB configuration We support Option 1 In LTE, PBCHs between serving cell and interference cells are collided, but it did not cause big problem for real network. Therefore, we think that UE can maintain the acceptable performance of SSBs even if the SSBs from interfering cells are fully collided with serving cell. In addition, we don’t think that it is practical to configure serving cell and all interference cells SSB in different time/frequency resources. In intel’s comments, in case 16QAM modulation for PDSCH, PBCH performance degradation don’t cause big problem for PDSCH performance. So, in order to avoid PDSCH performance degradation, it is acceptable for us to use 16QAM modulation for PDSCH in this topic. |
| ZTE | Issue 1-1-1: Network type Option 1. Are we going to define two sets of performance requirements for sync and async scenarios respectively? Issue 1-1-2: SSB configuration Given the NR design on SSB, we are not sure why SSBs have to be overlapping while the design itself can avoid this? Issue 1-1-3: Propagation condition The listed two options are not really in parallel options. We are fine with : TDLA30-10 for HetNet and TDLC300-100 for HomoNet. |
| Huawei | Issue 1-1-1: Network type Support Option 1. Asynchronized FDD will lead to different interference within one slot as Apple pointed out due to the random PMI selection for interference cells. We share the same views with Intel that there is no motivation to introduce such scenario since more practical deployment is synchronized. @ CTC: Should we study more advanced IRC processing for asynchronized? Based on our understanding, it may be difficult for UE to know the boundary of different interference within one slot. LTE uses TM3 for interference cell which causes same interference within one slot, so it is not a big issue for UE to handle the asynchronized scenario with legacy IRC. But for NR, it is related to uneven interference processing that is still under discussion for Rel-18 performance enhancement, it is out of the scope of this WI. Issue 1-1-2: SSB configuration Support Option 2. Based on our evaluations, PBCH colliding has impact on PDSCH from the following two aspects: - PBCH colliding will cause poor PBCH(MIB) performance leading to cell access failure. As our simulations show, target SNR of PBCH is higher than that of PDSCH even if MCS 13 is used without combination - PSS/SSS and PBCH DMRS colliding will cause poor accuracy of time/frequency tracking which will degrade the PDSCH performance. The evaluation of such impact has been captured in our contribution RP-212486 and we observed up to 1.8dB for 64QAM and 2.1dB for 256QAM performance gain with SSB-IM under the low network load compared to without IM receiver, it means the interference from the SSB of neighbour cells in the real network due to the colliding SSB configuration has serious impact on PDSCH performance. Considering that the intention of this WI is to verify the performance with pure PDSCH colliding configuration, it is better to setup a clear test environment and avoid any other possible impacts on the performance degradation. Issue 1-1-3: Propagation condition We support only to consider TDLC300-100. The performance gain of IRC over MRC is similar for most cases between TDLA and TDLC, but our preference is to consider the worst propagation condition. We agree with CTC that TDLC will be more suitable to verify the correct Ruu calculation. |

#### Sub-topic 1-2: Target PDSCH parameters for scenario 1

| Company | Comments |
| --- | --- |
| Apple | Issue 1-2-1: MCS For criteria: We prefer to select based on SINR > -6dB and SNR>INR as the first criteria. The measurable / testable performance benefit doesn’t seem very critical to us given that MMSE-IRC is baseline receiver since Rel-15. For MCS: Performance results show that with MCS4 SINR is very low and not suitable to define requirements, so MCS 13 seems suitable. How many testcases do we plan to define to be able to over 16QAM and QPSK in different tests? We should also ensure that there are no PDCCH decoding errors at the chosen operating SINR/SNR with ICI. |
| CMCC | Issue 1-2-1: MCS For the proposals on MCS, both Option 1 and Option 2 are ok for us. Option 2 may need further evaluation. |
| China Telecom | Issue 1-2-1: MCS We have proposed to use higher MCS index within QPSK to try to solve the low SINR point issue. Our thinking was that both QPSK and 16QAM should be covered in the test since both can be scheduled under ICI scenario. Same time, considering the majority’s view, we can give our compromise to only use MCS13 in the MMSE-IRC demodulation test. |
| Nokia, Nokia Shanghai Bell | Issue 1-2-1: MCS Concerning criteria: Option 1 and 2 seems reasonable. |
| Intel | Issue 1-2-1: MCS Based on our understanding, Option 1 is the main criteria for MCS and Test setup selection, because it allows to verify that MMSE-IRC is used by UE in scenario with inter-cell interference (this is the main purpose of these tests). As for Option 2 (SINR is not lower than -6 dB), it is rather reasonable criteria to avoid any RRM related issues. As for Option 3 (SNR-INR > -3dB) or 4 (SNR > INR), we don’t see any technical justification to consider on of these criteria, because the handover procedure is rather flexible in NR and doesn’t restrict the difference between SNR and INR value. 3 dB threshold is used for intra-frequency NR cells for IDLE mode, but demodulation requirements are applied in RRC_CONNECTED mode. Also, in case we want to compare the values which will be measured by UE, probably, it is better to check the difference between S/(I1+I2+N) and I1/(S+I2+N). However, we think that using of Option 1 and Option 2 criteria is sufficient. As for MCS, based on previous meeting agreement, we think that it is sufficient to consider single MCS for MMSE-IRC requirements to have sufficient test coverage. Therefore, we support to consider MCS 13. |
| Qualcomm | Issue 1-2-1: MCS Based on the comments so far, it seems that companies can agree to using only MCS13. We prefer to define only one test rather than try to cover different MCS because it won’t change the UE receiver algorithm. |
| MediaTek | Issue 1-2-1: MCS For MCS, we support to use MCS13 as most of SINR for MCS4 is too low from the simulation results. Also, we think it is sufficient to use only MCS13 to define test case. |
| Docomo | Issue 1-2-1: MCS For MCS, our preference is Option 1. |
| ZTE | Issue 1-2-1: MCS Both options are fine. |
| Huawei | Issue 1-2-1: MCS Based on our simulation, MCS 13 should be considered since target SINR for MCS4 is lower than -6dB for most cases in our simulations that is too low. For option 3 and option 4 of criteria, we agree that it is flexible for network to inform UE the handover indication and threshold is not a fixed value. But to be more practical, it is better to set good conditions for serving cell to avoid potential cell handover. For RRM handover test specified in A.6.3.1.1 and A.6.3.1.2 of TS 38.133, the SNR =8dB and INR=8dB for known cell and SNR=8dB and INR=11dB for unknown cell are used as conditions which can be considered as the typical configurations for real network. To be safer and more practical, we propose to use SNR>INR1 as criteria. @ Intel: Thanks for your analysis, based on our understanding, we just only compare the ‘S’ and ‘I1’ in case SNR>INR1 is used, since the relation of ‘S’ and ‘I1’ is equivalent to that of S/(I1+I2+N) and I1/(S+I2+N). |

#### Sub-topic 1-3: Interference model for scenario 1

| Company | Comments |
| --- | --- |
| Apple | Issue 1-3-1: Deployment We prefer to only cover HomNet to define requirements. For verifying performance with MMSE-IRC for PDSCH demod, the only difference is different interference levels between HomNet and HetNet. We think HomNet assumption would be better suited for NR deployment. We don’t think different interference levels changes any processing in the receiver for MMSE-IRC, of course if there is more interference, there is more rejection with IRC. We prefer to have same settings for 2RX and 4RX for ease of test setup and not introduce requirements with different interference levels or deployment assumption. The interference levels for PDSCH demod and CSI requirements are different in our understanding. Not sure if INR for CQI reporting is from HomNet or HetNet deployment. Issue 1-3-2: INR values for Homogeneous deployment assumptions Thanks Intel for providing the INR values for 1 cell interference for different INR levels with 2 interferers. We are fine with option 3 or option 5. Issue 1-3-4: Number of explicitly modeled interference cells We propose to have 1 interference cell. UE processing is not impacted by more number of interferers. At the end its an interference level seen at the UE. It would simplify test setup if we use 1 interference cell. Issue 1-3-5: Time and frequency offsets for synchronized network Option 1 is okay for 2 interference cells and FDD. For 1 interference cell – time offset: 3 us for 15KHz, 1us for TDD 30KHz; Freq offset: 300 Hz for both TDD and FDD. |
| CMCC | Issue 1-3-1: Deployment As we mentioned in previous meetings, HetNet is widely deployed and we have already observed Inter-cell interference in this scenario. We also support CTC’s suggestion that cover different configuration under HomoNet and HetNet such as Rx configuration, propagation condition and so on. Issue 1-3-2: INR values for Homogeneous deployment assumptions We think the following options can be considered for Homogeneous: - Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell - Option 2: INRs 7.77 and 2.29 dB in case of 2 interference cells and FFS in case of 1 interference cell In order to guarantee the test coverage without introducing additional test cases, we propose to use Option 1 for synchronous network and use Option 2 for async network. Issue 1-3-3: INR values for HetNet deployment assumptions We support Option 1. Issue 1-3-4: Number of explicitly modeled interference cells We prefer Option 3. If companies concern with test burden or workload, we propose 2 interference cell for HomoNet and 1 interference for HetNet. Issue 1-3-5: Time and frequency offsets for synchronized network Ok with Option 1. |
| China Telecom | Issue 1-3-1: Deployment We support to cover HetNet scenarios. UE will suffer larger interference power level under HetNet scenarios. Considering the test coverage and try not to increase the test case number with different interference power, we are also ok to cover HomNet and HetNet tests for test cases with different Rx number, for example, using HomNet for 2Rx and HetNet in 4Rx cases. However, we prefer not to cover HetNet by CSI cases. We have agreed to use static channel model for the CQI test, and we do not think it will be sufficient to verify MMSE-IRC process. Issue 1-3-2: INR values for Homogeneous deployment assumptions Under the proposed interference power level in option 1, companies have shown ~1dB performance gain with MMSE-IRC. We think such performance gain can be sufficient for requirement definition. Therefore, we still prefer Option 1 to reuse the same interference power level in LTE test. Issue 1-3-3: INR values for HetNet deployment assumptions (if HetNet will be agreed for Issue 1-3-1) INRs 11.39 and 5.45 dB in case of 2 interference cells is fine for us. Need further check on the 1 interference cell scenario. Issue 1-3-4: Number of explicitly modeled interference cells Support Option 3 to cover 1 cell and 2 cells. We are ok to cover different cell number within different test cases for 4Rx and 2Rx to control the test case number. Issue 1-3-5: Time and frequency offsets for synchronized network |
| Nokia, Nokia Shanghai Bell | Issue 1-3-1: Deployment We can agree to option 2 and 3. Option 2 and 3 can be combined to decide on the deployment. i.e., 4 Rx for heterogenous while 2Rx and 4 Rx or 2 Rx only for homogenous Issue 1-3-2: INR values for Homogeneous deployment assumptions We can compromise to Option 4. Issue 1-3-3: INR values for HetNet deployment assumptions (if HetNet will be agreed for Issue 1-3-1) We support the recommended WF. Issue 1-3-4: Number of explicitly modeled interference cells IRC gain with 2 interferer cells is higher. So 2 interferer cells should be included. Hence we support Option 2 or 3. |
| Intel | Issue 1-3-1: Deployment We are fine to include the HetNet assumptions as additional test. However, we prefer to have same test setup for 2Rx and 4Rx UEs as the typical procedure for definition of demodulation requirements. One of potential option we can consider the following set of requirements: - HomoNet: All SSBs are in the same time/frequency resources, TDL-C channel model, 2 and 4 Rx - HetNet: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources, TDL-A channel model, 2 and 4 Rx To reduce the test effort, we can define the applicability rule that verification for one of scenarios is sufficient. Issue 1-3-2: INR values for Homogeneous deployment assumptions We support Option 3, because such INR values can be observed in NR deployment based on our system level analysis. These values are used in the LTE NAICS, for which the system level analysis was done in later release in comparison to system level analysis for LTE MMSE-IRC. Therefore, we assume that LTE NAICS values are closer to typical NR deployment rather than LTE MMSE-IRC values. Also, based on our NR system level analysis we don’t see any scenarios with INR values 5.43 and -1.50 dB. Issue 1-3-3: INR values for HetNet deployment assumptions We are fine to consider Option 1 as baseline. However, we first need to take a look in results from companies to understand whether we can fulfill the criteria from Issue 1-2-1 (at least, MMSE-IRC gain and SINR > -6 dB). In case not, we probably need to consider another values. Issue 1-3-4: Number of explicitly modeled interference cells We support the Option 2, because it allows to achieve trade off between test complexity and practical conditions. For most of LTE requirements with inter-cell interference, 2 cells modelling is considered. Issue 1-3-5: Time and frequency offsets for synchronized network Option 1 is fine for us |
| Qualcomm | Issue 1-3-1: Deployment We have similar comment as Apple. WE prefer to only consider homogeneous deployment since UE implementation is agnostic to the deployment. Also, we prefer to have same setup between 2Rx and 4Rx because we usually have an applicability rule between those 2 sets. Issue 1-3-2: INR values for Homogeneous deployment assumptions We prefer to use LTE values, i.e., Option 4. Issue 1-3-4: Number of explicitly modeled interference cells We prefer to have only 1 cell for simplified test setup. As we increase the number of interfering cells, combined interference will start losing its structure and will get closer to AWGN. While, with 1 interference cell, we are definitely testing the structured interference. So, we don’t see the advantage of having multiple cells in the test setup. Issue 1-3-5: Time and frequency offsets for synchronized network Same comment as Apple. Nov 4th Updated: Other: It seems that there is a large span in simulation results for intercell interference case. So, we would like to clarify a few assumptions with the companies. Can other companies confirm if below assumptions were made for their simulations? - Interfering cell has 2Tx and random precoding with PRB granularity of 2. - Interfering cell has same channel model as for serving cell and it is independently generated for each cell. Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant. |
| MediaTek | Issue 1-3-1: Deployment Prefer to define homogeneous deployment only. We share the same view as Qualcomm that IRC processing is agnostic to the deployment. Issue 1-3-2: INR values for Homogeneous deployment assumptions We are OK with Option 5. Issue 1-3-4: Number of explicitly modeled interference cells It is sufficient to consider only 1 interference cell or 2 interference cells. We slightly prefer Option 2 which consider only 2 interference cells. |
| Docomo | Issue 1-3-1: Deployment We slightly prefer to Option 1. And also we would like to clarify the applicability rule proposed by China Telecom in Option 3 at first. Considering TS 38.101-4 Table 5.1.1.2-1, if UE support 4Rx, UE can skip 2Rx requirement. Based on our understanding about Table 5.1.1.2-1, in case China Telecom’s proposal is introduced, we think that 4Rx UEs pass only one scenario (e.g. HetNet) test. Therefore, RAN4 cannot verify both scenario. Since we think that HomNet is also typical deployment, we should avoid such a situation. In addition, we prefer to have same settings for 2Rx and 4Rx UEs. Based on above, if HetNet is introduced, RAN4 should introduce ”HomNet requirement with 2Rx/4Rx” and “HetNet requirement with 2Rx/4Rx.” Issue 1-3-4: Number of explicitly modeled interference cells Our preference is Option 3, but Option 2 is acceptable for us. Issue 1-3-5: Time and frequency offsets for synchronized network We are fine with Option 1. |
| Huawei | Issue 1-3-1: Deployment We share the same views with QC and Apple, UE is agnostic to the specific deployment that doesn’t affect IRC processing. Furthermore, the INR values for HetNet is so high that more effort should be spent to select a suitable MCS to meet target SNR>INR1 Issue 1-3-2: INR values for Homogeneous deployment assumptions We prefer option 5. From the simulation results with MCS13 in our contributions we have following observations: - For case with 3.1dB, 1 interference cell, the performance difference is only 1dB for one interference cell with MCS 13, TDLC and 2RX. - For cases with INR1=5.43dB, INR2=-1.5dB, the target SNR is always higher INR, but the performance gain for IRC is only 1.3dB~2.2dB. - For cases with INR1= 7.77dB, INR2= 2.29dB, only cases with TDLA30-10, 4RX and TDLC300-100,4RX has the observation that target SNR is lower than INR, but the difference is small and can be ignored by addition of margin. Meanwhile, the performance gain for IRC is considerable (2.7dB~5.7dB). - For cases with INR1= 13.91dB, INR2=3.34dB, the target SNR for most cases is far lower than INR. From the above observations, INR1=7.77dB, INR2=2.29dB is the most suitable configuration among all the options. Issue 1-3-4: Number of explicitly modeled interference cells We prefer to consider 2 interference cells since 1 interference cell can’t bring enough performance gain for IRC over MRC based on the given INR value from our simulation results. Issue 1-3-5: Time and frequency offsets for synchronized network Option 1 is OK |

#### Sub-topic 1-4: Receiver assumptions

| Company | Comments |
| --- | --- |
| Apple | We support option 1. For the agreed simulation results we would like to see performance impact with and without TRS-IM/IC before we add such clarification. |
| CMCC | Option 1. Share similar view with Apple. |
| China Telecom | To address Huawei’s concern, we are ok to include such clarification in the simulation assumption. |
| Nokia, Nokia Shanghai Bell | We support option 1. We agree with the reasoning behind option 1. |
| Intel | Based on our analysis from RAN4 #99 meeting (R4-2109198), we didn’t observe significant performance difference in case Serving cell TRS colliding with TRS interference and Serving cell TRS with interference free for different INR conditions. Therefore, we don’t expect big impact of TRS-IC/IM on PDSCH performance. Same time, we are fine to add such clarification in the simulation assumptions. |
| Qualcomm | Ok with Option 1. |
| Ericsson | Option 1. Same view as Apple. |
| MediaTek | OK with Option 1. |
| Docomo | We are fine with Option 1. |
| Huawei | We share the logic behind Option 1 and welcome interesting companies to do evaluation to check the performance difference with and without TRS colliding configuration. Currently based on our evaluation results shown in RP-212486 and we observed about 3.2dB performance gain for 64QAM with TRS-IC under the low network load compared to without TRS-IC receiver, it means the interference from the TRS of neighbour cells due to the colliding TRS configuration has serious impact on PDSCH performance, so we prefer to add such clarification. Also as several companies pointed out for colliding SSB configuration, it is better to preclude all other potential impact on PDSCH performance degradation and focus on the IRC for interference handling due to colliding PDSCH. It should prevent the case that UE fails the test due to the poor FO/TO tracking accuracy rather than improper interference handling. |

#### Sub-topic 1-5: Release independency of requirements

| Company | Comments |
| --- | --- |
| Apple | We support to follow previous meeting agreement. |
| China Telecom | Fine to follow previous meeting agreement. |
| Qualcomm | Ok with recommended WF. |
| Ericsson | Fine to follow previous meeting agreement. |
| Docomo | We are fine to follow previous meeting agreement. |
| Huawei | Prefer to follow previous meeting agreement. |

## Summary for 1st round

### Open issues

#### Sub-topic 1-1: Common test parameters

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Network type | Tentative agreements: N/A Candidate options: - Option 1 (Apple, Qualcomm, MediaTek, ZTE, Huawei): Only consider synchronized network - Option 2 (CMCC, China Telecom, Nokia): Include FDD asynchronized network type in FR1. - Option 3A (CMCC, China Telecom, Nokia): Reuse LTE time and frequency offset configuration as the starting point (i.e. 0.33 ms and 0 Hz for cell 1 and 0.67 ms and 0 Hz for cell 1) - Option 3B (Ericsson, Nokia): Time offset relative to serving cell is 0.5 slot + half a OFDM symbol. Recommendations for 2nd round: Continue discussion to collect more comments and address the following questions: - Apple: We would like to understand if the UE is aware of the time offset for any advanced processing-we would like clarification from China Telecom on the following: Observation 1: For async networks, UE will suffer from completely different interference within one slot. In such case, UE should implement different IRC processing in the slot. Also, are we assuming the same simulation parameters for interferer as sync case for parameters other than time offset? - China Telecom: In our initial thinking, we are ok to reuse the simulation assumptions for FDD sync scenario other than time/frequency offset. We have agreed to use random precoding for the interference cell whose precoder will be updated per slot, therefore, the interference power at the target cell will be different in time domain. - Intel: We would like to check whether asynchronous network is practical scenarios and what is the benefit to consider such scenario. Based on our understanding, in case all cells, for example, are used GNSS as reference sync source, we don’t expect to have the asynchronous network. - ZTE: Are we going to define two sets of performance requirements for sync and async scenarios respectively? - Huawei: Asynchronized FDD will lead to different interference within one slot as Apple pointed out due to the random PMI selection for interference cells. We share the same views with Intel that there is no motivation to introduce such scenario since more practical deployment is synchronized. @ CTC: Should we study more advanced IRC processing for asynchronized? Based on our understanding, it may be difficult for UE to know the boundary of different interference within one slot. LTE uses TM3 for interference cell which causes same interference within one slot, so it is not a big issue for UE to handle the asynchronized scenario with legacy IRC. But for NR, it is related to uneven interference processing that is still under discussion for Rel-18 performance enhancement, it is out of the scope of this WI. |
| Issue 1-1-2: SSB configuration | Tentative agreements: N/A Candidate options: - Option 1 (CMCC, China Telecom, Docomo): All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources - Option 2 (Apple, Qualcomm, MediaTek, Huawei): Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources - Option 3 (Intel): Serving cell SSB and first dominant interference cell SSB are in the different time/frequency resources. Serving cell SSB and second dominant interference cell SSB are in the same time/frequency resources - Option 4 (Intel): Option 1 for HomoNet and Option 2 for HetNet (in case HetNet will be considered) Recommendations for 2nd round: Continue discussion and take into account comments from 1st round - Apple: We are not sure if the effect on performance if any can be captured in link levels simulations for PDSCH demod, but in actual testing might lead some issues with tracking loops. - CMCC: Common configuration in our practical network. - China Telecom: We do not think option 2 is a practical SSB configuration in the real network. And in LTE, tracking RS such as PSS SSS are naturally collided, which did not cause big problem. There are companies provided simulation results for PBCH demodulation performance degradation due to ICI. However, it is still not clear whether PBCH demodulation performance degradation will truly impact the time/frequency tracking accuracy and leads to PDSCH performance degradation. To try to address Huawei’s concern, we are ok to use option 1 plus an additional note that ‘Assume no SSB-IM/IC is considered for inter-cell MMSE-IRC requirements definition’ in the chairman note or the simulation assumption, which is similar with issue 1-4-1. - Intel: In case 16QAM modulation is used for PDSCH, we don’t expect any issue with testing for any of SSB mapping option. - Qualcomm: In general, system will have better performance if UE can at least get one reference signal which it can rely on for timing/frequency tracking. We already agreed to have colliding TRS. So, we prefer to have non-colliding SSB. - Docomo: In LTE, PBCHs between serving cell and interference cells are collided, but it did not cause big problem for real network. Therefore, we think that UE can maintain the acceptable performance of SSBs even if the SSBs from interfering cells are fully collided with serving cell. In addition, we don’t think that it is practical to configure serving cell and all interference cells SSB in different time/frequency resources. - ZTE: Given the NR design on SSB, we are not sure why SSBs have to be overlapping while the design itself can avoid this? - Huawei: Based on our evaluations, PBCH colliding has impact on PDSCH from the following two aspects: - PBCH colliding will cause poor PBCH(MIB) performance leading to cell access failure. As our simulations show, target SNR of PBCH is higher than that of PDSCH even if MCS 13 is used without combination - PSS/SSS and PBCH DMRS colliding will cause poor accuracy of time/frequency tracking which will degrade the PDSCH performance. The evaluation of such impact has been captured in our contribution RP-212486 and we observed up to 1.8dB for 64QAM and 2.1dB for 256QAM performance gain with SSB-IM under the low network load compared to without IM receiver, it means the interference from the SSB of neighbour cells in the real network due to the colliding SSB configuration has serious impact on PDSCH performance. Considering that the intention of this WI is to verify the performance with pure PDSCH colliding configuration, it is better to setup a clear test environment and avoid any other possible impacts on the performance degradation. |
| Issue 1-1-3: Propagation condition | Tentative agreements: N/A Candidate options: - Option 1 (Apple, CMCC for HetNet, Nokia for HetNet, ZTE for HetNet, [Qualcomm], [Ericsson for HetNet]): TDLA30-10 - Option 2 (CMCC for HomoNet, China Telecom, Nokia for HomoNet, Intel, ZTE for HomoNet, Huawei, [Ericsson for HomoNet]): TDLC300-100 Recommendations for 2nd round: - Check whether there is any technical concern to consider TDLC300-100 for HomoNet assumptions |

#### Sub-topic 1-2: Target PDSCH parameters for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-2-1: MCS | Tentative agreements: MCS 13 Recommendations for 2nd round: Double check whether this agreement is applicable to Sync HomoNet only or same MCS can be used for HetNet and Async scenarios is case introduced |

#### Sub-topic 1-3: Interference model for scenario 1

|  | Status summary |
| --- | --- |
| Issue 1-3-1: Deployment | Tentative agreements: N/A Candidate options: - Option 1 (Apple, Qualcomm, MediaTek, Docomo, Huawei): Only consider homogeneous network deployment - Option 2 (CMCC, China Telecom, Nokia, Intel, [Ericsson]): Include HetNet deployment - CMCC: Cover different configuration under HomoNet and HetNet such as Rx configuration, propagation condition and so on - China Telecom: Cover HomNet and HetNet tests for test cases with different Rx number - Nokia: 4 Rx for heterogenous while 2Rx and 4 Rx or 2 Rx only for homogenous - Intel: Consider the following set of requirements - HomoNet: All SSBs are in the same time/frequency resources, TDL-C channel model, 2 and 4 Rx - HetNet: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources, TDL-A channel model, 2 and 4 Rx - To reduce the test effort, we can define the applicability rule that verification for one of scenarios is sufficient. GTW agreement: Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases: - One test case applied for Homogenous for each duplex mode and 2Rx/4Rx - One test case applied for HetNet for each duplex mode and 2Rx/4Rx - If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode Recommendations for 2nd round: N/A |
| Issue 1-3-2: INR values for Homogeneous deployment assumptions | Tentative agreements: N/A Candidate options: - Option 1 (Intel, Apple): Consider INRs 7.77 and 2.29 dB (2 interference cells) or INR 5.49 dB (1 interference cell) - Option 2 (CMCC for Sync Network, China Telecom, Nokia, Qualcomm, [Ericsson]): INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell - Option 3 (Apple, CMCC for Async Network, MediaTek, Huawei): Use INR1=7.77dB, INR =2.29dB for INR values. Recommendations for 2nd round: Check whether we can keep the following two options for further discussion: - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell |
| Issue 1-3-3: INR values for HetNet deployment assumptions (if HetNet will be agreed for Issue 1-3-1) | Tentative agreements: N/A Candidate options: - Option 1 (CMCC, Nokia, Intel): For HetNet, use INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. - Intel: We first need to take a look in results from companies to understand whether we can fulfill the criteria from Issue 1-2-1 (at least, MMSE-IRC gain and SINR > -6 dB). - Option 2 (China Telecom, [Ericsson]): INRs 11.39 and 5.45 dB Recommendations for 2nd round: Check whether we can consider the following option as baseline: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell. |
| Issue 1-3-4: Number of explicitly modeled interference cells | Tentative agreements: N/A Candidate options: - Option 1 (Apple, Qualcomm): Define requirements with 1 interference cell. - Option 2 (Nokia, Intel, MediaTek, Docomo, Huawei): Use explicit modelling of 2 interference cells - Option 3 (CMCC, China Telecom, Nokia, Docomo): Cover both 1 cell and 2 cells. - CMCC: 2 interference cell for HomoNet and 1 interference for HetNet - China Telecom: Cover different cell number within different test cases for 4Rx and 2Rx to control the test case number. Recommendations for 2nd round: Continue discussion. |
| Issue 1-3-5: Time and frequency offsets for synchronized network | Tentative agreements: - FDD 15 kHz - Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled) - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled) - TDD 30 kHz - Time offset: The serving cell is [3 or 1] us for interfering cell 1 and [-1 or FFS] us for interfering cell 2 (in case modeled) - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and FFS Hz for interfering cell 2 (in case modeled) Candidate options for time offset for TDD: - Option 1 ([China Telecom], CMCC, Intel, Docomo, Huawei): The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively - Option 2 (Apple, QC): For 1 interference cell – 1us for TDD 30KHz Recommendations for 2nd round: Continue discussion on assumptions for TDD case. |

#### Sub-topic 1-4: Receiver assumptions

|  | Status summary |
| --- | --- |
| Issue 1-4-1: TRS-IC/IM | Tentative agreements: N/A Candidate options: - Option 1 (China Telecom, Apple, CMCC, Nokia, Qualcomm, Ericsson, MediaTek, Docomo): If there are companies’ simulation results show that the interfered TRS will impact the UE synchronizing, and leads to PDSCH demodulation performance degradation, we are ok to add such clarification only in the simulation assumptions. - Option 2 (China Telecom, Intel, Huawei): Add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition Recommendations for 2nd round: Continue discussion. |

#### Sub-topic 1-5: Release independency of requirements

|  | Status summary |
| --- | --- |
| Issue 1-5-1: Release independency of Demodulation requirements | Tentative agreements: RAN4 discuss whether the UE demodulation with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption Recommendations for 2nd round: N/A |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2120707 |  |

### Open issues comments collection

Issue 1-1-1: Network type

- Address the following questions/comments

  - Apple: We would like to understand if the UE is aware of the time offset for any advanced processing-we would like clarification from China Telecom on the following:

Observation 1: For async networks, UE will suffer from completely different interference within one slot. In such case, UE should implement different IRC processing in the slot.

Also, are we assuming the same simulation parameters for interferer as sync case for parameters other than time offset?

    - China Telecom: In our initial thinking, we are ok to reuse the simulation assumptions for FDD sync scenario other than time/frequency offset. We have agreed to use random precoding for the interference cell whose precoder will be updated per slot, therefore, the interference power at the target cell will be different in time domain.

  - Intel: We would like to check whether asynchronous network is practical scenarios and what is the benefit to consider such scenario. Based on our understanding, in case all cells, for example, are used GNSS as reference sync source, we don’t expect to have the asynchronous network.

  - ZTE: Are we going to define two sets of performance requirements for sync and async scenarios respectively?

  - Huawei:

Asynchronized FDD will lead to different interference within one slot as Apple pointed out due to the random PMI selection for interference cells. We share the same views with Intel that there is no motivation to introduce such scenario since more practical deployment is synchronized.

@ CTC: Should we study more advanced IRC processing for asynchronized? Based on our understanding, it may be difficult for UE to know the boundary of different interference within one slot. LTE uses TM3 for interference cell which causes same interference within one slot, so it is not a big issue for UE to handle the asynchronized scenario with legacy IRC. But for NR, it is related to uneven interference processing that is still under discussion for Rel-18 performance enhancement, it is out of the scope of this WI.

| Company | Comment |
| --- | --- |
| Apple | We would like to understand if UE is aware of async interference as advanced receiver processing is assumed. |
| China Telecom | We prefer to cover async scenario for MMSE-IRC test. Firstly, we cannot assume that all NR networks are well synchronized. Secondly, async scenarios has additional impact to the UE IRC performance as we have exclaimed. Same time, to Huawei and Apple, we do not think UE needs be aware of the async interference, or even estimate the boundary before processing the IRC. With the same propagation condition on the different slots for the neighbor cell, UE will still observe similar ICI power on each of the DMRS symbols to process IRC, which means MMSE-IRC can still work under async scenarios. |
| Huawei | @ CTC: Due to the random PMI selection per slot in the interference cell, UE will observe different interference from symbol to symbol. Although the power may be same, the direction are different, which will cause large performance degradation in the serving cell UE. We still prefer not consider asyn scenario |
| CMCC | We also prefer to cover async scenario. Sync network cannot be always guaranteed in real network. Besides, in LTE, async scenario has also been considered, we would like to reuse LTE's assumption. |
| MediaTek | We share the same view as Huawei that it may cause larger performance degradation with current IRC processing. We prefer not to consider asynchronous scenario. |
| Ericsson | We support CTC’s comments. FDD async network is a practical NW deployment. In legacy LTE IRC receiver, RAN4 had already defined such test cases for FDD async. |
| Intel | Based on comments, it looks like that synchronous network can not be guarantied in the real scenarios. We are fine to at least make some analysis, first, to identify whether any issue with MMSE-IRC processing for asynchronous NR scenarios can be observed based on comment from Huawei. |
| Nokia | We do not have a strong opinion about FDD async, but a slight preference to not delay the progress of sync scenarios with this discussion. |
| Qualcomm | We also prefer not to consider async scenario. |
| Apple 2 | Thanks CTC for the clarification. What is the time offset we are considering for async case and we expect the UE to do no additional processing compared to MMSE-IRC or expect some advanced IRC processing for async case? |
| China Telecom 2 | Thanks Apple for the question. As we commented in the first round, we are fine to use the LTE’s assumption for FDD async as a start point: Interference Cell 1; Interference Cell 2. Time offset relative to target cell; 0.33(ms); 0.67(ms). Frequency shift relative to target cell; 0(Hz); 0(Hz). We do not expect some additional IRC processing for async scenarios either, but async scenario has additional impact to the UE IRC performance. We agree with Intel’s suggestion to have analysis to verify MMSE-IRC receiver can still achieve performance gain over MMSE receiver under async scenarios. |

Issue 1-1-2: SSB configuration

- Check views on the following options

  - Option 1: Use SSB Option 1 for all test

  - Option 2: Use SSB Option 2 for all test

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

    - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

| Company | Comment |
| --- | --- |
| Apple | We support option 2 for all tests |
| China Telecom | We support option 1 for all tests. Till now there is no evidence that UE synchronizing will be impacted due to ICI under the agreed simulation assumptions. Furthermore, as we have decided to use only MCS 13 for all tests, even PBCH decoding will not be impacted due to ICI. |
| Huawei | We support option 2 for all tests. We should create a clean environment for PDSCH test. Same time, from our simulations, the PBCH performance can’t be guaranteed when without combination is considered. |
| CMCC | It seems that Option 3A is a good compromise solution, and it also align with our deployment. Option 1 is our configuration in Homogeneous network, and Option 2 is one of configurations in our heterogeneous network. |
| MediaTek | We support Option 2 for all cases. It is a safer setting to verify the performance of PDSCH. |
| Ericsson | Option 1 It is a common configuration in the practical network. As we mentioned, the correlation of different SSS sequence is nearly zero by RAN1’s sequence design. In real field, PDBCH decoding is based on multiple trails, thus, there is also no performance impact for MIB decoding. We encourage the companies to propose the simulation evaluation on the performance loss for SSB colliding and RAN4 can consider to add a further margin when defining the requirements. |
| Intel | We think that Option 3A or 3B can be considered to have sufficient test coverage. |
| Qualcomm | We support Option 2 for all tests. Same comments as 1st round. |
| Docomo | We support Option 1. At this meeting, RAN4 agreed to apply MCS=13 for all tests. So, we don’t think that PBCH performance degradation caused by SSB collisions have big effect for PDSCH performance． |

Issue 1-3-4: Number of explicitly modeled interference cells

- Check views on the following options

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cell for homogeneous deployment assumptions and 1 interference for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions

| Company | Comment |
| --- | --- |
| Apple | We support option 1 - 1 interference for all test cases. Like deployment scenario, there is no change to receiver processing whether 1 or 2 interferer cells are modeled. Moreover with 2 interference cells the interference becomes white and hence higher INR level is needed to achieve similar performance as with 1 interferer. It only increases the test complexity, but doesn’t test anything in receiver with 2 interferers configured. |
| China Telecom | We think both 1 cell and 2 cells should be covered in the IRC test. Tests with 2 interference cells verifies the IRC receiver’s ability of mitigating interference from 2 different directions. And both 1 cell and 2 cells are also covered in the LTE IRC tests. We support option 3 to save the test case number and option 3A is our first preference. |
| Huawei | We support option 2. As our simulation shows, 1 interference cell has small performance gain for IRC over MRC. Same time, we have agreed to use 1 interference cell in CSI test. We don’t think the interference will become white with only 2 interference cells |
| CMCC | We prefer Option 3A |
| MediaTek | We support Option2. From our simulation results, the performance gain between MRC and IRC receiver is small with 1 interference cell. |
| Ericsson | Option 2. |
| Intel | Based on our system level analysis and assumptions, which is used for many LTE requirements with inter-cell interference, modelling of 2 interference cells is reasonable assumption to verify UE performance under conditions rather close to practical. Therefore, our first preference is Option 2. However, to move forward we are fine to consider Option 3B. |
| Nokia | We support option 2 of having 2 interferer cells for all test or downscale to option 3A as we do not see 2 interference cells for heterogeneous setup as a likely scenario. |
| Qualcomm | We support Option 1. We think that 1 interfering cell is enough to test the structured interference. As we add more interfering cells, it distorts the structure. |
| Docomo | Our preference is Option 2. In LTE, both 1 cell and 2 cells are covered for IRC test. So, we think that it is not enough to cover 1 cell for specifying UE performance under close to practical network. However, for the sake of progress, Option 3A is acceptable for us . |

Issue 1-3-5: Time and frequency offsets for synchronized network

- Check views of synchronization assumptions for TDD case

  - Current option

    - Time offset: The serving cell is [3 or 1] us for interfering cell 1 and [-1 or FFS] us for interfering cell 2 (in case modeled)

    - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and FFS Hz for interfering cell 2 (in case modeled)

  - Other options?

| Company | Comment |
| --- | --- |
| Apple | TO: 1 us for interference cell 1, -0.25 us for interference cell 2 if modeled. Freq shift: -100Hz for interference cell 2 if modeled |
| Huawei | Could Apple please elaborate where these values from ? |
| Ericsson | Time offset: The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively Frequency offset: The serving cell is 300 Hz and -100 Hz for interfering cell 1 and cell 2 respectively. |
| Intel | We suggest to use same assumptions for FDD and TDD, because delay between signals from different cells does not depend on duplex mode or SCS. |
| Qualcomm | We prefer to keep the TO within CP. We are ok to reuse the values from mTRP as suggested by Apple. |
| Apple | The proposed values are for TDD with 30KHz and come from the values we used for mTRP. With 3us TO, its larger than CP with 30KHz SCS and doesn’t suit the synchronized assumption. @Intel, if same values are preferred for FDD and TDD, we can use 3us and -0.25 us for FDD as well. |

Issue on simulation assumptions from Qualcomm

- Can other companies confirm if below assumptions were made for their simulations?

  - Interfering cell has 2Tx and random precoding with PRB granularity of 2.

  - Interfering cell has same channel model as for serving cell and it is independently generated for each cell.

  - Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant.

| Company | Comment |
| --- | --- |
| Apple | We confirm that we used these assumptions in our simulations. |
| China Telecom | We agree with using the above simulation assumptions. |
| Huawei | We agree with using the above simulation assumptions. |
| CMCC | We agree with using the above simulation assumptions. |
| MediaTek | We confirm that we use the above assumptions for simulations. |
| Ericsson | We agree with using the above simulation assumptions. |
| Intel | We would like to confirm above assumptions |
| Nokia | We agree, these assumptions have been used in our simulations already. |
| Qualcomm | We confirm all the above assumptions. |
| Docomo | We agree with using the above simulation assumptions. |

# Topic #2: MMSE-IRC receiver for inter-cell interference – CSI reporting requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117433 | Apple | Proposal #1: Do not introduce PMI reporting requirements for MMSE-IRC in ICI. Proposal #2: Define requirements for CQI reporting with NZP CSI-RS overlapping on target and interference cell. Proposal #3: Re-use the test metric as LTE CQI reporting in ICI. Proposal #4: Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing. |
| R4-2117641 | Nokia, Nokia Shanghai Bell | Observation 1: The PMI reporting may be impacted by a relative strong interference cell when UE is at cell edge Proposal 1: Introduce the PMI reporting requirements with inter-cell interference scenario. Observation 2: There is only a small throughput improvement when NZP CSI-RS from interference cell overlaps with CSI-IM compared to overlap with PDSCH. Observation 3: Based on these simulation results, we don’t observe a significant benefit of option 2 versus option 1. Proposal 2: Based on our observations, we propose to go with option 1. Observation 4: Option 2 was not selected as baseline in RAN4#100e, as one contributor asked for more time to confirm provided simulation results. Proposal 3: If the provided simulation results are not disputed, by the remaining contributor asking for more time to check in the last meeting, then option 2 shall be as used baseline. |
| R4-2117737 | CMCC | Proposal 1: Only consider CSI-IM on target cell overlapping with PDSCH from interference. Proposal 2: NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference. |
| R4-2118000 | Intel Corporation | Proposal 1: Do not define PMI reporting requirements for MMSE-IRC receiver for scenario with inter-cell interference. Proposal 2: Consider the following NZP CSI-RS assumptions for CQI requirements: NZP CSI-RS on target cell overlaps with PDSCH from interference Proposal 3: Consider the following interference modelling assumptions for CQI requirements for further downselection: - Option 1: 2 interference cells, INR1 13.91 dB, INR1 4.34 dB - Option 2: 1 interference cell, INR 10.04 dB - Option 3: 2 interference cells, INR1 7.77 dB, INR2 2.29 dB - Option 4: 1 interference cell, INR 5.49 dB |
| R4-2118407 | Ericsson | Observation 1: The PDSCH Tput performance is the same for possible NZP CSI-RS overlapping scenarios. Observation 2: NZP CSI-RS overlapping had already deployed in real field. Observation 3: RAN1 is discussing ‘false-PMI’ issue for multiple cells in Rel-17 FeMIMO. Proposal 1: RAN4 should consider the real deployment and use the following CSI-RS configuration: - Target cell NZP CSI-RS colliding with interf. cell NZP CSI-RS Proposal 2: RAN4 should consider the following serving cell CSI-IM configurations: - Scenario 1: serving cell CSI-IM colliding with Interf. cell CSI-IM - Scenario 2: serving cell CSI-IM colliding with Interf. cell PDSCH Proposal 3: RAN4 to define the PMI reporting tests for MMSE-IRC inter-cells’ requirement or change some PDSCH demodulation test case with follow PMI. Proposal 4: RAN4 to confirm the following interference configuration as the baseline for CQI reporting test. - 1 TX with static channel - INR configuration: 10.04 dB (DIP -0.41dB) |
| R4-2118409 | Ericsson | Observation 1: Compared with MRC-based CQI reporting, IRC-based CQI reporting receiver can have obvious throughput gain about 2dB gain in 70% of maximum throughput SNR point. Observation 2: The BLER is greater than or equal to 10% which implies a reasonable CQI reporting to NW. |
| R4-2118862 | China Telecom | Proposal 1: For this meeting, make decision not to define PMI reporting requirements with inter-cell interference scenario unless some PMI calculation difference can be shown. Observation 1: Since we have decided to use 1Tx with static channel model for the interference cell, there will be no difference between neighbor cell’s PDSCH and NZP CEI-RS in terms of interference level. Proposal 2: For target CSI-IM, we propose only to consider overlapping with PDSCH from interference, and for NZP CSI-RS on target cell, we slightly prefer overlaps with NZP CSI-RS from interference since it is more practical. |
| R4-2119045 | Huawei, HiSilicon | Proposal 1: Use following configurations for CQI testing: - Serving cell: - MIMO configuration: 2T2R, 2T4R, ULA Low - Propagation conditions: TDLA30-5 - Interference cell: - INR: 10.04dB (DIP: -0.41dB) - Other parameters: - Reuse from Rel-15 CQI test with fading channel - Test metric: - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ; - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%. - SINR=-2dB, γ can be set to [2] for 2RX and [3] for 4RX. Observation 1: As RAN 4 has agreed to use 1TX for interference cell, there is no difference between option 1 and option 2 for both CSI-IM configuration and NZP CSI-RS configuration. Observation 2: No big performance difference between two scenarios that serving cell’s CSI-IM is overlapping with neighboring cell’s CSI-IM and serving cell’s CSI-IM is overlapping with neighboring cell’s PDSCH. Proposal 3: Use following configurations: - Serving cell’s CSI-IM is overlapping with interference cell’s PDSCH. Serving cell’s NZP CSI-RS is overlapping with interference cell’s NZP CSI-RS Observation 3: No obvious performance difference for PMI selection with MRC processing and that with IRC processing. Proposal 4: Not to define the PMI requirements |
| R4-2119404 | MediaTek inc. | Proposal 1: NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference. Proposal 2: Not to introduce PMI reporting requirements with inter-cell interference scenario. |
| R4-2119548 | Qualcomm Incorporated | Proposal 1: Do not define PMI reporting test cases for Intercell Interference scenario. Proposal 2: Do not consider other scenarios for CSI-IM apart from collision with PDSCH interference. Proposal 3: Use 1x2/1x4 ULA Low antenna configuration on serving cell for defining CQI reporting requirements under intercell interference scenario. Proposal 4: Use 1Tx PDSCH interference colliding with target cell NZP CSI-RS. |

## Open issues summary

### Sub-topic 2-1: CSI requirements

Issue 2-1-1: Whether to define PMI reporting requirements

- Background

  - Option 1: Yes

  - Option 2: Need further discussion

  - Option 3: No

- Proposals

  - Option 1 (Apple, Intel, China Telecom, Huawei, MediaTek, Qualcomm): No

    - China Telecom: For this meeting, make decision not to define PMI reporting requirements with inter-cell interference scenario unless some PMI calculation difference can be shown.

  - Option 2 (Nokia, Ericsson): Yes

- Recommended WF

  - Check whether RAN4 can use Option 1 (Not to define PMI requirements)

  - Collect views on the following Ericsson observation: “RAN1 is discussing ‘false-PMI’ issue for multiple cells in Rel-17 FeMIMO.”

### Sub-topic 2-2: Interference Model

Issue 2-2-1: CSI-IM on target cell

- Background

  - CSI-IM overlaps with PDSCH from interference is the baseline scenario for defining CSI reporting requirement.

  - Other scenarios are FFS.

- Proposals

  - Option 1 (Nokia, CMCC, China Telecom, Huawei, Qualcomm): Only consider overlapping with PDSCH from interference

  - Option 2 (Ericsson): RAN4 should consider the following serving cell CSI-IM configurations:

    - Scenario 1: serving cell CSI-IM colliding with Interf. cell CSI-IM

    - Scenario 2: serving cell CSI-IM colliding with Interf. cell PDSCH

- Recommended WF

  - Check whether RAN4 can use Option 1 (Only consider overlapping with PDSCH from interference)

Issue 2-2-2: NZP CSI-RS on target cell

- Background

  - Option 1: Overlaps with PDSCH from interference

  - Option 2: Overlaps with NZP CSI-RS from interference

  - Option 3: Make further down selection based on simulation results and also take practical network configuration into account.

    - if using target NZP CSI-RS overlaps with neighbor NZP CSI-RS, same port number for the 2 cells shall be configured.

- Proposals

  - Option 1 (Intel, Qualcomm): Overlaps with PDSCH from interference

  - Option 2 (Apple, Nokia, CMCC, Ericsson, China Telecom, Huawei, MediaTek): Overlaps with NZP CSI-RS from interference

- Recommended WF

  - Check whether Option 2 (Overlaps with NZP CSI-RS from interference) can be considered

### Sub-topic 2-3: Simulation assumption for CQI requirements

Background

- Serving cell

  - Reuse assumptions from Sections 6.2.2.1.2.1, 6.2.2.2.2.1, 6.2.2.2.2.1, 6.2.3.2.2.1 in TS 38-101-4.

- Interference cell

  - 1 TX with static channel

  - NZP CSI-RS and CSI-IM is based on outcome of Issue 2-2-1 and Issue 2-2-2

  - INR configuration

    - Option 1: 10.04 dB (DIP -0.41dB)

    - Other options are not precluded

      - Based on the discussion of Deployment and Number of explicitly modeled interference cells in MMSE-IRC receiver for inter-cell interference – Demodulation requirements.

Issue 2-3-1: Antenna configuration for serving cell

- Proposals

  - Option 1 (Huawei): 2T2R, 2T4R, ULA Low

  - Option 2 (Qualcomm): 1x2/1x4 ULA Low antenna configuration

- Recommended WF

  - Check views on options above

Issue 2-3-2: Channel model for serving cell

- Proposals

  - Option 1 (Huawei): TDLA30-5

- Recommended WF

  - Check views on option above

Issue 2-3-3: Antenna configuration for interference cell (s)

- Proposals

  - Option 1 (Ericsson): 1 Tx

- Recommended WF

  - Confirm Option 1 based on previous meeting agreement

Issue 2-3-4: Channel model for interference cell (s)

- Proposals

  - Option 1 (Ericsson): Static

- Recommended WF

  - Confirm Option 1 based on previous meeting agreement

Issue 2-3-5: INR values

- Proposals

  - Option 1 (Intel): Consider the following interference modelling assumptions for CQI requirements for further downselection:

    - Option 1: 2 interference cells, INR1 13.91 dB, INR1 4.34 dB

    - Option 2: 1 interference cell, INR 10.04 dB

    - Option 3: 2 interference cells, INR1 7.77 dB, INR2 2.29 dB

    - Option 4: 1 interference cell, INR 5.49 dB

  - Option 2 (Ericsson, Huawei): INR configuration: 10.04 dB (DIP -0.41dB)

- Recommended WF

  - Check views on options above

Issue 2-3-6: Other parameters

- Proposals

  - Option 1 (Huawei): Reuse from Rel-15 CQI test with fading channel

- Recommended WF

  - Confirm Option 1 based on previous meeting agreement

Issue 2-3-7: Test metric

- Proposals

  - Option 1 (Apple, Huawei): Re-use the test metric as LTE CQI reporting in ICI.

    - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ;

    - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to 2%.

- Recommended WF

  - Check whether Option 1 is acceptable

Issue 2-3-8: Requirements definition

- Proposals

  - Option 1 (Apple): Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing.

  - Option 2 (Huawei): SINR=-2dB, γ can be set to [2] for 2RX and [3] for 4RX.

- Recommended WF

  - Check view on Option 1

  - Final values will be decided based on agreed simulation assumptions

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 2-1: CSI requirements

| Company | Comments |
| --- | --- |
| Apple | Issue 2-1-1: Whether to define PMI reporting requirements We support option 1. Could Ericsson provide some clarification on their observation/ false-PMI issue? |
| China Telecom | Issue 2-1-1: Whether to define PMI reporting requirements Option 1. |
| Intel | Issue 2-1-1: Whether to define PMI reporting requirements Based on our analysis we support not to define the PMI requirements. Same time, we need some time to double check this ‘false-PMI’ issue. Additional information from Ericsson will be very helpful. |
| Qualcomm | Issue 2-1-1: Whether to define PMI reporting requirements Prefer Option 1. We are not clear on Ericsson’s comment on ‘false-PMI’ issue. If RAN1 is already discussing that issue in FeMIMO WI, shouldn’t RAN4 discuss it as part of that WI when RAN1 is done? If that issue is important enough, RAN1 will address it. |
| Ericsson | Issue 2-1-1: Whether to define PMI reporting requirements RAN1 is discussing ‘false-PMI’ issue for multiple cells in Rel-17 FeMIMO TEI (See R1-2019722 TEI proposal #4). When UE is in the cell edge, the UE may report the false PMI based on the strongest interference cell other than PMI based on serving cell. We also encouraged interesting companies to further check RAN1 colleagues. Since many companies (including operators, UE vendors, and infra vendors) in RAN1 also prefer to discuss/solve in RAN4, we can compromise to not define PMI reporting requirements in this Further enhancement on NR demodulation performance WI, but we want to add the note ‘RAN4 will continue the discussion on the PMI reporting performance with inter-cell interference condition in Rel-17 FeMIMO WI performance part’. |
| Nokia, Nokia Bell Labs | Issue 2-1-1: Whether to define PMI reporting requirements Based on companies feedback, we can also support option 1. The issue of wrongly choosing PMI in certain scenarios is important but as long as it will be discussed in FeMIMO we can compromise to option 1. |
| Huawei | Support not to define the PMI requirements. Whether “false-PMI” issue needs to be discussed in R17 FeMIMO WI performance part, it should depend on RAN1 discussion and conclusion, also it is contribution driven, maybe it is not so suitable to mandate further actions for another WI in this WI discussion. |

#### Sub-topic 2-2: Interference Model

| Company | Comments |
| --- | --- |
| Apple | Issue 2-2-1: CSI-IM on target cell We support the recommended WF. Issue 2-2-2: NZP CSI-RS on target cell We support the recommended WF. |
| CMCC | Issue 2-2-1: CSI-IM on target cell Option 1 is ok for us. Issue 2-2-2: NZP CSI-RS on target cell We support Option 2. |
| China Telecom | Issue 2-2-1: CSI-IM on target cell We support the recommended WF. Issue 2-2-2: NZP CSI-RS on target cell We support the recommended WF. |
| Intel | Issue 2-2-1: CSI-IM on target cell Support recommended WF Issue 2-2-2: NZP CSI-RS on target cell We are fine with recommended WF |
| Qualcomm | Issue 2-2-1: CSI-IM on target cell Ok with recommended WF. Issue 2-2-2: NZP CSI-RS on target cell If it is agreed to use 1Tx on interfering cell, we are ok with Option 2. |
| Ericsson | Issue 2-2-1: CSI-IM on target cell We support the recommended WF. Issue 2-2-2: NZP CSI-RS on target cell We support the recommended WF. |
| MediaTek | Issue 2-2-1: CSI-IM on target cell Support the recommend WF. Issue 2-2-2: NZP CSI-RS on target cell Support the recommend WF. |
| Docomo | Issue 2-2-1: CSI-IM on target cell We Support the recommended WF. |
| Nokia, Nokia Bell Labs | Issue 2-2-1: CSI-IM on target cell We support the recommended WF. Issue 2-2-2: NZP CSI-RS on target cell We support the recommended WF. |
| Huawei | Issue 2-2-1: CSI-IM on target cell Support the recommended WF Issue 2-2-2: NZP CSI-RS on target cell We are fine with the recommended WF |

#### Sub-topic 2-3: Simulation assumption for CQI requirements

| Company | Comments |
| --- | --- |
| Apple | Issue 2-3-1: Antenna configuration for serving cell We evaluated performance for 2TX and see reasonable measurable gains. We are fine to further evaluate 1TX. Issue 2-3-2: Channel model for serving cell Option 1 is fine. Issue 2-3-3: Antenna configuration for interference cell (s) We support the recommended WF. Issue 2-3-4: Channel model for interference cell (s) We support the recommended WF. Issue 2-3-5: INR values We don’t see the necessity to have 2 interference cells with static channel. 1 interfernece cell is sufficient to verify CQI reporting with MMSE-IRC. We support to use the values from LTE – INR: 10.04 dB Issue 2-3-6: Other parameters We support the recommended WF. Issue 2-3-7: Test metric Option 1 is fine. Issue 2-3-8: Requirements definition SNR and  values can be decided based on simulation assumptions. Support option 1. |
| CMCC | Issue 2-3-1: Antenna configuration for serving cell We are ok with Option 1. Issue 2-3-3: Antenna configuration for interference cell (s) Support the recommended WF. Issue 2-3-4: Channel model for interference cell (s) Support the recommended WF. Issue 2-3-5: INR values Option 2 is fine for us. Issue 2-3-6: Other parameters Support the recommended WF. Issue 2-3-7: Test metric Option 1 is acceptable for us. |
| China Telecom | Issue 2-3-1: Antenna configuration for serving cell 2T2R and 2T4R are used for the existing NR CQI FR1 test, therefore, we prefer to reuse the same antenna configuration for MMSE-IRC based CQI test. ULA high for 2RX and XPL high for 4RX were used for the existing NR CQI FR1 test, we prefer to reuse the same antenna correlation for MMSE-IRC based CQI test. Issue 2-3-2: Channel model for serving cell Option 1 is fine for us. Issue 2-3-3: Antenna configuration for interference cell (s) Option 1 is fine for us. Issue 2-3-4: Channel model for interference cell (s) Option 1 is fine for us. Issue 2-3-5: INR values We did not see the need to introduce 2 interference cells with 1Tx and static channel model which will only change the interference power level. We prefer to reuse the INR value in LTE, i.e., INR configuration: 10.04 dB (DIP -0.41dB) Issue 2-3-6: Other parameters Support the recommended WF. Issue 2-3-7: Test metric Option 1 is fine for us. |
| Intel | Issue 2-3-1: Antenna configuration for serving cell Both options are fine for us. Issue 2-3-2: Channel model for serving cell Option 1 is fine for us Issue 2-3-5: INR values To move forward and reduce simulation effort, we are fine to focus on scenario with 1 interference cell and INR 10.04 dB for CQI requirements. Issue 2-3-6: Other parameters Option 1 is fine for us Issue 2-3-7: Test metric Option 1 is fine for us Issue 2-3-8: Requirements definition Option 1 is fine for us. |
| Qualcomm | Issue 2-3-1: Antenna configuration for serving cell We prefer Option 2 because when we have 2x2 Low, there are two observations and UE needs to solve the equation for two inputs, then UE can’t do much to cancel out the interference. With 1x2 Low, there are two observations and UE needs to solve for only 1 input, then UE can also do something to cancel out interference, which will improve the performance further. Issue 2-3-2: Channel model for serving cell Ok with Option 1. Issue 2-3-3: Antenna configuration for interference cell (s) Ok with recommended WF. Issue 2-3-4: Channel model for interference cell (s) Ok with recommended WF. Issue 2-3-5: INR values Same comments as other companies. Prefer Option 2. Issue 2-3-6: Other parameters Ok with recommended WF. Issue 2-3-7: Test metric Ok with Option 1 in principle. But we prefer to have X% for the 2nd bullet instead of 2% and it should be confirmed based on simulation results. Issue 2-3-8: Requirements definition SINR and  values should be decided based on the simulation results. |
| Ericsson | Issue 2-3-1: Antenna configuration for serving cell OK with Option 1. Issue 2-3-2: Channel model for serving cell OK with Option 1. Issue 2-3-3: Antenna configuration for interference cell (s) OK with Option 1. Issue 2-3-4: Channel model for interference cell (s) OK with Option 1. Issue 2-3-5: INR values Option 2 is fine for us which aligns with legacy LTE test. Issue 2-3-6: Other parameters OK with Option 1. Issue 2-3-7: Test metric OK to include both Tput ratio and BLER. We suggest to FFS the BLER value and further check the reasonable value based on simulation Issue 2-3-8: Requirements definition It’s too early to define the Tput ratio γ. We suggest to conclude the value after the simulation. @Apple, Could you further explain the meaning of option 1? |
| MediaTek | Issue 2-3-1: Antenna configuration for serving cell Both options are fine to us. However, we would like to mention that the CRI-RS port is 1 for the interference cell as we agreed 1Tx in the last meeting. If we use 2T2R in the serving cell based on the Table 6.2.2.1.1.1-1. Do we need to modify the “Number of CSI-RS ports (X)” from 2 to 1? Issue 2-3-2: Channel model for serving cell OK with Option 1. Issue 2-3-3: Antenna configuration for interference cell (s) OK with the recommend WF. Issue 2-3-4: Channel model for interference cell (s) OK with the recommend WF. Issue 2-3-5: INR values Prefer Option 2. Issue 2-3-7: Test metric Agree with Option 1 but the  value and BLER requirement can be confirmed later based on the simulation results. Issue 2-3-8: Requirements definition OK with Option 1. |
| Docomo | Issue 2-3-1: Antenna configuration for serving cell Option 1 is fine for us. Issue 2-3-4: Channel model for interference cell (s) Option 1 is fine for us. Issue 2-3-6: Other parameters Option 1 is fine for us. |
| Nokia, Nokia Bell Labs | Issue 2-3-1: Antenna configuration for serving cell We are OK with both options for initial simulations. We are also fine to restrict the to single layer, if this helps achieving agreement here. Issue 2-3-2: Channel model for serving cell We are OK with option 1 Issue 2-3-3: Antenna configuration for interference cell (s) We are OK with option 1 Issue 2-3-4: Channel model for interference cell (s) We are OK with Option 1. Issue 2-3-5: INR values We are OK with option 2 Issue 2-3-6: Other parameters We are OK with recommended WF Issue 2-3-7: Test metric We are OK with option 1 and agree with companies suggesting to finalize BLER value based on simulation results. Hence we should put the 2% in []. Issue 2-3-8: Requirements definition Concerning option 1, we agree that the MMSE-IRC requirements should not be achievable with legacy MMSE receivers. However, this can be discussed if the simulation results point towards such a questionable outcome. |
| Huawei | Issue 2-3-1: Antenna configuration for serving cell We prefer option 1 Issue 2-3-2: Channel model for serving cell Option 1 Issue 2-3-3: Antenna configuration for interference cell (s) Option 1 Issue 2-3-4: Channel model for interference cell (s) Option 1 Issue 2-3-5: INR values We prefer Option 2 Issue 2-3-6: Other parameters OK with recommended WF Issue 2-3-7: Test metric OK with recommended WF Issue 2-3-8: Requirements definition OK with recommended WF |

## Summary for 1st round

### Open issues

#### Sub-topic 2-1: CSI requirements

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Whether to define PMI reporting requirements | Tentative agreements: - Not to define PMI reporting requirements in the Rel-17 Further enhancement on NR demodulation performance WI - FFS whether to add the following note: ‘RAN4 will continue the discussion on the PMI reporting performance with inter-cell interference condition in Rel-17 FeMIMO WI performance part’ Candidate options: - Option 1 (Ericsson): Define note - Option 2 (Huawei): Note is not needed - Huawei: it should depend on RAN1 discussion and conclusion, also it is contribution driven Recommendations for 2nd round: Continue discussion on whether to define the note from Ericsson. |

#### Sub-topic 2-2: Interference Model

|  | Status summary |
| --- | --- |
| Issue 2-2-1: CSI-IM on target cell | Tentative agreements: Only consider overlapping with PDSCH from interference Recommendations for 2nd round: N/A |
| Issue 2-2-2: NZP CSI-RS on target cell | Tentative agreements: Overlaps with NZP CSI-RS from interference Recommendations for 2nd round: N/A |

#### Sub-topic 2-3: Simulation assumption for CQI requirements

|  | Status summary |
| --- | --- |
| Issue 2-3-1: Antenna configuration for serving cell | Tentative agreements: N/A Candidate options: - Option 1 (Huawei, Apple, CMCC, Intel, Ericsson, MediaTek, Docomo, Nokia): 2T2R, 2T4R, ULA Low - Option 2 (Qualcomm, Apple, Intel, MediaTek, Nokia): 1x2/1x4 ULA Low antenna configuration - Option 3 (China Telecom): 2T2R ULA high and 2T4R XPL high Recommendations for 2nd round: Continue discussion taking into account comments from the first round: - China Telecom: 2T2R and 2T4R are used for the existing NR CQI FR1 test, therefore, we prefer to reuse the same antenna configuration for MMSE-IRC based CQI test. ULA high for 2RX and XPL high for 4RX were used for the existing NR CQI FR1 test, we prefer to reuse the same antenna correlation for MMSE-IRC based CQI test. - Qualcomm: We prefer Option 2 because when we have 2x2 Low, there are two observations and UE needs to solve the equation for two inputs, then UE can’t do much to cancel out the interference. With 1x2 Low, there are two observations and UE needs to solve for only 1 input, then UE can also do something to cancel out interference, which will improve the performance further. - MediaTek: However, we would like to mention that the CRI-RS port is 1 for the interference cell as we agreed 1Tx in the last meeting. If we use 2T2R in the serving cell based on the Table 6.2.2.1.1.1-1. Do we need to modify the “Number of CSI-RS ports (X)” from 2 to 1? - Nokia: We are also fine to restrict the to single layer, if this helps achieving agreement here. |
| Issue 2-3-2: Channel model for serving cell | Tentative agreements: TDLA30-5 Recommendations for 2nd round: N/A |
| Issue 2-3-3: Antenna configuration for interference cell (s) | Tentative agreements: 1 Tx Recommendations for 2nd round: N/A |
| Issue 2-3-4: Channel model for interference cell (s) | Tentative agreements: Static Recommendations for 2nd round: N/A |
| Issue 2-3-5: INR values | Tentative agreements: One cell with 10.04 dB (DIP -0.41dB) Recommendations for 2nd round: N/A |
| Issue 2-3-6: Other parameters | Tentative agreements: Reuse from Rel-15 CQI test with fading channel Recommendations for 2nd round: N/A |
| Issue 2-3-7: Test metric | Tentative agreements: - Re-use the test metric as LTE CQI reporting in ICI. - the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ; - when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP, the average BLER for the indicated transport formats shall be greater than or equal to X%. Recommendations for 2nd round: N/A |
| Issue 2-3-8: Requirements definition | Tentative agreements: SINR, γ and X values should be decided based on the simulation results Candidate options: - Option 1 (Apple, Intel. MediaTek): Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing. - Ericsson: Could you further explain the meaning of option 1? Recommendations for 2nd round: - Discuss whether we need to define any criteria for definition of SINR, γ and X values. Option 1 below is one of the examples. |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2120708 |  |

### Open issues comments collection

Issue 2-1-1a: whether to add the following note: ‘RAN4 will continue the discussion on the PMI reporting performance with inter-cell interference condition in Rel-17 FeMIMO WI performance part’

- Option 1: Define note (Ericsson)

- Option 2: Note is not needed (Huawei)

  - Huawei: it should depend on RAN1 discussion and conclusion, also it is contribution driven

| Company | Comment |
| --- | --- |
| Apple | We support option 2. We agree with Huawei. We don’t need to decide what will be discussed in FeMIMO WI performance part here. |
| CMCC | Share similar view with Apple, Option 2. |
| Huawei | Option 2 |
| MediaTek | Support Option 2. |
| CTC | Option 2. |
| Intel | Slightly prefer Option 2. We think that Note suggested by Ericsson is rather straightforward and aligned with typical 3GPP work procedure. Based on outcome of RAN1 discussion, RAN4 will work on the scope of requirements for Rel-17 FeMIMO WI. |
| Qualcomm | Prefer Option 2. |
| Nokia | The issue of PMI reporting performance is important but we don't think it is necessary to define a note here. So we can support option 2, however if it is decided not to be discussed in FeMIMO, it should be re-addressed. The discussion from eMIMO on this topic was postponed to FeMIMO. |
| Ericsson | We are ok to compromise to not have this note. And we are ok with RAN4 will not define the PMI reporting performance with inter-cell interference condition in Rel-17 performance enhancement WI. |
| Docomo | We support Option 2. |

Issue 2-3-1: Antenna configuration for serving cell

- Option 1 (Huawei, Apple, CMCC, Intel, Ericsson, MediaTek, Docomo, Nokia): 2T2R, 2T4R, ULA Low

- Option 2 (Qualcomm, Apple, Intel, MediaTek, Nokia): 1x2/1x4 ULA Low antenna configuration

- Option 3 (China Telecom): 2T2R ULA high and 2T4R XPL high

| Company | Comment |
| --- | --- |
| Apple | We support option 1 or 2. For option 1 we would like to mention that we need to likely use fixed precoder and we do observe gains > 2 with 2 TX. Also, with 2TX we should configure the interference to transmit PDSCH on CSI-RS slots or OCNG, then 2 port CSI-RS on target UE will see interference from NZP CSI-RS and PDSCH/OCNG, otherwise NZP CSI-RS might see uneven interference on 2 ports. For option 3 with high correlation we might not be able to verify performance of MMSE-IRC correctly to reject interference as signal is correlated even without introducing ICI. |
| CMCC | Option 1 is our first preference, the suggestion from Apple is fine for us. |
| Huawei | We support option 1 or option 2. Based from our understanding, IRC processing is only sensitive to number of receiving antennas, at the UE side, there will always be one synthetic interference direction when there are more than one transmission antennas. From our simulations, the IRC performance gain for 2TX is obvious, but we are OK to use 1T2R/1T4R low. For option 3, we have same views with Apple that high antenna correlation may degrade the capability of rejecting the interference from signal. Meanwhile, Considering that we have agreed to use CSI-IM overlapping with PDSCH from interference, based on our understanding, we should configure interference to transmit PDSCH on CSI-IM/CSI-RS slots or OCNG regardless 1 TX or 2TX |
| MediaTek | We support Option 1 or 2 with ULA low configuration. As for ULA high configuration, the performance will be dominate by the chosen precoder. Hence, we prefer to only consider ULA low. |
| CTC | We are also fine with option 1. And we agree with Huawei that to ensure UE can observe the interference on the CSI-RS, we should configure interference to transmit PDSCH on CSI-IM/CSI-RS slots regardless 1 TX or 2TX |
| Intel | All options are fine for us. Probably Option 1 or 2 is better from interference rejection capability. |
| Qualcomm | For Option 1, we have a concern on other agreement where we agreed that NZP CSI-RS will collide with NZP CSI-RS from interferer. But, with Option 1, we will have 2 ports of NZP CSI-RS while interfering cell will only have 1. So, it can’t collide with NZP CSI-RS for half of the ports. Also, as we mentioned in our 1st round comment, with 1x2 configuration, UE will have better chance of handling the interference. |
| Ericsson | We are ok with either option 1 and 2. |
| Docomo | Our preference is Option 1. |

Issue 2-3-8: Requirements defination

- Agreement:

  - SINR, γ and X values should be decided based on the simulation results

  - Criteria: Test metric for CQI reporting in ICI is chosen such that it cannot be met with MMSE processing (example).

- Discuss whether we need to define any criteria for definition of SINR, γ and X values.

| Company | Comment |
| --- | --- |
| Apple | For the TP ratio metric γ, it can be chosen such that it cannot be achieved if MMSE processing is used for CSI reporting. SINR shall be >-4 dB. |
| Huawei | Beside the metric on TP ratio, BLER for IRC processing should be also considered. 2% may be feasible. |
| CTC | We agree with the criteria we should choose to ensure that it cannot be met with only MMSE processing. We can further decide the exact value based on more simulation results. |
| Intel | We agree with suggested criteria to select the SINR and γ values. |
| Qualcomm | We are ok to list this criteria as one of the options. But we also need to have reasonable values for gamma and X for the SINR chosen. We can discuss this in the next meeting since it does not impact the simulation campaign. |
| Ericsson | We are fine with listed criteria to be an example, and to be decided in the next meeting. |

# Topic #3: MMSE-IRC receiver for intra-cell inter-user interference

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117434 | Apple | Observation #1: With rank 2 on target UE and variable rank on interference UE the test set-up would be very complicated. Observation #2: Depending on signal power assumption the SIR and rank combination the SIR level would be either 0dB or 3dB. Proposal #1: Define requirements with either rank combination 2+1 or 2+2 for 4x4 case. Observation #3: With random precoder we observe 0.6~0.8 dB degradation in MCS4 and 1.5~2dB degradation with MCS13 with random precoder compared to orthogonal precoder for co-scheduled UEs. Observation #4: There is severe performance degradation with random precoder and 2 layers per co-scheduled UE, especially with TDLC channel. Proposal #2: Define requirements with orthogonal precoder for co-scheduled UEs. Observation #5: For 2TX with 1 layer per co-scheduled UE there is no significant performance delta between various DMRS port mapping between co-scheduled UEs. Proposal #3: For 2TX configure co-scheduled UEs on same CDM group with number of CDM groups without data as 1. Observation # 6: With XPL-Low defined with α1=0, α2=0, β=0, γ=0, there would be no difference between ULA low and XPL-Low correlation models. Proposal #4: Define requirements with ULA Low MIMO correlation model for all antenna configurations. Observation #7: In LTE for simultaneous transmission, the transmit powers are normalized by the total number of layers. Proposal #5: Use the following assumptions for MU-MIMO requirements: Signal power assumption - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal. SNR definition - SNR = (STargetUE+ SInterfUE)/N Observation #8: For 2x2 there is no significant performance delta between MMSE-IRC and MMSE for MCS4 and there is performance improvement with MMSE-IRC over MMSE for MCS13. Proposal #6: Define requirements for MU-MIMO with 2Tx with MCS13 and TDLA channel. Observation #9: Performance with TDLC channel is degraded by over 2 dB compared to TDLA channel for MCS13 with 2 layers per UE and MCS 19 with 2+1 rank combination. Observation #10: With TDLC and 2 layers per UE with MCS-19, MMSE-IRC cannot achieve good performance. Proposal #7: For 4x4 define requirements with either 2+1 or 2+2 rank combination with TDLA channel. |
| R4-2117435 | Apple | TP to TR 38.833: Interference Modeling for intra-cell inter-user interference |
| R4-2117738 | CMCC | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO |
| R4-2118001 | Intel Corporation | Observation #1: For PMI matrix selection for co-scheduled UE Option 1 (Select the precoder to ensure orthogonality) provides better performance in comparison to Option 2 (Random) for all considered scenarios and different receiver assumptions. Observation #2: For DMRS ports mapping for scenario with Rank 1+1 configuration Option 2 (different CDM groups mapping) and Option 3 (variable CDM groups mapping) allows to verify correct interfere-plus-noise covariance matrix estimation for 2 Rx UE for scenarios with different CDM groups for target and interference UEs. Observation #3: Test setup for Option 3 (variable CDM groups mapping) is more complicated in comparison to Option 2 (different CDM groups mapping) for DMRS ports mapping for scenario with Rank 1+1 configuration. Observation #4: For scenario with Rank 2 MCS 19 serving PDSCH and TDL-C channel model, MMSE-IRC does not allow to achieve maximum throughput in case of Rank 2 interference PDSCH signal Proposal 1: Consider the following assumptions for MU-MIMO modelling for requirements definition: - Precoder selection for interference UE: Option 1 (Select the precoder to ensure orthogonality) - DMRS ports mapping DMRS ports for case with rank 1+1: - First priority: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Second priority: Variable DMRS port mapping - Rank for target and interference PDSCH for 4 Rx UE test - First priority: Rank 2 (Target UE) + Rank 2 (Co-schedule UE) - Second priority: Rank 2 (Target UE) + Variable rank (Co-scheduled UE) - Same DMRS scrambling ID for target UE and co-scheduled UE Observation #5: For scenario with 2 TX antenna - MMSE-IRC performance benefits over MMSE-MRC is rather close for different propagation conditions for the most of scenarios except 4 Rx with MCS 13 - MMSE-IRC performance benefits over MMSE-MRC for scenario with 1+1 rank configuration and MCS 4 is not higher than 1 dB Observation #6: For scenario with 4 TX antenna - MMSE-MRC cannot reach the 70% of maximum throughput for most of the considered cases - MMSE-IRC cannot reach the 70% of maximum throughput for scenarios the following scenarios: - TDL-C channel model, Rank 2 Interference UE signal, MCS 13 target PDSCH and random PMI selection options - TDL-C channel model, Rank 2 Interference UE signal and MCS 19 target PDSCH Proposal 2: Consider the following assumptions for General PDSCH parameters: - Correlation model: ULA Low - Propagation conditions: TDL-C - MCS: MCS 13 in case of target UE Rank 1 and Rank 2 - Signal power assumptions: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal - SNR assumptions: SNR = (STargetUE+ SInterfUE)/N |
| R4-2118002 | Intel Corporation | TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO |
| R4-2118410 | Ericsson | Observation 1: No performance difference for configuring the interference UE between the same CDM group and different CDM groups for rank(1,1). Observation 2: No performance difference between same and different scrambling ID when paired UEs are in the same CDM group. Proposal 1: RAN4 to test rank (2, 1) and (2, 2) either independently or using a combination. Proposal 2: RAN4 to define the MMSE-IRC performance based on random PMI selection for both target and interference UE in intra-cell inter-users, with ensuring the selected PMI matrix shall not be identical to the precoding matrix applied for the UE under test. Proposal 3: RAN4 to define the MU-MIMO rank 1 test case with variable DMRS port mapping. - The number of CDM groups shall be 2 for same CDM group - Example: the modulation symbols of the signal under test are mapped to port 0, and the modulation symbols of the signal for interfering UE are mapped randomly onto antenna port 1, 2, or 3. Proposal 4: RAN4 to define the MU-MIMO rank 1 test case with variable scrambling ID configurations. Proposal 5: RAN4 to not define TDLC300-100 for the case 64QAM, 2layers on target UE. Proposal 6: RAN4 to consider the following MCS to define the requirements. - Rank 1: MCS 13 - Rank 2: MCS 13 for Rank 2 interference signal and MCS 19 for Rank 1 interference signal Proposal 7: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal for signal power assumption. Proposal 8: Use SNR = (STargetUE+ SInterfUE)/N for SNR assumption. |
| R4-2118411 | Ericsson | Observation 1: Compared with MRC, IRC receiver can have about 2dB gain in 70% of maximum throughput SNR point. |
| R4-2118412 | Ericsson | draftTP to TR38.833: receiver structure for intra-cell inter-user IRC |
| R4-2118863 | China Telecom | Observation 1: The reported RI for the co-scheduled UE mainly depends on its estimated SINR and channel condition. Therefore, companies’ system-level simulation will be necessary to find such a reasonable percentage before adopting it in the real test case. Proposal 1: Not to consider variable rank for the co-scheduled UE. Observation 2: In the situation that we use rank 2 for the target UE and rank 1 for the co-scheduled UE, if we assume BS signal power is equally allocated for each layer, target UE’s SNR will be 3dB larger than that of the co-scheduled UE. We have concern on whether BS will pair such 2 UEs within the real MU-MIMO transmission. Proposal 2: For the signal power assumption, the average total signal power should be the same for each paired UE regardless of the rank allocation. Proposal 3: Ok to use rank 2+2 to save the phase I simulation workload, and it can also avoid the misalignment on the different power allocation for each rank. Proposal 4: Support to cover both option 1 and option 2 for phase I evaluation, also ok to select one option in the phase II both considering the simulation result and realistic BS implementation. Observation 3: With using the same CDM group, the Rnn estimation accuracy will be decreased. Therefore, using the same CDM group will have additional test point in terms of Rnn estimation. Proposal 5: Support to cover both same and different CDM groups for phase I, to cover our study on both possible DMRS ports allocation in the TR. Proposal 6: Use same CDM group in the phase II to test the Rnn estimation accuracy. Proposal 7: As for the proposed variable DMRS port mapping, encourage companies to have more discussion on how option 3 can be performed the real test based on FRC. Proposal 8: Set the number of CDM groups without data configuration as 1. Proposal 9: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. Proposal 10: Use TDLC300-100 channel model to better verify the Rnn estimation accuracy for the IRC receiver. |
| R4-2118864 | China Telecom | TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario |
| R4-2119046 | Huawei, HiSilicon | Observation 1: Selecting orthogonal PMI matrix for paired UEs during the test is not near to the real network behaviour without emulating the network’s real precoding process. Observation 2: The target SNR for most cases with random PMI selection is at reasonable range based on our simulation results Observation 3: Without network assistance, interference plus noise covariance matrix can’t be estimated correctly if number of CDM groups without data of target UE 1 is set to 1 and number of CDM groups without data of co-scheduled UE 2. Observation 4: Interference + noise covariance can be calculated by using two CDM groups for Option 2; While for Option 1, with the assumption that number of CDM groups without data for paired UEs is 1, it can be calculated by using only one CDM group, which will bring slight performance degradation. Observation 5: Scrambling ID configuration has negligible effect on performance. Observation 6: Different scrambling configuration will cause extra signalling overhead for the test without any performance changes. Proposal: Define the intra cell inter user MMSE-IRC receiver with following assumptions: - Propagation conditions: TDLC300-100 - PMI selection for interference UE: Random and Select the PMI matrix randomly from the codebook to ensure it is not equal to PMI matrix of target UE. - Same scrambling ID - MIMO correlation: ULA Low as baseline and XPL Low if time allows - Rank1+1: - Same CDM groups - Number of CDM groups without data: 1 - MCS13 - Rank2+1: - MCS19 - Rank2+2: - MCS13 - Signal power assumptions: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal - SNR assumptions: SNR = (STargetUE+ SInterfUE)/N |
| R4-2119047 | Huawei, HiSilicon | For rank 1+1： Observation 1: There is no obvious much gain for MMSE-IRC receiver over MMSE-MRC receiver for MCS4, but much gain can be achieved for MCS 13. Observation 2: The performance for case with different CDM groups is better than that with same CDM groups. For rank 2+1: Observation 3: The performance gain is always obvious for MMSE-IRC receiver over MMSE-MRC receiver. For rank 2+2: Observation 4: MMSE-IRC can’t work for case with MCS 19 under TDLC300-100. Observation 5: MMSE-IRC always has obvious performance gain over MMSE-MRC. |
| R4-2119049 | Huawei, HiSilicon | TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver |
| R4-2119402 | MediaTek inc. | TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO |
| R4-2119405 | MediaTek inc. | Observation 1: Except the case 2+2 (MCS19, TDLC300-100), all cases including 1+1, 2+1 and 2+2 with MMSE-IRC can attain the maximum achievable throughput and have compatible performance gain compared with MMSE-MRC receiver. Proposal 1: For rank 1+1, consider the test case with MCS13. For rank larger than 2, consider the test case with rank 2+2 only. Proposal 2: Use only TDLA30-10 to define test cases. Proposal 3: Select the PMI matrix from the codebook of co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal. Proposal 4: Consider same CDM group for target UE and interference UE for the case with rank 1+1. Proposal 5: Consider to set “Number of CDM groups without data” to 1 for the case with rank 1+1. Proposal 6: Consider different scrambling sequence for all cases or variable scrambling ID during the test. |
| R4-2119549 | Qualcomm Incorporated | Proposal 1: Consider only rank combinations of 1+1 and 2+2 in the evaluation. Proposal 2: Use different CDM group for target and interfering UE in 1+1 case. Proposal 3: Use random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality. Proposal 4: Use same DMRS scrambling ID for all co-scheduled UEs. Proposal 5: Consider only TDLA30-10 for the evaluation. |

## Open issues summary

### Sub-topic 3-1: Inter-user interference modeling

Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2

- Background

  - Option 1: Rank 2 (Target UE) + Rank 1 (Co-schedule UE)

  - Option 2: Rank 2 (Target UE) + Rank 2 (Co-schedule UE)

  - Option 3: Rank 2 (Target UE) + Variable rank (Co-scheduled UE)

- Proposals

  - Option 1 (Apple, Huawei): Rank 2 (Target UE) + Rank 1 (Co-schedule UE)

  - Option 2 (Apple, Intel - First Priority, China Telecom, Huawei, MediaTek, Qualcomm): Rank 2 (Target UE) + Rank 2 (Co-schedule UE)

  - Option 3 (Intel - Second Priority): Rank 2 (Target UE) + Variable rank (Co-scheduled UE)

  - Option 4 (Ericsson): Option 1 and Option 2 or Option 3

- Recommended WF

  - Check whether RAN4 can use Option 2

Issue 3-1-2: Precoder selection for interference UE

- Background

  - Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

  - Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

    - Option 2A : Use following method to randomly select PMI matrix for interference UE that is not identical to that of Target UE for rank 2+1

      - 1) Randomly select the PMI matrix in codebook with rank2 and rank1 respectively.

      - 2) Normalize the PMI matrix for each layer for both target UE and co-scheduled UE to make the norm of each PMI matrix of each layer equal to 1/3.

      - 3) If the PMI matrix of rank 1 equals to the PMI matrix of any one layer of rank 2, reselect PMI matrix for Rank 1 with PMI index plus 1 and go back to step 2)

  - Option 3: Cover both Option 1 and Option 2 for phase 1 evaluation.

- Proposals

  - Option 1 (Apple, Intel, MediaTek, Qualcomm): Select the precoder to ensure orthogonality

  - Option 2 (Ericsson, Huawei): Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

  - Option 3 (China Telecom): Support to cover both option 1 and option 2 for phase I evaluation, also ok to select one option in the phase II both considering the simulation result and realistic BS implementation.

- Recommended WF

  - Check the following procedure:

    - Cover both option 1 and option 2 for phase I evaluation

    - Downselection from option 1 and option 2 for phase II is FFS

  - Check views on downselection criteria

Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration

- Background

  - Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

  - Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

  - Option 3: Variable DMRS port mapping during the test.

    - FFS the percent of each mapping and other details

    - Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE

- Proposals

  - Option 1 (Apple, China Telecom – for phase I and II, Huawei, MediaTek): DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

  - Option 2 (Intel - first priority, China Telecom – for phase I, Qualcomm): DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

  - Option 3 (Intel - second priority, Ericsson): Variable DMRS port mapping during the test.

    - China Telecom: As for the proposed variable DMRS port mapping, encourage companies to have more discussion on how option 3 can be performed the real test based on FRC.

- Recommended WF

  - Check the following procedure:

    - Cover both option 1 and option 2 for phase I evaluation

    - Downselection from option 1, 2 and 3 for phase II is FFS

  - Check views on downselection criteria

Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group

- Background

  - Option 1: 1 for target UE and co-scheduled UE.

  - Option 2: 2 for target and co-scheduled UE

    - Note: It depends on issue with DMRS ports mapping.

- Proposals

  - Option 1 (Apple, China Telecom, Huawei, MediaTek): 1 for target UE and co-scheduled UE.

  - Option 2 (Ericsson): 2 for target and co-scheduled UE.

- Recommended WF

  - Check the following WF

    - Use Option 1 in case Option 1 is agreed for Issue 3-1-3

    - Use Option 2 in case Option 3 is agreed for Issue 3-1-3

Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE

- Background

  - Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

  - Option 2: Same scrambling ID for all cases

  - Option 3: Configure variable scrambling ID during the test. FFS the details

- Proposals

  - Option 1 (China Telecom): Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

  - Option 2 (Intel, Huawei, Qualcomm): Same scrambling ID for all cases

  - Option 3 (Ericsson for Rank 1 tests, MediaTek): Configure variable scrambling ID during the test.

  - Option 4 (MediaTek): Different scrambling sequence for all cases

- Recommended WF

  - Check views on options above

### Sub-topic 3-2: PDSCH parameters

Issue 3-2-1: MIMO correlation for each UE

- Background

  - Option 1: Use ULA Low for 2TX and XPL Low for 4TX

    - FFS the correlation matrix for XPL Low

  - Option 2: ULA Low

- Proposals

  - Option 1 (Apple, Intel, Huawei): ULA Low

    - Huawei: ULA Low as baseline and XPL Low if time allows

- Recommended WF

  - Check whether RAN4 can use ULA Low as baseline

Issue 3-2-2: Propagation condition

- Background

  - Option 1: Only TDLA30-10

  - Option 2: Only TDLC300-100

  - Option 3: Further down select based on analysis

- Proposals

  - Option 1 (Apple, MediaTek, Qualcomm): Only TDLA30-10

  - Option 2 (Intel, China Telecom, Huawei): Only TDLC300-100

  - Option 3 (Ericsson): RAN4 to not define TDLC300-100 for the case 64QAM, 2layers on target UE.

- Recommended WF

  - Check views on options above.

Issue 3-2-3: MCS for target UE

- Background

  - For case with rank 1+1

    - Option 1: MCS 13

    - Option 2: Not consider 16QAM/MCS 13

  - For case with rank 2+1(if introduced)

    - Option 1: 13

    - Option 2: 19

  - For case with 2+2(if introduced)

    - Option 1: 13

    - Option 2: 19

    - Option 3: Not consider 64QAM

- Proposals

  - For case with rank 1+1

    - Option 1 (Apple, Ericsson, Huawei, Mediatek): MCS 13

  - For case with rank 2+1

    - Option 1 (Intel): 13

    - Option 2 (Ericsson, Huawei): 19

  - For case with rank 2+2

    - Option 1 (Intel, Ericsson, Huawei): 13

- Recommended WF

  - Check the following WF

    - Rank 1+1: MCS 13

    - Rank 2+1 (if introduced): Further discuss between MCS 13 and 19

    - Rank 2+2 (if introduced): MCS 13

Issue 3-2-4: Signal power assumptions

- Background

  - Option 1: Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1

  - Option 2: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- Proposals

  - Option 1 (Apple, Intel, Ericsson, Huawei): Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

  - Option 2 (China Telecom): For the signal power assumption, the average total signal power should be the same for each paired UE regardless of the rank allocation.

- Recommended WF

  - Check views on options from Proposals section

Issue 3-2-5: SNR assumptions

- Background

  - Option 1: SNR = STargetUE/N

  - Option 2: SNR = (STargetUE+ SInterfUE)/N

- Proposals

  - Option 1 (Apple, Intel, Ericsson, Huawei): SNR = (STargetUE+ SInterfUE)/N

- Recommended WF

  - Check whether RAN can use the following assumptions: SNR = (STargetUE+ SInterfUE)/N

## Companies views’ collection for 1st round

### Open issues

#### Sub-topic 3-1: Inter-user interference modeling

| Company | Comments |
| --- | --- |
| Apple | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 We are fine to go with Option 2. Issue 3-1-2: Precoder selection for interference UE We provided results for both options for phase 1 evaluation and suggest using option 1 for requirements definition. We observe that in many cases with TDLC channel random precoder doesn’t achieve max TP at reasonable SNR. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We support option 1 or option 2. We don’t support option 3 as it makes test setup complicated. We are not ok with having option 3 as an option in phase 2 as it was not evaluated in study phase, and we need to first evaluate it. Issue 3-1-4: We support option 1. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Option 2. We think in practical network same scrambling IDs would be used for co-scheduled UEs. |
| CMCC | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 Ok with Option 2. Issue 3-1-2: Precoder selection for interference UE Option 2 is preferred. We think it is more practical. Also ok with the procedure in recommended WF. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group The recommended WF should also add the condition ‘in case Option 2 is agreed’ - Use Option 1 in case Option 1 is agreed for Issue 3-1-3 - Use Option 2 in case Option 3 and Option 2 are agreed for Issue 3-1-3 |
| China Telecom | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 Support to go with Option 2 to avoid misalignment for Issue 3-2-4. Issue 3-1-2: Precoder selection for interference UE Support the recommended WF. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Cover both option 1 and option 2 for phase I evaluation. And prefer option 2 for requirement definition to test the Rnn estimation accuracy under IUI. Prefer to exclude option 3, the rank 2 percentage is hard to be decided and it is not clear how to perform the test based on FRC. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group We support option 1. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Option 1. We do not think UE should assume the network have the same scrambling ID if the co-scheduled UE is in the different CDM group. |
| Intel | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 We support to consider Option 2 for requirements definition Issue 3-1-2: Precoder selection for interference UE We support recommended way forward and we think that it is aligned with previous agreements. As for down selection for Phase II, we suggest to use Option 1 because for several considered scenarios using of Option 2 doesn’t allow to achieve the maximum throughput or even 70% of maximum throughput with MMSE-IRC receiver. We think that it is closer to practical condition where scheduler tries to select PMIs for co-scheduled UEs to improve the system performance. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We are fine with recommended WF. As for downselection from option 1, 2 and 3, we think that Option 3 can be considered as good way forward taking into account such diverse views. As for issue from China Telecom, we don’t see any issue with FRC in case Number of CDM groups without data will be equal to 2. We are also fine not to consider Option 3, in case RAN4 can reach consensus on Option 1 or Option 2. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group We are fine with correction from CMCC. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Option 1 or Option 2 is fine for us. |
| Qualcomm | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 Ok with Option 2. Issue 3-1-2: Precoder selection for interference UE Similar comments as Intel and Apple. Prefer to define requirements only based on Option 1. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We slightly prefer Option 2 because it will improve the quality of channel estimation since target and co-scheduled UEs are in different CDM group. We strongly oppose Option 3 since that is not practical. Issue 3-1-4: Ok with recommended WF with CMCC’s correction. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Prefer Option 2. We also think that in practical network, same scrambling IDs would be used for co-scheduled UEs. Also, as RAN4 is considering to define the requirements for enhanced receivers in future where joint channel estimation of target and co-scheduled UE will be required, i.e., need to have same scrambling ID, we prefer to have baseline requirements defined in Rel-17 to be also based on same scrambling ID so that RAN4 can show gains for the same setup. We strongly oppose Option 3 since that is not practical. |
| Ericsson | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 Option 3. From our understanding, all the rank 2 combinations are possibly deployed and important to real network. Thus, we suggest defining requirements for all the possible rank combinations. To reduce the number of test cases, RAN4 may consider to merge the interfering configurations. For example, - Probability of occurrence of transmission rank in interfering cells - 80% for Rank 1 and 20% for Rank 2 If companies don’t want to introduce a variable test case, our 2nd priority is to cover both option 1 and option 2. From our evaluation, there is great performance difference comparing rank combination 2+1 and 2+2. Besides, 64QAM is not suitable for rank (2,2) since it can not reach the maximum throughput, but it works for rank (2,1). Therefore, we suggest to define one test case for rank (2,2) with MCS=13 and one test case for rank (2,1) with MCS=19. Issue 3-1-2: Precoder selection for interference UE Option 2. RAN4 has agreed an important criterion on how to down selection of test as follow. Criteria for down selection of test parameters - The agreed baseline performance evaluation metric, i.e., evaluate the gain of MMSE-IRC over MMSE in terms of 70% max throughput performance of the target UE, under the same simulation setup, will be used for down-selection of critical test parameter for both phase I and phase II.. Based on our simulation for orthogonal PMI selection, there is no performance gain for IRC receiver over MRC receiver. Furthermore, in real deployment, the network is unlikely only based on the target UE’s feedback to decide the PMI (e.g., UL reference signal for TDD). Network cannot always guarantee to choose the optimal paired UEs for MU-MIMO which is different with the intention of option 1. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Our 1st priority is to have a test to cover all the possible DMRS ports combination for interfering UE. An example to define the test case is: the modulation symbols of the signal under test are mapped to port 0, and the modulation symbols of the signal for interfering UE are mapped randomly onto antenna port 1, 2, or 3. We’re also fine with recommended WF. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group Fine with recommended WF. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Option 3. We’re also fine with option 1. There is no performance for same or different DMRS scrambling IDs. It’s better to test whether UE supports both same and different scrambling IDs. |
| MediaTek | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 Support the recommended WF. Issue 3-1-2: Precoder selection for interference UE Prefer Option 2. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Slightly prefer Option 1. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group OK with the recommend WF and we are fine with the correction form CMCC. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE We think it is more practical to have different scrambling ID but we can compromise to Option 1. |
| ZTE | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 For defining performance requirements, Option 2 is fine. Issue 3-1-2: Precoder selection for interference UE Option 2. Orthogonality cannot be always guaranteed, so if we go for Option 1, we may have too optimistic performance gains Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration Fine with the recommended WF. Issue 3-1-4: Fine with the recommended WF. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE Option 1: same scrambling ID for the same CDM group, and different scrambling ID for different CDM groups. |
| Huawei | Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 We prefer to consider both option 1 and option 2 to cover all typical scenarios in real network @ Ericsson. Based on our understanding, it is feasible to use variable rank configurations for interference in inter cell scenario because different interference rank don’t have impact on performance. While for MU-MIMO, interference ranks have large impact on performance, so it is better to separate the test into different cases with different rank configurations to verify UE supporting different interference. Issue 3-1-2: Precoder selection for interference UE We support option 2 because: - Random PMI selection leads to larger interference so larger performance gain of IRC over MRC can be observed compared to orthogonal PMI selection - From our simulations, 70% of maximum throughput with random PMI selection can be reached for all cases except rank 2+2 with TDLC300-100 and MCS19. - It is impossible to always schedule paired UEs with ideal orthogonal PMI only based on UE feedback. Too optimistic and strict performance requirements will be defined based on ideal orthogonal PMI selection that deviates from the minimum requirements definition that RAN4 should focus on. Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration We support option 1 because: - Channel estimation with interference of co-scheduled UE can be verified. - Interference plus noise covariance matrix estimation with one CDM groups can be verified. While above two aspects can’t be verified for Rank 1. We don’t support option 3 since the test setup is complex, while the test purpose can be achieved with fixed number of DM-RS ports. Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group We support option 1 As discussed in our paper, different Number of CDM groups without data configurations for paired UEs will cause incorrect Ruu estimation. From the point of resource utilization, we prefer option 1. Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE We support option 2 because: - Different DMRS scrambling ID needs high layer signaling which will bring additional test complexity - Scrambling ID configuration has negligible impact on performance if legacy IRC processing is used. - We agree with QC that it helps to compare the performance for E-MMSE-IRC with MMSE-IRC in Rel-18 performance enhancement stage. |

#### Sub-topic 3-2: PDSCH parameters

| Company | Comments |
| --- | --- |
| Apple | Issue 3-2-1: MIMO correlation for each UE ULA Low for all. We don’t see a difference between XPL-Low and ULA low if the parameters are set to 0 for XPL-Low. Issue 3-2-2: Propagation condition We support option 1. We don’t think TDLC300-100 is suitable for MU-MIMO where typically PMI feedback from UEs are used at gNB for precoder selection. Also with MMSE-IRC, TDLSC has degraded performance in some cases. Issue 3-2-3: MCS for target UE We support the recommended WF. Issue 3-2-4: Signal power assumptions Option 1 Issue 3-2-5: SNR assumptions We support the recommended WF. |
| CMCC | Issue 3-2-1: MIMO correlation for each UE Support ULA low. Issue 3-2-2: Propagation condition TDLA30-10 and TDLC300-100 can be covered by different test cases. We can take Option 1 as compromise. Issue 3-2-4: Signal power assumptions Option 2, we share similar view with China Telecom, while if we use RANK 2+RANK 2, then Option 2 is equal to Option 1. Issue 3-2-5: SNR assumptions Option 1 is clearer to us. We can compromise to Option 2 if other companies concern about the definition. |
| China Telecom | Issue 3-2-1: MIMO correlation for each UE We need check on the correlation matrix for ULA low and XPL low under the proposed 1=0, 2=0, =0 and . We can come back to this in the second round. China Telecom Update: Based on our calculation, we are fine to only consider ULA low. Issue 3-2-2: Propagation condition Support TDLC300-100 to better verify the Rnn estimation accuracy. Issue 3-2-3: MCS for target UE Based on companies’ simulation results, with rank 2+2 and MCS 19, the SNR point @70% TP is either very high or N/A. Therefore, we are ok to only consider MCS13 for rank 2+2 case for phase II. Same time, we suggest to capture our simulation results for both MCS13 and MCS19 in the TR for phase I. Issue 3-2-4: Signal power assumptions The only concern we have on option 1 is that, if Rank 2 (Target UE) + Rank 1 (Co-schedule UE) is used, With Option 1, the co-scheduled UE’s SNR will be 3dB lower than the target UE, we are not sure if it is a practical UE pairing. Issue 3-2-5: SNR assumptions We support the recommended WF. |
| Intel | Issue 3-2-1: MIMO correlation for each UE Support recommended WF Issue 3-2-2: Propagation condition In case requirements will be defined with orthogonal precoder, we suggest to consider TDL-C channel model, because the higher MMSE-IRC gains are observed for several scenarios. In case requirements will be defined with random precoder, we suggest to consider TDL-A channel model because MMSE-IRC can not reach the 70% on max throughput for scenario with TDL-C and Rank 2+2 configuration, based on our results. Issue 3-2-3: MCS for target UE Recommended WF is fine for us. As for Rank 2+1, in case this scenario will be considered as dedicated test, we are fine use MCS 19. Same time, in case scenario with variable rank will be considered, same MCS should be used for scenario with Rank 2+1 and Rank 2+2. Therefore, we suggest to use MCS 13 for this case. Issue 3-2-4: Signal power assumptions We assume that Option 1 is more typical assumption because we consider equal power split between layers. In case Option 2 is considered, it means that per layer signal quality for different UEs will be different and we think that it is rather unfair assumption. Issue 3-2-5: SNR assumptions Support recommended WF. |
| Qualcomm | Issue 3-2-1: MIMO correlation for each UE Same comment as Apple. Ok with recommended WF. Issue 3-2-2: Propagation condition Prefer option 1. Low mobility scenario is more friendly to MU-MIMO since reported PMI will be more relevant in that case compared to higher Doppler scenario. Issue 3-2-3: MCS for target UE Ok with recommended WF. Prefer MCS13 for all cases. Issue 3-2-4: Signal power assumptions Prefer Option 1. Issue 3-2-5: SNR assumptions We prefer Option 1. Regardless of the option chosen, the performance is not going to change. It’s just a matter of how we capture it in the spec. In the end, RAN4 is defining the requirements for target UE. So, in our opinion, we should define the requirement based on target UE SNR only and define the power ratio between the two UEs as discussed in Issue 3-2-4. We believe that this way will be clearer to the reader of the spec. |
| Ericsson | Issue 3-2-1: MIMO correlation for each UE Option 1. Same view as Apple. Issue 3-2-2: Propagation condition Option 3. Based on our simulation results, it seems TDLC is not suitable for rank2 test, but it can achieve max Tput for rank 1 case. Thus, we suggest to define rank 1 with TDLC300-100 and rank 2 with TDLA30-10. Issue 3-2-3: MCS for target UE We support the recommended WF. Issue 3-2-4: Signal power assumptions Fine with option 1. Issue 3-2-5: SNR assumptions We support the recommended WF. |
| MediaTek | Issue 3-2-1: MIMO correlation for each UE Support the recommended WF. Issue 3-2-2: Propagation condition Prefer Option 1. We also think low mobility environment is more suitable for MU-MIMO. Issue 3-2-3: MCS for target UE OK with the recommended WF. |
| Huawei | Issue 3-2-1: MIMO correlation for each UE Ok with recommended WF. Issue 3-2-2: Propagation condition We think that TDLC300-100 should be covered: - For some cases, performance gain for TDLC300-100 of IRC over MRC is higher than that of TDLA30-10 - Performance for scenarios with large Doppler should be verified. Issue 3-2-3: MCS for target UE Based our simulation results, we support: For rank 1+1: MCS 13 For rank 2+1: MCS 19 For rank 2+2: MCS 13 Issue 3-2-4: Signal power assumptions Option 1 Issue 3-2-5: SNR assumptions Option 1 |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117435 | Intel: TP can be revised based on outcome of Issue 3-1-2 and Issue 3-2-4. Another comments: - NL is used in the text and NLc is used in equation. Probably it is better to align naming. Based on our understanding, to align per layer power, we need to apply scaling sqrt(NLi/NLc) to each Wi, because each Wi has normalization coefficient 1/sqrt(NLi*NTX) |
|  | China Telecom: 1) It may be clearer to put the ‘NL is the total number layers across all users’ into the second paragraph, since it is the first time NL appears. 2) Based on the last meeting’s agreement, we will capture in the TP the background of not selecting 8Tx or 16Tx. So we can add the following wording within the interference modeling part: ‘RAN4 recognize that under practical MU-MIMO scenario, BS with larger antenna scales like 8Tx, 16TX is more likely to be used. However, with the random precoder selection method for the target and the co-scheduled UEs, the performance for 8Tx and 16Tx is too poor to show performance gain for the inter-user interference suppressing receiver. Therefore, for this study, we use 2Tx and 4Tx to serve the same purpose. For UEs capable of inter-user interference suppression ability discussed in this TR, they can also be used in the deployments with larger number of Tx ports configuration.’ |
|  | Intel2: Update of our previous comment. We’ve realized that for Phase I we plan to cover scenarios with different precoder options. Therefore, it is better to provide details of random and orthogonal PMI selection methodologies. |
|  | Apple2: Thanks for the comments, Intel. We plan to add random and orthogonal in the revision. Thanks China Telcom for the comment. We don’t recall such agreement to add this in TR. Could you please point it out? Perhaps this is not correct for interference modeling part. |
| R4-2117738 | Intel: TP will be revised to include the observations of simulation results |
|  | Company B |
| R4-2118002 | Intel: TP will be revised to include the simulation results |
|  | Qualcomm: A minor suggestion from our side: Please provide simulation results spreadsheet as an attachment instead of embedding it since MCC doesn’t allow embedded documents. |
|  | Intel2: Thank you for information. We will take it into account in the revised version. |
| R4-2118412 | Apple: We don’t think the sub-clause for Network assistance and QCL information is needed. Re-wording suggestion: In this section, we provide details of the enhanced receiver for evaluating performance in the presence of inter-cell intra-user interference. |
|  | China Telecom: Regarding the interference plus noise matrix estimation method, the current TP only include the situation when the 2 paired UEs’ DMRS using the same CDM group. When the 2 paired UEs’ DMRS using the different CDM group, in our understanding, the Rnn can be estimated by simply power estimation on the other REs. So, related to Issue 3-1-3, if we agree to cover both DMRS port assignation, we think both Rnn estimation methods may need to be covered. |
| R4-2118864 | Intel: TP can be revised to include the outcome of Phase I study. |
|  | Qualcomm: 10CBW and 40CBW should be 10MHz CBW and 40MHz CBW. |
|  | China Telecom: Will be updated based on more consensus on phase I parameters and the outcome will be added. To QC: Thanks for the comment and it will be revised. |
| R4-2119049 | Intel: Correlation model can be updated based on outcome of Issue 3-2-1. Based on our understanding, MCS 4 is also considered for Phase I evaluation. Therefore, we can include it in the simulation assumptions. |
|  | Qualcomm: 2+1/2+2 may have to be modified based on outcome of Issue 3-1-1. |
|  | China Telecom: We are also fine to capture MCS4 as a phase I evaluation. |
|  | Intel2: @Qualcomm. We think that for Phase I (i.e. results which will be captured in TP) we can cover 2+1 and 2+2 based on agreement from R4-2108666. Same time, we will continue the discussion on scenarios for requirements definition which is handled by Issue 3-1-1. |
| R4-2119402 | China Telecom: To align with R4-2117435, better to add a sub-clause 4.1.1 for scenario. |
|  | Company B |

## Summary for 1st round

### Open issues

#### Sub-topic 3-1: Inter-user interference modeling

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Rank of interference PDSCH for rank of target UE is 2 | Tentative agreements: N/A Candidate options: - Option 1 (Apple, CMCC, China Telecom, Intel, Qualcomm, MediaTek, ZTE): Only Rank 2 (Target UE) + Rank 2 (Co-schedule UE) - Option 2 (Ericsson – First Priority): Rank 2 + Variable Rank - Option 3 (Ericsson – Second Priority, Huawei): Rank 2 + Rank 1 and Rank 2 + Rank 2 GTW agreement: Option 1. Recommendations for 2nd round: N/A |
| Issue 3-1-2: Precoder selection for interference UE | Tentative agreements: Cover both option 1 and option 2 for phase I evaluation and make further downselection between these options for Phase II Candidate options: - Option 1 (Apple, Intel, Qualcomm, MediaTek): Select the precoder to ensure orthogonality - Ericsson: Based on our simulation for orthogonal PMI selection, there is no performance gain for IRC receiver over MRC receiver. - Option 2 (CMCC, Ericsson, ZTE, Huawei): Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE. - Apple: We observe that in many cases with TDLC channel random precoder doesn’t achieve max TP at reasonable SNR. - Intel: For several considered scenarios using of Option 2 doesn’t allow to achieve the maximum throughput or even 70% of maximum throughput with MMSE-IRC receiver - Huawei: From our simulations, 70% of maximum throughput with random PMI selection can be reached for all cases except rank 2+2 with TDLC300-100 and MCS19. Recommendations for 2nd round: Continue discussion in the second round and check companies results. |
| Issue 3-1-3: DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration | Tentative agreements: Cover both option 1 and option 2 for phase I evaluation and make further downselection between option 1, 2 and 3 for Phase II Candidate options: - Option 1 (Apple, Intel, MediaTek, Huawei): DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group - Option 2 (Apple, China Telecom, Intel, Qualcomm): DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups - Option 3 (Intel, Ericsson): Variable DMRS port mapping during the test. Recommendations for 2nd round: Continue discussion taking into account 1st round comments - Apple: We are not ok with having option 3 as an option in phase 2 as it was not evaluated in study phase, and we need to first evaluate it. - China Telecom: And prefer option 2 for requirement definition to test the Rnn estimation accuracy under IUI. - Qualcomm: We slightly prefer Option 2 because it will improve the quality of channel estimation since target and co-scheduled UEs are in different CDM group. - Ericsson: Our 1st priority is to have a test to cover all the possible DMRS ports combination for interfering UE. An example to define the test case is: the modulation symbols of the signal under test are mapped to port 0, and the modulation symbols of the signal for interfering UE are mapped randomly onto antenna port 1, 2, or 3. - Huawei: We support option 1 because: - Channel estimation with interference of co-scheduled UE can be verified. - Interference plus noise covariance matrix estimation with one CDM groups can be verified. While above two aspects can’t be verified for Rank 1. We don’t support option 3 since the test setup is complex, while the test purpose can be achieved with fixed number of DM-RS ports. |
| Issue 3-1-4: Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group | Tentative agreements: N/A Candidate options: - Option 1 (Apple, China Telecom, Huawei): 1 for target UE and co-scheduled UE. - Option 2 (CMCC, Intel, Qualcomm, Ericsson, MediaTek, ZTE): - Use Option 1 (1 for target UE and co-scheduled UE) in case Option 1 is agreed for Issue 3-1-3 - Use Option 2 (2 for target UE and co-scheduled UE) in case Option 2 or Option 3 is agreed for Issue 3-1-3 Recommendations for 2nd round: Check whether we can consider Option 2 and make decision based on outcome of Issue 3-1-3 |
| Issue 3-1-5: DMRS scrambling ID for target UE and co-scheduled UE | Tentative agreements: N/A Candidate options: - Option 1 (China Telecom, Intel, Ericsson, MediaTek, ZTE): Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2 (Apple, Intel, Qualcomm, Huawei): Same scrambling ID for all cases - Option 3 (Ericsson): Configure variable scrambling ID during the test. Recommendations for 2nd round: Continue discussion and check whether RAN4 can use Option 1. |

#### Sub-topic 3-2: PDSCH parameters

|  | Status summary |
| --- | --- |
| Issue 3-2-1: MIMO correlation for each UE | Tentative agreements: ULA Low as baseline Recommendations for 2nd round: N/A |
| Issue 3-2-2: Propagation condition | Tentative agreements: N/A Candidate options: - Option 1 (Apple, CMCC, Intel for random PMI, Qualcomm, MediaTek): Only TDLA30-10 - Option 2 (China Telecom, Intel for orthogonal PMI, Huawei): Only TDLC300-100 - Option 3 (Ericsson): Rank 1 with TDLC300-100 and Rank 2 with TDLA30-10. Recommendations for 2nd round: Continue discussion taking into account 1st round comments - Apple: We don’t think TDLC300-100 is suitable for MU-MIMO where typically PMI feedback from UEs are used at gNB for precoder selection. Also with MMSE-IRC, TDLSC has degraded performance in some cases. - China Telecom: Support TDLC300-100 to better verify the Rnn estimation accuracy. - Intel: In case requirements will be defined with random precoder, we suggest to consider TDL-A channel model because MMSE-IRC can not reach the 70% on max throughput for scenario with TDL-C and Rank 2+2 configuration, based on our results. - Qualcomm: Low mobility scenario is more friendly to MU-MIMO since reported PMI will be more relevant in that case compared to higher Doppler scenario. - Ericsson: Based on our simulation results, it seems TDLC is not suitable for rank2 test, but it can achieve max Tput for rank 1 case. - MediaTek: We also think low mobility environment is more suitable for MU-MIMO. - Huawei: For some cases, performance gain for TDLC300-100 of IRC over MRC is higher than that of TDLA30-10. Performance for scenarios with large Doppler should be verified. |
| Issue 3-2-3: MCS for target UE | Tentative agreements: Use the following MCS for requirements definition - For case with rank 1+1: MCS 13 - For case with rank 2+1: FFS between MCS 13 and 19 - For case with rank 2+2: MCS 13 Candidate options: - For case with rank 2+1 - Option 1 (Intel, Qualcomm): 13 - Option 2 ([Ericsson], Huawei): 19 Recommendations for 2nd round: Continue discussion |
| Issue 3-2-4: Signal power assumptions | Tentative agreements: N/A Candidate options: - Option 1 (Apple, Intel, Qualcomm, Ericsson, Huawei): Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal - Option 2 (CMCC, China Telecom): For the signal power assumption, the average total signal power should be the same for each paired UE regardless of the rank allocation. Recommendations for 2nd round: Discuss this issue as a part of TP preparation. |
| Issue 3-2-5: SNR assumptions | Tentative agreements: N/A Candidate options: - Option 1 (CMCC, Qualcomm): SNR = STargetUE/N - Qualcomm: Regardless of the option chosen, the performance is not going to change. It’s just a matter of how we capture it in the spec. In the end, RAN4 is defining the requirements for target UE. So, in our opinion, we should define the requirement based on target UE SNR only and define the power ratio between the two UEs as discussed in Issue 3-2-4. We believe that this way will be clearer to the reader of the spec. - Option 2 (Apple, CMCC, China Telecom, Intel, Ericsson, Huawei): SNR = (STargetUE+ SInterfUE)/N GTW agreement: Option 2 with further working on the clarification when defining performance requirements into specification. Recommendations for 2nd round: N/A |

## Discussion on 2nd round

### WFs comments collection

| WF number | Comments collection |
| --- | --- |
| R4-2120709 | China Telecom: Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE • Depends on issue with DMRS ports for case with rank 1+1. If same CDM groups is used, set number of CDM groups without data to 1, otherwise, set it to 2. CTC: We generally agree with this agreement, but we prefer FFS on this issue if variable DMRS port mapping is used. Signal power assumptions CTC: For this issue, currently we can accept to keep open and have the additional agreement that ‘no need to be discussed in phase II based on the agreed rank 1+1 and rank 2+2 test setup’ Which is more close to the status we are. |
|  | Huawei: For your first comment, we modify it to FFS for in case of variable DMRS port mapping is used but based from our understanding, it would be better to use 2 CDM groups without data to avoid different tbSize. For your second comment, we delete the issue “Signal power assumptions” for discussion since we have agreed to not consider rank 2+1 |
|  | Intel: Based on second round discussion, we have suggestion to modify the title for the following issue “Number of CDM groups without data configuration for case with rank 1+1if same CDM group is agreed for target UE and co-scheduled UE” to make it more clear, because under this issue now we cover scenarios with same and different CDM groups. |
|  | Qualcomm: In the below snippet, we are not sure what “otherwise” case means since we already have values for same and different CDM groups. So, highlighted part should be removed. Number of CDM groups without data configuration for case with rank 1+1 • Depends on issue with DMRS ports for case with rank 1+1. If same CDM groups is used, set number of CDM groups without data to 1, if different CDM groups are used, set number of CDM groups without data to 2,otherwise, FFS. |
|  | Huawei: Thanks for the comments. "Otherwise" means variable DMRS port mapping based on the comments from CMCC that there is no agreement for such configuration. |
|  | Qualcomm: Thank you for the explanation. Current wording is fine with us. |

### Open issues comments collection

Issue 1. Precoder selection for interference UE

- Option 1: Select the precoder to ensure orthogonality

  - Ericsson: Based on our simulation for orthogonal PMI selection, there is no performance gain for IRC receiver over MRC receiver.

- Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

- Option 3: Cover both in different cases. e.g. Random for rank 1+1 and orthogonal for rank 2+2

| Company | Comment |
| --- | --- |
| CMCC | Option 2. We also observed that Option 2 achieves larger performance gain. |
| Huawei | Option 2. We have following reasons: 1. Random PMI selection leads to larger interference so larger performance gain of IRC over MRC can be observed compared to orthogonal PMI selection 2. From our simulations, 70% of maximum throughput with random PMI selection can be reached for all cases except rank 2+2 with TDLC300-100 and MCS19. 3. In real network, BS always derive precoding matrix by SRS-based channel estimation for TDD and PMI feedback from UE for FDD which can guarantee good orthogonality of signal and interference in the receiving side. But orthogonal PMI selection without any prior information of channel can’t guarantee the good orthogonality at the receiving side so it is not impractical in real world. Therefore, from the point of test, too optimistic and strict performance requirements will be defined based on orthogonal PMI selection that deviates from the minimum requirements definition that RAN4 should focus on. |
| China Telecom | We support option 1 and option 3 is also fine for us. According to the agreed simulation assumption, i.e., Rank 2+2 and MCS 13, orthogonal PMI selection can achieve better performance than random selection. Reasonable MMSE-IRC performance gain can be shown when rank 2+2 from all companies’ simulation results. |
| MediaTek | We prefer Option 1. We agree with Huawei that, without any prior information of channel, it cannot guarantee orthogonality between target UE and co-scheduled UE with orthogonal PMI selection. However, we also think it is not practical to select purely random but not identical PMI. gNB would try to select PMIs for co-scheduled UEs to improve the system throughput. |
| Intel | First priority – Option 1. Second priority – Option 3. Based on third comments from Huawei, we think that under typical conditions BS has information to generate the precoder to reduce the inter-UE interference. Also, based on results for Rank 2+2 with TDL-C channel model, we can observe that using of orthogonal precoder will lead to significant MMSE-IRC performance improvement. |
| Qualcomm | Prefer Option 1. Similar comment as MediaTek. |
| Apple | We support option 1. In actual deployments PMI feedback would be used and some orthogonalization to reduce inter-user interference would be employed, hence it would be reasonable to assume orthogonal precoders. We observe that for some cases random precoder doesn’t reach max TP at reasonable SNR, especially with TDLC300-100 channel. The performance with orthogonal precoder is better than random in all cases. We are not defining requirements to compare the performance gain between MMSE and MMSE-IRC. With MCS13 MMSE doesn’t even reach 70% max TP in most cases. |
| Ericsson | Option 2. Because option 2 is close to the real network deployment. Network cannot always guarantee to choose the optimal paired UEs for MU-MIMO which is different with the intention of option 1. |

Issue 2. DMRS ports for 1 target and 1 interfering UE scenario and 1+1 rank configuration

- Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

- Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

- Option 3: Variable DMRS port mapping during the test.

| Company | Comment |
| --- | --- |
| Huawei | We support option 1 because: 1. Channel estimation with interference of co-scheduled UE can be verified. 2. Interference plus noise covariance matrix estimation with one CDM groups can be verified. While above two aspects can’t be verified for Rank 2+2. We don’t support option 3 since the test setup is complex, while the test purpose can be achieved with fixed number of DM-RS ports. |
| China Telecom | Support Option 1 and we prefer not consider option 3. We share same view with Huawei. |
| MediaTek | Option 1. |
| Intel | Based on our understanding, these two aspects mentioned by Huawei for Option 1 are already verified by Rel-15 PDSCH requirements, because the processing is same for SU-MIMO and MU-MIMO test cases. Same time, with Option 2, we can verify correct interference estimation for 2 Rx UE which is not covered by Rel-15 requirements. We think that to cover different aspects for 2 Rx UE testing, it is better to consider Option 3. |
| Qualcomm | We can compromise to Option 1, if needed. But we strongly oppose Option 3. We don’t want to unnecessarily complicate the test setup. |
| Apple | We don’t support option 3 as it makes test setup very complicated. Option 1 or option 2 is fine. Slight preference for option 1 since different aspects not covered in 4x4 test cases will be tested. |
| Ericsson | We prefer to cover both option 1 and option 2 for phase I evaluation and make further down selection between option 1, 2 and 3 for Phase II. |

Issue 3. Number of CDM groups without data configurations for case with rank 1+1 that target UE and co-scheduled UE are located in the same CDM group

- Option 1: 1 for target UE and co-scheduled UE.

- Option 2:

  - Use Option 1 (1 for target UE and co-scheduled UE) in case Option 1 is agreed for Issue 2

  - Use Option 2 (2 for target UE and co-scheduled UE) in case Option 2 or Option 3 is agreed for Issue 2

| Company | Comment |
| --- | --- |
| Huawei | Option 2 is fine for us |
| China Telecom | Option 2 is fine for us. But we prefer FFS if option 3 in issue2 is used. |
| MediaTek | Option 2. |
| Intel | We are fine with Option 2. But probably the Issue title should be updated to “Number of CDM groups without data configurations for case with rank 1+1”, because for scenario with Option 2 in Issue 2 we don’t have target UE and co-scheduled UE are located in the same CDM group. |
| Qualcomm | Ok with Option 2. |
| Apple | Ok with option 2 |
| Ericsson | OK with option 2. |

Issue 4. DMRS scrambling ID for target UE and co-scheduled UE

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Option 2: Same scrambling ID for all cases

- Option 3: Configure variable scrambling ID during the test.

| Company | Comment |
| --- | --- |
| Huawei | We support option 2 because: 1. Different DMRS scrambling ID needs high layer signaling which will bring additional test complexity 2. Scrambling ID configuration has negligible impact on performance if legacy IRC processing is used. 3. Same scramble ID helps to compare the performance for E-MMSE-IRC with MMSE-IRC in Rel-18 performance enhancement stage. |
| China Telecom | We can agree with using same scrambling ID at least when paired UEs are in the same CDM group. If paired UEs are in different CDM groups, we need more feedback from infra vendors whether gNB will always configure the same scrambling ID for all the UEs within the cell. |
| MediaTek | As there is a possibility to have different scrambling ID in the paired UEs, we prefer Option 1 for test coverage. @Huawei: 1. We think that it won’t increase the test complexity. For the target UE, it does not need to change its scrambling ID, e.g., ID_1. We just need to use different scrambling ID, e.g., ID_2, for the co-scheduled UE. For the both case of same or different scrambling ID, target UE only needs to know its own scrambling ID. 2. We agree that the performance difference is small between same and different scrambling ID. Hence, if there is no concern on test complexity. Why can’t we go for different scrambling ID? |
| Intel | Options 1 and 2 are fine for us. |
| Qualcomm | Prefer Option 2. It is simpler test setup and will also help comparing the performance with E-MMSE-IRC requirements in future release. |
| Apple | We prefer option 2. Same reason as Qualcomm. |
| Ericsson | We are fine with option 1 and 3. There is no performance for same or different DMRS scrambling IDs. It’s better to test whether UE supports both same and different scrambling IDs. |

Issue 5. Propagation condition

- Option 1: Only TDLA30-10

- Option 2: Only TDLC300-100

- Option 3: Rank 1 with TDLC300-100 and Rank 2 with TDLA30-10.

| Company | Comment |
| --- | --- |
| CMCC | We think Option 1 is more suitable for MU-MIMO scenario, while larger MMSE-IRC gain can be achieved by Option2. Option 3 can be our compromise. |
| Huawei | Option 2, We think that TDLC300-100 should be covered: 1. For some cases, performance gain for TDLC300-100 of IRC over MRC is higher than that of TDLA30-10 2. Performance for scenarios with large Doppler should be verified, if UE pass the test with TDLC, it can pass the test with TDLA |
| China Telecom | Support option 2 for better verify the Rnn estimation under frequency selective channel condition. Option 3 can be our compromise. |
| MediaTek | We think Option 1 is more suitable for MU-MIMO. Also, based on the simulation results, the gain for TDLA30-10 of IRC over MRC is large enough to verify the UE performance. |
| Intel | We support Option 1 with random PMI and Option 2 with orthogonal PMI. Option 3 is good way forward solution. |
| Qualcomm | Prefer Option 1 as low mobility scenario is more suitable for MU-MIMO because PMI reporting is more relevant by the time it gets applied to ensure orthogonality. |
| Apple | We prefer option 1. Same reason as MediaTek, Qualcomm. |
| Ericsson | Option 3. |

Issue 6. MCS

- Recommended WF: Use the following MCS for requirements definition

  - For case with rank 1+1: MCS 13

  - For case with rank 2+2: MCS 13

| Company | Comment |
| --- | --- |
| CMCC | The recommended WF is fine for us |
| Huawei | The recommended WF is fine for us |
| China Telecom | The recommended WF is fine for us |
| MediaTek | The recommended WF is fine for us |
| Intel | WF is fine for us |
| Qualcomm | Ok with recommended WF. |
| Apple | Ok with recommended WF. |
| Ericsson | OK with recommended WF. |

Issue 7. Signal power assumptions (Discuss this issue as a part of TP preparation.)

- Option 1: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- Option 2: For the signal power assumption, the average total signal power should be the same for each paired UE regardless of the rank allocation

| Company | Comment |
| --- | --- |
| CMCC | Since we agree to define requirements for RANK1+1 and RANK2+2, from our view, Option 1 is equal to Option 2. We can compromise to Option 1. |
| Huawei | Option 1 is more feasible for clarification in TP |
| China Telecom | We have agreed to use RANK1+1 and RANK2+2, no need to discuss in phase II. For rank 2+1 for phase I, as the chairman suggested in the GTW session, we can keep open and capture all the simulation results in the TR with either option company used. |
| Intel | We have additional comment for concern from China Telecom in the first round about 3 dB SNR difference for different UEs in scenario with Rank 2 + Rank 1. We think that it is rather reasonable assumption, because if UEs are assigned with different Ranks then it means that such UEs have different quality of propagation conditions and it is not clear why UEs with same SNR conditional will be scheduled with the different Ranks. Probably it is better to align on this issue to simplify the capturing of 2+1 results in the TP. |
| Qualcomm | Prefer Option 1. Similar comment as Intel for power difference. We are ok to capture the different assumptions in TP. |
| Apple | We support option 1 for Phase 2 requirements definition. Agree with Intel and Qualcomm on power difference. we can capture the simulation results in TR with different assumptions. Request companies to indicate the assumptions used in their simulation results to understand which results to group/ average together. |
| Huawei 2 | We would like to provide our understanding for this issue and CTC’s concern: Based from our understanding, we should also consider the interference between layers for SNR definition. Take rank 2+1 as an example, n For, option 1, assume the power of each layer is S, noise power is N, the SINR of each UE is equal to S/(2S+N). n For option 2, assume the power of each layer of target UE is S, power of interference UE is2S, noise is N, then SINR of target UE is S/(3S+N), the SINR of interference UE is 2S/(2S+N). We can observe that it is fairer to allocate the same power for each layer rather than each UE considering the same SINR can be achieved for each UE |
| Ericsson | We prefer to go with option 1 in phase II, and further indicate the assumption in simulation results if needed. |
| China Telecom 2 | Thank you for the comments and discussion. Our main concern is that it may be not practical for BS to pair UEs with different channel conditions within the same group. Additional power for the co-scheduled UE just to ensure same SNR per UE. We understand the situation that may be there is no company provide simulation results with option 2 power assumption. And if so, we can list the 2 options within the simulation assumption part of the TR and point out in the result summary part that only option 1 is used. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2120710 (Revision of R4-2117435) | China Telecom: 2) Based on the last meeting’s agreement, we will capture in the TP the background of not selecting 8Tx or 16Tx. So we can add the following wording within the interference modeling part: ‘RAN4 recognize that under practical MU-MIMO scenario, BS with larger antenna scales like 8Tx, 16TX is more likely to be used. However, with the random precoder selection method for the target and the co-scheduled UEs, the performance for 8Tx and 16Tx is too poor to show performance gain for the inter-user interference suppressing receiver. Therefore, for this study, we use 2Tx and 4Tx to serve the same purpose. For UEs capable of inter-user interference suppression ability discussed in this TR, they can also be used in the deployments with larger number of Tx ports configuration.’ |
|  | Apple2: We don’t recall such agreement to add this in TR. Could you please point it out? Perhaps this is not correct for interference modeling part. |
|  | China Telecom: To Apple, the agreement is captured in last meeting’s chairman note as follows: Agreement: Consider 2 Tx and 4Tx only for defining RAN4 test cases – Include the background information for the decision of choosing 2Tx/4Tx for this test case into TR. – With above test configuration, there is no limitation for the number Tx ports configuration deployed by NW under MU-MIMO scenario. Our first thinking was to capture our proposed wording in the interference model section since it was the first time NTX appears. Or it may be also reasonable to put it in the simulation parameter section. We are open to discuss with other companies. |
|  | Intel: @Apple: Based on our comment from the first round, we suggest to have scaling sqrt(NLi/NL) for each W. Because W1 has scaling factor sqrt(1/(NL1*NTx)) and W2 has scaling factor sqrt(1/(NL2*NTx)), but W should have scaling factor sqrt(1/(NL*NTx)). @China Telecom: We think that it is better to capture this information in TP from MediaTek (Scenario for inter-user interference suppression for MU-MIMO) or in TP from Huawei (Simulation assumptions), because it related not only to interference modelling. |
|  | Apple2 : Thanks for clarification @Intel. We have provided updated version. @China Telecom Thanks for the clarification and additional information. We should probably capture this in simulation assumptions section/ TP |
|  | China Telecom2: Thanks Intel and Apple for the comments. We can also agree to put this part into the simulation assumptions TP, |
|  | Huawei: We have difference understanding of random precoder specified in R4-2120710 In TP R4-212070, we copy the following wording: Random precoder: W2 is randomly selected from the codebook ensuring that it is not identical to W1 It may be “W2 is randomly selected from the codebook ensuring that no column of W2 is identical to any column of W1”. |
| R4-2120711 (Revision of R4-2117738) | China Telecom: We think companies’ results with‘Inf’ for the MRC, and with exact SNR value for the IRC receiver, should also be counted as‘Performance gain is observed’. |
|  | Intel: We think that probably it will be clearer to provide the observations based on average results rather than per company observations. Also, it is better to differentiate performance gain which is less than 1 dB and which is higher than 1 dB to identify scenarios with testable MMSE-IRC performance. |
|  | Apple2: We suggest to provide conclusions based on average results. Also, it should be ULA Low for all cases. XPL Low is not defined in 38.101-4. |
|  | CMCC: All the comments above are captured in our TP-v3 For Intel and Apple's comments, we captured those in this latest TP, the observations are based on average results, and give the information of whether the performance gain is larger or smaller than 1dB. For CTC's comment, we give the observations for the cases that ‘Inf’ for the MRC, and with exact SNR value for the IRC receiver" as "the performance gain is observed, the average gain is [TBA]" |
|  | Intel: We think that probably for the following scenarios we have certain alignment that MMSE-MRC can not reach the 70% of Max throughput: - For RANK 1+1, 2T2R ULA low MIMO configuration, MCS 13 with same CDM group and different CDM groups - For RANK 2+1, 4T4R ULA low MIMO configuration, MCS 19 Therefore, we suggest to add the following sentence “MMSE-MRC cannot reach 70% of maximum throughput and MMSE-IRC gain cannot be estimated” instead of “the performance gain is observed, the average gain is [TBA]” for the above scenarios. |
| R4-2120712 (Revision of R4-2118002) | CMCC: For draft R4-2119048, some calculation formulas contain error, such as the MMSE-MRC Span/Avg in AC8, AC12, AE8, AE12 and so on. Maybe further check is needed, and the TP should also be updated accordingly. |
|  | Intel: Based on offline check with CMCC, formulas are correct in the latest version of draft R4-2119048 and align with procedure which we described above. |
| R4-2120713 (Revision of R4-2118412) | Apple: Propose re wording In this section, we provide the system equations for evaluating the performance of inter-cell intra-user interference mitigation. In 2nd para of 4.2.1 editorial- co-scheduled UE's |
|  | Qualcomm: In the 1st paragraph, it should be “intra-cell inter-user interference” instead of “inter-cell intra-user”. |
|  | Huawei: Thank for preparing the TP. The receiver structure seems not correct Based from our understanding, djk,l should be NLjx1 vector where NLj is number of layers of UEj, Hjk,l, j={2,…NUE} should be (NRx x NLj) channel matrix. If Hjk,l is not include the precoding matrix descripted in the TP, it should be same for all the UEs |
|  | Ericsson: One additional comment to Apple’s comments: In 2nd para of 4.2.1 editorial- co-scheduled UE's Current wording subject to general situations(multiple co-scheduled UEs if introduced) and we also indicate that Nue = 2 in the following paragraph. So we think here we don’t need to modify the ‘ UEs’ ’to ‘ UE’s ’ |
| R4-2120714 (Revision of R4-2118864) | Qualcomm: “MMSE-IRC receiver without network processing is considered as the reference receiver structure. DMRS based interference covariance estimation is assumed.” In the above sentence, network processing should be replaced by “network assistance”. |
|  | Apple: We only have evaluation results for FDD in our understanding. The conclusion should be updated accordingly. |
|  | China Telecom: Since many of the result summary are ‘TBA’ in the R4-2120711 TP, we also leave the conclusion for simulation result as ‘TBA’. |
| R4-2120715 (Revision of R4-2119049) | Intel: Probably it is also better to keep 2+1 scenario for simulation assumptions in TP for Phase I, because we have collection of such simulation results and it is aligned with our previous agreement that 1+1, 2+1 and 2+2 will be considered for Phase I evaluation. |
|  | Apple: Do we need to have 2 options for scr ID? We don’t think companies submitted simulation results with both or at least this is not differentiated in the results. |
|  | China Telecom: Based on the discussion in R4-2120710, the suggested wording for 8Tx/16Tx are captured in the updated v2 version. |
| R4-2120716 (Revision of R4-2119402) |  |

# Topic #4: CR work split

In this section we collect companies views on CR work split and plan the assignment.

## Open issues summary

Moderator note: Each company can provide the company name to the table below in the row with interested topic and section. In case, company have any comments on missing section, work split of the section or other issues, please use the form in Section 4.2.

| Topic | Section | Company |
| --- | --- | --- |
| MMSE-IRC receiver for inter-cell interference – Demodulation requirements | General and applicability section | [Apple], Huawei |
|  | PDSCH requirements Moderator note: This part can be split into two blocks – FDD and TDD depending on number of volunteers | Apple, CMCC |
|  | [Annex A: FRC] Moderator note: Necessity is based on outcome of Issue 1-2-1 | [Intel] |
|  | Annex B: Interference modelling assumptions | Nokia |
| MMSE-IRC receiver for inter-cell interference – CSI requirements | General and applicability section | Huawei |
|  | CQI requirements Moderator note: This part can be split into two blocks – FDD and TDD depending on number of volunteers | China Telecom, Qualcomm Ericsson |
|  | [PMI requirements] Moderator note: Necessity is based on outcome of Issue 2-1-1 | Ericsson |
| MMSE-IRC receiver for intra-cell inter-user interference | General and applicability section | ZTE |
|  | PDSCH requirements Moderator note: This part can be split into two blocks – 2 RX and 4 RX depending on number of volunteers | [CMCC], China Telecom |
|  | [Annex A: FRC] Moderator note: Necessity is based on outcome of Issue 3-2-3 | [Intel] |
|  | Annex B: MU-MIMO Beamforming Model | [Intel], Huawei |

## Companies views’ collection for 1st round

| Company | Comments |
| --- | --- |
| XXX |  |
| CMCC | We are happy to take care of PDSCH TDD or FDD requirements for inter-cell interference demodulation. If only one company is needed for requirement part, we are also happy to prepare the PDSCH requirements for intra-cell inter-user interference demodulation. |
| China Telecom | Thanks for initiating the work split. We are happy to take the CR drafting work for the CQI requirements for the MMSE-IRC receiver for inter-cell interference – CSI requirements. And we can also take the PDSCH requirements for MMSE-IRC receiver for intra-cell inter-user interference. |
| Intel | We have also included the possible additional section for MU-MIMO: FRC. |
| ZTE | We are interested in intra-cell inter-user interference, and would like to take General and applicability section. |

## Summary for 1st round

Tentative plan for CR work split is presented below.

| Topic | Section | Company |
| --- | --- | --- |
| MMSE-IRC receiver for inter-cell interference – Demodulation requirements | General and applicability section | [Apple, Huawei] |
|  | PDSCH requirements - FDD | Apple |
|  | PDSCH requirements - TDD | CMCC |
|  | [Annex A: FRC] Moderator note: Based on current status, R.PDSCH.1-2.1 FDD and R.PDSCH.2-2.1 TDD can be reused. Therefore, definition of new FRC is not needed. We can double check it in 2nd round. |  |
|  | Annex B: Interference modelling assumptions | Nokia |
| MMSE-IRC receiver for inter-cell interference – CSI requirements | General and applicability section | Huawei |
|  | CQI requirements - FDD | Qualcomm |
|  | CQI requirements - TDD | Ericsson |
|  | [PMI requirements] Moderator note: Based on 1st round discussion, this section is not needed |  |
| MMSE-IRC receiver for intra-cell inter-user interference | General and applicability section | ZTE |
|  | PDSCH requirements – 2 Rx | CMCC |
|  | PDSCH requirements – 4 Rx | China Telecom |
|  | Annex A: FRC Moderator note: At least we need to define FRC for MCS13 with Rank 2 and 2 CDM groups | Intel |
|  | Annex B: MU-MIMO Beamforming Model | [Intel, Huawei] |

Recommendations for 2nd round:

- Confirm plan and resolve the conflict for two sections

- Check the following Moderator suggestion to balance the workload:

| Topic | Section | Company |
| --- | --- | --- |
| MMSE-IRC receiver for inter-cell interference – Demodulation requirements | General and applicability section | [Apple, Huawei] |
|  | PDSCH requirements - FDD | Apple |
|  | PDSCH requirements - TDD | CMCC |
|  | [Annex A: FRC] Moderator note: Based on current status, R.PDSCH.1-2.1 FDD and R.PDSCH.2-2.1 TDD can be reused. Therefore, definition of new FRC is not needed. We can double check it in 2nd round. |  |
|  | Annex B: Interference modelling assumptions | Nokia |
| MMSE-IRC receiver for inter-cell interference – CSI requirements | General and applicability section | Huawei |
|  | CQI requirements - FDD | Qualcomm |
|  | CQI requirements - TDD | Ericsson |
|  | [PMI requirements] Moderator note: Based on 1st round discussion, this section is not needed |  |
| MMSE-IRC receiver for intra-cell inter-user interference | General and applicability section | ZTE |
|  | PDSCH requirements – 2 Rx | [CMCC -> Intel] |
|  | PDSCH requirements – 4 Rx | China Telecom |
|  | Annex A: FRC Moderator note: At least we need to define FRC for MCS13 with Rank 2 and 2 CDM groups | Intel |
|  | Annex B: MU-MIMO Beamforming Model | [Intel, Huawei] |

## Discussion on 2nd round

Confirm plan and resolve the conflict for two sections

Check the following Moderator suggestion to balance the workload:

| Topic | Section | Company |
| --- | --- | --- |
| MMSE-IRC receiver for inter-cell interference – Demodulation requirements | General and applicability section | [Apple, Huawei] |
|  | PDSCH requirements - FDD | Apple |
|  | PDSCH requirements - TDD | CMCC |
|  | [Annex A: FRC] Moderator note: Based on current status, R.PDSCH.1-2.1 FDD and R.PDSCH.2-2.1 TDD can be reused. Therefore, definition of new FRC is not needed. We can double check it in 2nd round. |  |
|  | Annex B: Interference modelling assumptions | Nokia |
| MMSE-IRC receiver for inter-cell interference – CSI requirements | General and applicability section | Huawei |
|  | CQI requirements - FDD | Qualcomm |
|  | CQI requirements - TDD | Ericsson |
|  | [PMI requirements] Moderator note: Based on 1st round discussion, this section is not needed |  |
| MMSE-IRC receiver for intra-cell inter-user interference | General and applicability section | ZTE |
|  | PDSCH requirements – 2 Rx | [CMCC -> Intel] |
|  | PDSCH requirements – 4 Rx | China Telecom |
|  | Annex A: FRC Moderator note: At least we need to define FRC for MCS13 with Rank 2 and 2 CDM groups | Intel |
|  | Annex B: MU-MIMO Beamforming Model | [Intel, Huawei] |

| Company | Comments |
| --- | --- |

## Summary for 2nd round

Agreement on CR work split is presented below.

| Topic | Section | Company |
| --- | --- | --- |
| MMSE-IRC receiver for inter-cell interference – Demodulation requirements | General and applicability section | Apple |
|  | PDSCH requirements - FDD | Apple |
|  | PDSCH requirements - TDD | CMCC |
|  | Annex B: Interference modelling assumptions | Nokia |
| MMSE-IRC receiver for inter-cell interference – CSI requirements | General and applicability section | Huawei |
|  | CQI requirements - FDD | Qualcomm |
|  | CQI requirements - TDD | Ericsson |
| MMSE-IRC receiver for intra-cell inter-user interference | General and applicability section | ZTE |
|  | PDSCH requirements – 2 Rx | Intel |
|  | PDSCH requirements – 4 Rx | China Telecom |
|  | Annex A: FRC Moderator note: At least we need to define FRC for MCS13 with Rank 2 and 2 CDM groups | Intel |
|  | Annex B: MU-MIMO Beamforming Model | Huawei |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel Corporation |  |
| WF on CSI requirements for inter-cell interference MMSE-IRC | Ericsson |  |
| WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117435 | TP to TR 38.833: Interference Modeling for intra-cell inter-user interference | Apple | Revised |  |
| R4-2117738 | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO | CMCC | Revised |  |
| R4-2118002 | TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO | Intel Corporation | Revised |  |
| R4-2118412 | draftTP to TR38.833: receiver structure for intra-cell inter-user IRC | Ericsson | Revised |  |
| R4-2118864 | TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario | China Telecom | Revised |  |
| R4-2119049 | TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver | Huawei, HiSilicon | Revised |  |
| R4-2119402 | TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO | MediaTek inc. | Revised |  |

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2120707 | WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC | Intel Corporation | Agreeable |  |
| R4-2120707 | WF on CSI requirements for inter-cell interference MMSE-IRC | Ericsson | Agreeable |  |
| R4-2120709 | WF on MMSE-IRC receiver for intra-cell inter-user interference | Huawei, HiSilicon | Agreeable |  |
| R4-2120710 (Revision of R4-2117435) | TP to TR 38.833: Interference Modeling for intra-cell inter-user interference | Apple | Agreeable |  |
| R4-2120711 (Revision of R4-2117738) | TP to TR 38.833: Summary of link level evaluation for inter-user interference suppression for MU-MIMO | CMCC | Agreeable |  |
| R4-2120712 (Revision of R4-2118002) | TP to TR 38.833: Link level simulation results for inter-user interference suppression for MU-MIMO | Intel Corporation | Agreeable |  |
| R4-2120713 (Revision of R4-2118412) | draftTP to TR38.833: receiver structure for intra-cell inter-user IRC | Ericsson | Agreeable |  |
| R4-2120714 (Revision of R4-2118864) | TP to TR 38.833: Conclusion for phase I evaluation on inter-user interference suppression for MU-MIMO scenario | China Telecom | Agreeable |  |
| R4-2120715 (Revision of R4-2119049) | TP: Introduction of simulation assumptions for intra cell inter user MMSE-IRC receiver | Huawei, HiSilicon | Agreeable |  |
| R4-2120716 (Revision of R4-2119402 | TP to TR 38.833 Scenario for inter-user interference suppression for MU-MIMO | MediaTek inc. | Agreeable |  |