3GPP TSG-RAN WG4 Meeting # 101-e	R4-2120757

# Electronic Meeting, November 01-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.124 | CR | - | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 36.124: correction of the frequency range for the RI test, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It was observed, that the E-UTRA UE EMC specification is not following the IEC 61000-4-3 specification in terms of the frequency range applicable to the radiated immunity testing. Please note, that all other RAN4 EMC specifications already reflect the IEC 61000-4-3 correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the frequency range applicable to the Radiated Immunity testing (to be up to 6 GHz), as per IEC 61000-4-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | E-UTRA UE EMC specification would be misaligned with the IEC specification, and other RAN4 EMC specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 9.2, 9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 7.2	Immunity

Table 2: Immunity applicability

|  |  | Equipment test requirement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phenomenon | Application | Equipment connected to fixed AC or DC power installations | Equipment connected to vehicular DC supplies | Equipment powered by integral battery | Reference subclause in the present document | Reference standard |
| RF electromagnetic field (80 MHz to 6000 MHz) | Enclosure | applicable | applicable | applicable | 9.2 | IEC 61000-4-3 [17] |
| Electrostatic discharge | Enclosure | applicable | applicable | applicable | 9.3 | IEC 61000-4-2 [18] |
| Fast transients common mode | Signal and control ports, DC and AC power input ports | applicable | not applicable | not applicable | 9.4 | IEC 61000-4-4 [19] |
| RF common mode 0,15 MHz to 80 MHz | Signal and control ports, DC and AC power input ports | applicable | applicable | applicable | 9.5 | IEC 61000-4-6 [20] |
| Transients and surges, vehicular environment | DC power input ports | not applicable | applicable | not applicable | 9.6 | ISO 7637 Part 1 [6] And ISO 7637 Part 2 [7] |
| Voltage dips and interruptions | AC mains power input ports | applicable | not applicable | not applicable | 9.7 | IEC 61000-4-11 [21] |
| Surges, common and differential mode | DC and AC power input ports | applicable | not applicable | not applicable | 9.8 | IEC 61000-4-5 [22] |

----------------------------- Next modified section ------------------------------

## 9.2	RF electromagnetic field (80 MHz - 6000 MHz)

The test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of UE and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [17]:

-	for UE and ancillary equipment the following requirements shall apply;

-	the test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	the stepped frequency increments shall be 1 % of the momentary frequency;

-	when using the max hold detector method (see ANNEX A) at each test frequency step initially an unmodulated test signal shall be applied. Then the test modulation shall be applied;

-	the test shall be performed over the frequency range 80 MHz to 6000 MHz;

-	responses in stand alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	the frequencies selected during the test shall be recorded in the test report.

----------------------------- End of modified section ------------------------------