3GPP TSG-RAN WG4 Meeting #102-e              	R4-2203546

Electrical Meeting, 21th Feb-3rd March, 2022

Agenda item:	10.9.4.3.2

Source:	Samsung

Title:	Simulation results of UL timing adjustment requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on UL timing adjustment requirement for Rel-17 FR2 HST were further discussed and finalized. The related agreement was captured in the WF [1]

In this contribution, we just resubmitted our ideal and impairment simulation results for PUSCH UL timing adjustment requirement derived.

# 2	Simulation

In this section, the ideal and impairment simulation results are provided for alignment. The simulation assumption can be referred as following table.

Table 3-1: Simulation assumption for UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Channel bandwidth |  | 120KHz SCS: 50MHz and 200MHz |
| MCS |  | 16 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 0 |
|  | Additional DM-RS position | 1+0 1+1 1+1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | B |
|  | Allocation length | 10 |
| Frequency domain resource assignment | RB assignment | 50MHz CBW: 16RBs for each UE, Moving UE RBs: 0~15; Stationary UE RBs: 16~31 200MHz CBW: 66RBs for each UE, Moving UE RBs: 0~65, Stationary UE RBs: 66~131 |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 16 for 50MHz and 66 for 200MHz |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | For TDD: - last symbol in slot #3 in radio frames for 120KHz |
|  | SRS resource allocation | 120 kHz SCS: - 50MHz CBW (32RBs)~C_SRS =9, B_SRS=0 - 200MHz CBW (132RBs)~C_SRS =33, B_SRS=0 |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS= 10 respectively. |  |  |

Table 3-2: Ideal and impairment simulation results for UL timing adjustment requirements

| Tx/Rx | SCS, BW | MCS | Mapping type | Symbol length | DMRS configuration | PT-RS | Channel | RS assignment | SNR@70 of TP (ideal) | SNR@70 of TP (Impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 120KHz50MHz | MCS16 | B | 10 | 1+0 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~15 | 7.5 | 10.0 |
|  |  |  |  |  |  |  |  | Stationary UE: 16~31 |  |  |
| 1T2R | 120KHz50MHz | MCS16 | B | 10 | 1+1 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~15 | 7.1 | 9.6 |
|  |  |  |  |  |  |  |  | Stationary UE: 16-31 |  |  |
| 1T2R | 120KHz 50MHz | MCS16 | B | 10 | 1+1+1 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~15 | 7.3 | 9.8 |
|  |  |  |  |  |  |  |  | Stationary UE: 16~31 |  |  |
| 1T2R | 120KHz 200MHz | MCS16 | B | 10 | 1+0 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~65 | 7.2 | 9.7 |
|  |  |  |  |  |  |  |  | Stationary UE: 66~131 |  |  |
| 1T2R | 120KHz 200MHz | MCS16 | B | 10 | 1+1 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~65 | 7.2 | 9.7 |
|  |  |  |  |  |  |  |  | Stationary UE: 66~131 |  |  |
| 1T2R | 120KHz 200MHz | MCS16 | B | 10 | 1+1+1 | K_PTRS=2, L_PTRS=1 | AWGN | Moving UE: 0~65 | 7.2 | 9.7 |
|  |  |  |  |  |  |  |  | Stationary UE: 66~131 |  |  |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided for requirement derived

# References

R4-2203006, WF on BS demodulation requirement for FR2 HST,  Nokia, Samsung