3GPP RAN WG4 Meeting #102		R4-2203662

Electronic meeting, February 21 – March 3, 2022

Agenda item:	9.2.3

Source:	Apple

Title:	On the VLP mode for the NR-U operation

WI/SI:	NR_6GHz_unlic_full

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Following the publication of the 6GHz regulatory decision for different regions, such as US FCC and EU/CEPT, RAN plenary agreed several Rel-17 WIs hat aims at adding 3GPP support for the lower and full 6GHz band [2][3].

One of the unique features of the unlicensed 6GHz band is that regulatory requirements usually define several operational modes: standard power (SP) with the frequency usage coordination, low power indoor (LPI), and very low power (VLP) applicable to both indoor and outdoor. The existing 3GPP core NR-U functionality supports SP and LPI modes, and most technical discussions in Rel-16 and Rel-17 have been focusing on the LPI mode. The reason is that all the countries enable the LPI mode in the 6GHz band with similar regulatory requirements, whereas the SP mode can be used only in a few countries. As for the VLP mode, it was discussed several times during Rel-17, but no conclusion was reached on whether 3GPP can/should introduce its support.

In this discussion paper we present our further considerations on the VLP mode for the 6GHz band, and which technical aspects shall be discussed further to enable this mode of operation to the 3GPP core specifications.

# 2	VLP mode for the 6GHz band

As mentioned in the Introduction part, depending on the local regulatory requirements the 6GHz band can have different operation modes, such as SP, LPI and VLP. While the LPI mode is defined by all countries, which have decided to open this band for the license-exempt operation, the VLP mode support is not homogeneous sometimes having quite distinctive regulatory requirements. Figure 2-1 below presents a summary of different countries and which 6GHz band modes they allow in different frequencies.

Figure 2-1: Summary of the 6GHz modes in different countries.

Referring to the comments expressed for the VLP mode during Rel-17 technical discussions, the following aspects shall be carefully analysed further for the VLP mode:

-	Use cases that the 3GPP core specifications need to support. While the LPI mode suits well a use case with the AP/bridge and client devices, the VLP mode is more applicable to the client-to-client communication, whereupon quite many regulatory requirements even explicitly prohibit usage of AP/bridge devices.

-	Fluid regulatory requirements. There are countries/regions which are still in the process of refining the VLP mode parameters and releasing further updates. As an example, while the LPI mode was already supported some time ago in Chile, the VLP mode has been added only recently harmonising its usage with other LAM countries. The US FCC are also considering addition of the VLP mode to the 6GHz band.

-	3GPP power class for the VLP mode. This aspect was raised during ongoing Rel-17 discussions. Current NR-U functionality defines only PC5 with +20dBm transmission power. While PC5 can obviously support lower transmission power needed for VLP, there is an open question whether 3GPP has to introduce a new power class specifically designed for the VLP mode; and if so, whether it should be +14dBm or +17dBm.

-	Harmonised introduction of different regulatory requirements into the 3GPP specifications. As summarised in Annex A below, while most countries define maximum EIRP for the VLP mode as +14dBm, there are regions that allow +17dBm maximum transmission power.  Furthermore, the maximum PSD may vary a lot from country to country, such as -8dBm/MHz in Canada versus 11dBm/MHz in UK and no restrictions at all in Morocco and Chile. Even though it is in principle possible to support individual country VLP requirements, it would be very impractical and, when possible, similar requirements should be harmonised in the 3GPP specifications.

-	Minimisation of NS values. At least for the LPI mode there has been an intention to minimise number of new NS values. For instance, existing NS_53 was re-used for all LAM countries including Peru and Chile, even though these countries could have had a separate NS flag. So, new flags were added only for EU/CEPT, Canada and South Korea. Similarly, same exercise should be performed for the VLP mode in all countries where it is defined. In fact, with introduction of new LPI NS values – NS_59 and NS_60 – there will be only three spare values for band n96. Accounting for the fact that new NS values might be needed for LPI in other countries/regions, RAN WG4 should take a holistic approach on which new VLP NS values are needed unless we conclude that the NS field should be extended.

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Taking all these considerations into account, our view is that introduction of the VLP mode will require non-trivial technical discussions, which it will be impossible to conduct within remaining time in Rel-17 before the target release deadline in March 2022. Thus, if there is an interest in adding support for this mode to the 3GPP core specification, we suggest considering the corresponding WI for Rel-18.

Proposal:	Consider the VLP mode in a new spectrum Rel-18 WI for the 6GHz band.

# 3	Conclusions

In this discussion paper we have presented our further considerations on the so-called VLP mode for the 6GHz band. Our view is that introduction of the VLP mode will require non-trivial technical discussions, which it will be impossible to conduct within Rel-17 timeframe before the target release deadline in March 2022. Thus, if there is an interest in adding support for this mode to the 3GPP core specification, we suggest considering the corresponding WI for Rel-18.

Proposal:	Consider the VLP mode in a new spectrum Rel-18 WI for the 6GHz band.

# 3	References

RP-1912926, "WID on NR-based Access to Unlicensed Spectrum", Qualcomm Inc.

RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

RP-212302, "Introduction of operation in full unlicensed band 5925-7125MHz ", Apple

3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.13.0

Annex A: Summary of the 6GHz band regulations

| Region | Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP |  | 14dBm |  |  |
|  | UAE | LPI | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP |  | 14 dBm |  |  |
| Region 2 | US | SP | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP |  | 17 dBm |  |  |
|  | Columbia | LPI | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |