3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203720

# Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.24.2

Source:	Qualcomm Inc.

Title:	On NR SL relay RRM Requirement

Document for:	Discussion

# Introduction

In this contribution, we present our view on the open issues of R17 NR SL relay RRM requirement from [1].

# Discussion

## Interruptions from SL to Uu

The following issue was open from RAN4#101bis-e discussion

Whether to further consider the additional conditions on interruption requirements of SL relay?

Candidate options:

  - Option 1: The interruption requirements in this clause shall not apply if at least one of the following conditions is met:

- T310 timer is running for RLF on PCell,

- Performing candidate beam detection on PCell/serving cell as specified in section 8.5.5 and 8.5.6,

- While receiving paging and

- While receiving system information.

  - Option 2: NO, discuss the R17 impact just in R17 SL enhancement WID

Note that discover reconfiguration can be controlled by network as other RRC procedures when UE is in coverage. In SL relay scenario, when UE is out of coverage, Uu link is not active anymore and no interruption should be considered. Therefore, network has full control of discover reconfiguration when interruption is possible, and we don’t see the need for adding restrictions.

Observation 1: network has full control of discover reconfiguration when interruption is possible, and therefore there is no need to add restrictions.

Proposal 1: No need to specify the additional interruption avoidance rule for SL relay.

# Conclusions

Observation 1: network has full control of discover reconfiguration when interruption is possible, and therefore there is no need to add restrictions.

Proposal 1: No need to specify the additional interruption avoidance rule for SL relay.

# Reference

[1] R4-2202706