3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203761

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.6.5.3

Source:	Apple Inc.

Title:	Discussion on SDR requirements with 1KQAM

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e SDR requirements with 1024QAM in NR DL were discussed and way forward [1] was agreed.

- Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, where - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

In this contribution we present simulation results and propose suitable MCS values for SDR requirements with 1024QAM.

# 2. Discussion

In RAN4#101-e it was agreed to add MCS indices based on the methodology agreed in Rel-15 for SDR requirements.

  - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj)

Where MCSupperboundj was agreed as in table below.

Table 1: MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj |
| --- | --- | --- | --- |
| 1 | 10 | 1 | 25 |
| 1 | 10 | 0.8 | 21 |
| 1 | 10 | 0.75 | 19 |
| 1 | 10 | 0.4 | 9 |
| 2 | 10 | 1 | 25 |
| 2 | 10 | 0.8 | 21 |
| 2 | 10 | 0.75 | 19 |
| 2 | 10 | 0.4 | 9 |

The options for MCS index for 2RX and 4RX were discussed and captured in [1]:

- Add the MCS indexes for SDR test with 1024QAM based on Table 2 for 2Rx.

  - Rank 1

    - Option 1: MCS25

    - Option 2: MCS23

  - Rank 2

    - Option 1: MCS23

    - Option 2: Not define SDR tests

- Add the MCS indexes for SDR test with 1024QAM based on Table 3 for 4Rx.

  - Rank 1

    - Option 1: MCS25

    - Option 2: MCS23

  - Rank 2

    - Option 1: MCS23

    - Option 2: FFS

Using the simulation parameters from TS38.101-4 Table 5.5A-1, and TX EVM=2.5%, we evaluate the practical MCS with 1024QAM modulation order.

Figure 1: SDR results with 1024QAM with 2RX

Figure 1: SDR results with 1024QAM with 4RX

The summary of simulation results is provided in the table below.

Table 2: Simulation results for SDR with 1KQAM

| Num. RX | Num. Layers | MCS | SNR @85% Max TP |
| --- | --- | --- | --- |
| 2 | 1 | 23 | 27.4 |
|  |  | 24 | 29.5 |
|  |  | 25 | 31.5 |
|  | 2 | 23 | 30.5 |
|  |  | 24 | 32.5 |
|  |  | 25 | 34.6 |
| 4 | 1 | 23 | 23.5 |
|  |  | 24 | 25.5 |
|  |  | 25 | 28.7 |
|  | 2 | 23 | 27.6 |
|  |  | 24 | 29.0 |
|  |  | 25 | 32.5 |

For 2RX: With 1 layer, the operating SNR with MCS 23 is feasible, but with MCS 24 and 25 it’s on the higher side. With 2 layers, the operating SNR with all MCS is > 30 dB and doesn’t seem feasible.

Observation #1: With 2RX the operating SNR with 1 layer is feasible with MCS 23. The operating SNR with 2 layers and 2RX is high for all MCS.

For 4RX: With 1-layer operating SNR is reasonable for all MCS. With 2 layers operating SNR with MCS 23 is feasible, but with MCS 24 and 25 it’s on the higher side.

Observation #1: With 4RX the operating SNR is reasonable for all MCS with 1 layer. The operating SNR with 2 layers is feasible with MCS 23.

Based on the observations above we propose the following MCS index for SDR tests with 1024QAM:

MCS Index for 2RX

Rank 1: MCS 23

Rank 2: Do not introduce

MCS Index for 4RX

Rank 1: MCS 25

Rank 2: MCS 23

We propose to introduce the following MCS indices for SDR test with 1024QAM for 2RX and 4RX.

Table 2	MCS indexes for SDR test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 23 | 23 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |

Table 3	MCS indexes for SDR test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

Proposal #1: For 1024QAM SDR requirements with 2RX use MCSpracticalj as 23 for 1 layer, no requirements for 2 layers. The MCS index table is introduced as:

MCS indexes for SDR test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 23 | 23 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |

Proposal #2: For 1024QAM SDR requirements with 4RX use MCSpracticalj as 25 for 1 layer, 23 for 2 layers. The MCS index table is introduced as:

MCS indexes for SDR test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# 3. Conclusion

In this paper, we provide we present simulation results and propose suitable MCS values for SDR requirements with 1024QAM. Our observations and proposals are captured below:

Observation #1: With 2RX the operating SNR with 1 layer is feasible with MCS 23. The operating SNR with 2 layers and 2RX is high for all MCS.

Observation #1: With 4RX the operating SNR is reasonable for all MCS with 1 layer. The operating SNR with 2 layers is feasible with MCS 23.

Proposal #1: For 1024QAM SDR requirements with 2RX use MCSpracticalj as 23 for 1 layer, no requirements for 2 layers. The MCS index table is introduced as:

MCS indexes for SDR test with 1024QAM for 2Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 23 | 23 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |

Proposal #2: For 1024QAM SDR requirements with 4RX use MCSpracticalj as 25 for 1 layer, 23 for 2 layers. The MCS index table is introduced as:

MCS indexes for SDR test with 1024QAM for 4Rx

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.