3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203866

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.10.2.2

Source: 		Qualcomm Incorporated

Title: 			RRM requirements for HO with PSCell

Document for:	Discussion

# 1.	Introduction

In the last RAN4 meeting, the RRM requirements for HO with PSCell has been discussed and agreed in [1]. However, there are open issues from the last meeting. Therefore, we continue to share our views of the RRM requirements for HO with PSCell in this paper.

# 2. 	Discussion

During the previous meetings, RAN4 agreed that defining separate delay requirements for PScell addition and HO delay requirements. However, there is still uncertainty about the terminology.

PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

- Proposal 1. PScell addition delay requirements are only applicable at NR-SA to ENDC. Otherwise HO delay requirements are applied for both Pcell HO and PScell changes with different parameters accordingly.

PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms,	 for NR-SA to ENDC

HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms,  	otherwise

Issue: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel

- Proposal 2. Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [5,10]  ms.

- Proposal 3. Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) in general. Except T_processing=30ms for NR-SA to ENDC.

Candidate options from the previous meeting is replicated as

- Option 1: (CATT, Apple, Huawei, OPPO, Xiaomi, Ericsson)-> case study - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) - Option 1a: (Huawei, MTK, vivo) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [X] ms. - X=FFS and can be different for different HO with PSCell scenarios. - Option 2a: (Ericsson, Intel) - Tprocessing applies independently for PCell and PSCell, and follows legacy (20ms or 40ms depending on whether target is same or different FR). - Tprocessing = 20ms for NR target with NR source cell in same FR (without FR mode switch) - Tprocessing = 40ms for NR target with NR source cell in different FR (FR mode switch) - Tprocessing = 40ms for NR target without NR source cell (inter-RAT HO, or PSCell addition) - Tprocessing = 20ms for E-UTRA target with E-UTRA source cell - Tprocessing = 40ms for E-UTRA target without E-UTRA source cell (inter-RAT HO, or PSCell addition) - Option 3: (MTK, Huawei) - Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + 10ms - Option 4: (Qualcomm) - Tprocessing = 30ms for NR-SA to EN-DC - Due to UE mobility processing on dual cell groups, even for parallel processing based latency requirements, the total latency should be relaxed based on the following principle: - [X]ms of additional margin should be provided compared to max(latency for PCell handover without PSCell change, latency for PSCell change without PCell handover) - FFS on [X], e.g. 5.

- Agreement - Introduce extra margin Y ms for sequential processing case comparing to parallel processing case for UE SW processing and RF warm-up for [PCell handover] and PSCell addition/change - Y = [10] ms Note: no extra interruption is required

- Proposal 4. Tprocessing for HO with PSCell for sequential processing case is Tprocessing for parallel processing case + 10ms.

Issue: Processing timeline for NR SA to EN-DC

RAN2 LS reply [R2-2201902]

As timing reference of target PScell is target Pcell, sequential processing is applied in this case. Otherwise, parallel processing can be performed.

- Proposal 5. Sequential processing is applied when SMTC of target unknown PScell is provided by to UE in the container obtained from target E-UTRAN PCell. Otherwise, parallel processing can be applied.

# 3.	Conclusion

- Proposal 1. PScell addition delay requirements are only applicable at NR-SA to ENDC. Otherwise HO delay requirements are used for both Pcell HO and PScell changes with different parameters accordingly.

PSCell addition delay= TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms,	 for NR-SA to ENDC

HO delay = TRRC_delay +Tinterrupt = TRRC_delay +Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms,  	otherwise

- Proposal 2: Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) + [5 or 10]ms for parallel cases.

- Proposal 3. Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change) for parallel cases. Except T_processing=30ms for NR-SA to ENDC.

- Proposal 4. Tprocessing for HO with PSCell for sequential processing case is Tprocessing for parallel processing case + 10ms.

- Proposal 5. Sequential processing is applied when SMTC of target unknown PScell is provided by to UE in the container obtained from target E-UTRAN PCell. Otherwise, parallel processing can be applied.

# 4.    References

- R4-2202598, WF on further RRM enhancement for NR and MR-DC – HO with PSCell, VIVO, RAN4 #101-bis-e