3GPP TSG-RAN WG4 Meeting # 102-e		                       R4-2203919

Electronic Meeting, February 21 – March 3, 2022

Title: 	System parameters for 6GHz licensed band

Source: 	CATT

Agenda item:	9.3.2

Document for:	Approval

- Introduction

In RAN4#101-bis-e meeting, system parameters for 6GHz licensed band were discussed and a WF [1] was approved. This contribution will provide our views on the remaining issues, including channel raster and sync raster.

- Discussion

In RAN4#101-bis-e meeting, RAN4 reached agreements on the operating band and channel bandwidth for 6GHz licensed band. But the channel raster and sync raster are still FFS. The way forwards are captured in [1] as follows:

2.4 Channel raster Candidate options: - Option 1: follow the legacy approach: 15kHz and 30kHz; - Option 2: based on 5MHz frequency block, FFS on the granularity of NR-ARFCN 2.5 Synchronization raster This should depend on agreement of the minimum channel bandwidth and channel raster agreement, companies’ further view are welcome in 2nd round.

SCS based channel raster is a normal way to locate carrier frequency for licensed bands and also enables more flexibility compared to 5MHz channel raster. So it would be more straightforward and appropriate to follow the legacy approach for 6425-7125MHz since it is operated as a licensed band. In our previous paper, we propose to not restrict the use of this band for the countries and regions beyond RCC countries if need is identified. Considering the potential usage in other areas, finer channel raster, i.e. 15kHz and 30kHz with more flexibility is preferred.

Proposal 1: To follow the legacy approach by using the channel raster of 15kHz and 30kHz as Table 2-1.

Table 2-1: Applicable NR ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n10x | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |

Based on the agreements on channel bandwidth, the minimum channel bandwidth for 6425-7125MHz is 20MHz.

The range of GSCN can be calculated as below:

First = ((6425 + 0.82 + 3.6) - 3000) / 1.44 + 7499 = 9880.54   9881

Last = ((7125 - 0.82 - 3.6) - 3000) / 1.44 + 7499 = 10360.51   10360

For the step size, a few companies have the concern on cell search time if a smaller step size is adopt. For 6425-7125MHz, there are 486 sync raster entries if the step size is 1 with 1.44MHz. In general, the step size can be 1~7 based on the calculation of ΔFSS,Raster. However, a smaller step size could enable more flexibility for sync raster entry. The step size of <1> is specified for the majority of operating bands in FR1. So it is also preferred to have the same step size for sync raster entries for 6425-7125MHz.

Figure 2-1: Possible shifts for the PBCH as the transmitted carrier is shifted on the RF carrier raster.

Number of sync raster entries (7125-6425) / 1.44 = 486.11  486

ΔFSS,Raster ≤ BWRB,carrier – BWPBCH + ΔFCH,Raster

ΔFSS,Raster ≤ 18.36MHz – 7.2MHz + 0.030MHz = 11.19MHz

Maximum step size = 11.19/1.44 = 7.77  7

Based on GSCN definition, it is proposed to introduce the applicable SS raster entries in table 2-2 for 6425-7125MHz.

Proposal 2: To introduce the applicable SS raster entries in Table 2-2 for 6425-7125MHz.

Table 2-2: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n10x | 30 kHz | Case C | 9881 – <1> – 10360 |

- Conclusion

This contribution mainly discusses system parameters for 6GHz NR licensed band. The following proposals are derived:

Proposal 1: To follow the legacy approach by using the channel raster of 15kHz and 30kHz as Table 2-1.

Table 2-1: Applicable NR ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n10x | 15 | 828334 – <1> – 875000 | 828334 – <1> – 875000 |
|  | 30 | 828334 – <2> – 875000 | 828334 – <2> – 875000 |

Proposal 2: To introduce the applicable SS raster entries in table 2-2 for 6425-7125MHz.

Table 2-2: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n10x | 30 kHz | Case C | 9881 – <1> – 10360 |

- Reference

- R4-2202266, WF on system parameters for 6GHz licensed band, ZTE, RAN4#101-bis-e