3GPP TSG-RAN WG4 Meeting #102-e                                                    R4-2204010

# Electronic Meeting, February 21 - March 3, 2022

Source:	ZTE Corporation

Title:	TP to TR 38.862 on symbols and abbreviations

Agenda Item:	11.3.1

Document for:	Approval

1	Introduction

In RAN4#101-bis-e meeting, the TR 38.862 v050 has been agreed in [1]. According to the work plan of SID [2], the TR will be for approval at RAN#95 in March 2022. Some missing parts of the TR should be added. In this proposal, a TP to add symbols and abbreviations into the TR is proposed.

Proposal 1:	 It is suggested to adopt the following TP for symbols and abbreviations in TR 38.862.

2	References

[1] R4-2200613, TR 38.862 v050, Study on band combination handling in RAN4

[2] R4-211146, Revised SID: Study on band combination handling in RAN4, ZTE

Text Proposal

<< begin text proposal >>

<< Unchanged sections omitted >>

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c.

BWChannel	Channel bandwidth.

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz.

Max()	The largest of given numbers.

Min()	The smallest of given numbers.

NRB	Transmission bandwidth configuration, expressed in units of resource blocks.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCS	Bandwidth Combination Set.

BS	Base Station.

BW	Bandwidth.

CA	Carrier aggregation.

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands.

CC	Component carrier.

CR	Change Request.

DC	Dual Connectivity.

DL	Downlink.

E-UTRA	Evolved Universal Terrestrial Radio Access.

EN-DC	E-UTRA/NR DC.

FDM	Frequency Division Multiplexing.

FR	Frequency Range.

HPUE	High Power UE.

LTE	Long Term Evolution.

MOP	Maximum Output Power.

MR-DC	Multi-radio DC.

NBC	Non-backwards Compatible.

NE-DC	NR/E-UTRA DC.

NR	New Radio.

NR-DC	NR/NR DC.

PC	Power Class.

RF	Radio Frequency.

Rx	Receiver.

SCS	Subcarrier spacing.

SUL	Supplementary uplink.

SR	Status Report.

TDM	Time Division Multiplex.

TP	Text Proposal.

Tx	Transmitter.

UE	User Equipment.

UL	Uplink.

ULSUP	Uplink sharing from UE perspective.

V2X	Vehicle to Everything.

WI	Work Item.

WID	Work Item Description.

<< end text proposal >>