3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204091

# Electronic Meeting, February 21- March 3, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Discussion on NR-U MPR and A-MPR for type 1 waveforms

Agenda Item:	9.2.3 UE RF requirements [NR_6GHz_unlic_full-Core]

Document for:	Approval

# Introduction

In R4#101b-e, when A-MPR was discussed, it was raised that A-MPR for type 1 waveforms was not provided. It was then clarified that for NR-U MPR for Release 16, only type 2 waveforms were evaluated and thus, before any further discussion is pursued on A-MPR, MPR should be assessed and then aspects of type 1 waveforms in terms of in-band PSD should drive the A-MPR discussion. In this contribution, we make a high level analysis of the type 1 waveforms with respect to the PC5 NR-U SEM and ACLR requirements.

# Discussion

When NR-U has been introduced in release 16, the focus has been on enabling the technology in the same way it was done for eLAA in UL. The priority for this was evaluating the waveforms that were designed in RAN1 to provide the best power trade-off for partial allocation that, in most unlicensed band regulations, are subject to an in-band PSD requirement. The interlace waveforms (type 2) are specifically designed to spread the PSD across the channel bandwidth to allow the optimum transmitted power.

Together with fully allocated waveforms these type 2 interlaced waveforms are the only waveforms that have been evaluated to derive NR-U MPR and A-MPR for the relevant regulations in n46, n96 and n102.

In R4#101b-e, it was noted that type 1 are also valid waveforms for NR-U and A-MPR should be derived for these. Before discussing A-MPR, it should be first understood which MPR would apply to such waveforms.

## NRU MPR and partial allocation type 1 waveforms

The PC5 NR-U MPR in 38.101-1 for partial allocation is currently defined only for interlaced type 2 waveforms as specified in the NOTE 1 in Table 6.2F.2-1. Table 1 compares the PC3 NR MPR with PC5 N-RU MPR, and it can be seen that the NR-U partial MPR is always higher than all other types of allocations in NR MPR.

Table 1: Comparison of NR PC3 MPR and NR-U PC5 MPR

| Modulation |  | NR PC3 MPR (dB) / RB allocation |  |  | NR-U PC5 MPR (dB)/ RB Allocation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Edge | Outer | Inner | Full | Partial |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | 0 | 2 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1 |  | 0 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 2.5 |  |  | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 4.5 |  |  | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.5 |  |  | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.5 |  |  | ≤ 7.0 | ≤ 7.0 |

Given that PC3NR is 3 dB higher power than PC5 NR-U and that NR-U has a 3dB relaxed ACLR, one could speculate that partial allocation NR-U MPR should have margin if applied to type 1 partial allocations. In fact this may not be the case because NR and NRU use very different SEM requirements. Not only are the shape and values different, but moreover, the NR-U mask is relative to the peak in-band power in 1MHz while the NR mask is absolute.

The two masks are compared in Table 2 for a 20 MHz channel bandwidth, and they are very different, as an example when the in-band power in 1MHz is < 20dBm which is the case for 1RB for PC5 NR-U, the NR-U mask becomes more stringent the NR mask at 14 MHz offset and improvement with MPR is not rapid since the mask drops with the in-band power making it even more stringent than the NR mask.

Table 2: comparison of NR and NR-U spectrum emission mask

| ΔfOOB [MHz] | NR 20MHz |  | NR-U 20MHz |  | ΔfOOB [MHz] | NR 20MHz |  | NR-U 20MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 |  |  | -2.0 | dBR/100kHz* | 10.5 | -13.0 | dBm/MHz | -28.6 | dBR/MHz |
| 0.2 | -13.0 | dBm/200kHz | -4.0 | dBR/100kHz* | 11.5 | -13.0 | dBm/MHz | -29.8 | dBR/MHz |
| 0.3 | -13.0 | dBm/200kHz | -6.0 | dBR/100kHz* | 12.5 | -13.0 | dBm/MHz | -31.0 | dBR/MHz |
| 0.4 | -13.0 | dBm/200kHz | -8.0 | dBR/100kHz* | 13.5 | -13.0 | dBm/MHz | -32.2 | dBR/MHz |
| 0.5 | -13.0 | dBm/200kHz | -10.0 | dBR/100kHz* | 14.5 | -13.0 | dBm/MHz | -33.4 | dBR/MHz |
| 0.6 | -13.0 | dBm/200kHz | -12.0 | dBR/100kHz* | 15.5 | -13.0 | dBm/MHz | -34.6 | dBR/MHz |
| 0.7 | -13.0 | dBm/200kHz | -14.0 | dBR/100kHz* | 16.5 | -13.0 | dBm/MHz | -35.8 | dBR/MHz |
| 0.8 | -13.0 | dBm/200kHz | -16.0 | dBR/100kHz* | 17.5 | -13.0 | dBm/MHz | -37.0 | dBR/MHz |
| 0.9 | -13.0 | dBm/200kHz | -18.0 | dBR/100kHz* | 18.5 | -13.0 | dBm/MHz | -38.2 | dBR/MHz |
| 1 |  |  | -20.0 | dBR/100kHz* | 19.5 | -13.0 | dBm/MHz | -39.4 | dBR/MHz |
| 1.5 | -10.0 | dBm/MHz | -20.4 | dBR/MHz | 20.5 | -25.0 | dBm/MHz | -40.0 | dBR/MHz |
| 2 | -10.0 | dBm/MHz | -20.9 | dBR/MHz | 21.5 | -25.0 | dBm/MHz | -40.0 | dBR/MHz |
| 2.5 | -10.0 | dBm/MHz | -21.3 | dBR/MHz | 22.5 | -25.0 | dBm/MHz | -40.0 | dBR/MHz |
| 3 | -10.0 | dBm/MHz | -21.8 | dBR/MHz | 23.5 | -25.0 | dBm/MHz | -40.0 | dBR/MHz |
| 3.5 | -10.0 | dBm/MHz | -22.2 | dBR/MHz | 24.5 | -25.0 | dBm/MHz | -40.0 | dBR/MHz |
| 4 | -10.0 | dBm/MHz | -22.7 | dBR/MHz | 25.5 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 4.5 | -10.0 | dBm/MHz | -23.1 | dBR/MHz | 26.5 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 5.5 | -13.0 | dBm/MHz | -24.0 | dBR/MHz | 27.5 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 6.5 | -13.0 | dBm/MHz | -24.9 | dBR/MHz | 28.5 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 7.5 | -13.0 | dBm/MHz | -25.8 | dBR/MHz | 29.5 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 8.5 | -13.0 | dBm/MHz | -26.7 | dBR/MHz | >30 | -30.0 | dBm/MHz | -40.0 | dBR/MHz |
| 9.5 | -13.0 | dBm/MHz | -27.6 | dBR/MHz |  |  |  |  |  |

This region below 20 MHz ΔfOOB is subject to IM3 of the wanted signal and its image, and is critical for narrow allocations which are part of the type 1 RB allocations. The IMD5 region is for ΔfOOB between 20 and 40MHz and at an in-band PSD < 10dBm/MHz, again the NR-U mask becomes more stringent.

Observation on SEM: NR-U mask is more stringent than NR mask at larger offsets and notably in the ACLR1 region where IMD3 of narrow allocations with its own image becomes critical. It cannot be concluded that the MPR that allows narrow allocations to meet NR SEM would pass NR-U SEM.

## Applicable requirements for in-band PSD

Table 3 provides the in-band PSD requirements across NR-U bands and countries. If there are rare exceptions where an in-band PSD requirement is not set, the main driving cases have strict in-band PSD requirements.

Table 3: In-band PSD requirements in unlicensed bands

| Band | Country | CBW (MHz) | Frequency range (MHz) | IB PSD (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | Japan | all | 5150-5350 + 5470-5725 | 10 |
|  | China | 20 | 5170-5330 + 5490-5730 | 10 |
|  |  | 40 | 5170-5330 + 5490-5730 | 7 |
|  |  | 60, 80 | 5170-5330 + 5490-5730 | 4 |
|  | EU/CEPT | all | 5150-5350 + 5470-5725 | 11 |
|  | US | 20 | 5150-5230 + 5250-5350 + 5470-5850 / 5230-5250 | 10 / 4 |
|  |  | 40 | 5150-5230 + 5250-5350 + 5470-5850 / 5230-5250 | 7 / 4 |
|  |  | 60, 80 | 5150-5350 + 5470-5850 | 4 |
| n96 | US | all | 5925-6425 + 6525-6875 (SP) / 5925-7125 (LPI) | 17 (SP) / -1 (LPI) |
|  | Canada | all | 5925-6875 (SP) / 5925-7125 (LPI/VLP) | 23 (SP) / 5 (LPI) / -8 (VLP) |
|  | Brazil | all | 5925-7125 | -1 (LPI) / -5 (VLP) |
|  | Peru | all | 5925-7125 | -1 |
|  | Chile | all | 5925-7125 | -1 |
|  | Costa Rica | all | 5925-7125 |  |
|  | Colombia | all | 5925-7125 | -1 |
|  | South Korea | all | 5925-7125 (LPI) / 5925-6425 (VLP) | 2 (LPI) / 1 (VLP) |
| n102 | EU/CEPT | all | 5945-6425 | 10 (LPI) / 1 (VLP) |
|  | UK | all | 5925-6425 | 11 |
|  | UAE | all | 5925-6425 |  |
|  | Morocco | all | 5925-6425 |  |

One of the reasons one would want to use type 1 allocations, is to enable a lower number of RBs than for the interlace type 2 waveform. For example, using 1 to 9 contiguous RBs for 20MHz bandwidth, but, when using 1RB in a case with an in-band PSD requirement the PSD of that RB is limited to the same level than a 1RB/10 interlace patterns. Instead of an increased UL range, this results in a reduced UL range for the 1RB case since each RB of the 10x1RB/10 interlace will achieve the same SNR at the BS than 1RB and can benefit from lower coding rate by using more RBs.

Table 4 compares the in-band PSD in 1 MHz of type 1 partial allocations with type 2 interlaces at 20dBm (PC5) output power. The top half part of the table focusses on the type 1 partial waveforms with less than 10 RBs which are not feasible for type 2 interlaces while the bottom half compares the full, interlace type 2 and type 1 contiguous RBs for different NR-U channel bandwidths where for partial allocations with the same number of RBs.

Table 4: in-band PSD at 20dBm for different types of waveforms

|  |  |  | Waveform type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW (MHz) | SCS (kHz) | WF Coding | Full |  |  | #RB | Partial Type 2 (interlace) |  | Partial Type 1 (contiguous RBs) |  | Delta to interlace (dB) | Delta to Full (dB) |
|  |  |  | #RB | LogBW | dBm/MHz |  | logRB | dBm/MHz | BW | dBm/MHz |  |  |
| 20-100 | 15 | CP/DFT | NA | NA | NA | 1 | NA | NA | 0.18 | 20 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 2 | NA | NA | 0.36 | 20 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 3 | NA | NA | 0.54 | 20 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 4 | NA | NA | 0.72 | 20 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 5 | NA | NA | 0.9 | 20 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 6 | NA | NA | 1.08 | 19.7 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 7 | NA | NA | 1.26 | 19 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 8 | NA | NA | 1.44 | 18.4 | NA | NA |
|  |  | CP/DFT | NA | NA | NA | 9 | NA | NA | 1.62 | 17.9 | NA | NA |
| 20 | 15 | DFT | 100 | 12.6 | 7.4 | 10 | 10 | 10 | 1.8 | 17.4 | 7.4 | 10 |
|  |  | CP | 106 | 12.8 | 7.2 | 11 | 10.4 | 9.6 | 1.98 | 17 | 7.4 | 9.8 |
| 40 |  | DFT | 216 | 15.9 | 4.1 | 20 | 13 | 7 | 3.6 | 14.4 | 7.4 | 10.3 |
|  |  | CP | 216 | 15.9 | 4.1 | 22 | 13.4 | 6.6 | 3.96 | 14 | 7.4 | 9.9 |
| 60 | 30 | DFT | 162 | 17.7 | 2.3 | 32 | 15.1 | 4.9 | 11.52 | 9.4 | 4.5 | 7.1 |
|  |  | CP | 162 | 17.7 | 2.3 | 33 | 15.2 | 4.8 | 11.88 | 9.3 | 4.5 | 7 |
| 80 |  | DFT | 216 | 18.9 | 1.1 | 40 | 16 | 4 | 14.4 | 8.4 | 4.4 | 7.3 |
|  |  | CP | 217 | 18.9 | 1.1 | 44 | 16.4 | 3.6 | 15.84 | 8 | 4.4 | 6.9 |
| 100 |  | DFT | 270 | 19.9 | 0.1 | 54 | 17.3 | 2.7 | 19.44 | 7.1 | 4.4 | 7 |
|  |  | CP | 273 | 19.9 | 0.1 | 55 | 17.4 | 2.6 | 19.8 | 7 | 4.4 | 6.9 |

Observations on in-band PSD:

- For the type 1 partial allocations < 10RBs, the in band PSD is from 18 to 20dBm which is higher than the majority of the allowed in-band PSD

- Type 1 partial allocations with the same number of RBs than the corresponding interlace in a given BW has 7.5 dB higher PSD for 15kHz SCS and 4.5 dB higher PSD for 30kHz SCS

- The full allocation results in to the lowest in-band PSD but with lower BS SNR than interlaces but with similar BS SNR than type 1 partial RBs.

Based on the above observations, there is no UL range or SNR benefit in using type 1 partial allocations compared to type 2 interlaces when there is an in-band PSD limit.

Proposal: Since there is no benefit in using type 1 partial allocations for cases where A-MPR is dictated by in-band PSD we propose that release 17 PC5 NR-U MPR and A-MPR is only based on fully allocated waveforms, type 2 interlace waveforms and wideband operation full and partial allocated sub-band operation. Partial type 1 waveforms MPR and A-MPR may be studied in a later release if justified by cases without in-band PSD limitations.

# Conclusions

In this contribution, we discuss the attributes of type 1 and type 2 partial allocations in the scope of PC5 NR-U MPR and A-MP and make the following proposal.

Proposal: Since there is no benefit in using type 1 partial allocations for cases where A-MPR is dictated by in-band PSD we propose that release 17 PC5 NR-U MPR and A-MPR is only based on fully allocated waveforms, type 2 interlace waveforms and wideband operation full and partial allocated sub-band operation. Partial type 1 waveforms MPR and A-MPR may be studied in a later release if justified by cases without in-band PSD limitations.

This proposal is based on following observations for MPR related to SEM and in-band PSD for A-MPR.

Observation on SEM: NR-U mask is more stringent than NR mask at larger offsets and notably in the ACLR1 region where IMD3 of narrow allocations with its own image becomes critical. It cannot be concluded that the MPR that allows narrow allocations to meet NR SEM would pass NR-U SEM.

Observations on in-band PSD:

- For the type 1 partial allocations < 10RBs, the in band PSD is from 18 to 20dBm which is higher than the majority of the allowed in-band PSD

- Type 1 partial allocations with the same number of RBs than the corresponding interlace in a given BW has 7.5 dB higher PSD for 15kHz SCS and 4.5 dB higher PSD for 30kHz SCS

- The full allocation results in to the lowest in-band PSD but with lower BS SNR than interlaces but with similar BS SNR than type 1 partial RBs.

# References