3GPP TSG RAN WG4 Meeting #102-e	R4-2204133

Online, 21th Feb. – 3rd Mar. 2022

Agenda item:	9.21.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-31: EN-DC_11_n3-n77-n79

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 3B NR of DC_11_n3-n77-n79 was approved in RAN#94 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_11_n3-n77-n79

6.X.1	Operating bands for DC

Table 6.X.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | CA_n3-n77-n79 |

6.X.2	Inter-band DC Configurations

Table 6.X.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |

6.X.3	Co-existence studies

Co-existence study for DC_11_n3-n77-n79 was covered by the studies for the fallback modes of DC_11_n3-n77, DC_11_n3-n79 and DC_11_n77-n79.

No additional MSD requirement needs to be defined for this dual connectivity configuration.

6.X.4	∆TIB and ∆RIB values

For DC_11_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n77-n79 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

6.X.5	MSD

As mentioned in 6.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

3	Reference

[1]  RP-212885  Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL), ZTE Corporation