3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204169

# Electronic Meeting, February 21 – March 3, 2022

Source: 	MediaTek Inc.

Title: 	Draft text proposal to update TS 38.101-5 on clause 7.5 NTN UE ACS

Agenda Item:	10.13.4.2

Document for:	Approval

Introduction

Regarding TR 38.863-020 [2] and NR NTN WF [1] from RAN4#101-bis-e, consensus about the NR NTN UE ACS was reached. In the contribution, the draft CR text is proposed for NR NTN UE ACS.

Discussion

From [2], the agreed ACLR and ACS of NR-NTN are shown in Table below.

Table 6.5-2 ACLR and ACS of TN

| TN |  | Values |
| --- | --- | --- |
| UE | ACLR | 30 dB |
|  | ACS | 33 dB |

Table 6.5-3 ACLR and ACS of NR-NTN

| NR-NTN |  | Values |
| --- | --- | --- |
| UE | ACLR | 30 dB |
|  | ACS | 33 dB |

Observation 1: From [2], it was agreed that the UE ACS requirements for TN and NR-NTN are the same.

Proposal 1: To reuse the NR UE ACS definition and requirements from TS 38.101-1 for NR NTN UE.

Observation 2: Two options are provided here for text proposal. Option 2 is preferred as it is straightforward.

Conclusion

In the contribution, the CR of NR NTN UE ACS is proposed.

Observation 1: From [2], it was agreed that the UE ACS requirements for TN and NR-NTN are the same.

Proposal 1: To reuse the NR UE ACS definition and requirements from TS 38.101-1 for NR NTN UE

Observation 2: Two options are provided here for text proposal. Option 2 is preferred as it is straightforward.

- 4 References

[1] R4-2203130, “WF on [307] NTN_Solutions_Part2”, Samsung

[2] 3GPP TR 38.863-020

- Option 1 : Draft proposed CR text compared with TS 38.101-1

<Start of change>

- 7.5	Adjacent channel selectivity

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

The NR UE ACS definition in clause 7.5 from TS 38.101-1 is applicable for the NR NTN UE.

ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The NR NTN UE shall fulfil the minimum requirements specified in Table 7.5-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz.

Table 7.5-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| ACS | dB | 33 | 30 | 27 |

<For bands n255 and n256, we can also just refer to ACS requirement for bands below 2700MHz in TS 38.101-1.>

<End of change>

- Option 2 : Draft proposed CR text

<Start of change>

- 7.5	Adjacent channel selectivity

The NR UE ACS definition and requirements in clause 7.5 from TS 38.101-1 are applicable for the NR NTN UE for NTN bands of the corresponding FDL_high and FUL_high range.

<End of change>