3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204374

Electronic Meeting, February 21 – March 3, 2022

Agenda Item: 4.1.7

Source: MediaTek Inc.

Title: Discussion on FR2 inter-frequency relative RSRP accuracy

Document for: Discussion

- Introduction

In the last meeting, this issue was discussed but not yet reached conclusion. In this paper, we provide the background of the current requirement and propose a solution for FR2 inter-frequency relative RSRP accuracy.

- Discussion

The background based on our understanding is provided in section 2.1. Besides, because, in the last meeting, some companies think the discussion should be extended for the upper bound of the relative accuracy requirement. Thus, our views on the lower bound and upper bound are provided in section 2.2 and 2.3, respectively.

2.1 Background

For the inter frequency relative RSRP accuracy in FR2, there are two test cases in TS 38.133:

- A.5.7.1.2 EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

- A.7.7.1.2 SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

We will take A.5.7.1.2 as an example to illustrate the accuracy uncertainty issues caused by

- Mis-alignment between fine beam and rough beam

- Different antenna gains on different bands

The current test requirement for this SS-RSRP relative accuracy is provided for reference, as in Table A.5.7.1.2.3-2.

Table A.5.7.1.2.3-2: SS-RSRP relative accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 3 – Cell 2 | SSB_RP3 - SSB_RP2 -δ ≤ Reported RSRP(dB) ≤ SSB_RP3 - SSB_RP2 +δ –(X) |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP relative accuracy requirement from Table 10.1.5.1.2-1 Note 3: Void Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

- Mis-alignment between fine beam and rough beam

According to the test parameters in Table A.5.7.1.2.2-2, Cell 2 (the serving cell) is at AoA1 which is randomly picked from the subset of angles which fulfil the spherical coverage requirement, while Cell 3 (neighboring cell) is at AoA2 along the Rx beam peak direction.

Table A.5.7.1.2.2-2: SS-RSRP inter-frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Angle of arrival configuration | 1~4 |  | Setup 4b according to clause A.3.15.4.2 |  | Setup 4b according to clause A.3.15.4.2 |  |
|  |  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |

Note that, the beam peak direction (AoA2) is determined based on fine beam assumption during EIS 3D scan, and it is not the beam peak for rough beam which is used for L3 measurement as for SS-RSRP. On the other hand, since AoA1 is randomly picked from the spherical coverage (based on fine beam), it is possible that UE may achieve worse rough beamforming gain at AoA2 (i.e. at the fine beam peak) than at AoA1 (selected from spherical coverage).

For example, in the figure 1,

- When the fine beam is applied (the blue dots on the right-hand side), the fine beam peak is pointing to the AoA2. In this case, the gain at AoA2 will be better than AoA1, i.e., SSB_RP3 - SSB_RP2 > 0. Thus, UE can pass the test due to margin X.

- When the rough beam is applied (the red dots on the right-hand side), since the directions of fine beam and rough beam may be different, the rough beam peak may point to the AoA1, where AoA1 is randomly picked from spherical coverage 50%-tile (based on fine beam). In this case, the gain at AoA1 with rough beam may be better than AoA2 with rough beam, i.e., SSB_RP3 - SSB_RP2 < 0. Thus, UE may fail the test due to no margin in lower bound in accuracy requirement.

Figure 1. One example that the beam forming gain may be higher at AoA1 than at AoA2

Observation 1: In the FR2 inter-frequency RSRP relative accuracy test, the beam peak AoA2 for Cell 3 is based on fine beam, while SS-RSRP measurement can be conducted by rough beam.

Observation 2: UE may achieve worse rough beamforming gain at AoA2 (i.e. at the fine beam peak) than at AoA1 (selected from spherical coverage).

- Different antenna gain on different bands

Furthermore, in this inter-frequency test, Cell 2 and Cell 3 are selected from 2 different frequency layers, where the antenna gains could be different at different frequencies, especially for different bands. For example, Cell 2 from AoA1 (spherical coverage) is on f1 while Cell 3 from AoA2 (beam peak) is on f2, and then the antenna gain on f1 could be higher than that on f2.

In order to fulfil the current test requirement, UE is mandated to equalize the antenna gain between 2 frequencies or even 2 bands. Such a UE implementation does not seem to honestly reflect the exact signal power level difference experienced by UE. We believe that during Rel-15, the discussion was mainly focused on intra-band scenarios. There was no attention paid on how the requirement should be applied for inter-band scenarios.

Observation 3: Current SS-RSRP relative accuracy test requirement mandates UE to equalize the antenna gain difference between 2 frequencies or even 2 bands, which seems not well-discussion in Rel-15.

Proposal 1: For the relative inter-frequency accuracy requirement, the following two additional margins should be considered:

- Mis-alignment between fine beam and rough beam

- Different antenna gain on different bands

2.2 lower bound

To evaluate the problem found in the section 2.1, the measurement result on rough beamforming gain based on the test setting of the A.5.7.1.2 in TS 38.133 is provided in Table 1.

Table 1. L3-RSRP measurement results based on test setting in A.5.7.1.2

| Serving cell | Neighbor cell | Peak on NBRNote2 (SSB_RP3, dBm/SCS) | 50% sphere coverage on Serv best Note3 (SSB_RP2, dBm/SCS) | 50% sphere coverage on Serv worst Note4 (SSB_RP2, dBm/SCS) |
| --- | --- | --- | --- | --- |
| n258 24.3Ghz | n258 24.4Ghz | -71 | -69.5 | -76 |
| n260 37.07Ghz | n260 37.17Ghz | -74 | -72.5 | -82 |
| n261 27.93Ghz | n261 28.03Ghz | -71 | -67.5 | -76 |
| Note 1: The frequency difference between two cells within one band is 100 MHz. Note 2: Peak on NBR: UE uses rough beam to measure the RSRP at beam peak Note 3: 50% sphere coverage on Serv best: UE uses rough beam to measure the RSRP for each AoAs within 50% sphere coverage and records the largest value of the RSRP. Note 4: 50% sphere coverage on Serv worst: UE uses rough beam to measure the RSRP for each AoAs within 50% sphere coverage and records the lowest value of the RSRP. |  |  |  |  |

Our derivation of the margins for intra-band and inter-band is provided as follows:

As we mentioned in last meeting, the accuracy requirement in current TS 38.133 considers the impacts of the baseband (BB) and RF. Now, two additional impacts (1) mis-alignment between fine beam and rough beam and (2) different antenna gain on different bands (only for inter-band) should be considered.

For intra-band, the margin includes three parts BB + RF + D, where D is the impact of mis-alignment between fine beam and rough beam. Considering the worst case in terms of (SSB_RP3 - SSB_RP2) within one band, the worst value is -71 – (-67.5) dB = -3.5 dB @ n261, as shown in Table 1. In this test case, because the side condition SNR is 6 dB which is relatively high, in our understanding, the baseband impact can be ignored, i.e., assume BB is negligible. Besides, the RF impact can also be ignored due to small separation in frequency  between two cells (100 MHz), i.e., RF is negligible. Thus, 3.5 dB difference is mainly due to the beam misalignment, i.e. D. In addition, in our understanding, additional margin 2 dB is needed due to insufficient sample (It is difficult to test all possible cases). As a result, for intra-band, totally additional margin in lower bound 3.5 (D) + 2 (insufficient sample) = 5.5 dB is proposed.

For inter-band, the margin includes four parts BB + RF + D + Ginter, where D is the same definition as intra-band and Ginter is the impact of different antenna gain on different bands. Considering the worst case in terms of (SSB_RP3 - SSB_RP2) among the pairs of two bands, the worst value is -74 – (-67.5) = -6.5 dB @ n260 & n261. In this test case, the BB is negligible due to the same reason in intra-band. And the D is assumed as 3.5 dB, which is derived from intra-band as above. Thus, 6.5 – 3.5 (D) – 0 (BB) = 3 dB, which is contributed by RF + Ginter. However, to us, it is difficult to separate the impacts of RF and Ginter. Considering the worst case, all 3 dB is assumed to be contributed by Ginter. In addition, additional margin 2 dB is needed due to insufficient sample (It is difficult to test all possible cases). As a result, for inter-band, totally additional margin in lower bound 3.5 (D) + 3 (Ginter) + 2 (insufficient sample) = 8.5 dB is proposed.

Proposal 2: For the test case of FR2 inter-frequency relative RSRP accuracy, to add 5.5 (D) dB and 8.5 (D+ Ginter) dB margin in the lower bound for intra-band and inter-band, respectively, where

- Mis-alignment between fine beam and rough beam (D) = 5.5 dB

- Different antenna gain on different bands (Ginter) = 3 dB

2.3 upper bound

For the upper bound, the main difference from lower bound is X, i.e., no X in the lower bound while X takes place in upper bound.

In the following, the accuracy uncertainty issues caused by mis-alignment between fine beam and rough beam, and different antenna gains on different bands can be discussed by the following two factors.

- Different antenna gain on different bands (Ginter): The Ginter should be also applied for the upper bound because the impact due to different bands is not considered in X.

- Mis-alignment between fine beam and rough beam  (D): To our understanding, no extra D margin in upper bound is needed, i.e., D=0. Because the X factor for beamforming gain uncertainty is sufficient based on our measurement results in Table 1.

The X calculated from Table 7.3.2.3-1 and Table 7.3.4.3-1 in TS 38.101-2, and the Peak on NBR - 50% sphere coverage on Serv worst (to get the maximum value in upper bound) are summarized as Table 2. (Note: In Table 2, the X is derived based on CBW = 50 MHz which is same as test setting in A.5.7.1.2)

Table 2. the relation between -X and (Peak on NBR - 50% sphere coverage on Serv worst) in upper bound based on our measurement results in Table 1.

| Operating band (Neighbor cell - Serving cell) | -X | Measurement results: Peak on NBR - 50% sphere coverage on Serv worst |
| --- | --- | --- |
| N258 – n258 | 10.9 | 5 |
| N260 - n260 | 12.6 | 8 |
| N261 – n261 | 10.9 | 5 |
| N258 – n260 | 15.2 | 11 |
| N258 – n261 | 10.9 | 5 |
| N260 – n261 | 8.3 | 2 |
| N260 – n258 | 8.3 | 2 |
| N261 – n258 | 10.9 | 5 |
| N261 – n260 | 15.2 | 11 |

For upper bound, it can be observed from the measurement results, the (peak on NBR - 50% sphere coverage on Serv worst) is smaller than (-X), i.e., the current X already covers the uncertainty in the upper bound even though the rough beam is used in the measurement results. Thus, to avoid too-relaxed requirement, we prefer not to add extra margin for mis-alignment between fine beam and rough beam in upper bound, i.e. D can be excluded.

Proposal 3: For the test case of FR2 inter-frequency relative RSRP accuracy, to add 3 (Ginter) dB margin in the upper bound for inter-band, where

- Different antenna gain on different bands (Ginter) = 3 dB

- Summary

In this paper, we raise the issue we found in current test requirements for FR2 SS-RSRP relative accuracy test cases (A.5.7.1.2 and A.7.7.1.2). We have the following observations and proposals.

Observation 1: In the FR2 inter-frequency RSRP relative accuracy test, the beam peak AoA2 for Cell 3 is based on fine beam, while SS-RSRP measurement can be conducted by rough beam.

Observation 2: UE may achieve worse rough beamforming gain at AoA2 (i.e. at the fine beam peak) than at AoA1 (selected from spherical coverage).

Observation 3: Current SS-RSRP relative accuracy test requirement mandates UE to equalize the antenna gain difference between 2 frequencies or even 2 bands, which seems not well-discussion in Rel-15.

Proposal 1: For the relative inter-frequency accuracy requirement, the following two additional margins should be considered:

- Mis-alignment between fine beam and rough beam

- Different antenna gain on different bands

Proposal 2: For the test case of FR2 inter-frequency relative RSRP accuracy, to add 5.5 (D) dB and 8.5 (D+ Ginter) dB margin in the lower bound for intra-band and inter-band, respectively, where

- D (Mis-alignment between fine beam and rough beam) = 5.5

- Ginter (Different antenna gain on different bands) = 3

Proposal 3: For the test case of FR2 inter-frequency relative RSRP accuracy, to add 3 (Ginter) dB margin in the upper bound for inter-band, where

- Different antenna gain on different bands (Ginter) = 3 dB

- Annex

The reference for reference sensitivity (Table 7.3.2.3-1) and EIS spherical coverage for power class 3(Table 7.3.4.3-1) in TS 38.101-2 are provided as follows:

Table 7.3.2.3-1: Reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3 | -85.3 | -82.3 | -79.3 |
| n258 | -88.3 | -85.3 | -82.3 | -79.3 |
| n259 | -84.7 | -81.7 | -78.7 | -75.7 |
| n260 | -85.7 | -82.7 | -79.7 | -76.7 |
| n261 | -88.3 | -85.3 | -82.3 | -79.3 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

Table 7.3.4.3-1: EIS spherical coverage for power class 3

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |