3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2204506

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.13.4.1

Source: 		Qualcomm Incorporated

Title: 	TP on TR 38.863 for NTN UE Tx requirements

Document for:	Approval

# Introduction

Per agreed work split in [1], the paper is to provide the TP for TR 38.863 on NTN UE Tx requirements.

# 2. Discussion

1) MPR/A-MPR

As discussed in [3], it was agreed ACLR/ACS of TN UE can be used for NTN in last RAN4 meeting [1]. Therefore, the MPR of TN UE for PC3 shall be applied for NTN UE as well.

For A-MPR, it is proposed to reuse the A-MPR values defined in NS_24 and NS_100 for n256 and To reuse the A-MPR values defined in NS_56 for n255.

2) Transmit modulation quality

It was agreed the QPSK, 16QAM and 64QAM shall be considered for NTN UE. Therefore, for the EVM, the same requirements specified for TN UE QPSK, 16QAM, and 64QAM.can be reused for NTN UE.

For the carrier leakage and in-band emission, the same requirements specified for TN UE can be reused for NTN UE.

3) SEM

It was agreed that to use same requirement of TN UE for NTN UE ACLR [1]. Therefore, it is reasonable to reuse the SEM of TN UE for NTN UE.

Proposal 1: To approve the following TP on NTN UE Tx requirements.

# 3.	TP on NTN UE Tx requirements

=============================Start of the changes=======================================

### 7.4.2	 UE Transmission characteristics for satellite access

#### 7.4.2.1 	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Rel-17 for satellite access.

#### 7.4.2.2	Conducted transmitter characteristics

##### 7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

##### 7.4.2.2.2 	MPR/AMPR

For MPR, the same requirements specified in Table 6.2.2-1 in TS 38.101-1 [17] can be reused for NTN satellite access UEs.

For A-MPR, the requirements of NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 [17] for band n65 can be reused for n256. The requirements of NS_56 specified in Table 6.2.3.1-1 in TS 38.101-1 [17] for band n24 can be reused for n255.

##### 7.4.2.2.3 	Output power dynamics

For Transmit OFF power and Power control, the framework and requirements for existing TN UE can be reused for satellite UE. The transmit OFF power is -50 dBm for 20MHz.

Since handheld UE always transmits the maximum output power for satellite access scenario based on the NR NTN calibration summary in R4-2115628, FFS whether relaxed value needed or not based on existing TN UE requirements

##### 7.4.2.2.4 	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the Satellite Access Node.

NOTE:	The requirements is applicable only when PVT ephemeris updated at least once [10] seconds.

##### 7.4.2.2.5 	Transmit modulation quality

For the EVM (Error Vector Magnitude), the same requirements specified in TS 38.101-1 [17] sub-clause 6.4.2.1 for QPSK, 16QAM, and 64QAM can be reused for NTN satellite access UEs.

For the carrier leakage, the same requirements specified in TS 3.101-1 [17] sub-clause 6.4.2.2 can be reused for NTN satellite access UEs.

For the in-band emission, the same requirements specified in TS 3.101-1 [17] sub-clause 6.4.2.3 can be reused for NTN satellite access UEs.

##### 7.4.2.2.6 	Spectrum emission mask

For SEM of NTN satellite access UEs, the same requirements of TN UE specified in TS 3.101-1 [17] sub-clause 6.5.2.2 can be reused.

##### 7.4.2.2.7 	ACLR

For ACLR, the same ACLR for TN NR UE which is specified in TS 3.101-1 [17] sub-clause 6.5.2.4.1 can be applied for NTN satellite access UEs.

The requirement of ALCR is specified in Table 7.4.2.2.7-1.

Table 7.4.2.2.7-1: NTN satellite access UE ACLR requirement

|  | Power class 3 |
| --- | --- |
| NTN UE ALCR | 30dB |

##### 7.4.2.2.8 	Spurious emissions

The spurious emissions limits specified in TS 38.101-1[17] sub-clause 6.5.3 would also be applicable to NTN satellite access UEs.

##### 7.4.2.2.9 	Transmit intermodulation

The transmit intermodulation limits specified in TS 38.101-1[17] sub-clause 6.5.4 (Table 7.4.2.2.9-1) would also be applicable to NTN satellite access UEs.

The requirement of transmit intermodulation is specified in Table 7.4.2.2.9-1.

Table 7.4.2.2.9-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel center | BWChannel | 2*BWChannel |
| Interference CW signal level | -40 dBc |  |
| Intermodulation product | < -29 dBc | < -35 dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCS's for the channel BW as defined in Table 6.5.2.4.1-1 from TS 38.101-1[17] |  |
| Measurement offset from channel center | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

=============================End of the changes=======================================

# 4.	References

- R4-2203130, WF on [307] NTN_Solutions_Part2, Samsung

- R4-2203132, TR 38.863 v0.2.0

- R4-2204504, Discussion on NTN UE RF requirements, Qualcomm Incorporated