3GPP TSG-RAN WG4 Meeting # 102-e 		R4-2204525

Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.12.2.2

Source: 	CMCC

Title: 	Discussion on R17 demodulation enhancement for intra-cell inter-user interference suppressing

Document for:	Discussion

- Introduction

In RAN4#101bis-e meeting, we discussed the interference modeling for intra-cell inter-user interference MMSE-IRC receiver. A way forward was agreed in [1], updated simulation results and summary has been captured in TR 38.833 [2]. In this contribution, we provide our views on UE feature list.

- Discussion

UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

- Option 1: No need to introduce new UE feature, requirements release independent from Rel-15

- Option 2: Mandatory with or without UE capability signaling

- Option 3: Optional with or without UE capability and applicable from Rel-17

- Option 4: Optional without UE capability and applicable from Rel-15

Based on our knowledge, MMSE-IRC is the baseline receiver for NR UEs from Rel-15. In Rel-15, we didn’t define the performance requirements for interference scenario due to limited time unit. In Rel-17, we continue developing the requirements which should have been developed in Rel-15. Therefore, the requirements should be release independent from Rel-15. Regarding UE capability signaling, we support Option 1 since MMSE-IRC is the baseline receiver. No new UE feature is needed for baseline.

However, considering of the R15/16 UEs in the market may not support MMSE-IRC receiver, we can compromise to introduce UE feature to move forward. For R15/16 UE, the feature can be optional without UE capability signaling, For the UE from Rel-17, the feature should be mandatory without UE capability signaling.

Proposal 1: The performance requirements should be release independent from Rel-15.

Proposal 2: For UE feature, we prefer the following schemes

- 1st priority: no need to introduce new UE feature.

- 2nd priority: optional without UE capability signaling for Rel-15 and Rel-16, mandatory without UE capability signaling from Rel-17

- Conclusion

In this contribution, we discuss the UE feature for intra-cell inter-user interference MMSE-IRC receiver, our proposal is:

Proposal 1: The performance requirements should be release independent from Rel-15.

Proposal 2: For UE feature, we support the following schemes

- 1st priority: no need to introduce new UE feature.

- 2nd priority: optional without UE capability signaling for Rel-15 and Rel-16, mandatory without UE capability signaling from Rel-17

- Reference

[1] R4-2203011, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei, HiSilicon.

[2] 3GPP TR 38.833: “Further enhancement on NR demodulation performance”