3GPP TSG-RAN WG4 Meeting # 102-e                                                       R4-2204570

Electronic Meeting, Feb. 02 – Mar. 03 2022

Agenda item:	10.1.3.3

Source:	CMCC BUPT

Title:		PDP validation results for CDL-C UMa and reference values for UMi

Document for:	Approval

# Introduction

In the last RAN4 #101-e-bis meeting, PDP reference value and pass/fail limits for CDL-C Uma are determined [1], in this contribution, we give the specific measurement value and the differences to target for CDL-C Uma firstly, and reference values for CDL-C UMi channel model are also declared. It is noted that the target values for CDL-C UMi are also achieved using summing cluster method as CDL-C UMa channel model.

# Details

This contribution is divided into two parts, PDP measurement results for CDL-C UMa are given in the first part, and we add the reference value for CDL-C UMi in the second part.

## 2.1 Measurement results for CDL-C UMa

The PDP validation results of 2.45GHz and 3.6GHz are as follows, and the specific difference of delay and power between the measurement results and reference results are shown in Table1 to Table 4. It can be observed that except the cluster around 300ns of beam1, the differences are all within +/- 1dB, which meaning a good match.

Fig.1 original measured PDP results of CDL C UMa 2.45GHz Beam1

Fig.2 original measured PDP results of CDL C UMa 2.45GHz Beam2

Fig.3 original measured PDP results of CDL C UMa 3.6GHz Beam1

Fig.4 original measured PDP results of CDL C UMa 3.6GHz Beam2

Table 1. Measurement value of CDL-C UMa 2.45GHz beam1

| Combined Clusters index | Target Delay(ns) | Measured Delay(ns) | Delay gap(ns) | Target Power(dB) | Measured Power(dB) | Power gap(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -34.3 | -33.3 | -1 |
| 2-5 | 80 | 80 | 0 | -19.5 | -18.9 | -0.6 |
| 6-8 | 235 | 230 | 5 | 0.0 | 0.0 | 0 |
| 9-10 | 290 | 290 | 0 | -33.0 | -31.0 | -2 |
| 11 | 450 | 450 | 0 | -35.8 | -35.8 | 0 |
| 12 | 480 | 480 | 0 | -34.0 | -33.0 | -1 |

Table 2. Measurement value of CDL-C UMa 2.45GHz beam2

| Combined Clusters index | Target Delay(ns) | Measured Delay(ns) | Delay gap(ns) | Target Power(dB) | Measured Power(dB) | Power gap(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -27.9 | -27.3 | -0.6 |
| 2-5 | 80 | 75 | 5 | 0.0 | 0.0 | 0 |
| 6-8 | 235 | 235 | 0 | -18.4 | -18.7 | 0.3 |
| 9-10 | 290 | 290 | 0 | -27.8 | -27.4 | -0.4 |
| 11 | 450 | 450 | 0 | -27.9 | -28.5 | 0.6 |
| 12 | 480 | 480 | 0 | -28.0 | -27.2 | -0.8 |

Table 3. Measurement value of CDL-C UMa 3.6GHz beam1

| Combined Clusters index | Target Delay(ns) | Measured Delay(ns) | Delay gap(ns) | Target Power(dB) | Measured Power(dB) | Power gap(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -34.2 | -33.8 | -0.4 |
| 2-5 | 80 | 80 | 0 | -19.3 | -18.6 | -0.7 |
| 6-8 | 235 | 230 | 5 | 0.0 | 0.0 | 0.0 |
| 9 | 290 | 290 | 0 | -34.7 | -32.9 | -1.8 |
| 10 | 450 | 450 | 0 | -35.8 | -35.6 | -0.2 |
| 11 | 480 | 480 | 0 | -34.7 | -34.6 | -0.1 |

Table 4. Measurement value of CDL-C UMa 3.6GHz beam2

| Combined Clusters index | Target Delay(ns) | Measured Delay(ns) | Delay gap(ns) | Target Power(dB) | Measured Power(dB) | Power gap(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | -27.8 | -26.8 | -1.0 |
| 2-5 | 80 | 75 | 0 | 0.0 | 0.0 | 0.0 |
| 6-8 | 235 | 235 | 5 | -18.3 | -18.4 | 0.1 |
| 9-10 | 290 | 290 | 0 | -28.9 | -28.8 | -0.1 |
| 11 | 450 | 450 | 0 | -28.1 | -27.7 | -0.4 |
| 12 | 480 | 480 | 0 | -28.8 | -28.3 | -0.5 |

Note: the power gap is calculated using the target power minus the measured power.

Observation 1: Except the cluster around 300ns of beam1, the differences between the reference and measurement values are all within +/- 1dB, and the difference of 2.45GHz and 3.6GHz cluster around 300ns is -2dB and -1.8dB, respectively. The delay differences are all within 5ns. All of the PDP measurement results meet the pass/fail limits requirement.

## 2.2 Reference values for CDL-C UMi

Reference values of 2.45GHz and 3.6GHz beam1 are illustrated in Table5 and Table 6, respectively.

Table 5. Reference values of CDL-C UMi 2.45GHz beam1

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -30.7 |
| 2-5 | 20 | -19.2 |
| 6-9 | 65 | 0 |
| 10 | 80 | -33.0 |
| 11-12 | 130 | -31.4 |

Table 6. Reference values of CDL-C UMi 3.6GHz beam1

| Combined Clusters index | Delay(ns) | Power(dB) |
| --- | --- | --- |
| 1 | 0 | -30.7 |
| 2-5 | 20 | -19.2 |
| 6-9 | 65 | 0 |
| 10 | 80 | -33.1 |
| 11-12 | 130 | -31.4 |

Proposal 1: Adopt the results in Table 5 and Table 6 as the CDL-C UMi PDP reference values.

# 3	Conclusions

In this contribution, CDL-C UMa PDP validation results of CMCC & BUPT joint lab are given in a more specific way, and we propose the PDP reference values of CDL-C Umi. Here are the observations and the proposals.

Observation 1: Except the cluster around 300ns of beam1, the differences between the reference and measurement values are all within +/- 1dB, and the difference of 2.45GHz and 3.6GHz cluster around 300ns is -2dB and -1.8dB, respectively. The delay differences are all within 5ns. All of the PDP measurement results meet the pass/fail limits requirement.

Proposal 1: Adopt the calculating results in Table 5 and Table 6 as the CDL-C UMi PDP reference values.

# 4 References

- R4-2203063, WF on NR MIMO OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Jan. 17-25 2022