3GPP TSG RAN WG4 Meeting #102-e	R4-2204649

Electronic Meeting, Feb. 21- Mar. 3, 2022

Agenda item:			10.23.2.2

Source:	vivo

Title:	Further discussion on reference point for Te requirements

Document for:	Discussion

- Introduction

In the past RAN4 meetings, there were discussions on reference point for Te requirements. The related issues and conclusions were captured in WFs [1, 2, 3, 4].

In the past meetings, we provided our views on the reference downlink timing for Te requirements in [5, 6, 7]. In this contribution, we further provide our views on the reference downlink timing for Te requirements.

# Discussion

In the last meeting, options of TP for reference point of downlink timing definition were discussed and captured in the WF [4].

RAN4 to continue discussion using the following 3 TP options: Option 1: [Apple, Nokia, vivo] The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna Option 2: [Ericsson, Huawei, QC] The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame from the reference cell arrives at the UE antenna Option 3: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna WF: Next meeting is last meeting to reach agreement. All 3 options are open for discussion.

We fully support Option 1. There are multiple reasons that reference point for existing Te requirements can only be based on ‘first detected path’.

TS 38.133 is mostly a requirements spec for UE. It is described from UE perspective. ‘First path’ is not appropriate to define Te requirements as it is unknown to UE. It is not possible for UE to meet requirements that is not controllable by UE, unless relaxed requirements that covers all scenarios are defined.

However, the Te requirements are specified based on UE ‘first detected path’. Multipath detection error is not taken into consideration for downlink timing detection error in Te. This was also confirmed by several contributions. In [5, 6], simulation results are provided for first path detection error. It is observed that multipath detection error cannot be included into existing Te requirements for some typical channel models used for defining demodulation requirements.

If ‘first path’ is used, then the applicability of the requirements must be changed, i.e., the requirements are only applicable to AWGN and LoS channel maybe, which is undesirable since timing requirements should be generic and should cover NLoS channel which is more typical in practical network.

Furthermore, if Te reference point is based on ‘first path’, there is severe issue regarding UE uplink timing autonomous adjustment. The requirements for UE uplink timing autonomous adjustment in TS 38.133 are as follows.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be before the downlink timing of the reference cell.

UE needs to adjust uplink transmission timing if downlink timing is changed. The adjustment can only be conducted based on UE first detected path because there is no way for UE to know the ‘first path’. After adjustment, UE is required to meet Te requirements. The propagation conditions can change during UE autonomous adjustment. If Te requirements are based on ‘first path’ but UE adjust uplink timing based on UE ‘first detected path’, then UE will fail to meet Te requirements after UE autonomous adjustment. Moreover, UE may not need to conduct autonomous adjustment at all if Te reference point is based on ‘first path’, which should not be changing in our view.

As for the testability of ‘first detected path’ raised by companies, we don’t think there should be any issue. The LTE system has been deployed and commercialized successfully for more than decade. The LTE timing requirements are based on ‘first detected path’ and UE in the field passed all the necessary conformance tests regarding timing requirements. So, we do believe there is no testability issue at all.

On the contrary, if reference point requirements are to be changed to ‘first path’. It changes the requirements and tests need to be designed/updated accordingly. It means new tests should be introduced. Then RAN5 has to define new tests for the new requirements. However, we don’t see justification for such change.

In summary, the comparison of definition of reference point of Te can be summarized in Table 1 below.

Table 1. Comparison of definition of reference point of Te

|  | First detected path | First path |
| --- | --- | --- |
| Requirement’s applicability | Static channel, LoS channel, Multipath fading channel | Static channel, LoS channel maybe |
| UE autonomous adjustment | UE can always meet autonomous adjustment requirements as it is based on UE first detected path. | UE could fail to meet autonomous adjustment requirements when in multipath-fading channel or when there is channel variation. UE may not need to conduct autonomous adjustment at all as first path is not supposed be changing. |
| System impact | NW does not know the downlink multi-path detection error at UE side in fading channel, as it is not included in Te requirements. | NW does not know the downlink detection error at UE side in fading channel, as there are no applicable requirements. |
| Test design feasibility | Feasible in AWGN channel Tests has already been designed | Feasible in AWGN channel Tests need to be revised and re-designed. |
| RAN5 Test | Already implemented | New test implementation is needed. |

Therefore, we believe that definition of Te reference point should be based on UE ‘first detected path’. It is also useful to further clarify that the reference point is at the UE antenna.

  - Proposal 1: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.

For option 3, it would be relaxed Te requirements compared to option 1. We don’t see any reason to make the change as in option 3. If it is the only way out concluding the issue, it would be fine.

# Summary

In this contribution, we further provided our views on the reference downlink timing for Te requirements. Based on analysis following observations and proposals are present.

  - Proposal 1: The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell at the UE antenna.

- References

- R4-2108368	WF on RRM for NR IIoT and URLLC, Nokia, Nokia Shanghai Bell

- R4-2115371	WF on NR_IIOT_URLLC_enh_RRM, Nokia, Nokia Shanghai Bell

- R4-2120335	WF on NR enhanced IIoT and URLLC support, Nokia, Nokia Shanghai Bell

- R4-2202782	WF on NR_IIOT_URLLC_enh RRM requirements, Nokia, Nokia Shanghai Bell

- R4-2112558	Discussion on reference point for Te requirements, vivo

- R4-2117765	Further discussion on reference point for Te requirements, vivo

- R4-2201369	Further discussion on reference point for Te requirements, vivo