3GPP TSG-RAN WG4 Meeting #102-e	R4-2204716

Electronic Meeting, 21 February – 3 March 2022

Source: 	Ericsson

Title:  	RRC Idle/Inactive and connected state mobility requirements for HST FR2

Agenda Item:	10.9.3.2

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#101-bis-e meeting, with outcome in terms of agreements and open issues captured in WFs [2].

In this contribution, we focus on issues RRC Idle/Inactive and connected state mobility specifications due to HST FR2 deployment.

# Disucssion

## Sub-topic 2-1: RRC CONNECTED and IDLE state mobility requirements

Issue 2-1-4: IDEL/INACTIVE state Cell Re-selection

Agreement: Defined enhanced requirements for Cell reselection in IDLE/INACTIVE mode for DRX 320 ms in HST FR2 deployments: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: [When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraRAT-MeasurementEnhancement-r16].. N1 refers to the number of Rx beams and equals 2 for Set 1, and 6 for Set 2. Way forward: - The companies are encouraged to check the Notes and to reformulate Note 2.

Proposal 1:  No need to consider more types of enhancement, besides of [highSpeedMeasFlag-FR2].

# Conclustion

Proposal 1:  No need to consider more types of enhancement, besides of [highSpeedMeasFlag-FR2].

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2202594 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell