3GPP TSG-RAN WG4 Meeting #102-e	R4-2204718

Electronic Meeting, 21 February – 3 March 2022

Source: 	Ericsson

Title:  	Signalling characteristics requirements for HST FR2

Agenda Item:	10.9.3.4

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#101-bis-e meeting, with outcome in terms of agreements and open issues captured in WFs [2].

In this contribution, we focus on issues main signaling specifications due to HST FR2 deployment.

# Disucssion

According to the approved WF, signalling requirements including BFD&BFR and Link recovery were agreed as follows:

Issue 3-1-2: TCI switching delay

Way forward: FFS how to avoid inter-symbol interference during TCI switching for FR2 HST scenario.

Regarding observations in R4-2201770, inter-symbol interference occurs when TCI state switches between RRHs. In principle, there are several impacts to normal transmission or reception caused by timing jump:

- DL inter-symbol interference

- Scheduling restriction due to SSB collision with adjacent data symbol

- UL data loss (also inlcude inter-symbol interference) before UL timing recovery by RA based solution or UE autonomous adjusment solution

Some companies proposed to prolong TCI state switch delay for symbols to avoid inter-symbol interference on PDCCH and PDSCH in last meeting. To our understanding, the issue only happens in worst case, e.g. propagation for Dmin distance shall exceed CP length a lot, in this sense, the extended delay may be overdesigned for most of deployment cases.

Proposal 1: Performance degradation is expected upon inter-symbol interference during TCI switching for FR2 HST scenario. No necessity to limit receiving in RRM requirements.

# Conclustion

Proposal 1: Performance degradation is expected upon inter-symbol interference during TCI switching for FR2 HST scenario. No necessity to limit receiving in RRM requirements.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2202594 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell